3GPP TSG-RAN WG1 Meeting #95	        		            R1-1813880

Spokane, USA, November 12th-16th, 2018

Agenda item:	7.2.2.1

Source: 	Samsung

Title: 	Evaluation Results for NR-U

Document for:	Discussion and Decision

# Introduction

In RAN1 #94bis [1], the following agreements were made:

Agreement:

When submitting the evaluation result, the following has to be provided as well

  - the TxOP assumptions of WiFi and NR-U

  - Is RTS/CTS enabled for WiFi

  - PD/ED threshold assumptions

  - Max modulation order supported in each technology

  - MIMO scheme and number of MIMO layers used for both technologies

  - WiFi MAC layer A-MPDU/A-MSDU aggregation level, MPDU size

  - NR-U SCS,

  - WiFi guard interval

  - NR UE processing time capability (#1 or #2)

  - NR PDSCH/PUSCH mapping type, PDCCH monitoring configuration

  - Link adaptation assumptions

  - NR assumption on self scheduling or using cross carrier scheduling

Agreement:

For the coexistence evaluation of sub-7GHz bands other than 5GHz band, previously agreed to use technology neutral assumptions. Companies are encouraged to provide simulation results together with assumption on the technology neutral channel access mechanism.

Agreement:

The template in Annex B of TR 38.889 v0.1.1 is used to capture results in the TR at least for 5 GHz.

This contribution discusses the evaluation results of Wi-Fi/Wi-Fi, NR-U/Wi-Fi, and NR-U/NR-U coexistence performance in 5 GHz unlicensed spectrum, and the coexistence performance impacts of energy detection thresholds and enabling preamble detection for NR-U. This contribution is a revision to R1-1812973.

# Coexisting Evaluation Results for Indoor Sub-7 GHz NR-U

An important design objective for NR-U is to ensure fair coexistence within NR-based operations in the unlicensed band, and between NR-based operation in unlicensed and other incumbent RATs in the unlicensed band. This section provides the evaluation results for indoor sub-7 GHz NR-U, and demonstrates that NR-U can provide fair coexistence with Wi-Fi, and that Wi-Fi performance can be improved under NR-U/Wi-Fi coexistence over the baseline Wi-Fi/Wi-Fi coexistence.

The coexistence performance evaluation has been performed by following the simulation assumptions from Table 1. The outdoor evaluations will use the same simulation assumptions unless otherwise mentioned. Additional simulation parameters have also been provided in the Appendix.

TABLE 1. Simulation assumptions

| Parameter | Value |
| --- | --- |
| TxOP assumption of Wi-Fi and NR-U | 4ms for both Wi-Fi and NR-U |
| Is RTS/CTS enabled for Wi-Fi | No |
| PD/ED threshold assumptions | For Wi-Fi, PDT = -82 dBm, EDT = -62 dBm; for NR-U EDT = -72 dBm (baseline) |
| Max modulation order supported in each technology | 256 QAM for both Wi-Fi and NR-U |
| MIMO scheme and number of MIMO layers used for both technologies | NR-U with array radiation pattern according to TR38.802 [2] and max BF gain of 5 dBi; omni-directional for Wi-Fi |
| WiFi MAC layer A-MPDU/A-MSDU aggregation level, MPDU size | MPDU size = 3250 bytes by default, 1ms per MPDU |
| NR-U SCS | 30 kHz |
| Wi-Fi guard interval | 0.8 us |
| UE processing capability | Capability #1 |
| NR PDSCH/PUSCH mapping type, PDCCH monitoring configuration | Mapping type B in the starting slot of TxOP; per-symbol PDCCH monitoring for flexible starting position |
| Link adaptation assumptions | No fast link adaptation utilizing multiple switching points within COT for NR-U |
| NR assumption on self-scheduling or cross carrier scheduling | Cross-carrier scheduling in UL |

Table 2 provides the detailed Wi-Fi and NR-U indoor coexistence evaluation results with 20 MHz and FTP traffic type 3. In particular, Wi-Fi and NR-U consist both DL traffic and UL traffic with a 50/50 ratio, and the traffic arrival rate 𝜆 from Table 2 is in the unit of files per second. In addition, in order to achieve fairness in terms of channel access, both Wi-Fi and NR-U only perform omni-directional LBT regardless of the coexistence scenario when obtaining the results of Table 2.

TABLE 2. Wi-Fi and NR-U coexistence evaluation with 20 MHz and FTP traffic for indoor scenario

| Tdoc / Source | Reported parameters |  | Low load BO range for Wi-Fi in WiFi+WiFi: 10%~25% |  |  |  | Medium load BO range for Wi-Fi in WiFi+WiFi: 35%~50% |  |  |  | High load BO range for Wi-Fi in WiFi+WiFi: above 55% |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Wi-Fi in WiFi+ WiFi | Wi-Fi in WiFi+ NR-U | NR-U in WiFi+ NR-U | NR-U in NR-U+ NR-U | Wi-Fi in WiFi+ WiFi | Wi-Fi in WiFi+ NR-U | NR-U in WiFi+ NR-U | NR-U in NR-U+ NR-U | Wi-Fi in WiFi+ WiFi | Wi-Fi in WiFi+ NR-U | NR-U in WiFi+ NR-U | NR-U in NR-U+ NR-U |
| R1-1813880 / Source 1 | DL: UPT CDF [Mbps] | 5% | 2.077 | 2.086 | 3.529 | 12.000 | 0.103 | 0.436 | 4.184 | 4.700 | 0.04 | 0.267 | 0.057 | 0.797 |
|  |  | 50% | 30.508 | 35.679 | 63.412 | 63.704 | 12.735 | 21.654 | 38.000 | 39.07 | 1.62 | 4.936 | 14.045 | 20.731 |
|  |  | 95% | 52.927 | 56.945 | 74.133 | 74.078 | 49.774 | 53.152 | 73.624 | 74.058 | 33.656 | 32.256 | 42.991 | 66.636 |
|  |  | Mean | 28.461 | 31.821 | 54.791 | 55.576 | 17.523 | 22.865 | 36.523 | 39.550 | 8.016 | 9.708 | 16.644 | 23.482 |
|  | DL: Delay CDF [s] | 5% | 0.050 | 0.05 | 0.054 | 0.054 | 0.050 | 0.050 | 0.054 | 0.054 | 0.050 | 0.05 | 0.054 | 0.054 |
|  |  | 50% | 0.086 | 0.056 | 0.058 | 0.058 | 0.183 | 0.100 | 0.100 | 0.106 | 0.800 | 0.624 | 0.333 | 0.219 |
|  |  | 95% | 1.760 | 1.121 | 0.308 | 0.378 | 5.535 | 1.913 | 0.801 | 0.698 | 6.56 | 5.244 | 1.787 | 1.403 |
|  |  | Mean | 0.362 | 0.289 | 0.131 | 0.112 | 1.060 | 0.418 | 0.206 | 0.194 | 1.809 | 1.301 | 0.516 | 0.392 |
|  | UL: UPT CDF [Mbps] | 5% | 1.450 | 1.895 | 8.945 | 19.057 | 0.216 | 0.396 | 2.490 | 5.646 | 0.022 | 0.018 | 1.738 | 2.071 |
|  |  | 50% | 35.542 | 39.754 | 57.249 | 57.542 | 16.897 | 23.044 | 31.288 | 38.075 | 3.016 | 4.215 | 14.792 | 21.480 |
|  |  | 95% | 53.493 | 53.399 | 73.471 | 73.5533 | 53.622 | 48.525 | 72.538 | 73.230 | 32.975 | 40.011 | 43.120 | 59.95 |
|  |  | Mean | 30.904 | 33.153 | 51.827 | 54.91 | 20.958 | 23.444 | 33.162 | 39.735 | 9.049 | 10.498 | 18.290 | 24.595 |
|  | UL: Delay CDF [s] | 5% | 0.050 | 0.050 | 0.054 | 0.054 | 0.050 | 0.050 | 0.054 | 0.054 | 0.050 | 0.05 | 0.055 | 0.054 |
|  |  | 50% | 0.070 | 0.059 | 0.070 | 0.064 | 0.132 | 0.111 | 0.130 | 0.103 | 0.805 | 0.754 | 0.279 | 0.209 |
|  |  | 95% | 2.165 | 1.334 | 0.434 | 0.204 | 5.584 | 3.672 | 1.309 | 0.739 | 5.748 | 5.106 | 1.563 | 1.388 |
|  |  | Mean | 0.360 | 0.295 | 0.128 | 0.092 | 1.04 | 0.647 | 0.299 | 0.203 | 1.505 | 1.4 | 0.474 | 0.426 |
|  | 𝜌DL |  | 0.988 | 0.993 | 0.992 | 0.997 | 0.874 | 0.944 | 0.945 | 0.962 | 0.733 | 0.8385 | 0.870 | 0.932 |
|  | 𝜌UL |  | 0.99 | 0.99 | 0.986 | 0.988 | 0.894 | 0.940 | 0.936 | 0.977 | 0.747 | 0.771 | 0.944 | 0.969 |
|  | BO |  | 16.5% | 13.4% | 12.2% | 7.1% | 48.6% | 34.5% | 37% | 26.3% | 76% | 70.5% | 65.4% | 52.1% |
|  | 𝜆 |  | 0.15 |  |  |  | 0.22 |  |  |  | 0.3 |  |  |  |  |
|  |  |  | Additional comments: |  |  |  |  |  |  |  |  |  |  |  |  |

It can be observed from Table 2 that for both DL traffic and UL traffic, Wi-Fi will have better throughput performance and delay performance under the Wi-Fi/NR-U coexistence scenario as opposed to the Wi-Fi/Wi-Fi coexistence scenario, which can demonstrate that NR-U is a friendly neighbor with Wi-Fi similar as LTE-LAA. In addition, Table 2 also demonstrates that the throughput and delay performance for NR-U can be further improved under the NR-U/NR-U coexistence scenario compared to the Wi-Fi/NR-U coexistence scenario.

Observation 1: Wi-Fi has better performance under Wi-Fi/NR-U coexistence than Wi-Fi/Wi-Fi coexistence for indoor scenario.

Observation 2: NR-U can coexist well with each other in unlicensed spectrum for indoor scenario.

# Coexisting Evaluation Results for Outdoor Sub-7 GHz NR-U

In this section, we provide the evaluation results for outdoor sub-7 GHz with 15kHz SCS assumption. Table 3 and 4 provide the evaluation results for coexistence with outdoor scenario 1 and 2, respectively. Specifically, Wi-Fi and NR-U have only DL traffic, and the traffic arrival unit 𝜆 means the unit of files per second.

TABLE 3. Wi-Fi and NR-U with DL-only coexistence evaluation with 20 MHz and FTP traffic for outdoor scenario 1

| Tdoc / Source | Reported parameters |  | Low load BO range for Wi-Fi in WiFi+WiFi: 10%~25% |  |  |  | Medium load BO range for Wi-Fi in WiFi+WiFi: 35%~50% |  |  |  | High load BO range for Wi-Fi in WiFi+WiFi: above 55% |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Wi-Fi in WiFi+ WiFi | Wi-Fi in WiFi+ NR-U | NR-U in WiFi+ NR-U | NR-U in NR-U+ NR-U | Wi-Fi in WiFi+ WiFi | Wi-Fi in WiFi+ NR-U | NR-U in WiFi+ NR-U | NR-U in NR-U+ NR-U | Wi-Fi in WiFi+ WiFi | Wi-Fi in WiFi+ NR-U | NR-U in WiFi+ NR-U | NR-U in NR-U+ NR-U |
| R1-1813880 / Source 1 | DL: UPT CDF [Mbps] | 5% | 2.686 | 3.225 | 11.137 | 27.422 | 0.897 | 1.313 | 2.133 | 12.965 | 0.360 | 0.525 | 1.368 | 3.483 |
|  |  | 50% | 9.141 | 10.743 | 42.626 | 56.850 | 5.653 | 7.154 | 21.888 | 45.770 | 3.850 | 4.780 | 11.284 | 23.963 |
|  |  | 95% | 26.689 | 29.385 | 73.956 | 75.709 | 21.635 | 24.113 | 63.013 | 74.355 | 15.410 | 19.642 | 40.608 | 62.633 |
|  |  | Mean | 11.335 | 12.798 | 43.159 | 55.530 | 7.480 | 9.017 | 25.785 | 44.736 | 5.380 | 6.515 | 14.891 | 27.253 |
|  | DL: Delay CDF [s] | 5% | 0.077 | 0.069 | 0.053 | 0.051 | 0.116 | 0.099 | 0.056 | 0.053 | 0.145 | 0.114 | 0.076 | 0.056 |
|  |  | 50% | 0.267 | 0.216 | 0.094 | 0.066 | 0.490 | 0.371 | 0.186 | 0.091 | 0.640 | 0.528 | 0.346 | 0.173 |
|  |  | 95% | 0.876 | 0.905 | 0.451 | 0.179 | 2.692 | 1.731 | 1.039 | 0.320 | 3.647 | 3.015 | 1.426 | 0.875 |
|  |  | Mean | 0.348 | 0.323 | 0.152 | 0.090 | 0.761 | 0.586 | 0.311 | 0.133 | 1.077 | 0.899 | 0.476 | 0.273 |
|  | 𝜌 |  | 0.965 | 0.982 | 0.973 | 0.988 | 0.898 | 0.910 | 0.852 | 0.982 | 0.786 | 0.859 | 0.782 | 0.918 |
|  | BO |  | 0.152 | 0.127 | 0.078 | 0.042 | 0.348 | 0.299 | 0.258 | 0.092 | 0.547 | 0.497 | 0.443 | 0.260 |
|  | 𝜆 |  | 0.1 |  |  |  | 0.15 |  |  |  | 0.2 |  |  |  |
|  |  |  | Additional comments: |  |  |  |  |  |  |  |  |  |  |  |

TABLE 4. Wi-Fi and NR-U with DL-only coexistence evaluation with 20 MHz and FTP traffic for outdoor scenario 2

| Tdoc / Source | Reported parameters |  | Low load BO range for Wi-Fi in WiFi+WiFi: 10%~25% |  |  |  | Medium load BO range for Wi-Fi in WiFi+WiFi: 35%~50% |  |  |  | High load BO range for Wi-Fi in WiFi+WiFi: above 55% |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Wi-Fi in WiFi+ WiFi | Wi-Fi in WiFi+ NR-U | NR-U in WiFi+ NR-U | NR-U in NR-U+ NR-U | Wi-Fi in WiFi+ WiFi | Wi-Fi in WiFi+ NR-U | NR-U in WiFi+ NR-U | NR-U in NR-U+ NR-U | Wi-Fi in WiFi+ WiFi | Wi-Fi in WiFi+ NR-U | NR-U in WiFi+ NR-U | NR-U in NR-U+ NR-U |
| R1-1813880 / Source 1 | DL: UPT CDF [Mbps] | 5% | 3.519 | 4.356 | 12.183 | 19.404 | 1.597 | 2.191 | 5.388 | 5.999 | 0.696 | 0.663 | 0.567 | 1.845 |
|  |  | 50% | 11.330 | 13.595 | 52.143 | 57.899 | 8.117 | 9.779 | 29.397 | 31.273 | 5.940 | 6.412 | 18.017 | 19.871 |
|  |  | 95% | 34.731 | 36.983 | 75.486 | 75.607 | 29.071 | 31.368 | 67.317 | 69.602 | 28.158 | 27.612 | 52.986 | 62.053 |
|  |  | Mean | 14.302 | 16.122 | 49.356 | 53.813 | 10.426 | 13.254 | 31.864 | 33.551 | 8.857 | 9.250 | 20.917 | 23.771 |
|  | DL: Delay CDF [s] | 5% | 0.066 | 0.057 | 0.053 | 0.052 | 0.080 | 0.066 | 0.055 | 0.054 | 0.088 | 0.082 | 0.063 | 0.058 |
|  |  | 50% | 0.187 | 0.167 | 0.076 | 0.066 | 0.334 | 0.279 | 0.136 | 0.127 | 0.454 | 0.410 | 0.204 | 0.193 |
|  |  | 95% | 0.641 | 0.506 | 0.377 | 0.264 | 1.590 | 1.343 | 0.628 | 0.627 | 2.497 | 2.478 | 1.174 | 1.196 |
|  |  | Mean | 0.256 | 0.217 | 0.132 | 0.108 | 0.518 | 0.443 | 0.215 | 0.206 | 0.747 | 0.709 | 0.343 | 0.342 |
|  | 𝜌 |  | 0.982 | 0.986 | 0.985 | 0.971 | 0.935 | 0.966 | 0.898 | 0.890 | 0.847 | 0.877 | 0.782 | 0.836 |
|  | BO |  | 0.106 | 0.096 | 0.06 | 0.06 | 0.323 | 0.265 | 0.217 | 0.223 | 0.509 | 0.470 | 0.442 | 0.389 |
|  | 𝜆 |  | 0.1 |  |  |  | 0.175 |  |  |  | 0.25 |  |  |  |
|  |  |  | Additional comments: |  |  |  |  |  |  |  |  |  |  |  |

We can see from the results of Table 3 and 4 that Wi-Fi performance of Wi-Fi/NR-U coexistence scenario is better compared to that of the Wi-Fi/Wi-Fi coexistence scenario for both outdoor scenario 1 and scenario 2. This means that Wi-Fi can also enjoy the benefit from the coexistence with NR-U in outdoor scenario similar as LTE-LAA. Furthermore, Table 3 and 4 show that the NR-U performance in the NR-U/NR-U coexistence scenario is better compared to that in the Wi-Fi/NR-U coexistence scenario.

Observation 3: Wi-Fi has better performance under Wi-Fi/NR-U coexistence than Wi-Fi/Wi-Fi coexistence for outdoor scenario.

Observation 4: NR-U can coexist well with each other in unlicensed spectrum for outdoor scenario.

# Coexistence performance with variable ED threshold or preamble detection for NR-U

The Wi-Fi/NR-U and NR-U/NR-U coexistence performance in previous sections are obtained under the baseline channel access scheme that NR-U uses energy detection with a -72 dBm ED threshold for the 20 MHz channel. In this section, the coexistence performance under different NR-U energy detection thresholds or enabling preamble detection (PD) for NR-U is investigated for the indoor scenario.

For Wi-Fi/NR-U coexistence, besides the baseline scheme with a -72dBm ED threshold for NR-U, two other coexistence schemes for NR-U are considered. The first one is for NR-U to use a lower and more conservative ED threshold of -82 dBm, and the other one is for NR-U to enable Wi-Fi preamble such that NR-U and Wi-Fi can detect each other through the preamble detection mechanism with -82 dBm threshold. Figure 1 provides the Wi-Fi DL/UL throughput performance under load/medium/high load scenarios, wherein different color represents the Wi-Fi mean throughput performance when it coexists with another Wi-Fi network, NR-U with -72dBm ED threshold, NR-U with -82 dBm ED threshold, and NR-U with preamble respectively. In addition, the actual detection performance for preamble is implemented when enabling the preamble for NR-U. For example, in order to achieve a detection accuracy of 90%, the preamble SINR shall be at least 2 dB.

Figure 1 shows that Wi-Fi throughput performance can be increased under all the three considered Wi-Fi/NR-U coexistence scenarios over the baseline Wi-Fi/Wi-Fi coexistence scenario. For Wi-Fi/NR-U coexistence performance, compared to coexisting with an NR-U network that uses a -72 dBm ED threshold as in Section 2, the Wi-Fi DL/UL throughput can be further improved when it coexists with an NR-U network that uses either a -82 dBm ED threshold, or the preamble detection scheme. Furthermore, Wi-Fi can achieve comparable or even better DL/UL throughput performance when it coexists with NR-U that uses a -82 dBm ED threshold, compared to when Wi-Fi coexists with NR-U that enables Wi-Fi preamble. This demonstrates that for the considered indoor scenario wherein a significant fraction of weak serving links (e.g., serving link RSSI < =72 dBm) exists, it is sufficient for NR-U to use energy detection with a more conservative ED threshold (e.g., -82 dBm) to well coexist with Wi-Fi. In addition, since Wi-Fi detects the preamble enabled NR-U using the -82 dBm PD threshold as opposed to the -62 dBm ED threshold, Wi-Fi will have decreased channel access chance that can lead to lower throughput values when it coexists with preamble enabled NR-U rather than ED only NR-U with -82 dBm ED threshold.

Figure 1. Wi-Fi mean throughput performance under Wi-Fi/NR-U coexistence scenario

In addition to the Wi-Fi throughput performance, the corresponding NR-U throughput performance under the three Wi-Fi/NR-U coexistence scenarios for the indoor setup are also provided in Figure 2. It can be validated from Figure 2 that depending on the energy detection threshold, NR-U with only energy detection can achieve comparable or even better mean throughput performance compared to enabling Wi-Fi preamble for NR-U.

Figure 2. NR-U mean throughput performance under Wi-Fi/NR-U coexistence scenario

Observation 5: Comparable Wi-Fi/NR-U coexistence performance can be achieved between when NR-U enables Wi-Fi preamble and when NR-U only uses energy detection.

For NR-U/NR-U coexistence scenario, the performance impacts of different ED threshold and enabling preamble for NR-U are also investigated for the indoor scenario. Compared to the baseline wherein NR-U uses -72 dBm ED threshold, Figure 3 illustrates that a lower ED threshold value of -82 dBm can provide better coexistence performance for the considered indoor deployment in medium and high load cases. In particular, the NR-U throughput gain of using -82 dBm ED threshold over the -72 dBm ED threshold increases as the traffic load increases. By contrast, when Wi-Fi preamble is enabled for NR-U, NR-U is only able to achieve similar throughput performance as ED-only NR-U with -72 dBm ED threshold, and lower throughput performance compared to when NR-U uses energy detection with the -82 dBm ED threshold. This demonstrates that enabling Wi-Fi preamble for NR-U would degrade the NR-U/NR-U coexistence performance.

Observation 6: Enabling Wi-Fi preamble for NR-U would degrade NR-U/NR-U coexistence performance.

Figure 3. NR-U mean throughput performance under NR-U/NR-U coexistence scenario

Since comparable Wi-Fi performance can be achieved when Wi-Fi coexists with ED-only NR-U and when Wi-Fi coexists with preamble enabled NR-U, and that NR-U/NR-U coexistence performance would degrade when Wi-Fi preamble is enabled, it is sufficient for NR-U to support only energy detection for fair Wi-Fi/NR-U and NR-U/NR-U coexistence. The specific energy detection threshold can be up to network implementation subject to the unlicensed regulation allowance, which can be adjusted based on the network setup/utilization such that a fair coexistence performance can be achieved.

# Conclusions

This contribution considered the evaluation results for NR unlicensed system, and have made the following proposals and observations.

Observation 1: Wi-Fi has better performance under Wi-Fi/NR-U coexistence than Wi-Fi/Wi-Fi coexistence for indoor scenario.

Observation 2: NR-U can coexist well with each other in unlicensed spectrum for indoor scenario.

Observation 3: Wi-Fi has better performance under Wi-Fi/NR-U coexistence than Wi-Fi/Wi-Fi coexistence for outdoor scenario.

Observation 4: NR-U can coexist well with each other in unlicensed spectrum for outdoor scenario.

Observation 5: Comparable Wi-Fi/NR-U coexistence performance can be achieved between when NR-U enables Wi-Fi preamble and when NR-U only uses energy detection.

Observation 6: Enabling Wi-Fi preamble for NR-U would degrade NR-U/NR-U coexistence performance.

# Appendix

The additional simulation parameters for the evaluations in this contribution is provided by Table 5.

TABLE 5: Additional Default Simulation Parameters for Sub-7 GHz NR-U

| Carrier Frequency | 5GHz |
| --- | --- |
| Carrier Channel Bandwidth | 20MHz |
| Number of users per operator | 5 per gNB/AP |
| SCS | 30 kHz (15kHz for outdoor) |
| Channel Model | NR InH Mixed Office model (NR UMi street canyon for outdoor) |
| gNB Tx Power | 18 dBm (to compensate 5 dBi max BF gain for same EIRP as Wi-Fi) |
| AP Tx Power | 23 dBm |
| UE/STA Tx Power | 18 dBm (across all antennas) |
| gNB/AP antenna array configuration | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = dV = 0.5 λ |
| UE/STA antenna array configuration | (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1), dH = dV = 0.5 λ |
| AP Antenna gain | 0dBi |
| UE/STA Antenna gain | 0dBi |
| BS/AP Noise Figure | 5dB |
| UE/STA Receiver Noise Figure | 9dB |
| Minimum received power from serving cell for UE dropping | -82dBm |
| Energy detection threshold | -72 dBm for NR-U, -62 dBm for Wi-Fi |
| UE receiver | MMSE-IRC as the baseline receiver |
| Traffic model | FTP model 3 with traffic arrival rate of λ = 0.5 (36.889 Table A.1.1) |

# References

[1] RAN1 Chairman’s Note, 3GPP TSG RAN WG1 Meeting #94b, Chengdu, China, October 8th –12th, 2018.

[2] TR 38.802, Study on New Radio Access Technology Physical Layer Aspects, V14.2.0, Sep. 2017.