3GPP TSG RAN WG1 Meeting #96	R1-1901513

Athens, Greece, February 25-March 1, 2019

Agenda Item:	6.2.1.6

Source:	Huawei, HiSilicon

Title:	On MPDCCH performance improvement

Document for:	Discussion and decision

# Introduction

In RAN1 #95 meeting, the following agreements were concluded on MPDCCH performance improvement [1].

Agreement

For distributed MPDCCH, predefined mapping between CRS ports and MPDCCH DMRS ports is supported.

Agreement

For localized/distributed MPDCCH, the predefined mapping between CRS ports and MPDCCH ports is based on precoder cycling across time and frequency domain. FFS the details.

Agreement

Reuse existing LTE Rel-8 codebooks for precoding of MPDCCH is the starting point.

- Unless performance gains are idenfied further optimization on top of Rel-8 codebook will not be introduced

Agreement

For UE in Connected/Idle mode, the power offset between CRS and DMRS antenna ports of MPDCCH is indicated by SIB.

Agreement

- For UE in Idle mode, the configuration of CRS for MPDCCH performance improvement is indicated via SIB/SI. FFS the details of configuration.

- For UE in Connected mode, the configuration of CRS for MPDCCH performance improvement is indicated via UE-specific RRC signalling. FFS the details of configuration.

In this contribution, further considerations on MPDCCH performance improvement are provided.

# Discussion

## Mapping between CRS ports and DMRS ports of localized MPDCCH

For the mapping between CRS and DMRS of localized MPDCCH, it was agreed that predefined mapping and mapping based on CSI report can be supported.

For predefined mapping, the number of CRS ports and DMRS ports should be considered. For localized MPDCCH transmission, a single DMRS port is supported. If the number of CRS port is one, so one port CRS can correspond to one port DMRS. If the number of CRS port is two or four, two or four CRS ports can correspond to one DMRS port, and the mapping matrix can be predefined in the specification.

For the mapping based on CSI report, the eNB can semi-statically adjust the mapping matrix. The specification can specify available matrix set, and informs the suitable mapping matrix to the UE by RRC signaling.

For predefined mapping or mapping based on CSI report, the mapping matrix can be changed in frequency domain and/or time domain. The eNB can configure the granularity in frequency domain and time domain for the matrix adjustment.

Proposal 1: The granularity of matrix adjustment can be configured in frequency domain and time domain.

## Mapping between CRS ports and DMRS ports of distributed MPDCCH

For the distributed MPDCCH transmission, two ports DMRS can be used, and the transmit diversity can be obtained by distributed transmission. Therefore, the predefined mapping between CRS ports and DMRS ports is preferred. If the number of CRS port is two, each CRS port can correspond to one each DMRS port.

If the number of CRS port is four, two CRS ports can correspond to one DMRS port.

Proposal 2: If the number of CRS port is less than the number of DMRS port, the CRS should not be used for MPDCCH performance improvement.

# Conclusions

In this contribution, based on the agreement in previous meeting, further considerations on MPDCCH performance improvement are provided and following proposals are given:

Proposal 1: The granularity of matrix adjustment can be configured in frequency domain and time domain.

Proposal 2: If the number of CRS port is less than the number of DMRS port, the CRS should not be used for MPDCCH performance improvement.

# References

- RAN1 Chairman notes, RAN1 #95, Spokane, USA, November 12 – 16, 2018.