3GPP TSG RAN WG1 Meeting #96	R1-1901528

Athens, Greece, February 25 – March 1, 2019

Agenda Item:	7.2.2.2.4

Source:	Huawei, HiSilicon

Title:	Transmission with configured grant in NR unlicensed band

Document for:	Discussion and decision

Introduction

In the previous RAN1 AH meeting #1901 [1], the following agreements were made:

Agreement:

For configured grant resource configuration in time domain, the following alternatives are to be studied with more detailed proposal and analysis, strive to down-select in RAN1#96:

- Alt. 1: Bitmap based approach as baseline with potential enhancement

  - Companies are encouraged to provide detailed design in next meeting

- Alt. 2: NR Rel-15 based time domain resource allocation approach as baseline with potential enhancement

  - Companies are encouraged to provide detailed design in next meeting

Agreement:

- Support multiple UE starting time offsets with sub-symbol granularity with FeLAA AUL approach as the baseline

  - FFS: Enhancements specific to NRU

- Companies are encouraged to provide views and analysis on the following issues:

  - Whether to support allowing the UE to start transmission later than the starting symbol as indicated in configured grant based on LBT outcome

    - If yes, multiple starting positions within a slot for a configured grant configuration;

      - Alt. 1: subset of symbols

      - Alt. 2: any symbol

      - FFS: gNB knowledge of starting symbol, whether UE indicates to gNB

      - FFS signaling details

      - FFS: whether similar design for scheduled grant and configured grant

  - Whether the ending symbol can be punctured

  - Whether the position of the ending symbol can be shifted depending on the starting position due to LBT procedures

Conclusion:

The following aspects should be discussed further as part of the channel access discussions

- Contention window adjustment

- Details of COT sharing related to NRU configured grant including details and limitations on UE-initiated COT sharing with gNB and configured grant UL transmissions within gNB acquired COT

Agreement:

CG-UCI should at least include the following information:

- HARQ ID

- NDI

- RV

- COT sharing information, FFS details

FFS: other information including UE IDIn the following sections, we discuss the support of uplink transmission with configured grant based on Type 1 and Type 2 in NR-U and the further modifications required with respect to NR in aspects such as time-frequency resource configuration, HARQ enhancements, UCI on PUSCH, MCOT sharing as well as transmission adaptation. This is a revision of R1-1900063.

Time-frequency Resource Configuration

Time-domain resource configuration

It was agreed in the TR [1] that allowing consecutive configured grant resources in time without any gaps in between the resources and non-consecutive configured grant resources (not necessarily periodic) with gaps in between the resources is beneficial and should be considered for NR in unlicensed spectrum.

Therefore, we hereby discuss the two alternative approaches identified in the previous meeting for potential down selection in this meeting:

Alt. 1: Bitmap based approach as baseline with potential enhancement

Alt. 2: NR Rel-15 based time domain resource allocation approach as baseline with potential enhancement

Alt.1: In FeLAA AUL, an RRC configured bitmap of X=40 bits is used to indicate the allowed time resources for AUL transmission on subframe level. Although such a mechanism seems to provide some flexibility to assign or exclude certain subframes for configured UL transmission within the span of the 40 msec, it is worth noting that the whole pattern repeats as well every 40 msec until it is semi-statically re-configured. If a similar approach is adopted in NR-U, the allocation of periodic time domain resources, as per the TR and in line with NR’s time-domain resource configuration for transmission with CG, becomes quite constrained. This is due to the fact that for any desired CG periodicity of P units, the bitmap length of X units has to be an integer multiple of P, i.e. X has satisfy the formulae X mod P = 0. This in fact implies that the choice of the bitmap length X is not flexible enough to support the allocation of periodic time-domain resources of an arbitrary value of P.

It is also worth noting that, to simplify the collision avoidance with the eNB’s periodic attempts to transmit the critical DRS signals, the bitmap length in AUL was selected to match the base DRS periodicity out of the {40, 80, 160} msec periodicities.

Observation 1:The choice of the bitmap length X is not flexible enough to support the allocation of periodic time-domain resources of an arbitrary value of P due to the constraint X mod P = 0 in addition to other factors determining X such as the DRS period.

Compared to NR signaling 5 bits to indicate the index of the selected resource periodicity from a predefined set of a wide range of periodicities, and 4 to 6 bits to indicate the configuration of the time resource, the signaling overhead of the bitmap approach increases linearly with the bitmap size X. Furthermore, to indicate the same time-domain resource duration/periodicity under the higher numerologies supported in NR-U, e.g., 30 KHz and 60 KHz SCSs, the required bitmap length is 2X and 4X, respectively. Thus, leading to an excessive signaling overhead when the resource allocation granularity of one slot is preserved.

Observation 2: When the resource allocation granularity of one slot is preserved at higher SCSs, the bitmap approach is not scalable as it results in excessive signaling overhead.

Even with the same SCS, for the bitmap to indicate the time-domain resource duration/periodicity while exploiting a finer resource allocation granularity such a mini-slot of n symbols, the required bitmap length is scaled up by the ratio of 14/n. Thus, leading as well to an excessive signaling overhead when finer resource allocation granularity is desired.

Observation 3: When a finer resource allocation granularity such a mini-slot of n symbols is desired, even for the same SCS, the bitmap approach is not scalable as it results in scaling up the signaling overhead by a factor of 14/n.

Therefore, to overcome the excessive signaling overhead, several companies have proposed to reinterpret the indication bit of the bitmap so that it maps to multiple slots at higher SCSs or multiple mini-slots when a finer granularity is used. However, this comes at the expense of undermining the resource allocation flexibility and fine granularity.

Observation 4: To overcome the excessive signaling overhead of the bitmap approach, the resource allocation flexibility and fine granularity are undermined.

Alt. 2: In NR R15, an RRC signaling indicates the full time domain resource allocation for type 1 including periodicity, offset in the frame, start symbol and length of PUSCH and K-repetition of the configured grant resource. For type 2, only periodicity and K-repetition in time domain are given through RRC signaling. The other time domain related parameters are given through DCI activation scrambled with CS-RNTI. In the following we provide details on how theses parameters can used/enhanced for the operation in the unlicensed spectrum in NR-U:

- Periodicity P: NR already offers a set of periodicity values for each of the SCSs and CP types supported in NR-U using the same indication field. Each set spans a wide range of values, i.e., from 2 symbols to 640 msec. Although the smallest periodicity values are not useful for the generic eMBB traffic anticipated in NR-U, this configuration parameter provides enough flexibility

- repK : For URLLC, this parameters indicated one of {1, 2, 4, 8} repetitions of the same TB within the configured period. Given the absence of an indication of the total duration of the configured time resource in NR R15, the K-repetition parameter can be reinterpreted as the number of configured resources within the configured period, possibly in conjunction with the SCS configured for the respective active UL BWP.

- timeDomainOffset: This parameter, whether configured through RRC or DCI, is already suitable for configuring the starting slot of the configured periodic resource across different numerologies.

- startSymbolAndLength (SLIV): In NR, this parameter indicates the starting symbol and the length of the PUSCH within a slot for URLLC packets. In NR-U this parameter can be reused to indicate mainly the duration of each PUSCH or can be replaced to indicate multiple starting positions of the UL burst, i.e., multiple resource boundaries over the time resource as discussed in further details in Section 2.4.2.1. In the latter case, the PUSCH duration either spans the symbols between two consecutive burst starting positions or defaults to a full slot as discussed in later details. Whereas, the UE can start the configured grant transmission from any configured resource boundary and occupy any number of the configured resources.

As such, the time domain resource allocation approach of Alt. 2 provides enough flexibility to support periodic allocation of consecutive resources which are preferred from the perspective of reducing the LBT overhead. The proposed enhancements provide the necessary parameters for the case of eMBB traffic and for improving the resource utilization in the unlicensed spectrum through multiple transmission opportunities. Meanwhile, additional non-periodic resources can be exploited using gNB-triggered transmissions with CG as discussed in Section 2.4.2.

Proposal 1: NR-U should consider to enhance the time domain allocation of configured grant based on NR R15 (Alt. 2 in the last agreement). Basic enhancements should at least include:

- Total duration of the time-domain resources within a period

  - Reinterpretation of the K-repetition as the number of configured resources within a period

  - FFS: Reinterpretation is based on the SCS configured for the respective active UL BWP

- Multiple burst starting positions within a period with configurable granularity such as slot or mini-slot

Frequency-domain resource configuration

In any case, an UL TCG in NR-U needs also to comply with the regulatory requirements in the unlicensed spectrum such as the minimum OCB and maximum PSD requirements, and therefore, the resource configuration should include the frequency interlace(s) to be used within the configured frequency resources.

Since one of the key features of NR-U is the wideband operation, a CG UE can be configured with a wideband carrier and a wideband UL BWP thereof that spans multiple subbands, e.g., each subband is a 20MHz unlicensed channel. In order to increase the resiliency of the TCG in NR-U to LBT failure, the CG UE can be configured with a frequency-domain resource, e.g., one or more frequency interlaces, across multiple subbands. As agreed in the SI phase, subband LBT is supported and thus the CG UE can perform multiple subband LBT procedures, e.g., following eLAA multi-channel access. Based on the results of the subband LBT procedures, the CG UE can transmit on one or more subbands for which the LBT procedure(s) are successful. Moreover, the CG UE may select the number of subbands to use for its TCG based on the traffic type or the TB size.

Proposal 2: For the uplink transmission with configured grant in NR-U, configuration of frequency-domain resources includes one or more frequency interlaces.

Proposal 3: Frequency-domain resources are configured across multiple subbands of a wideband UL BWP configured to the UE for transmission with configured grant in NR-U.

Proposal 4: Based on the results of the subband LBT procedures, the CG UE can transmit on one or more subbands for which the LBT procedures are successful.

- FFS: The CG UE selects a number of such subbands to use for its TCG based on the traffic type and/or the TB size

Multiple resource configurations per cell for NR-U UE

Compared to NR in the licensed band, the anticipated UL traffic intensity and traffic diversity for transmission with configured grant is much larger in NR-U. This is because it is beneficial for uplink traffic of various traffic classes to overcome the LBT overhead incurred by scheduled UL in the unlicensed spectrum. Particularly, in the absence of a licensed anchor cell for transmission of SR or cross-carrier UL grant in some deployment scenarios targeted in NR-U.

As such, an NR-U UE can be transmitting uplink packets from different traffic classes (QCIs) with different latency and/or bit rate requirements for which a single resource configuration, e.g., periodicity and TBS, may not be adequate. As such, in RAN1 meeting #95 [2], it was agreed for enhanced configured grant transmission in NR R16 that multiple active configured grant configurations for a given BWP of a serving cell should be supported at least for different services/traffic types and/or for enhancing reliability and reducing latency.

Therefore, it is important to support multiple resource configurations per UE for the uplink transmission with configured grant to satisfy the diverse QoS requirements anticipated in NR-U. We note that, due to the dependency of HARQ process information on the time-domain resources in NR R15, multiple resource configurations are only allowed on different cells for TCG. However, such dependency ought to be removed in NR-U with the agreement to introduce UCI on PUSCH carrying HARQ process ID, NDI, and RVID. Thus, full flexibility can be exploited when multiple resource configurations are supported per cell per CG UE.

Proposal 5: Multiple active resource configurations per cell per BWP should be supported for uplink transmission with configured grant in NR-U.

Improved resource utilization subject to LBT

Alignment of transmission start time of multiple UEs

Configuring dedicated time/frequency resource to each UE is likely to result in resource waste and decreased user capacity. This is due to the fact that burst traffic may result in the pre-configured dedicated resources to be underutilized. But more importantly, a UE with data ready to transmit may not gain access to the pre-configured resources as a result of LBT failure.

Therefore, one technique to improve the resource utilization with pre-configured resources is to configure a selected set of UEs with same time-domain resources, and either orthogonal or same frequency interlaces on the same LBT subband, while aligning their transmission starting points to avoid mutual blocking during the LBT.

We note that NR R15 already supports the configuration of multiple CG UEs with the same time-frequency resources. If a collision occurs in such case, the receiving gNB can identify the UEs using other pre-configured resources, such as DMRS, and resolve the collision in the spatial or code domains. As such, resource utilization can be improved despite the burst traffic and channel availability in NR-U leading to increased user capacity and reduced latencies.

Proposal 6: To improve the CG resource utilization in NR-U, gNB should align the transmission starting points of UEs configured with the same time-domain resources, and either orthogonal or same frequency interlaces, to avoid mutual blocking during the LBT.

Proposal 7: Given that NR-U naturally inherits NR’s support multiple CG UEs configured with the same time-frequency resources per cell, NR-U should support the use of other configured resources, e.g., DMRS, to resolve intra-cell collisions.

Multiple transmission opportunities

Due to the uncertainty of the unlicensed channel availability in NR-U, a UE with data ready to transmit may not gain access to the pre-configured resources as a result of LBT failure. Not only does missing the pre-configured transmission opportunity lead to further underutilized resources, but it also results in excessive latencies that may compromise the gains of bypassing the UL grant procedure. This is because the UE that missed the pre-configured periodic transmission opportunity needs to defer its channel access for the remaining time span of the configured period until the following transmission opportunity.

Multiple transmission occasions (Potential burst starting positions):

In the previous meeting, companies were encouraged to provide views and analysis on the following issues whether to support allowing the UE to start transmission later than the starting symbol as indicated in configured grant based on LBT outcome and if multiple starting positions are provided in such case within a slot following either: Alt. 1: subset of symbols; or Alt. 2: any symbol.

We observe that one technique to improve the resource utilization with configured resources, with reduced complexity at both the UE side and the gNB’s blind decoding side, is to configure the UE with multiple transmission occasions over the configured time-domain resource within a CG period. In other words, potential UL burst starting positions are defined in course granularity (subset of symbols as in Alt. 1) over the time-domain resource. Before the beginning of the configured period, the UE performs the LBT procedure towards accessing the first transmission occasion/burst starting position. If the LBT is successful, the UE may start

gNB LBT

Feedback Transmission

UL Transmission Using Remaining CG Parameters

DL Transmission

transmitting one or more PUSCHs up to the end of the CG resource within the configured period. However, if the LBT fails, the UE does not defer the channel access for the remaining period. Rather, it resumes its channel access attempt by performing LBT towards accessing the second transmission occasion, and so on and so forth as shown in Fig. 1. The UE misses the CG transmission opportunity for this period only if the LBT before the last transmission occasion fails. As such, resource underutilization and excessive latencies associated with missing the pre-configured transmission opportunities due to LBT failures are avoided.

As discussed in our companion contribution on UL channel design [3], the choice of the subset of symbols can be based on the SCS in use to preserve the channel access opportunities. For instance the burst starting positions can be aligned to the boundaries of 2-symbol mini-slots in the case of 15 or 30 KHz SCS, and aligned to the boundaries of 7-symbol mini-slots in the case of 60 KHz SCS.

The least UE complexity and gNB blind decoding can be achieved when each PUSCH is configured to span the number of symbols between two consecutive burst starting positions. In such case, no puncturing or rate matching is needed as a result of the LBT. Also, the ending position of the PUSCH is pre-known to the gNB. This option can be denoted as Opt 1-a. However, such savings come at the expense of increased usage of the HARQ processes. The other option can be denoted as Opt 1-b in which each PUSCH is configured to span a full slot ending at the slot boundaries. In such case, the ending position of the PUSCH is also pre-known to the gNB and fewer HARQ processes are utilized yet puncturing/rate matching may apply to at least the first PUSCH starting the UL burst wherein CBG-based (re)transmission are beneficial as captured in the TR.

Fig. 1 Multiple pre-configured transmission occasions/course burst starting positions within the periodicity of the configured grant.

Proposal 8: NR-U should support the configuration of multiple transmission occasions/potential burst starting positions every subset of symbols (Alt. 1) within the CG time resource to avoid latency and resource waste due to LBT failure and reduce the gNB’s blind detection.

    - Subset of 2 symbols is used with 15 and 30 KHz SCSs

    - Subset of 7 symbols is used with 60 KHz SCS

Proposal 9: To reduce UE complexity and gNB blind decoding, consider the following two options for the configuration of the PUSCH resource based on Alt. 1:

    - Opt 1-a: Each PUSCH is configured to span the number of symbols between two consecutive burst starting positions.

      - No puncturing or rate matching needed while the PUSCH ending position is pre-known to the gNB

    - Opt 1-b: Each PUSCH is configured to span a full slot ending at the slot boundaries.

    - Puncturing/rate matching may apply to at least the first PUSCH at the beginning of the UL burst wherein.

      - CBG-based  (re)transmission should be considered

As agreed in the SI phase [3], it is identified to be beneficial to consider UE multiplexing and collision avoidance mechanisms between configured grant transmissions and between configured grant and scheduled grant transmissions. In line with this agreement and our earlier discussion in Subsection 2.4.1, the configuration of multiple burst starting positions in course granularity within the CG periodicity enables frequency multiplexing of a group of CG UEs that are configured with the same time-domain resources and orthogonal frequency interlaces. This can be achieved by configuring blanking symbol(s) at some of these burst starting positions as fine granularity resource configuration and thus avoiding mutual blocking among the group of UEs despite the burst traffic and/or the LBT failures as shown in Fig 2.

Fig. 2 Enabling UE multiplexing and collision avoidance with CG and scheduled grant using blanking symbols at selected pre-configured transmission occasions/UL burst starting positions.

Proposal 10: NR-U should support CG UE multiplexing and collision avoidance between CG and scheduled grant using blanking symbols at selected pre-configured transmission occasions/UL burst starting positions. During the blanking period, following FeLAA techniques can be applied

    - Random UE-selected offsets for collision avoidance of full BW CG UEs

    - Random UE-selected offsets for collision avoidance between CG and Scheduled UL

For example, the following new time-domain resource configuration parameters can be configured, via RRC for Type 1-based, and via DCI for Type 2-based NR-U TCG:

- Number of potential burst start positions within the CG time resource

- Number of, or interval between, selected occasions with blanking symbols for UE multiplexing

- Starting point, or number of blanking symbols, at selected transmission occasion/potential burst starting position

It is worth noting that these blanking symbols can also serve the purpose of coexistence with DL and/or scheduled grant UL transmissions when applied as a starting offset at the beginning of the CG UL burst. Furthermore, when UCI is transmitted on each PUSCH, it should indicate the number of slots within the PUSCH as well as the number of OSs for the starting and ending points of the PUSCH. Unlike in AUL, if the pre-configured blanking symbols for UE multiplexing overlap with the symbols to be decided by the UE whether to be blanked or not, before the PUSCH starting position or after the PUSCH ending position, the pre-configured blanking overrides the UE’s decision to protect the UE multiplexing opportunity.

Proposal 11: For NR-U to support NR-U multiple time-domain transmission opportunities along with CG UE multiplexing/collision avoidance with scheduled UL, the following parameters can be configured, via RRC for Type 1-based, and via DCI for Type 2-based NR-U TCG:

- Number of potential burst start positions within the CG time resource

- Number of, or interval between, selected occasions with blanking symbols

- Starting point, or number of blanking symbols, at selected transmission occasions

gNB-triggered transmission with configured grant:

In addition to periodic configured-grant transmissions, the gNB could also dynamically allow a group of UEs to transmit on additional resources in accordance with the configured grant (except for periodicity and time offset) by sending a DL common alignment signal such as a short DCI scrambled by a group ID or a CORESET containing multiple UE-specific DCIs. In fact, such a mechanism has some similarities to the two-step UL grant introduced in eLAA in the sense that transmission resources and parameters are signaled to the UE before a DCI signal triggers the UL transmission. However, a key difference is that the UE is pre-configured with the transmission resources and parameters through a configured grant using higher layer signaling such as RRC.  Beside the increased flexibility in coping with UL traffic variations and LBT failures at the UE side as shown in Fig. 3, such a common alignment signal allows the configured-grant UEs to either transmit immediately or use one shot CAT2 LBT by sharing the gNB-acquired MCOT, e.g., in a B-IFDMA manner.

Fig. 3 Using a DL group common alignment signal, gNB can trigger the uplink transmission based on configured grant from one or more UEs to improve resource utilization and reduce latency

Moreover, using the gNB-triggered transmission with configured grant, the gNB naturally avoids collisions with its DL signals/channels and scheduled UL via implementation. Meanwhile, the benefit of multiplexing the transmissions of the aligned UEs, e.g., in frequency domain using orthogonal interlaces, is preserved.

Furthermore, it was agreed in the SI phase [3] that a gap of up to 16 us should be allowed between the end of the DL transmission and the immediate transmission of feedback to accommodate for the transceiver turnaround time. However, in many cases, the UE processing delay can be larger than 16 us and thus the immediate feedback might not be possible unless the UE fills up such larger gap with some useful pre-configured transmission. While some options to fill the gap could be SRS or CSI transmission, we observe that this is an efficient use case for gNB-triggered transmission with CG wherein a DCI scheduling the DL transmission can trigger the transmission with CG without LBT within 16 us from the end of the DL transmission and for a duration sufficiently larger than the processing delay. As shown in Fig.4, the UE can transmit the feedback immediately after the gNB-triggered transmission without wasting the no-LBT opportunity.

Fig. 4 gNB-triggered transmission with CG can be used to fill the gap between DL transmission and UL feedback for scenarios with processing delay longer than 16 μs.

Proposal 12: NR-U should support gNB-triggered transmission with CG parameters (less periodicity and offset) to enhance the UE’s channel access opportunities, meanwhile the following benefits are achieved:

- gNB naturally avoids collisions with DL signals/channels and scheduled UL

- Multiplexing of multiple triggered CG UEs due to the Alignment signal

Proposal 13: A gNB-triggered transmission with CG can be used to fill the gap between DL transmission and UL feedback for scenarios with processing delay longer than 16 μs.

HARQ Enhancements

HARQ retransmission

In NR R15, HARQ retransmission of configured grant is scheduled by gNB. In the previous RAN1 meetings, HARQ retransmission via same CG resources without explicit grant is also supported. Considering there is no one to one mapping between the CG resource and the HARQ ID, it is more flexible to allow configured grant and grant based transmission share the same HARQ ID pool.

In NR R15 CG, the retransmission is scheduled by UL grant scrambled by CS-RNTI with NDI=1, and UL grant scrambled with C-RNTI always schedules initial transmission regardless of the NDI value. If the legacy NR mechanism is reused, there is a risk of DTXACK issue. E.g., if the gNB miss detects a CG PUSCH transmitted by a UE and associated with a HARQ ID, and schedules the UE for an initial transmission with UL grant scrambled by C-RNTI using the HARQ ID, the UE would misunderstand that the gNB has correctly received the CG PUSCH and then flush the buffer.

As it has been identified that introducing UCI on PUSCH to carry HARQ process ID, NDI, and RV is beneficial, gNB in such case will have the knowledge of the HARQ ID and NDI value from a successfully detected PUSCH, and can use this information for grant-based new (initial) transmission or retransmission. The differentiation of the initial transmission and retransmission can be based on the comparison of NDI reported by UCI and that included in UL grant. E.g., an initial transmission is scheduled if the NDI indicated in UL grant is toggled compared with the NDI reported in UCI of the CG PUSCH, while a retransmission is scheduled if the NDI is non-toggled. Thus the DTXACK issue disappears in the case above since the NDI in UL grant would be non-toggled compared to that reported in the UCI. This rule is also adopted by FeLAA specifications.

Proposal 14: For scheduling a retransmission for an HARQ process that was initially transmitted with configured grant, the differentiation between the initial transmission and retransmission can be based on the comparison of NDI reported by UCI and that included in UL grant.

HARQ ID determination

It was concluded in the SI phase [4] that removing dependencies of HARQ process information on the timing is beneficial for NR-U transmission with CG. Introducing UCI on PUSCH to carry HARQ process ID, NDI, RVID as in LTE-AUL was pointed out as one solution to achieve that. In such case, the UE transmits its selected HARQ process ID in a CG-UCI that is multiplexed with the PUSCH and the HARQ process ID needs to be encoded and decoded separately before the data to enable the soft combining of the packet at the gNB. This however implies less reliability of the CG-UCI compared to a dedicated physical control channel, especially with collisions among transmissions with configured grant or interference from hidden nodes.

As a complementary solution for HARQ ID/UE ID determination for transmissions with configured grant in NR-U (including initial transmission, and retransmissions), some flexible mapping mechanism between the HARQ ID and the set of configured grant resources/parameters used for the transmission of the associated TB can beneficial. For example, the HARQ process ID can be determined/verified based on the frequency interlace and/or the DMRS cyclic shift (such that initial and retransmission of a UE TB can be identified). This in fact allows for a more flexible utilization of the configured grant resources, and furthermore, supports multiple resource configurations per cell per UE as discussed earlier in Section 2.1.1.

Proposal 15: Given that NR-U naturally inherits NR’s support of multiple CG UEs configured with the same time-frequency resources per cell, critical information in CG-UCI are subject to collisions and NR-U should thus consider the following

- Support as well the use of other configured resources, e.g., DMRS, to identify/verify the UE ID and/or the HARQ process ID

- Inclusion of UE ID in the CG-UCI is not beneficial in such case

HARQ feedback indication

To overcome the uncertainty of HARQ timing in the unlicensed spectrum, LTE-AUL uses a bitmap to indicate the ACK status of all the HARQ processes in a UE-specific DCI (AUL-DFI), regardless of whether these processes are in use or not. It was concluded in the SI phase that introducing a DFI including HARQ feedback for configured grant transmission is beneficial in NR-U.

However, compared to gNB transmitting the DFI for each UE individually, savings in signaling overhead, LBT overhead, and UE energy consumption can be achieved when group common signaling is employed for ACK/NACK feedback. For instance, the gNB can provide pending HARQ feedback for one or more PUSCH transmitted with configured grant within previous UL COTs using a GC-DCI scrambled with a group RNTI. Also, for the UEs configured with the same time-domain resources and with orthogonal interlaces, the HARQ feedback would be needed concurrently; the gNB can the HARQ feedback simultaneously for such UEs using the GC-DFI without incurring the latency and LBT overhead associated with sending individual UE-specific DFIs.

Optionally, for transmission of such PDCCH, the gNB can share the UE-acquired MCOT as discussed in the following section. Similar to AUL-DFI, further benefits can be attained if some transmission adaptation parameters are included in that GC-DCI, as discussed in Section 2.4.

Proposal 16: NR-U should consider HARQ-ACK feedback using group common DFI for uplink transmission with configured grant.

MCOT Sharing and LBT Requirements

Typically, a transmission with configured grant based on either Type 1 or Type 2 would require the UE to perform CAT4 LBT before the pre-configured time-domain resources based on the priority class of the traffic. That applies as well to the case when multiple occasions are employed as discussed in Section 2.1.1, wherein the UE proceeds with the CAT4 LBT towards the following transmission occasion if a previous occasion has been missed due to LBT failure.

However, when the gNB transmits a DL Alignment signal to trigger a transmission from one or more UEs based on the configured grant, only the gNB is required to perform CAT4 LBT. Whereas, the UEs with data to transmit can share the gNB-acquired MCOT and either access the channel immediately using CAT2 LBT, if the PUSCH transmission can start after gap that is less than 16 μsec, or using CAT2 LBT otherwise. In such case, the UEs transmitting in response to the DL Alignment signal limit their COT by the gNB-acquired MCOT.

As discussed in the previous section, the gNB can provide HARQ feedback for one or more PUSCH transmitted with CG within the UE-acquired MCOT in either a GC-DCI or frequency multiplexed UE-specific DCIs. It means that gNB could share one or more UE-acquired MCOT(s) and access the channel with CAT2 LBT. For this UL MCOT sharing, the following two options could be considered:

- Option 1: gNB independently inherits multiple UE-acquired MCOTs (if any).

- Option 2: gNB inherits a common MCOT (in both time domain and frequency domain) among different UEs.

From a fairness point of view, the transmission is only allowed to be intended for the UE(s) who acquired the MCOT.

Fig. 5 Option 2: gNB inherits a common MCOT (in both time and frequency domains) among different UEs

Proposal 17: For gNB to sharing the MCOT acquired by CG UE(s), the following options should be supported in NR-U:

- Option 1: gNB independently inherits multiple UE-acquired MCOTs (if any).

- Option 2: gNB inherits a common MCOT (in both time and frequency domains) among different UEs.

Proposal 18: MCOT sharing is supported for transmission with configured grant in NR-U:

- gNB sharing UE-acquired MCOT for transmission of HARQ-ACK and/or PDSCH to that UE

- UE sharing gNB-acquired MCOT when a gNB-triggered transmission with CG is employed

CG-UCI on PUSCH

In order to support asynchronous HARQ in LTE-AUL, AUL-UCI is transmitted on each PUSCH in AUL carrying UE-selected HARQ parameters such as HARQ process ID, NDI, and RV, UE identification (C-RNTI). In addition, AUL-UCI uses one bit to inform the eNB of the actual PUSCH start point (OS#0 or #1) based on LBT result, another bit for the actual PUSCH end point (OS#12 or #13), and a third bit to indicate whether UL-to-DL MCOT sharing is possible. However, eNB sharing the AUL COT is only allowed to transmit PDCCH on 2 symbols since the priority class of the PUSCH is unknown to the eNB for transmission of PDSCH of same or higher priority class to the same UE.

Therefore, in order to introduce CG-UCI in NR-U, enhancements are needed to the UCI content and associated gNB/UE procedures to commensurate the enhanced resource allocation/granularity and the enhanced COT-sharing for NR-U CG as discussed in the previous sections. These enhancements include:

- PUSCH Start Point and PUSCH End Point:

  - Opt 1: Extended to indicate multiple blanked OSs

  - Opt 2: Reuse legacy field plus pre-defined mapping to number of OSs based on numerology

- Remaining MCOT Duration: Allows the gNB to share the UL COT for PDSCH transmission

UE to select PUSCH start and end point and indicate them along with remaining COT duration in CG-UCI. However, unlike LTE-AUL, the UE is expected to indicate the PUSCH start and end points only if the respective symbols do not lay within the blanking symbols pre-configured over the CG resource. In other words, the preconfigured blanking overrides the UE’s criteria for selection of PUSCH Start/End points.

Proposal 19:  Indication of PUSCH start and end points in CG-UCI should be enhanced to indicate multiple blanked OFDM symbols. The following options should be considered:

- Opt 1: Extended to indicate multiple blanked OSs

- Opt 2: Reuse legacy field plus pre-defined mapping to number of OSs based on numerology

Proposal 20:  Remaining MCOT Duration should be indicated in the C-UCI to allow the gNB to share the UL COT from one or more CG UEs for transmission of PDSCH.

Transmission Adaptation

In NR, gNB may only update the transmission parameters using re-activation DCI for type 2 whereas only RRC re-configuration can be used in case of Type 1. In order to cope with the interference uncertainties, e.g., from hidden nodes, faster adaptation of the pre-configured transmission parameters is beneficial. This in fact can be achieved in either a UE-directed manner or a gNB-directed manner.

UE-directed: Using this transmission adaptation mechanism, the UE spontaneously updates its configured-grant transmission parameters such as MCS, RI and PMI, and indicates the changes to the gNB within the uplink burst. Two possible indication techniques can be considered:

- Using a pre-configured pool of pilot signals to indicate the change, e.g., DMRS and cyclic shifts

- Introducing a new UCI format that can be multiplexed with PUSCH to indicate the UE updated transmission parameters. Note that although a new AUL-UCI has been introduced in FeLAA, its contents are limited to HARQ parameters and PUSCH timing indication.

gNB-directed: Using that transmission adaptation mechanism enables the gNB to update the configured grant transmission parameters, e.g., based on received signal quality such as SINR or BLER, and indicate the new parameters in a DCI format as follows:

- Using GC-DCI which reduces the signalling overhead and allows for combine the updated parameters with other DL control information such as HARQ-ACK feedback. In addition, TPC commands can be transmitted to allow for closed-loop power control for which a DCI format similar to LTE Format 3 needs to be adopted.

- Using UE-specific DCI which is similar to the AUL-DFI introduced in FeLAA to carry the HARQ feedback as well as TPMI.

Proposal 21: CG UE should indicate updated transmission parameters such as MCS in the CG-UCI

Conclusions

In this contribution, we discussed the uplink transmission with configured grant in NR-U and the changes required with respect to NR in aspects such as time-frequency resource configuration, HARQ enhancements, MCOT sharing and LBT mechanism as well as transmission adaptation. The following proposals were made:

Observation 1:The choice of the bitmap length X is not flexible enough to support the allocation of periodic time-domain resources of an arbitrary value of P due to the constraint X mod P = 0 in addition to other factors determining X such as the DRS period.

Observation 2: When the resource allocation granularity of one slot is preserved at higher SCSs, the bitmap approach is not scalable as it results in excessive signaling overhead.

Observation 3: When a finer resource allocation granularity such a mini-slot of n symbols is desired, even for the same SCS, the bitmap approach is not scalable as it results in scaling up the signaling overhead by a factor of 14/n.

Observation 4: To overcome the excessive signaling overhead of the bitmap approach, the resource allocation flexibility and fine granularity are undermined

Proposal 1: NR-U should consider to enhance the time domain allocation of configured grant based on NR R15 (Alt. 2 in the last agreement). Basic enhancements should at least include:

- Total duration of the time-domain resources within a period

  - Reinterpretation of the K-repetition as the number of configured resources within a period

  - FFS: Reinterpretation is based on the SCS configured for the respective active UL BWP

- Multiple burst starting positions within a period with configurable granularity such as slot or mini-slot

Proposal 2: For the uplink transmission with configured grant in NR-U, configuration of frequency-domain resources includes one or more frequency interlaces.

Proposal 3: Frequency-domain resources are configured across multiple subbands of a wideband UL BWP configured to the UE for transmission with configured grant in NR-U.

Proposal 4: Based on the results of the subband LBT procedures, the CG UE can transmit on one or more subbands for which the LBT procedures are successful.

- FFS: The CG UE selects a number of such subbands to use for its TCG based on the traffic type and/or the TB size

Proposal 5: Multiple active resource configurations per cell per BWP should be supported for uplink transmission with configured grant in NR-U.

Proposal 6: To improve the CG resource utilization in NR-U, gNB should align the transmission starting points of UEs configured with the same time-domain resources, and either orthogonal or same frequency interlaces, to avoid mutual blocking during the LBT.

Proposal 7: Given that NR-U naturally inherits NR’s support multiple CG UEs configured with the same time-frequency resources per cell, NR-U should support the use of other configured resources, e.g., DMRS, to resolve intra-cell collisions.

Proposal 8: NR-U should support the configuration of multiple transmission occasions/potential burst starting positions every subset of symbols (Alt. 1) within the CG time resource to avoid latency and resource waste due to LBT failure and reduce the gNB’s blind detection.

    - Subset of 2 symbols is used with 15 and 30 KHz SCSs

    - Subset of 7 symbols is used with 60 KHz SCS

Proposal 9: To reduce UE complexity and gNB blind decoding, consider the following two options for the configuration of the PUSCH resource based on Alt. 1:

    - Opt 1-a: Each PUSCH is configured to span the number of symbols between two consecutive burst starting positions.

      - No puncturing or rate matching needed while the PUSCH ending position is pre-known to the gNB

    - Opt 1-b: Each PUSCH is configured to span a full slot ending at the slot boundaries.

    - Puncturing/rate matching may apply to at least the first PUSCH at the beginning of the UL burst wherein.

      - CBG (re)transmission should be considered

Proposal 10: NR-U should support CG UE multiplexing and collision avoidance between CG and scheduled grant using blanking symbols at selected pre-configured transmission occasions/UL burst starting positions. During the blanking period, following FeLAA techniques can be applied

    - Random UE-selected offsets for collision avoidance of full BW CG UEs

    - Random UE-selected offsets for collision avoidance between CG and Scheduled UL

Proposal 11: For NR-U to support NR-U multiple time-domain transmission opportunities along with CG UE multiplexing/collision avoidance with scheduled UL, the following parameters can be configured, via RRC for Type 1-based, and via DCI for Type 2-based NR-U TCG:

- Number of potential burst start positions within the CG time resource

- Number of, or interval between, selected occasions with blanking symbols

- Starting point, or number of blanking symbols, at selected transmission occasions

Proposal 12: NR-U should support gNB-triggered transmission with CG parameters (less periodicity and offset) to enhance the UE’s channel access opportunities, meanwhile the following benefits are achieved:

- gNB naturally avoids collisions with DL signals/channels and scheduled UL

- Multiplexing of multiple triggered CG UEs due to the Alignment signal

Proposal 13: A gNB-triggered transmission with CG can be used to fill the gap between DL transmission and UL feedback for scenarios with processing delay longer than 16 μs.

Proposal 14: For scheduling a retransmission for an HARQ process that was initially transmitted with configured grant, the differentiation between the initial transmission and retransmission can be based on the comparison of NDI reported by UCI and that included in UL grant.

Proposal 15: Given that NR-U naturally inherits NR’s support of multiple CG UEs configured with the same time-frequency resources per cell, critical information in CG-UCI are subject to collisions and NR-U should thus consider the following

- Support as well the use of other configured resources, e.g., DMRS, to identify/verify the UE ID and/or the HARQ process ID

- Inclusion of UE ID in the CG-UCI is not beneficial in such case

Proposal 16: NR-U should consider HARQ-ACK feedback using GC-DFI for uplink transmission with configured grant.

Proposal 17: For gNB to share the MCOT acquired by CG UE(s), the following options should be supported in NR-U:

- Option 1: gNB independently inherits multiple UE-acquired MCOTs (if any).

- Option 2: gNB inherits a common MCOT (in both time and frequency domains) among different UEs.

Proposal 18: MCOT sharing is supported for transmission with configured grant in NR-U:

- gNB sharing UE-acquired MCOT for transmission of HARQ-ACK and/or PDSCH to that UE

- UE sharing gNB-acquired MCOT when a gNB-triggered transmission with CG is employed

Proposal 19:  Indication of PUSCH start and end points in group common UCI should be enhanced to indicate multiple blanked OFDM symbols. The following options should be considered:

- Opt 1: Extended to indicate multiple blanked OSs

- Opt 2: Reuse legacy field plus pre-defined mapping to number of OSs based on numerology

Proposal 20:  Remaining MCOT Duration should be indicated in the C-UCI to allow the gNB to share the UL COT from one or more CG UEs for transmission of PDSCH.

Proposal 21: CG UE should indicate updated transmission parameters such as MCS in the CG-UCI

- References

3GPP TR 38.889 (v16.0.0): "Study on NR-based access to unlicensed spectrum ".

RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #95, Spokane, USA, November 12th – 16th, 2018.

R1-1901524, “UL channels and signals in NR unlicensed band”, Huawei, Hisilicon

3GPP TR 38.889 (v16.0.0): "Study on NR-based access to unlicensed spectrum ".

Qualcomm Inc, “New WID on NR-based Access to Unlicensed Spectrum” , RP-182878, Sorrento, Italy, Dec. 10-13, 2018