3GPP TSG RAN WG1 #96 Meeting	R1- 1901530

Athens, Greece, February 25 – March 1, 2019

Agenda Item:	7.2.2.3

Source:	Huawei, HiSilicon

Title:	Generic design of spectral-domain modulation sequences for 5G NR-U PRACH waveforms

Document for:	Discussion and decision

Introduction

Four potential alternatives for NR-U PRACH frequency resource allocation have been included into the Technical Report TR 38.889 [1, pp. 42]. They might be considered as the starting point for further design of NR-U PRACH waveforms in the work item for NR unlicensed [2].

In this contribution we present a generic construction of spectral-domain modulation sequences for NR-U PRACH that can be adjusted to any of above-mentioned four frequency resource allocation alternatives. We show by examples that the resulting sets of PRACH preambles have favorable performance in terms of peak-to-average power ratio (PAPR), auto-/cross-correlation and PRACH capacity for any of four frequency resource allocation alternatives captured in the TR 38.889.

Basic Construction of Modulation Sequences

Let us assume that the elements of a complex-valued k-th spectral-domain modulation sequence{cku}, k=0, 1,…,K-1, u=0, 1,…,At-1, are used as the DFT coefficients Zm,  kΩm at frequencies Ωm, where

Ωm={f|f=δi+jm,l}                                                           (1)

i=0,1,…,t-1,l=0,1,…,A-1, 0≤jm,0<jm,1<…<jm,A-1≤ δ-1

is the m-th (m=0, 1,…,B-1, B≤δ/A) set of allowed frequencies for PRACH over a frequency band consisting of N = δt discrete frequencies, which is further divided into t interlacing subbands each containing δ consecutive frequencies. It is noted that Ωm implies a repetitive PRACH resource allocation, i.e., the same allowed RE locations {jm,l| 0≤jm,0<jm,1<…<jm,A-1≤ δ-1} are repeated in all the t subbands. The variable A represents the number of subcarriers for PRACH per interlacing subband, while B is the number of orthogonal PRACH frequency allocations per interlacing subband.

Then we define the sequence {cku} as

cku=bnu mod A ar(u),        k=0, 1,…,K-1,     K=RA                           (2)

where {aru}, r=0, 1,…, R-1, is a constant-envelope sequence of length At, and {bn(l)} , n=0, 1,…,A-1, l=0, 1,…,A-1 is the set of A (near) orthogonal sequences.

The generic sequence (2) is applicable to the PRACH design under any of the four frequency resource allocation alternatives in [1], as the PRACH resource allocations in these alternatives can all be represented using a common set of parameters δ, t, A, B and Ωm with different realizations. Given a specific allowed frequency set  Ωm, a PRACH preamble waveform can be obtained by scaled IDFT of a block-interleaved comb spectrum as

zm,kw=1Nf=0N-1Zm,kfWN-wf,   w=0,1,2, …, N-1				(3)

where WN=exp⁡(--12πN), Zm,kf takes value according to (2) if f∈Ωm and is 0 otherwise. The sequences zm,kw and  Zm,kf have the same energy E=At.

On the basis of (1) to (3), different combinations of the allowed frequency set Ωm, the (long) sequence {aru} and the (short) sequence {bnu mod A} can be adopted to generate a set of PRACH preambles with low PAPR and low correlation between them. In this contribution, the following two variants of the generic sequence construction will be considered.

Variant 1:

- aru=Wt(ru mod A)u/A,  u=0, 1, …, At-1, where r is relatively prime to A;

- bn(l) is a column of a pre-determined A×Nb (near) orthogonal matrix B=bl,jl=0,1,…, A-1,  j=0,1,…Nb-1.

Variant 2:

- aru=WAtru(u+(At mod 2))/2,  u=0, 1, …, At-1, where r is relatively prime to At;

- bnl=artnWArn(l).

It should be noted that the construction in Variant 2 leads to a set of sequences {cku} which are effectively cyclically shifted versions of a single ZC sequence aru, with cyclic shift gap NCS=t, i.e., arubnu mod A=aru+tn.

Performance Evaluations

Simulation scenarios

Timing Detection window: According to the simulation assumption agreed in [3], the maximum round-trip delay is taken to be 1.15 μs, corresponding to 300 m inter site distance (ISD), and the maximum channel delay is taken to be 0.87 μs, corresponding to the agreed TDL-C channel model with 100 ns desired delay spread. This gives that a timing detection window of 1.15+0.87 μs = 2.02 μs should be assumed at the receiver in all the PRACH schemes below. Consequently, our proposed PRACH schemes should guarantee good auto/cross-correlation within this timing detection window. Mathematically, the normalized periodic cross-correlation (NPCC) function between any two PRACH preambles {zm,r,nk} and {zm',r',n'k} in (3) is defined as

θzm,r,n,  zm',r',n'p=1Atf=0N-1Zm,r,nfZm',r',n'*fWNpf,p≥0,θzm',r',n',zm,r,n*-pp<0.					(4)

Specifically, (4) reduces to a normalized periodic auto-correlation (NPAC) function when zm,r,nk= zm',r',n'k, ∀k. Since the generic sequence construction in Section 2 involves repetitive PRACH resource allocation, it can be proved that the generated PRACH preamble waveform always has a zero auto-correlation zone (ZAZ) of size t-1, i.e., θzm,r,n,  zm,r,np=0, p=1, 2, …, t-1, which corresponds to a time span of  1/(δ∆f) with ∆f being the spacing of the frequencies. When this ZAZ time span is less than the required timing detection window, at least one auto-correlation side-peak on each side of the main peak will fall within the timing detection window, and the allowed frequency set, i.e., Ωm, should be carefully selected to suppress the amplitude of these auto-correlation side-peaks to a sufficiently low level.

Frequency resource allocation: We assume a maximum number of 106 PRBs and ∆f=15 kHz SCS and the following agreed frequency resource allocation alternatives.

- Alternative 1 (Uniform PRB-level interlace mapping), we choose to (uniformly) allocate every other PRB to PRACH, i.e., the maximum number of frequencies available for PRACH is 12REsPRB ×1062 PRBs=636 REs. Consequently, we let t=A=23, which is the maximum prime number satisfying t=A and tA≤636. With every other PRB allocated to PRACH, the A=23 REs per interlacing subbband span 4 PRBs. Hence we have δ=4 PRBs=48 REs and B=1, and the RE allocation per subband can be selected as {13, 14, …, 23, 36, 37, …, 47}, which guarantees sufficiently low auto-correlation side-peaks within the timing detection window as shown in Fig. A-1(a) of Appendix A;

- Alternative 2 (Non-uniform PRB-level interlace mapping), we set the size of a subband to be δ=5 PRBs=60 REs and allocate the 0th, 1st and 3rd PRBs (i.e., 36 REs in total) per subband to PRACH. In this case, the PRBs allocated to PRACH are non-uniformly distributed, and the number of subbands, t, is at most 1065=21. Consequently, we let t=A=19, which is the maximum prime number less than 21. With 36 REs per interlacing subband allocated to PRACH and A=19, we choose to let B=2, and the two allowed near-orthogonal frequency sets in each subband can be selected as {0, 1, …, 5, 12, 13, …, 17, 36, 37, …, 42} and {6, 7, …, 11, 18, 19, …, 23, 41, 42, …, 47}, both of which can guarantee sufficiently low auto-correlation side-peaks within the timing detection window as shown in Fig. A-2(a) of Appendix A;

- Alternative 3 (Uniform RE-level interlace mapping), we choose to allocate every third RE (i.e., at most 12 REs/PRB×106PRBs3=424 REs) to PRACH. Consequently, we let t=A=19, which is the maximum primitive number satisfying t=A and tA≤424. Since the A=19 REs per interlacing subband have spacing of 3 REs (i.e., allocated to every third RE) , we have that the size of an interlacing subband is δ=3A=57, and that B=3 FDMed preambles can be accommodated with allowed frequency sets per subband being {0, 3, 6, …, 54}, {1, 4, 7, …, 55} and {2, 5, 8, …, 56}, respectively;

- Alternative 4 (Non-interlaced mapping), we choose to set A=139, i.e., the same as the NR short PRACH sequence length.  Consequently, we can let δ=139 REs12 REs/PRB=12 PRBs=144 REs,t=106δ=8, and B=1. The allowed frequency set in each subband is then given by {3, 4, …,141}.

Modulation sequence design:

- For Variant 1, the matrix B is generated from a DFT matrix of proper size with certain rows(s) removed (if necessary) to match the length A, and then with certain columns removed in order to reduce the maximum pairwise NPCC within the detection time window to be less than a pre-defined level (e.g., 15%) that ensures the resultant probability of false detection no higher than the predefined target value (i.e., 0.1%). For example, under Alternative 1 with A=23, we select a 24×24 DFT matrix and remove its last row as well as all the odd columns to obtain a matrix B of size A×Nb=23×12, i.e.,  B=bl,j={W242lj|l=0, 1,…22,j=0,1,…,11}. The detailed examples of matrix B for all the four frequency allocation alternatives are listed in Table 1.

- For Variant 2, and Alternatives 1, 2 and 3, the ZC sequence length LZC is selected to be the minimum prime number not less than the size of allowed frequency set Ωm, i.e., At. For the Alternative 4, where the same ZC sequence is repeated in different subbands, we select LZC=139, i.e., the same as the number of allowed REs per subband. It is equivalent to a generic sequence construction where aru is an all-1 sequence and bnl is a length-LZC=139 ZC sequence with certain cyclic shift. For convenience, we still refer to such a construction as Variant 2. For all the four alternatives, the cyclic shift gap NCS is numerically determined to reduce the maximum pairwise NPCC within the detection time window to be less than the pre-defined level (e.g., 15%). The detailed values of  LZC and NCS for each alternative are listed in Table 1.

Table 1. Parameter settings of considered PRACH schemes under different alternatives.

|  |  | Alt-1 | Alt-2 | Alt-3 | Alt-4 |
| --- | --- | --- | --- | --- | --- |
| δ |  | 48 | 60 | 57 | 144 |
| t |  | 23 | 19 | 19 | 8 |
| A |  | 23 | 19 | 19 | 139 |
| B |  | 1 | 2 | 3 | 1 |
| RE allocation(s) within each subband {jm,l} |  | {13, 14, …, 23, 36, 37, …, 47} | {0, 1, …, 5, 12, 13, …, 17, 36, 37, …, 42} or {6, 7, …, 11, 18, 19, …, 23, 41, 42, …, 47} | {0, 3, 6, …, 54}, {1, 4, 7, …, 55} or {2, 5, 8, …, 56} | {3, 4, …141} |
| Variant 1 | Nb | 12 | 6 | 9 | 23 |
|  | B | bl,j=W242lj | bl,j=W203lj | bl,j=W192lj | bl,j=W1396lj |
| Variant 2 | LZC | 541 | 367 | 367 | 139 |
|  | NCS | 45 | 46 | 40 | 6 |

Results summary

In Appendix A, we plot the PAPR and auto/cross-correlation of all the 8 PRACH schemes listed in Table 1, and the main observations from them are summarized in Table 2 below. It can be seen that the generic sequence construction can always achieve satisfactory performance in terms of PAPR, auto-/cross-correlation and PRACH capacities. In particular, the Variant 1 can bring lower cross correlation for all the four alternatives for the frequency resource allocation, and lower PAPR for the Alternatives 1, 2 and 4.

Table 2. Performance of generic sequence constructions under different alternatives.

|  | Generic sequence construction | Max. PAPR (dB) |  | Max. NPAC (NFFT=δt) | Max. NPCC (NFFT=δt) | No. of preambles (No. of cells) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | NFFT=δt | NFFT=105 |  |  |  |
| Alt-1 | Variant 1 | 0 | 6.0860 | 0.0435 | 0.0435 | 6348 (92 cells) |
|  | Variant 2 | 8.8271 | 8.9384 |  | 0.0963 | 6480 (90 cells) |
| Alt-2 | Variant 1 | 0 | 6.6424 | 0.1442 | 0.0776 | 4104 (38 cells) |
|  | Variant 2 | 7.7262 | 8.8843 |  | 0.1350 | 5124 (72 cells) |
| Alt-3 | Variant 1 | 0 | 6.2466 | 0 | 0.0526 | 9234 (114 cells) |
|  | Variant 2 | 3.0671 | 3.0676 |  | 0.1130 | 9882 (135 cells) |
| Alt-4 | Variant 1 | 5.7023 | 6.7996 | 0.0359 | 0.1092 | 25576 (368 cells) |
|  | Variant 2 | 12.1317 | 12.1317 |  | 0.1141 | 3174 (46 cells) |

Observation 1: The generic sequence construction can provide satisfactory PAPR, auto-/cross-correlation and PRACH capacity.

Observation 2: Variant 1 brings lower cross-correlation for all the four alternatives for frequency resource allocation, and lower PAPR for the Alternatives 1, 2 and 4.

Conclusion

Based on the above results, we have the following two proposals:

Proposal 1: The PRACH preamble modulation sequence {ck(u)} is defined as

cku=bnu mod A ar(u),        k=0, 1,…,K-1,     K=RA

where {aru}, r=0, 1,…, R-1, is a constant-envelope sequence of length At, and {bn(l)} , n=0, 1,…,A-1, l=0,…,A-1 is the set of A (near) orthogonal sequences.

Proposal 2: The following two variants of the Proposal 1 should be considered:

Variant 1:

- aru=Wt(ru mod A)u/A,  u=0, 1, …, At-1, where Wt=exp⁡(--12πt) and r is relatively prime to A;

- bn(l) is a column of a pre-determined A×Nb (near) orthogonal matrix B=bl,jl=0,1,…, A-1,  j=0,1,…Nb-1.

Variant 2:

- aru=WAtru(u+(At mod 2))/2,  u=0, 1, …, At-1, where r is relatively prime to At;

- bnl=artnWArn(l).

- References

3GPP TR 38.889 v1.1.0: "Study on NR-based Access to Unlicensed Spectrum".

RAN Chairman’s Notes, 3GPP TSG RAN Meeting #82, Sorrento, Italy, Dec. 10-13, 2018

RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Ad-Hoc Meeting, Taipei, Jan. 21 – 25, 2019.

- Appendix A: Examples of modulation sequences for different frequency resource allocations

We show here the performance evaluation results for the eight PRACH waveforms listed in Table 1, including the auto-/cross correlations and the PAPR. The normalized periodic auto-correlation (NPAC) function of each preamble only depends on the allowed frequency allocation for PRACH, which is the same for both variants of sequence design. We follow the NR setting of 64 preambles per cell and generate 64 preambles for both variants of sequence design.

Figure A-1 contains the auto-correlation, PAPR and cross-correlation of the PRACH schemes based on the generic sequence constructions with two variants under the Alternative 1. It can be seen in Figure A-1(a) that the first auto-correlation side-peak is located  1δ∆f=1.38μs < 2.02 μs from the main-peak, and so falls within the timing detection window of 2.02μs. However, due to its low amplitude, i.e., only 0.0435, its impact on the mis-detection performance is negligible. Their PAPR values are compared in Figure A-1(b), and their pairwise maximum amplitude of normalized periodic cross-correlation (NPCC) within the timing detection window (i.e., 2.02 μs) are plotted in Figure A-1(c) and Figure A-1(d), respectively. The generic sequence construction with Variant 1 has lower PAPR and lower NPCC than that with Variant 2.

(a). Auto-correlation                                                              (b). PAPR

(c). Cross-correlation of Variant 1                                             (d). Cross-correlation of Variant 2

Figure A-1. Performance of generic sequence for the Alternative 1 for frequency resource allocation.

Figure A-2 contains the results for the Alternative 2 for frequency resource allocation. An auto-correlation side-peak of amplitude 0.1442 is observed on each side of the main-peak with time offset 1δ∆f=1.11μs < 2.02 μs and so falls within the timing detection window. This side-peak is low enough and does not impact the mis-detection performance. The generic sequence construction with Variant 1 has lower PAPR and lower NPCC than that with Variant 2.

Figure A-3 plots the results for the Alternative 3 for frequency resource allocation. In this case, the REs allocated to a PRACH preamble are uniformly distributed in the frequency band. Hence a zero auto-correlation zone (ZAZ) whose time span is much larger than the 2.02 μs timing detection window is achieved. Due to the uniform RE allocation for a PRACH preamble, the generic sequence construction with Variant 2 can maintain the low PAPR of ZC sequences (as low as 3.07 dB), while the Variant 1 produces higher PAPR (i.e., 6.25 dB). However, the generic sequence construction with Variant 1 achieves a much lower NPCC within the timing detection window than that with the Variant 2.

Figure A-4 plots the results for the Alternative 4 for frequency resource allocation. The Variant 1 produces much lower PAPR than the Variant 2.

(a). Auto-correlation                                                                            (b). PAPR

(c). Cross-correlation of Variant 1                            (d). Cross-correlation of Variant 2

Figure A-2. Performance of generic sequence for the Alternative 2 for frequency resource allocation.

(a).   Auto-correlation                                                                         (b). PAPR

(c). Cross-correlation of Variant 1                                    (d). Cross-correlation of Variant 2

Figure A-3. Performance of generic sequence for the Alternative 3 for frequency resource allocation.

(a).   Auto-correlation                                                                          (b). PAPR

(c). Cross-correlation of Variant 1                                   (d). Cross-correlation of Variant 2

Figure A-4. Performance of generic sequence for the Alternative 4 for frequency resource allocation.

Here the number of supported cells is calculated assuming 64 preambles per cell. Note that for Variant 2, one root index is not reused across cells, implying that some cyclically shifted versions of it may be abandoned.