3GPP TSG-RAN WG1 Meeting #96	Tdoc R1-1901599

Athens, Greece, February 25th – March 1st, 2019

Agenda Item:	7.2.6.4

Source:	Ericsson

Title:	Evaluation of URLLC Power Distribution Scenario

Document for:	Discussion, Decision

# Introduction

Power distribution is one of the main use cases considered for Rel. 16 eURLLC SI [1]. In this contribution we evaluate reliability and latency performance of URLLC power distribution use case using system level simulator. In addition, UL and DL geometry and link level results are presented. Assumptions on link and system level simulations are based on the assumption in [3] with additional adjustment to appropriately capture relevant deployment. The results show that it is possible to achieve the strict requirement of URLLC power distribution use case.

# Discussion

According to [3], requirements for power distribution use cases are given in Table 1 below.

Table 1: Assumption of reliability requirement for power distribution use case

| Use case (Clause #) | Reliability (%) | Latency (ms) | Data packet size and traffic model | Description |
| --- | --- | --- | --- | --- |
| Power distribution (22.804:5.6.4 &5.6.6) | 99.9999 | 5(end to end latency) Note: 2-3 ms air interface latency | DL & UL: 100 bytes ftp model 3 with arrival interval 100 ms | Power distribution grid fault and outage management |
|  | 99.999 | 15(end to end latency) Note: 6-7 ms air interface latency | DL & UL: 250 bytes Periodic and deterministic with arrival interval 0.833 ms Random offset between UEs | Differential protection |

In this contribution, we focus on the strict requirement, i.e., 99.9999% reliability within 3ms latency for the packet size of 100 bytes. We note that reliability requirement of 99.9999% at PHY layer assumes that higher layer mechanisms like PDCP duplication are not available. On the other hand, for deployments that have higher layer mechanisms like PDCP duplication, physical layer reliability requirement can be relaxed.

With this requirement in mind, we evaluate performance of URLLC for power distribution and differential protection use cases using system simulator.

## Power distribution grid fault and outage management

Based on system level simulation assumption in Table A-1 in the appendix B, we obtain DL and UL SINR geometry with full buffer assumption as shown in Fig. 1 in appendix A for calibration purposes. We note that the full buffer assumption here can be seen as a worst-case traffic assumption in terms of interference. Based on the results in Fig. A-1, the 5%-tile SINR are -0.35 dB (DL) and -0.44 dB (UL).

With the system level simulation assumption in Table A-1 for power distribution with full-buffer assumption, the 5%-tile SINR are -0.35 dB (DL) and -0.44 dB (UL) for ISD = 500m.

Based on system level simulation assumptions in Table A-2 and Table A-3 in the appendix B we obtain percentage of UEs meeting URLLC requirements according to Table 2. The UL power control setting is higher for Table A-2 and Table A-3 than for Table A-1. For both DL and UL slot-based single-transmission is used. For Table A-2 there is no eMBB users and PUSCH is grant-free using CG (Configured Grant) while for Table A-3 there are also eMBB users and PUSCH is SR-based.  Capability 2 processing times is used for UE. For gNB, the SR->grant time is 1 slot (1 PDCCH monitoring occasion per slot) and the PUSCH processing time is assumed to be 7 symbols.

Table 2: Percentage of UEs fulfilling 3 ms latency and 99.9999% reliability requirement with simulation assumptions in Table A-2 and Table A-3. l

|  | Scenario Table A-2 (no eMBB, CG PUSCH) | Scenario Table A-3 (with eMBB, GB PUSCH) |
| --- | --- | --- |
| Downlink | 99.8% | 95.8% |
| Uplink | 95.4% | 95.9% |

With the system level simulation assumption in Table A-2 (no eMBB, CG PUSCH) and Table A-3 (with eMBB, GB PUSCH) for power distribution with FTP model 3 a 3ms latency and 99.9999% reliability requirement can be met for more than 95% of UEs.

## Differential protection

Based on system level simulation assumption in Table A-4 in the appendix B we obtain percentage of UEs meeting URLLC requirements according to Table 3, where reliability requirement is 99.999%, and air interface one-way latency is 6ms. For this use case 7os allocation is used.

Table 3: Percentage of UEs fulfilling 6 ms latency and 99.999% reliability requirement with simulation assumptions in Table A-4.

|  | Average number of UEs per cell |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 4 |  | 6 |  | 8 |  | 10 |  |
|  | UE% | RU | UE% | RU | UE% | RU | UE% | RU |
| Downlink | 99.8% | 12.5% | 99.9% | 24.2% | 99.9% | 24.7% | 99.9% | 36.6% |
| Uplink | 96.2% | 8.7% | 95.7% | 16.3% | 95.2% | 20.6% | 94.4% | 24.8% |

With the system level simulation assumption in Table A-4 for differential protection with periodic traffic model a 6ms latency and 99.999% reliability requirement can be met for more than 99% of UEs in DL and more than 94.4% of UEs in UL.

# Conclusion

In section 2 we made the following observations:

Observation 1	With the system level simulation assumption in Table A-1 for power distribution with full-buffer assumption, the 5%-tile SINR are -0.35 dB (DL) and -0.44 dB (UL) for ISD = 500m.

Observation 2	With the system level simulation assumption in Table A-2 (no eMBB, CG PUSCH) and Table A-3 (with eMBB, GB PUSCH) for power distribution with FTP model 3 a 3ms latency and 99.9999% reliability requirement can be met for more than 95% of UEs.

Observation 3	With the system level simulation assumption in Table A-4 for differential protection with periodic traffic model a 6ms latency and 99.999% reliability requirement can be met for more than 99% of UEs in DL and more than 94.4% of UEs in UL.

# References

- RP-181477 New SID on Physical Layer Enhancements for NR URLLC, Huawei, HiSilicon, Nokia, Nokia Shanghai Bell

- ITU-R M.[IMT-2020-TECH PERF REQ] (WP5D-TD-0300)

- TR38.824 Study on physical layer enhancements for NR ultra-reliable and low latency case (URLLC) v.1.0.0

- 3GPP TS 38.214 V15.3.0

- R1-1900175 Latency Evaluation of Rel-15 URLLC, Ericsson, January 2019

# Appendix A. Additional simulation results

Figure A-1: DL and UL SINR distribution for different inter-site distance values for full-buffer traffic model.

Based on link level simulation assumption in Table A-5 in the appendix B, we present BLER performances of PDCCH and PDSCH/PUSCH.

For PDCCH, we consider DCI size =40 bits excluding CRC, AL4,8,16, and 1os CORESET. PDCCH BLER for different AL are given in Fig. A-2.

Figure A-2: PDCCH BLER with DCI size 40 for different AL

For data channels, we consider packet size of 100 bytes, transmission duration of 7 OFDM symbols with 1 DMRS symbol overhead. BLER for different MCSs supported within 40 MHz BW (e.g., MCS2 to MCS6 from Table 5.1.3.1-3 [4]) are given in Fig. A-3.

Figure A-3: BLER of data channels for different MCSs

# Appendix B. Simulation assumptions.

Table A-1: System level simulation assumption (power distribution use case)

| Parameters | Value |
| --- | --- |
| Layout | Single layer - Macro layer: Hex. Grid |
| Inter-BS distance | 500m |
| Carrier frequency | 4 GHz |
| Channel model | UMa in TR 38.901 |
| UE Tx power | 23dBm |
| BS antenna configurations | 8Tx/8Rx ports (M, N, P, Mg, Ng; Mp, Np) = (8, 4, 2, 1, 1; 1, 4), dH = 0.5λ, dV = 0.8λ; 9 degrees electrical antenna tilt |
| BS antenna height | 25m |
| BS antenna element gain + connector loss | 8 dBi |
| BS receiver noise figure | 5dB |
| UE antenna configuration | 2 Tx/4 Rx antenna ports |
| UE antenna height | Follow the modelling of TR 38.901 (e.g. 1.5m) |
| UE antenna gain | 3dBi |
| UE receiver noise figure | 9 dB |
| Total transmit power per TRxP | 49 dBm |
| Simulation bandwidth | 40 MHz DL and 40 MHz UL |
| Duplex mode | FDD |
| SCS | 30 kHz |
| UE distribution | 100% of users are outdoors |
| Traffic | Full buffer |

Table A-2: System level simulation assumption (power distribution use case)

| Parameters | Value |
| --- | --- |
| Layout | Single layer - Macro layer: Hex. Grid |
| Inter-BS distance | 500m |
| Carrier frequency | 4 GHz |
| Channel model | UMa in TR 38.901 |
| UE Tx power | 23dBm |
| BS antenna configurations | 8 Tx/8 Rx antenna ports: (M, N, P, Mg, Ng; Mp, Np) = (8, 4, 2, 1, 1; 1, 4), dH = 0.5λ, dV = 0.8λ; 3 degrees electrical antenna tilt |
| BS antenna height | 25m |
| BS antenna element gain + connector loss | 8 dBi |
| BS receiver noise figure | 5dB |
| UE antenna configuration | 2 Tx/4 Rx antenna ports, Isotropic |
| UE antenna height | Follow the modelling of TR 38.901 (e.g. 1.5m) |
| UE antenna gain | 0dBi |
| UE receiver noise figure | 9 dB |
| Total transmit power per TRxP | 49 dBm |
| BS receiver | MMSE-IRC |
| Number of UEs per cell | URLLC: 10 (on average) eMBB: 0 |
| Simulation bandwidth | 40 MHz |
| SCS | 30 kHz |
| UE distribution | 100% of users are outdoors Use 3km/h for modeling fading channel |
| UE power control | P0=-47.6 dBm, α=0.8 |
| HARQ/repetition | Single transmission, UL: configured grant Type2 |
| Channel estimation | No channel estimation error |
| Traffic | FTP model 3, arrival interval 100 ms, TBS=100 |
| URLLC requirements | Latency: 3 ms, Reliability 99.9999% |

Table A-3: System level simulation assumption (power distribution use case)

| Parameters | Value |
| --- | --- |
| Layout | Single layer - Macro layer: Hex. Grid |
| Inter-BS distance | 500m |
| Carrier frequency | 4 GHz |
| Channel model | UMa in TR 38.901 |
| UE Tx power | 23dBm |
| BS antenna configurations | 8 Tx/8 Rx antenna ports: (M, N, P, Mg, Ng; Mp, Np) = (8, 4, 2, 1, 1; 1, 4), dH = 0.5λ, dV = 0.8λ; 3 degrees electrical antenna tilt |
| BS antenna height | 25m |
| BS antenna element gain + connector loss | 8 dBi |
| BS receiver noise figure | 5dB |
| UE antenna configuration | 2 Tx/4 Rx antenna ports, Isotropic |
| UE antenna height | Follow the modelling of TR 38.901 (e.g. 1.5m) |
| UE antenna gain | 0dBi |
| UE receiver noise figure | 9 dB |
| Total transmit power per TRxP | 49 dBm |
| BS receiver | MMSE-IRC |
| Number of UEs per cell | URLLC: 10 (on average) eMBB: 10 (on average) |
| Simulation bandwidth | 40 MHz |
| SCS | 30 kHz |
| UE distribution | 100% of users are outdoors Use 3km/h for modeling fading channel |
| UE power control | P0=-70 dBm, α=0.8 |
| HARQ/repetition | Single transmission |
| Channel estimation | No channel estimation error |
| Traffic | FTP model 3, arrival interval 100 ms, TBS=100 |
| URLLC requirements | Latency: 3 ms, Reliability 99.9999% |

Table A-4: System level simulation assumption (Differential protection use case)

| Parameters | Value |
| --- | --- |
| Layout | Single layer - Macro layer: Hex. Grid |
| Inter-BS distance | 500m |
| Carrier frequency | 4 GHz |
| Channel model | UMa in TR 38.901 |
| UE Tx power | 23dBm |
| BS antenna configurations | 4 Tx/4 Rx antenna ports: (M, N, P, Mg, Ng; Mp, Np) = (8, 4, 2, 1, 1; 1, 2), dH = 0.5λ, dV = 0.8λ; 3 degrees electrical antenna tilt |
| BS antenna height | 25m |
| BS antenna element gain + connector loss | 8 dBi |
| BS receiver noise figure | 5dB |
| UE antenna configuration | 2 Tx/4 Rx antenna ports, Isotropic |
| UE antenna height | Follow the modelling of TR 38.901 (e.g. 1.5m) |
| UE antenna gain | 0dBi |
| UE receiver noise figure | 9 dB |
| Total transmit power per TRxP | 49 dBm |
| BS receiver | MMSE-IRC |
| Number of UEs per cell | URLLC: 4,6,8,10 (on average) eMBB: No |
| Simulation bandwidth | 40 MHz |
| SCS | 30 kHz |
| UE distribution | 100% of users are outdoors Use 3km/h for modeling fading channel |
| UE power control | P0=-70 dBm, α=0.8 |
| HARQ/repetition | Single transmission |
| Channel estimation | No channel estimation error |
| Traffic | Periodic and deterministic with arrival interval 0.833 ms Random offset between UEs Packet size is 250 bytes |
| URLLC requirements | Latency: 6 ms, Reliability 99.999% |

Table A-5: Link level simulation assumption (Urban macro scenario)

| Parameter | Value |
| --- | --- |
| Carrier frequency for evaluation | 4GHz |
| Channel model | TDL-C (delay spread: 300ns) as in 38.901 |
| UE speed | 3 km/h for modeling fading channel |
| BS antenna configuration | 2 Tx/4 Rx antenna ports |
| UE antenna configuration | 2 Tx/4 Rx antenna ports |
| System bandwidth | 40 MHz |
| Sub-carrier spacing | 30 kHz |
| Channel estimation | Practical |
| Receiver type | MMSE |