3GPP TSG RAN WG1 Meeting #96	Tdoc R1-1901600

Athens, Greece, February 25 – March 1, 2019

Agenda Item:	7.2.6.4

Source:	Ericsson

Title:	Evaluation of URLLC Transport Industry Scenario

Document for:	Discussion, Decision

# Introduction

After clarification of eURLLC study item a transport industry use case including remote driving was explicitly added [1]. In this contribution we evaluate reliability and latency performance of URLLC remote driving and ITS use cases by system level simulations. In addition, some link level simulations are done.

Assumptions on system and link level simulations are based on the technical report in [2] with some modifications. The assumptions are provided in appendix. The evaluation results show SINR distributions, and percentage of users satisfying latency and reliability requirements.

# Discussion

According to [2], requirements for transport industry use cases are given in Table 1 below.

Table 1: Assumption of reliability requirement for transport industry use case

| Use case (Clause #) | Reliability (%) | Latency (ms) | Data packet size and traffic model |
| --- | --- | --- | --- |
| Transport Industry (22.186: 5.5) Remote driving | 99.999 | 5 (end to end latency) Note: 3ms air interface latency | For UL: 2.5 Mpbs; Packet size 5220 bytes For DL: 1Mbps; Packet size 2083 bytes Note: Data arrival rate 60 packets per second for periodic traffic model |
| Transport Industry (23.501, 22.261) Intelligent transport system (ITS) | 99.999 | 10(end to end latency) Note: 7ms air interface latency | UL&DL: 1.1 Mbps, Packet size 1370 bytes Note: Data arrival rate 100 packets per second for periodic traffic model |

## Remote driving

Based on system level simulation assumption in Table A-1 in the appendix, the network is simulated for remote driving scenario. To avoid any confusions, it is important to note, that common hexagonal layout has been used.

Since 30 kHz SCS is being used, a 3-ms air interface latency budget corresponds to 6 slots. Within this time limit, there could be many transmission strategies proposed, including segmentation or slot aggregation due to the very large UL packets. In this contribution we consider the slot aggregation approach because it is simpler and easier to explain.

Taking into account the fact that mini-slot repetitions are not allowed and that transmissions may not cross border in Rel-15, the slot-based transmissions are used for this scenario where the number of PUSCH slots aggregated can be 1,2,4 or 8. However, since the PUSCH preparation time is 5.5 symbols (i.e., N2 = 5.5 symbols according to UE processing capability #2), the UE requires at a minimum 5.5 symbols from data arrival until PUSCH transmission. With an assumed one-slot gNB processing time we note that more than 2 repetitions are not possible since the alignment delay in UE can be up to one slot. Since traffic is predictable either configured grant or grant-based PUSCH is possible. For grant-based PUSCH, gNB can send a grant to be valid when data arrive in UE buffer without needing to wait for a SR from UE.

Due the smaller packets, DL is less challenging where slot-based single-transmission is used. Processing times in UE and gNB does not allow HARQ-based re-transmissions.

We note that more aggressive power control parameters in UL are applied to ensure URLLC probability. Another modification of agreed assumption is related to layout, where we show result for the hexagonal deployment instead.

The number of CCEs allocated for PDCCH per UE is typically 1 CCE wherein only a few percent of PDCCH transmissions use higher aggregation level. Highest aggregation used is 8 CCEs. The PDCCH is hence likely not the main limiting factor for this scenario. The 95-percentile of RB allocation size per UE is ~48 RBs for DL and ~82 RBs for UL. The mean and standard-deviation of percentage of UEs meeting URLLC requirements is according to Table 1.

Table 1: Percentage of UEs fulfilling URLLC requirements

|  | Mean | Standard deviation |
| --- | --- | --- |
| Downlink | 96.9% | 1.9% |
| Uplink | 60.5% | 4.8% |

With the system level simulation assumption in Table A-1 for transport industry use case:

- 95% of UEs fulfilling URLLC requirements can be reached for DL

- Only 60% of UEs fulfil URLLC requirements for UL with two repetitions. FFS if segmentation can perform better

Based on simulation results we may conclude that UL performance is the bottleneck in URLLC design.

## ITS

Similarly, based on system level simulation assumption in Table A-1 in the appendix, we provide simulation results of intelligent transport system scenario. The requirements here are 7 ms latency and 99.999% reliability.

We mainly reuse assumptions and principles from remote driving setup excluding UL scheduling strategy, where one slot transmission in enough.

We summarize simulation results for ITS scenario in the Table 2.

Table 2. Simulation results of intelligent transport system scenario with assumption from Table A-2.

|  | Average number of UEs per cell |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 4 UEs/cell |  | 6 UEs/cell |  | 8 UEs/cell |  | 10 UEs/cell |  |
|  | UE% | RU% | UE% | RU% | UE% | RU% | UE% | RU% |
| Downlink | 100% | 0.6% | 100% | 0.7% | 100% | 0.7% | 100% | 0.8% |
| Uplink | 99.4% | 0.6% | 99.2% | 0.7% | 98.8% | 0.8% | 98.8% | 1% |

Based on simulation result the following observation can be made:

According to system level simulations for ITS scenario 100% of users in DL can fulfil 7ms latency and 99.999% reliability requirements.

According to system level simulations for ITS scenario about 99% of users or more in UL can fulfil 7ms latency and 99.999% reliability requirements.

## Link level results

In addition to system level simulation results, we show the link level simulation results for PDSCH/PUSCH which are based on simulation assumption in the Appendix B to the paper (Table A-2). We simulated ITS and Remote driving scenarios with different packet sizes.

Due to big packet sizes and not extreme latency requirements the best option is to use slot-based transmissions (14 os). According to latency evaluation presented in [3] (table 4), there are following HARQ options available.

- For ITS scenario (7 ms latency) up to two HARQ retransmissions is available (5.18 ms latency) for both in DL and UL.

- For Remote driving scenario (3 ms latency) in UL and DL there is time for initial transmission only, but in principle if slot aggregation with AF=2 is used, one HARQ retransmission can be done.

We also simulated the case with half the packet size, assuming the large incoming packet can be fragmented, and the fragments are transmitted in a burst.

The preliminary simulation results for ITS scenario are shown in Figure A-1 and A-2, while results for Remote driving scenario are shown in Figure A-3 and A-4.

# Conclusion

This contribution provides the evaluation results for the transport industry scenario. In section 2 we made the following observation.

Observation 1	With the system level simulation assumption in Table A-1 for transport industry use case:

	95% of UEs fulfilling URLLC requirements can be reached for DL

	Only 60% of UEs fulfil URLLC requirements for UL with two repetitions. FFS if segmentation can perform better

Observation 2	According to system level simulations for ITS scenario 100% of users in DL can fulfil 7ms latency and 99.999% reliability requirements.

Observation 3	According to system level simulations for ITS scenario about 99% of users or more in UL can fulfil 7ms latency and 99.999% reliability requirements.

# References

- RP-181477 New SID on Physical Layer Enhancements for NR URLLC, Huawei, HiSilicon, Nokia, Nokia Shanghai Bell

- TR38.824 Study on physical layer enhancements for NR ultra-reliable and low latency case (URLLC) v.1.0.0

- R1-1900175 Latency Evaluation of Rel-15 URLLC, Ericsson, January 2019.

# Appendix A. Link level simulation results

Figure A-1: PDSCH BLER vs. SNR for ITS scenario.

Figure A-2: PUSCH BLER vs. SNR for ITS scenario.

Figure A-3: PDSCH BLER vs. SNR for Remote Driving scenario.

Figure A-4: PDSCH BLER vs. SNR for Remote Driving scenario.

# Appendix B. Simulation assumptions

Table A-1: System level simulation assumption

| Parameters | Value |
| --- | --- |
| Layout | Single layer - Macro layer: Hex. Grid 7 sites with 3 sectors |
| Inter-BS distance | 500m |
| Carrier frequency | 4 GHz |
| Channel model | UMa in TR 38.901 |
| UE Tx power | 23dBm |
| BS antenna configurations | 8Tx/8Rx ports; 3 degrees electrical antenna tilt |
| BS antenna height | 25m |
| BS antenna element gain + connector loss | 8 dBi |
| BS receiver noise figure | 5dB |
| UE antenna configuration | 2 Tx/4 Rx antenna ports |
| UE antenna height | Follow the modelling of TR 38.901 (3m) |
| UE antenna gain | 0dBi |
| UE receiver noise figure | 9 dB |
| Total transmit power per TRxP | 49 dBm |
| Simulation bandwidth | 40 MHz DL and 40 MHz UL |
| Duplex mode | FDD |
| SCS | 30 kHz |
| UE distribution | 100% of users are outdoors |
| UE speed | 60 km/h |
| UE power control | alpha = 0.8, target SINR = 50 dB |
| HARQ/repetition | DL: Single-transmission (14os), UL: 2 repetitions (2 slots) for remote driving or Single transmission (14os) for ITS |
| Channel estimation | Ideal |
| Traffic (Remote driving) | Periodic, 60 packets/sec, TBS DL: 5220 bytes, TBS UL: 2083 bytes |
| Traffic (ITS) | Periodic, 100 packets/sec, TBS in DL and UL: 1370 bytes. |

Table A-2: Link level simulation assumption

| Parameters | Value |
| --- | --- |
| Carrier frequency for evaluation | 4GHz |
| Channel model | TDL-C (delay spread: 300ns) as in 38.901 |
| UE speed | 60 km/h for remote driving and ITS; |
| BS antenna configuration | 4 Tx/4 Rx antenna ports |
| UE antenna configuration | 2 Tx/4 Rx antenna ports |
| System bandwidth | 40 MHz |
| Sub-carrier spacing | 30 kHz |
| Channel estimation | Practical |
| Receiver type | MMSE |
| Time allocation | 14 os |
| TBS | Remote driving: 5220/2610 bytes in UL and 2083/1042 bytes in DL ITS: 1370/685 bytes in UL and DL |
| Additional info | BLER curves are extrapolated by linear function in log domain |