3GPP TSG RAN WG1 #96	R1-1901663

# Athens, Greece, 25th February – 1st March, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | DRAFT | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft TS 38.214 CR on NZP CSI-RS configured with repetition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT_Core |  |  |  |  |  | Date: |  |  | 2019-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For beam management procedure P-3, higher layer parameter repetition shall be configured to "on" for the NZP-CSI-RS resource set. It is usually associated with a CSI-ReportConfig in which "reportQuantity" is set to "none". In RAN1 #95, the followings were agreed: Upon detection of a DCI format 0_1 with "UL-SCH indicator" set to "0" and with a non-zero "CSI request" where the associated "reportQuantity" in CSI-ReportConfig set to "none" for all CSI report(s) triggered by "CSI request" in this DCI format 0_1, the UE ignores all fields in this DCI except the "CSI request" and the UE shall not transmit the corresponding PUSCH as indicated by this DCI format 0_1. Hence, it is clear for SP-CSI or A-CSI on PUSCH with "reportQuantity" in CSI-ReportConfig set to "none". It is also concluded in RAN1 #95 that it is an error case where P-CSI or SP-CSI on PUCCH with "reportQuantity" in CSI-ReportConfig set to "none". In order to support periodic P-3 or semi-persistent P-3 with no PUSCH-based CSI report configured, it should be supported that higher layer configured periodic or semi-persisent NZP-CSI-RS resource set with repetition set to “on” while no P-CSI or SP-CSI on PUCCH is configured. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Repetition can be set to 'on' when reporting setting is not configured. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Periodic beam management P-3 would not be supported. Semi-persistent P-3 must be activated by a DCI. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: - If the network is implemented according to the CR and the UE is not, the UE may not conduct P-3 beam management measurement upon receiving the NZP CSI-RS with repetition set to ‘on’. - If the UE is implemented according to the CR and the network is not, there is no inter-operability problem and the network won’t configure NZP CSI-RS with repetition set to ‘on’ when there is no reporting settings are linked with the CSI-RS resource set. |  |  |  |  |  |  |  |  |

5.2.2.3.1	NZP CSI-RS

< Unchanged parts are omitted >

-	repetition in NZP-CSI-RS-ResourceSet is associated with a CSI-RS resource set and defines whether UE can assume the CSI-RS resources within the NZP-CSI-RS-ResourceSet are transmitted with the same downlink spatial domain transmission filter or not as described in Subclause 5.1.6.1.2. and can be configured when the higher layer parameter reportQuantity associated with all the reporting settings linked with the CSI-RS resource set is set to 'cri-RSRP' or 'none', or no reporting settings are linked with the CSI-RS resource set.

< Unchanged parts are omitted >