3GPP TSG RAN WG1 Ad-Hoc Meeting 1901		R1-1901665

Athens, Greece, 25th February – 1st March, 2019

Source:	vivo

Title:	Draft TS 38.213 CR on multiplexing HARQ-ACK/SR/CSI in PUCCH

Agenda Item:	7.1.3

Document for:	Discussion and Decision

- Discussion

In email discussion after RAN1#95, it is agreed to remove the following sentences for TS38.213 v 15.3.0 section 9.2.5.2,

A UE is configured by higher layer parameter maxCodeRate a code rate for multiplexing HARQ-ACK, SR, and periodic/semi-persistent CSI report(s) in a PUCCH transmission using PUCCH format 2, PUCCH format 3, or PUCCH format 4. If the PUCCH transmission uses PUCCH format 4 and if the total number of UCI bits is more than 115, the UE drops CSI reports as described in [6, TS38.214] until the total number of UCI bits is smaller than or equal to 115.

After deleting the sentence above, the UE behavior when exceeding maxCodeRate is as follows in the current spec.

Table 1. UE behavior for PUCCH format 4, total UCI payload results in exceeding maxCodeRate

TS38.213 v 15.4.0 section 9.2.5.2 … - if for Part 2 CSI report priority value(s), it is and , the UE selects the first Part 2 CSI reports, according to respective priority value(s) [6, TS 38.214], for transmission together with the HARQ-ACK, SR and Part 1 CSI reports , where is the number of Part 1 CSI report bits for the CSI report and is the number of Part 2 CSI report bits for the CSI report priority value, is a number of CRC bits corresponding to , and is a number of CRC bits corresponding to - else, the UE drops all Part 2 CSI reports and selects Part 1 CSI report(s), from the CSI reports in ascending priority value [6, TS 38.214], for transmission together with the HARQ-ACK and SR information bits where the value of satisfies and , where is a number of CRC bits corresponding to UCI bits, and is a number of CRC bits corresponding to UCI bits. …

According to this, for the following case (UE behavior in paragraph marked red in Table 1),

- if a UE is configured by higher layer parameter maxCodeRate a code rate for multiplexing HARQ-ACK, SR, and periodic/semi-persistent CSI report(s) in a PUCCH transmission using PUCCH format 4, and

- if the total UCI payload results in exceeding the maxCodeRate, and

- if UE drops all Part 2 CSI reports

The UE shall select NCSI-part1reported Part 1 CSI report(s). However, the specification does not state the case NCSI-part1reported=0, that is, the total UCI bits including ONLY first priority Part 1 CSI report and HARQ-ACK/SR results in exceeding maxCodeRate. Thus the specification is incomplete and the UE behavior for such case is unclear. The consequence is that gNB and UE may miss HARQ-ACK/SR transmission since the UE behavior is not clear.

Two alternatives can be considered as follows,

Alternative 1: UE is not expect to have NCSI-part1reported=0, that is to say, UE is not expected to have OACK+OSR+OCSI-part1,1+OCRC,CSI-part1,1>MRBPUCCH∙Nsc,ctrlRB∙Nsymb-UCIPUCCH∙Qm∙r

The cons of this alternative is that gNB configuration and scheduling flexibility is restricted.

Alternative 2: it is allowed to dropping all Part 1 CSI report and only transmit HARQ-ACK/SR. That is to say NCSI-part1reported=0 and NCSI-part2reported=0 is allowed when UE is configured by higher layer parameter maxCodeRate a code rate for multiplexing HARQ-ACK, SR, and periodic/semi-persistent CSI report(s) in a PUCCH transmission using PUCCH format 4. However, since usually the HARQ-ACK/SR payload is not large, it maybe not in the PUCCH RESET payload range before CSI/HARQ-ACK/SR multiplexing.

Considering specification complexity, alternative 1 is proposed to make the UE behavior of such case clear. The corresponding draft CR is attached.