3GPP TSG RAN WG1 Meeting #96							R1-1901689

Athens, Greece, 25th February – 1st March, 2019

Source:	vivo

Title:	On coexistence of NR and LTE sidelink for V2X

Agenda Item:	7.2.4.5

Document for:	Discussion and Decision

- Introduction

The feasibility and mechanism of coexistence of LTE and NR sidelink technologies have been discussed in the RAN1 1901 Ad-Hoc meeting, and the following agreements were achieved [1]:

Agreements: - For TDM solutions for in-device coexistence between LTE and NR V2X: - Time Alignment - Subframe boundary alignment is required between LTE and NR V2X sidelinks - Both LTE and NR V2X sidelinks are aware of the time resource index (e.g., DFN for LTE) in both carriers Agreements: - For long term time scale TDM solutions for in-device coexistence between LTE and NR V2X: - For a UE with coexistence impact, non-overlapping (in time domain) resource pools are (pre-)configured for NR V2X and LTE V2X sidelinks - No information is exchanged between LTE and NR sidelinks within the UE - Long term time scale TDM solution is feasible from RAN1 point of view - Note: although feasible, it is expected that such a solution may have impact on latency, reliability and data rate requirements for some applications - No additional modifications to LTE specifications are needed Agreements: Assuming SPS scheduling (mode -3 or mode-4) for LTE V2X, for short time scale TDM solutions for in-device coexistence for V2X, - For each occurrence of Tx/Tx overlap, one RAT is prioritized over another - This requires some information exchange between LTE and NR sidelinks within the UE - FFS: whether the information exchange between LTE and NR sidelinks can support this requirement - FFS: if there is impact to RAN1 LTE specification with this agreement - FFS: whether this solution can be up to UE implementation - For each occurrence of Tx/Rx overlap, one RAT is prioritized over another - This requires some information exchange between LTE and NR sidelinks within the UE - FFS: if there is impact to RAN1 LTE specification with this agreement - FFS: whether this solution can be up to UE implementation - FFS: If determination of priority for Rx operation is feasible and whether the information exchange between LTE and NR sidelinks can support this requirement Agreements: - Inter-band FDM Solutions for coexistence - For static power assignment of Pc,max for each carrier - Synchronization is not assumed for inter-band coexistence of NR sidelink and LTE sidelink. - This FDM solution is feasible for resolution of Tx/Tx coexistence conflicts - If the band separation is large enough (based on RAN4 indication), then this FDM solution for coexistence is feasible for Tx/Rx coexistence - If the band separation is NOT large enough, then this FDM solution is not feasible for resolution of Tx/Rx coexistence conflicts - For dynamic power sharing between carriers, - FFS details of FDM solutions and whether they are feasible

In this contribution, we provide our view on the coexistence issues of LTE and NR sidelink technologies.

- TDM solution

TDM solution has beneficial on no adjacent channel leakage nor power limitation between LTE and NR sidelink transmissions. However, TDM solution would inevitably degrade the latency and reliability to some extent. For long-term time-scale TDM solutions for in-device coexistence between LTE and NR V2X, no information exchange is required between LTE sidelink and NR sidelink within the UE, while for short-term time-scale TDM solutions, dynamic information exchange is inevitably required.

For resolution of Tx/Tx collision, there are two alternatives:

- Alt.1: Joint resource selection to avoid Tx/Tx collision

In this way, with short-term inter-module coordination between LTE sidelink and NR sidelink modules, a joint resource selection mechanism for LTE and NR sidelink can be performed in autonomous resource selection mode. The sensing and measurement results, as well as the selected resources, can be shared between modules, so that the sidelink transmissions for LTE and NR can be separated in time.

Given that the physical resource configurations may be distinct significantly between LTE sidelink and NR sidelink, the sensing procedure and monitoring may be sidelink-specific. However, the resource selection may be performed over the superset of candidate resources from both sidelink, so that the collision between LTE and NR sidelink transmissions can be avoided.

- Alt.2: Independent resource allocation with dropping based on QoS parameters

Independent resource allocation between LTE sidelink and NR sidelink can be applied for better spectrum utilization. In any case, if a collision between LTE and NR sidelink transmissions occurs, one of the transmission occasions should be dropped based on the QoS parameters. Thus, at least the QoS parameters of the associated packets should be exchanged. The drawback of this solution is the degradation on latency, reliability and data rate requirements due to packet dropping

On the other hand, for resolution of Tx/Rx collision, different resource allocation schemes of the receiving packet should be considered:

- Semi-static resource reservation

In this case, the Tx/Rx collision can be resolved by prioritizing the transmission or potential reception based on the QoS parameters. With semi-static resource reservation, the QoS parameters can be aware at the receiver UE given the reservation indication (e.g. SPS reservation in LTE) from the transmitter UE. In this case, the resource reservation period information should also be exchanged between sidelink modules.

- Dynamic resource assignment

If dynamic packet assignment is supported in NR, the receiver UE cannot predict the packet arrival and the associated QoS parameters. Consequently, the Tx would always prioritize the dynamic Rx practically, regardless of the QoS parameters. Nevertheless, this is not an essential issue for coexistence: such a half-duplex issue also happen within NR sidelink operation.

Proposal 1: For short-term time-scale TDM solutions, the information exchange between LTE sidelink and NR sidelink may include QoS parameters, sensing or measurement results, and resource reservation information.

- FDM solution

FDM solution has beneficial on supporting independent resource allocation between LTE and NR sidelink modules. Based on static power assignment, FDM solution is feasible for resolution of Tx/Tx coexistence conflicts, as well as Tx/Rx coexistence conflicts with sufficient frequency separation. In this case, no synchronization is required between LTE sidelink and NR sidelink.

On the other hand, dynamic power sharing between LTE sidelink and NR sidelink can also be used to mitigate the Tx/Tx collision issue in FDM. In the case of inter-band FDM coexistence with sufficient frequency separation, dynamic power sharing is feasible also for Tx/Rx coexistence. In any cases, subframe boundary alignment should be assumed between LTE sidelink and NR sidelink.

Proposal 2: For dynamic power sharing, subframe boundary alignment is assumed between NR sidelink and LTE sidelink.

If dynamic power sharing between LTE sidelink and NR sidelink is used, short-term information exchange between modules is required for scaling the transmission power, by prioritizing the transmission between different RATs. The prioritization of the transmissions can either base on the QoS parameters, or base on the RAT type (such as to avoid impact to LTE, similar to the EN-DC case).

Proposal 3: For dynamic power sharing of FDM exchange between LTE sidelink and NR sidelink, the power scaling may base on QoS parameters or the RAT type.

- Coexistence for in-coverage operation

If either TDM or FDM solution were applied to a UE, where scheduled resource allocation mode is applied to one sidelink, it would be beneficial to inform the network that coexistence issue may happen and to provide some assistance information to the network for proper scheduling. Moreover, the in-device conflict may also happen between Uu and sidelink, e.g. simultaneous sidelink transmission and uplink transmission. In this case, some coexistence issue would also happen, including:

- Collision between sidelink and uplink transmission;

- Self-interference issue between Uu and sidelink due to inter-modulation distortion and harmonic;

- Half duplex issue between Uu and sidelink

Proposal 4: The coexistence issues of power limited, self-interference and half-duplex need to be resolved for in-coverage operation.

To resolve these issues, assistance information between UE and network may be beneficial in this case. The assistance info, for example, may include the affected frequency band or carrier, or time domain resources, etc. If both sidelink are controlled by the network, some kinds of coordination on the network side can be assumed.

Proposal 5: A UE can inform the network that the coexistence issue would happen (either between different sidelink modules or between Uu and sidelink), and provides assistance information to the network to resolve the coexistence issues.

On the other hand, if the autonomous resource selection mode is enabled in one sidelink, the scheduling node of one RAT may not be aware of the autonomous resource selection result of the other RAT and vice versa, thus not possible to avoid collision between them. Therefore, it is helpful to take into account the resources reserved by the network in the other sidelink, during the resource sensing and reservation procedure. As a result, the problematic resource candidates can be excluded, and the potential in-device coexistence issues can be mitigated.

Proposal 6: During the sensing and reservation procedure in one sidelink, a UE should consider the resources reserved by the network in the other sidelink to mitigate the potential coexistence issue.

- Limited Tx/Rx capability

The limited Tx capability issue has been discussed for LTE CA in eV2X. A similar issue also occurs in the case of LTE and NR sidelink coexistence. The solution in LTE CA is done in the MAC, by either excluding the subframe exceeds its TX capability limitation, or triggering resource reselection until the resultant transmission resources can be supported. It may not be simply reused for the coexistence case, because a separate MAC entity is employed for each RAT and real-time coordination between may be challenging.

On the other hand, a UE with limited Rx capability may not be able to monitor all possible channels at the same time. Even if it is possible, it is suboptimal in terms of power consumption and UE complexity, and not always necessary. For instance, the UE may need to monitor the basic safety message in LTE sidelink all the time. However, for advanced V2X service, the UE may not always need to monitor the same channel. Therefore, some enhancements should be introduced to mitigate the limited Tx/Rx capability issue. This is very important in order to support pedestrian UE, where limited processing capability is assumed and exhaustive power consumption should be avoided.

Proposal 7: The issues of limited Tx/Rx capability need to be resolved for LTE and NR sidelink coexistence, especially for pedestrian UE.

If the SA and the data can be transmitted in different carriers or different sidelink, it is possible that the UE performs sensing on only one carrier can still be aware of the resource reservation on another carrier or another sidelink. As a result, only a limited set of carriers are required for sensing, which can mitigate the issue of limited Tx/Rx capability. Moreover, it is useful if a UE is required to keep monitoring on one carrier or sidelink for some specific services (e.g., basic safety), while still be able to sense and transmit on the other carrier or sidelink for other services.

Proposal 8: Cross-carrier/-sidelink sensing and reservation can be considered for mitigating the limited Tx/Rx capability issues.

- Conclusion

In the contribution, we provide our view on the coexistence issues of LTE and NR sidelink technologies with the following proposals:

Proposal 1: For short-term time-scale TDM solutions, the information exchange between LTE sidelink and NR sidelink may include QoS parameters, sensing or measurement results, and resource reservation information.

Proposal 2: For dynamic power sharing, subframe boundary alignment is assumed between NR sidelink and LTE sidelink.

Proposal 3: For dynamic power sharing of FDM exchange between LTE sidelink and NR sidelink, the power scaling may base on QoS parameters or the RAT type.

Proposal 4: The coexistence issues of power limited, self-interference and half-duplex need to be resolved for in-coverage operation.

Proposal 5: A UE can inform the network that the coexistence issue would happen (either between different sidelink modules or between Uu and sidelink), and provides assistance information to the network to resolve the coexistence issues.

Proposal 6: During the sensing and reservation procedure in one sidelink, a UE should consider the resources reserved by the network in the other sidelink to mitigate the potential coexistence issue.

Proposal 7: The issues of limited Tx/Rx capability need to be resolved for LTE and NR sidelink coexistence, especially for pedestrian UE.

Proposal 8: Cross-carrier/-sidelink sensing and reservation can be considered for mitigating the limited Tx/Rx capability issues.

- Reference

- Chairman’s Notes, 3GPP TSG RAN WG1 Ad-Hoc meeting 1901.