3GPP TSG RAN WG1 Meeting #96 R1-1901757 Athens, Greece, 25th February – 1st March, 2019 CR-Form-v11.2. CHANGE REQUEST. 38.214; CR; DRAFT; rev; -; Current version:; 15.4.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; X; Core Network. Title:; Draft CR on default QCL for PDSCH. Source to WG:; ZTE. Source to TSG:. Work item code:; NR_newRAT-Core; Date:; 2019-2-15. Category:; F; Release:; Rel-15. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16). Reason for change:; In TS38.214, the default QCL for PDSCH is determined according to PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest CORESET-ID in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored by the UE. However, the wording of “the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) in the TCI state with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET” is not clear: - Whether CORESET#0 without a configured TCI state should be considered? - If considered, any priority rules between a CORESET with a configured TCI state and another CORESET without a configured TCI state. It is due to the fact that the CSI-RS provided by the TCI state can provide more accurate channel tracking, CORESET without a configured TCI state should have a lower priority compared with other CORESET(s) with a configured TCI state. In order to have a unified solution, we should only describe the UE behaviours in the case of CORESET without a configured TCI state and the case of that with a configured TCI state.. Summary of change:; Adding the sentences to support the case of CORESET without a configured TCI state when determining default QCL for PDSCH.. Consequences if not approved:; Unclear description of default QCL assumption for PDSCH.. Clauses affected:; 5.1.5. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:.

### 5.1.5	Antenna ports quasi co-location

<Unchanged part omitted>

For both the cases when tci-PresentInDCI is set to 'enabled' and tci-PresentInDCI is not configured in RRC connected mode, if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL, as defined in [13, TS 38.306],

- if at least one CORESET is configured with a TCI state in a serving cell, the UE may assume that the DM-RS ports of PDSCH of the serving cell are quasi co-located with the RS(s) in the TCI state with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space and a configured TCI state with the lowest CORESET-ID in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored by the UE;

- otherwise, the UE may assume that the DM-RS ports of PDSCH of the serving cell are quasi co-located with the PDCCH DM-RS with respect to the QCL parameter(s) used for PDCCH transmission in the CORESET associated with a monitored search space with the lowest CORESET-ID in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored by the UE.

In this case, if the 'QCL-TypeD' of the PDSCH DM-RS is different from that of the PDCCH DM-RS with which they overlap in at least one symbol, the UE is expected to prioritize the reception of PDCCH associated with that CORESET. This also applies to the intra-band CA case (when PDSCH and the CORESET are in different component carriers). If none of configured TCI states contains 'QCL-TypeD', the UE shall obtain the other QCL assumptions from the indicated TCI states for its scheduled PDSCH irrespective of the time offset between the reception of the DL DCI and the corresponding PDSCH.

<Unchanged part omitted>