3GPP TSG RAN WG1 Meeting #96                                          R1-1901775

Athens, Greece, February 25th – March 1st, 2019

Source: 	ZTE Corporation

Title: 	Performance evaluation for factory automation

Agenda item:	7.2.6.4

Document for:	Discussion/Decision

Introduction

During RAN1#AdHoc1901 meeting, the simulation results and corresponding observations of power distribution were agreed to capture in the TR [1] and one email discussion on simulation results and corresponding observations of transport industry and Rel-15 enabled use cases were approved. The simulation results and corresponding observations of factory automation are left to be discussed in this meeting. Furthermore, companies are encouraged to report the number of packets simulated in each source.

In this contribution, we discuss the following two aspects,

- Simulation methodologies, including the number of packets simulated and simulation acceleration methods.

- Re-submit our geometry and simulation results of factory automation.

Simulation methodologies

As agreed in RAN1#94b [2], four representative use cases with different requirements can be evaluated for eURLLC simulation. In the last meeting, we submitted our geometry and corresponding simulation results of these four use cases [3][4][5][6][7]. Meanwhile companies are encouraged to report the number of packets simulated in each source as requested by Chairman. Table 1 presents the number of packets simulated in our previous contributions for each use case.

Table 1 Number of packets simulated in each source.

| Source | Use case | Reliability | Number of simulated packets per UE |
| --- | --- | --- | --- |
| R1-1900077 | Power distribution grid fault and outage management | 99.9999% | About 10^8 |
|  | Differential protection | 99.999% | About 10^7 |
| R1-1900078 | Factory automation | 99.9999% | About 10^8 |
| R1-1900079 | Rel-15 enabled use case (Urban) | 99.999% | About 10^7 |
|  | Rel-15 enabled use case (Indoor) | 99.9% | About 10^5 |
| R1-1900080 R1-1900238 | Remote driving and ITS | 99.999% | About 10^7 |

As we can see, some use cases require 10^-6 BLER, which means we have to simulate about 10^8 packets for these cases. Such a big number of packets will result in extremely long simulation time. To reduce the simulation time, some acceleration methods are adopted in our simulation platform to make the simulation time acceptable.

Acceleration method 1: TTI segmentation and parallel processing

Since each simulation sever has lots of cores (e.g., 64 or 128), these severs can support lots of process in parallel. In our simulation, we segment the required number of TTIs into several different TTI segmentations and each TTI segmentation is associated with one process. Each process is associated with one or more cores (different cores may be in different severs). All the TTI segmentations can be processed in parallel. All the TTI segmentations are simulated based on the same UE dropping, this can be guaranteed by setting the same random seed for UE dropping in each TTI segmentation.

To make sure the performance of parallel processing is equivalent to that without parallel process, channel continuity needs to be guaranteed. As depicted in Figure 1, the channel continuity is guaranteed by setting continuous TTI index and sufficient warm TTI in each TTI segmentation. By adopting this acceleration method, the simulation time can be significantly reduced.

Figure 1 TTI segmentation and parallel processing.

Acceleration method 2: Channel state update

According to our analysis and statistics, the channel state update module in our platform takes a large proportion of simulation time. To further reduce simulation time, the parallel processing method is also adopted for channel state update.

As we know, each UE’s channel state update is independent. Thus, channel state update of different UEs can be calculated in parallel. In our simulation, the channel state update will be processed in parallel if there is sufficient computation resource in this sever.

- Observation 1: Parallel processing can significantly reduce the eURLLC simulation time.

Geometry and simulation results of factory automation

This geometry and simulation results of factory automation in this section is a re-submit from our previous contribution [4].

Geometry

Based on previous discussion in RAN1#95, companies are encouraged to provide simulation results with the corresponding geometry. Both with-re-drop and without-re-drop are allowed. In this section, both DL and UL geometry without-re-drop for factory automation are present. The detailed simulation assumptions for DL/UL geometry are summarized in Table A-1 in the appendix. As depicted in Figure 2, the DL 5% Q-value for this scenario is -1.35 dB and the UL 5% Q-value for this scenario is -0.96 dB.

- Observation 2: For factory automation at 4 GHz,

- The 5% Q-value of DL geometry is -1.35 dB.

- The 5% Q-value of UL geometry is -0.96 dB.

Figure 2 Geometry of factory automation at 4 GHz.

Simulation results

In this section, we provide both percentage of UEs satisfying eURLLC requirements and maximum offered cell load. All the simulation results are based on the assumption of no re-drop. The detailed simulation assumptions for eURLLC SLS are summarized in Table A-1 in the appendix.

Downlink

As we agreed in RAN1#94b [8], for periodic traffic model for factory automation, it is assumed that the data for UEs in a group will arrive simultaneously in the evaluations. In our simulation, each 10 UEs are grouped together.

All the downlink transmission is grant-based with a latency-prioritized scheduling algorithm. The DL simulation results of factory automation at 4 GHz are summarized in Table 2.

Table 2 DL Simulation results of factory automation at 4 GHz.

| Description: Motion control Reliability: 99.9999% Latency: 2(end to end latency), 1 ms air interface latency Packet size: 32 bytes, periodic deterministic traffic model with data arrival interval 2 ms |  |  |
| --- | --- | --- |
| Scenario configurations | Offered cell load (Mbps) | UEs in outage |
| UE number per cell: 5 | 0.6108 | 0.00 % |

- Observation 3: For factory automation at 4 GHz with 5 UEs per cell, all UEs can satisfy the corresponding reliability and latency requirements for downlink.

Uplink

For uplink, both grant-based and grant-free simulation results are provided. For grant-based uplink transmission, the latency-prioritized scheduling algorithm is adopted. Furthermore, the N2 control-to-uplink delay is also taken into account, which is set as N2= 6.5 symbol. Considering the time-align delay, the maximum scheduling delay is up to 10 symbols in our simulation platform, which equals the sum of N2 and time-align delay. For grant-free uplink transmission, the N2 control-to-uplink delay can be ignored. In our simulation, Type-1 grant free configuration is used. To better fit in with the traffic model of this scenario, the grant-free configurations are set as P=4 slots, TO=1 slot, K=2.

- Grant-based

The grant-based UL simulation results of factory automation at 4 GHz is summarized in Table 3. As we can see, all the UEs can fully satisfy the corresponding reliability and latency requirements.

Table 3 Grant-based UL simulation results of factory automation at 4 GHz.

| Description: Motion control Reliability: 99.9999% Latency: 2(end to end latency), 1 ms air interface latency Packet size: 32 bytes, periodic deterministic traffic model with data arrival interval 2 ms |  |  |
| --- | --- | --- |
| Scenario configurations | Offered cell load (Mbps) | UEs in outage |
| UE number per cell: 5 | 0.6108 | 0.00% |

- Observation 4: For factory automation at 4 GHz with 5 UEs per cell, all UEs can satisfy the corresponding reliability and latency requirements for grant-based uplink transmission.

- Grant-free

The grant-free UL transmission results of factory automation at 4 GHz is summarized in Table 4. From the simulation results, all the UEs in this scenario can satisfy the corresponding reliability and latency requirements.

Table 4 Grant-free UL simulation results of factory automation at 4 GHz.

| Description: Motion control Reliability: 99.9999% Latency: 2(end to end latency), 1 ms air interface latency Packet size: 32 bytes, periodic deterministic traffic model with data arrival interval 2 ms |  |  |
| --- | --- | --- |
| Scenario configurations | Offered cell load (Mbps) | UEs in outage |
| UE number per cell: 5 | 0.6108 | 0.00% |

- Observation 5: For factory automation at 4 GHz with 5 UEs per cell, all UEs can satisfy the corresponding reliability and latency requirements for grant-free uplink transmission.

Conclusion

In this contribution, we present the number of packets simulated in our previous contributions and some acceleration methods that can reduce eURLLC simulation time. Moreover, the geometry and simulation results of factory automation at 4 GHz are re-submitted since they need to be discussed in this meeting.

To sum up, the following observations are made,

- Observation 1: Parallel processing can significantly reduce the eURLLC simulation time.

- Observation 2: For factory automation at 4 GHz,

- The 5% Q-value of DL geometry is -1.35 dB.

- The 5% Q-value of UL geometry is -0.96 dB.

- Observation 3: For factory automation at 4 GHz with 5 UEs per cell, all UEs can satisfy the corresponding reliability and latency requirements for downlink.

- Observation 4: For factory automation at 4 GHz with 5 UEs per cell, all UEs can satisfy the corresponding reliability and latency requirements for grant-based uplink transmission.

- Observation 5: For factory automation at 4 GHz with 5 UEs per cell, all UEs can satisfy the corresponding reliability and latency requirements for grant-free uplink transmission.

Reference

Draft Report of 3GPP TSG RAN WG1 #AH_1901 v0.1.0, Taipei, Jan. 21–25, 2019.

R1-1812101, Final Report of 3GPP TSG RAN WG1 #94bis v1.0.0, Chengdu, China, Oct. 8–12, 2018.

R1-1900077, Performance evaluation for power distribution at 4 GHz, Taipei, Jan. 21–25, 2019.

R1-1900078,	Performance evaluation for factory automation, Taipei, Jan. 21–25, 2019.

R1-1900079,	Performance evaluation for Rel-15 enabled use cases, Taipei, Jan. 21–25, 2019.

R1-1900080,	Performance evaluation for transport industry at 4 GHz, Taipei, Jan. 21–25, 2019.

R1-1900238,	Performance evaluation for urban macro scenarios at 700 MHz, Taipei, Jan. 21–25, 2019.

R1-1812101, “Final Report of 3GPP TSG RAN WG1 #94bis v1.0.0”, Chengdu, China, October 8–12, 2018.

Appendix

Table A-1 Simulation assumptions

| Parameters | Value |
| --- | --- |
| Inter-BS distance | 20m |
| Carrier frequency | 4 GHz |
| UE Tx power | 23dBm |
| BS antenna element gain + connector loss | 5 dBi |
| BS receiver noise figure | 5dB |
| BS antenna configurations | 8 Tx antenna ports (M, N, P, Mg, Ng; Mp, Np) = (2, 2, 2, 1, 1; 2, 2) |
| BS antenna height | 10 m |
| UE antenna configuration | 2 Rx: (M, N, P, Mg, Ng; Mp, Np) = (1, 1, 2, 1, 1; 1, 1) |
| UE antenna height | 1.5m |
| UE antenna gain | 0dBi |
| BS Tx power | 24 dBm per 20 MHz |
| BS receiver | MMSE-IRC |
| UE receiver noise figure | 9 dB |
| SCS | 30 kHz |
| Simulation bandwidth | 40 MHz |
| Layout | Single layer as defined in 38.802 Indoor floor: 12 BSs per 120 m x 50 m |
| Channel model | ITU InH for 4 GHz |
| Number of UEs per cell | 5 |
| UE distribution | 100% of users are indoor: 3 km/h |
| HARQ/repetition | Synchronous HARQ Max number of transmissions = 4 |
| UE UL power control | P0 = -90; alpha = 1 |
| Channel estimation | Ideal |
| Scheduling algorithm | Latency-prioritized scheduling algorithm Each slot has three PDCCH occasions. |