3GPP TSG RAN WG1 #96			R1-1901792

Athens, Greece, 25th February – 1st March, 2019

Source: MediaTek Inc.

Title: Low PAPR RS

Agenda Item: 7.2.8.5

Document for:  Discussion

# Introduction

The following agreements were made in RAN1 AH1901 meeting:

Agreement

For length 12,18, and 24 respectively, NR Rel-16 supports the binary CGS in the Table C1, C2, and C3 respectively followed by pi/2 BPSK modulation followed by DFT as DMRS sequence for π π/2 BPSK modulation for both PUSCH and PUCCH.

- The above is applicable to single-symbol DMRS configuration

- FFS: CGS for two-symbol DMRS configuration

Table C1, C2, and C3 can be found in R1-1901362

Agreement:
For length 6 CGS; 8-PSK is used
Decide the associated sequences in the next RAN1 meeting

Conclusion

Multiple companies in RAN1 have identified a power imbalance issue due to the some combinations of Rel.15 DMRS and some MIMO precoding matrices

- For example, it may occur for some cases of two symbol DMRS and two or four layer MIMO for a UE

- Some companies claim this can be avoided by implementation based solutions for PDSCH and PUSCH

Agreement
For one OFDM symbol DMRS and for PUSCH with pi/2 BPSK modulation, down select among the following alternatives

- Alt.0: Only a single DMRS port is supported (one comb is used)

- Alt.1: One DMRS port per comb is supported (in total 2 ports)

- Alt.2: Support two DMRS ports per comb (in total 4 ports)

- Study may take UL timing misalignment into account

In this contribution, we discuss length 6 CGS for time domain π/2-BPSK based DMRS, UE port multiplexing, and power imbalance issues.

# Low PAPR DMRS for DFT-s-OFDM

## Length 6 CGS

Due to a short length, binary sequence with length 6 cannot provide 30 candidates with desired properties. In RAN1 AH1901 meeting, it is agreed to use 8PSK for length 6 sequence.

In this section, we provide our results of 8PSK length 6 CGS in Table 1. The sequence sun is then passing to DFT-s-OFDM modulator to get DMRS waveform. The search strategy follows the same procedure as in [1] with auto-correlation in delay window [-1,1] and cross-correlation in window [-1,0,1]. 30 sequences are picked with less cross-correlation and avoid any pair of sequences are circular shift version to each other.

Table 1. 8PSK Length 6 CGS

sun=ejϕnπ/8

| u | ϕ0,…, ϕ(5) | PAPR (dB) |
| --- | --- | --- |
| 0 | 1 3 1 7 -3 -7 | 2.0214 |
| 1 | 1 3 1 -5 5 -7 | 1.9297 |
| 2 | 1 3 -3 7 5 -7 | 1.9507 |
| 3 | 1 5 3 -3 7 -7 | 1.9507 |
| 4 | 1 5 -1 -7 7 -7 | 1.9298 |
| 5 | 1 -3 1 -3 1 -3 | 1.8257 |
| 6 | 1 -3 3 -7 7 -7 | 2.0214 |
| 7 | 1 -3 -7 -3 1 5 | 0.7838 |
| 8 | 1 -3 -7 -3 1 -3 | 1.5331 |
| 9 | 1 -3 -7 -3 -7 -3 | 1.5333 |
| 10 | 1 -1 1 5 -5 7 | 1.9143 |
| 11 | 1 -1 1 -5 7 -3 | 1.9943 |
| 12 | 1 -1 3 7 1 5 | 1.6125 |
| 13 | 1 -1 3 7 1 -3 | 1.8358 |
| 14 | 1 -1 3 7 3 7 | 1.4624 |
| 15 | 1 -1 3 7 3 -3 | 1.4418 |
| 16 | 1 -1 3 7 -5 7 | 1.9852 |
| 17 | 1 -1 3 -7 5 -1 | 1.9942 |
| 18 | 1 -1 3 -3 1 5 | 1.7817 |
| 19 | 1 -1 3 -3 1 -3 | 1.7821 |
| 20 | 1 -1 3 -3 -7 -3 | 1.4418 |
| 21 | 1 -1 3 -1 3 7 | 1.6341 |
| 22 | 1 -1 3 -1 3 -3 | 1.7821 |
| 23 | 1 -1 3 -1 -7 -3 | 1.8359 |
| 24 | 1 -1 -7 5 -7 -3 | 1.9853 |
| 25 | 1 -1 -7 5 -5 -1 | 1.9144 |
| 26 | 1 -1 -7 -3 1 -3 | 1.6341 |
| 27 | 1 -1 -7 -3 -7 -3 | 1.4624 |
| 28 | 1 -1 -5 -1 3 -3 | 1.7817 |
| 29 | 1 -1 -5 -1 -7 -3 | 1.6126 |

Some statistics of PAPR, normalized auto-/cross-correlation are shown in Figure 1-Figure 3 and Table 2-Table 4, which compares some of current candidate 8PSK sequences in [2-5] and Rel-15 CGS. It shows the proposed sequences set has good auto-correlation and PAPR statistics.

Proposal 1: For length 6, NR Rel-16 supports sequences in Table 1 as DMRS for π/2-BPSK modulation PUSCH.

Figure 1. CCDF of power to average power ratio

Table 2. Statistics of PAPR (* with FDSS)

|  | mean | Max | Min |
| --- | --- | --- | --- |
| Rel-15 | 3.19 | 3.75 | 2.39 |
| R1-1900908* | 1.93 | 2.19 | 1.46 |
| R1-1900020* | 1.88 | 2.12 | 1.44 |
| R1-1900811* | 1.94 | 2.08 | 1.61 |
| R1-1900673* | 1.71 | 2.19 | 0.99 |
| Proposed* | 1.75 | 2.02 | 0.78 |

Figure 2. CDF of auto-correlation in [-1,1]

Table 3. Auto-correlation in [-1,1]

|  | mean | Max |
| --- | --- | --- |
| Rel-15 | 0 | 0 |
| R1-1900908 | 0.19 | 0.24 |
| R1-1900020 | 0.19 | 0.24 |
| R1-1900811 | 0.20 | 0.24 |
| R1-1900673 | 0.27 | 0.67 |
| Proposed | 0.09 | 0.18 |

Figure 3. CDF of cross-correlation in [-1,0,1]

Table 4. Cross-correlation in [-1,0,1]

|  | mean | Max |
| --- | --- | --- |
| Rel-15 | 0.38 | 0.91 |
| R1-1900908 | 0.38 | 0.96 |
| R1-1900020 | 0.37 | 0.93 |
| R1-1900811 | 0.39 | 0.92 |
| R1-1900673 | 0.36 | 1.00 |
| Proposed | 0.38 | 0.96 |

## DMRS ports multiplexing

In Rel-15, the orthogonality is assured by time domain cyclic shifts of a sequence which has constant amplitude in frequency domain. Furthermore, the shift values for each port are even distributed over the base sequence length (recall that the frequency pattern for the ports are [+,+,+,+,…] and [+,-,+,-,+,-,…] respectively for each port), so in time domain two ports are orthogonal not only when they are fully aligned but also within a certain delay window. In design of time domain π/2-BPSK sequence, finding a set of based sequence that have the same good correlation property as Rel-15 is unlikely to be possible. Three alternatives discussed in RAN1 meeting:

- Alt.0: Only a single DMRS port is supported (one comb is used)

- Alt.1: One DMRS port per comb is supported (in total 2 ports)

- Alt.2: Support two DMRS ports per comb (in total 4 ports)

Alt.0 is an extreme case that limits system capability to only support SU in one OFDM symbol. So Alt.0 should be excluded.

For Alt.1, it still can support MU but only up to 2 users (one for each CDM group). This somewhat downgrade from Rel-15 (4 users) while interference between the users can be avoid.

For Alt.2, though it supports the same MU capability as Rel-15, the (auto-/cross-) correlation property is not as good as in Rel-15 in general. As a result, inter-user interference (in the same CDM group) might be non-negligible especially when consider UL timing misalignment. For larger bandwidth allocation (PRB>4), the pseudo-random sequence with larger length can help to average out the interference. However, the CGS used for small PRB allocation [7] cannot assure the same auto-correlation for the 2nd port and the cross-correlation between the ports (either in the same cell or between cells). We will address part of this issue in later section when discuss how to choose TD-OCC for orthogonal port generation.

From our view, the system should try to maximize its MU capability while controls the interference under an acceptable level. Therefore, for larger bandwidth allocation (PRB>4), it could adopt Alt.2 to maximize number of ports; and for small bandwidth allocation (PRB≤4), adopt Alt.1 to minimize the interference.

Proposal 2: In Rel-16, for π/2-BPSK DMRS used in DFT-s-OFDM PUSCH, support Alt.2 for larger bandwidth allocation (PRB>4) and support Alt.1 for smaller bandwidth (PRB≤4).

### Orthogonal ports generation

In previous meeting, companies [1-6] consider orthogonal port generation by using an intra-symbol TD-OCC. The practical implementation could be on frequency domain (e.g., [1,4]) but the overall operation is equivalent to a TD-OCC, so in below, the problem of deriving orthogonal ports is formulated as choosing a TD-OCC. The pre-DFT sequence of the i-th port can be expressed as

sin=win⋅sn      i=0,1.      n=0,1,2,..

where sn is a base π/2-BPSK sequence designed with desired properties (PAPR, auto-correlation).

It is clear that w0=1,1,1,… can be used for port0. However,  w1n need to be carefully designed so that port1 can have the same desired properties as port0. Two main considerations for choosing w1:

- Consider PAPR, w1n should be in {+1,-1} so s1n is still a π/2-BPSK sequence.

- Consider autocorrelation, we show in Section 5 that by choosing w1n as a DFT basis, s1n will have the same amplitude of circular auto-correlation function as s0n.

Consider both factors above, a nature choice is w1=[+1,-1,+1,-,1,…] or equivalently w1n= -1n.

Figure 4 shows examples of auto-correlation of two ports generated by different w1 using CGS in [7]. The top subplot is using w1=[1,1,1,…, -1,-1,-1,…] and the bottom subplot is using w1n= -1n.

| (a) Length-12 (seqIdx=0) | (b) Length-18 (seqIdx=0) |
| --- | --- |
| (c) Length-24 (seqIdx=0) |  |

Figure 4. Auto-correlation function of DMRS ports

Proposal 3: For π/2-BPSK DMRS used in DFT-s-OFDM PUSCH, support two ports multiplexing by applying intra-symbol TD-OCC w0n=1, and  w1n= -1n

# Power imbalance

The existing power imbalance issues (between OFDM symbols and between RE in same OFDM symbol) increase signal dynamic range unnecessarily. On transmitter side, it decreases PA efficiency and could also has side effect on OOB spectrum emission (see [8]). On receiver side, large symbol-to-symbol power imbalance could also complicate AGC design. Therefore, we think it need to be fixed in Rel-16.

One approach is to apply a port-specific phase (e.g. pseudo-random phase) on each DMRS port. This can be done by an implementation-based solution, i.e., transmitter apply phase rotation on the transmitted signal and is transparent to receiver, or it can be explicitly specified in the specification. If we go with implementation-based solution in RAN1, then a downlink waveform with power imbalance is still an allowed waveform and UE receiver might still be expected to handle such a waveform to avoid handling such cases, then it should be clarified that a UE is not expected to handle waveforms with power imbalance.

We think RAN1 should notify RAN4 for the power imbalance issue and pursue further discussion based on RAN4’s feedback.

Proposal 4: RAN1 notifies RAN4 for the power imbalance issue. Pursuit a specification solution based on RAN4’s feedback.

# Conclusions

Proposal 1: For length 6, NR Rel-16 supports sequences in Table 1 as DMRS for π/2-BPSK modulation PUSCH.

Proposal 2: In Rel-16, for π/2-BPSK DMRS used in DFT-s-OFDM PUSCH, support Alt.2 for larger bandwidth allocation (PRB>4) and support Alt.1 for smaller bandwidth (PRB≤4).

Proposal 3: For π/2-BPSK DMRS used in DFT-s-OFDM PUSCH, support two ports multiplexing by applying intra-symbol TD-OCC w0n=1, and  w1n= -1n

Proposal 4: RAN1 notifies RAN4 for the power imbalance issue. Pursuit a specification solution based on RAN4’s feedback.

# Auto-correlation with DFT basis TD-OCC

Circular auto-correlation function for sequence sn can be written as

rk= m=0N-1sms*m+k%N
=m=0N-1smu-n-m%N,     where ut≜s*-t  (conjugated time reversal)
=s*u-n      (circular convolution)

Take DFT on both side of above equation, we have expression in frequency domain:

Rf=S-f⋅U-f
where Uf=S*-f.

Now consider w[n] is a DFT basis, i.e., wn=ej2πf0nN, for some f0∈[0,1,…N-1]. The sequence obtained by sn=snw[n] a circular shift version of sn in the frequency domain, i.e.,

Sf=S[f-f0]

sn has its conjugated time reversal sequence un which has frequency domain response

Uf=S*-f=S*-f-f0=U[f-f0]

Similarly, the autocorrelation function of sn denoted by rn is rn=(s*u)-n and we have in frequency domain:

Rf=S-f⋅U-f
=S[-f-f0]⋅U[-f-f0]
=Rf-f0

The last equation implies in time domain, auto-correlation function of sn:

rn=rnej2πk0nN  or  rn=rnw[n]

Therefore,  rn=rn for ∀n.

# References

[1] R1-1900220, “Low PAPR RS,” MediaTek, RAN1 Ad-Hoc Meeting 1901, Taipei, Taiwan, January, 2019

[2] R1-1900908, “Lower PAPR reference signals”, Qualcomm, RAN1 Ad-Hoc Meeting 1901, Taipei, Taiwan, January, 2019.

[3] R1-1900020, “Discussion on DMRS sequence design for low PAPR”, Huawei, HiSilicon, RAN1 Ad-Hoc Meeting 1901, Taipei, Taiwan, January, 2019.

[4] R1-1900811, “On Low PAPR RS”, InterDigital, RAN1 Ad-Hoc Meeting 1901, Taipei, Taiwan, January, 2019.

[5] R1-1900673, “Discussion on low PAPR RS”, NEC, RAN1 Ad-Hoc Meeting 1901, Taipei, Taiwan, January, 2019.

[6] R1-1900090, “Low PAPR RS”, ZTE, RAN1 Ad-Hoc Meeting 1901, Taipei, Taiwan, January, 2019

[7] R1-1901362, “Joint proposal on length-12, length-18, and length-24 CG sequences for pi/2 BPSK”, Qualcomm, Intel, ZTE, Sanechips, Huawei, HiSilicon, IITH, CEWiT, Reliance Jio, IITM, Tejas Networks, NEC, InterDigital, MTK, VIVO, RAN1 Ad-Hoc Meeting 1901, Taipei, Taiwan, January, 2019.

[8] R1-1810437, “Low PAPR RS,” MediaTek, RAN1 94bis, Chengdu, China, October 2018