3GPP TSG RAN WG1 Meeting #96                                                        R1-1901804

Athens, Greece, 25th Feb. - 1st Mar., 2019

Agenda Item: 7.2.9.2

Source: MediaTek Inc.

Title: NR UE Power Saving Designs

Document for: Discussion and Decision

# Introduction

In [1], there provide comprehensive power consumption evaluation results. For FR1 reference configuration and the three agreed traffic models, the power consumption distributions can be checked in Figure 1:

Observation 1: Among the non-data-transmission power states, PDCCH-only and sleep states consume the most power.

Proposal 1: NR power saving improvement should aim for:

- Reducing PDCCH-only power consumption by reducing the monitoring amount and the power consumption per monitoring occasion

- Optimizing the sleep power by reducing the wake-up number and increasing deeper sleeps

Figure 1: Power consumption distribution for different traffic models

In the subsequent sections, we will elaborate in power saving designs according the following order:

- Adaptation in frequency domain

- Adaptation in time domain

- Adaptation in antenna domain

- Adaptation to DRX operation with UE assistance

- Adaptation to DRX operation without UE assistance

- Adaptation to reduce PDCCH monitoring

- Joint Power Saving Adaptation by extended BWP framework

- Power saving signal/channel design and comparison

- Power saving procedures

- UE Energy Efficiency Measures and Joint Optimization

# Adaptation in Frequency Domain

## Dynamic BWP Switching for a Serving Cell

Dynamic BWP switching via DCI is introduced in Rel-15. One general concern is the additional latency due to A-CSI reporting after switching to a BWP of wider bandwidth. In section 2 of [1], we evaluate the power saving gain and the increment in per-packet latency where A-CSI reporting cause 7-slot delay to the first data scheduling after BWP switching. One can check the results quoted in Table 1 and arrive at the following observation:

Observation 2: The average increment in per-packet latency due to A-CSI reporting after switching to a BWP of wider bandwidth is only within 4% and 2% for FTP/Video and IM traffic models, respectively, since only few front packets are delayed. On the other hand, there can realize significant power saving gain of 23% and 24% accordingly.

Table 1: Metrics of Rel-15 DCI and timer based BWP switching

| Metric | FTP/Video Baseline & dynamic BWP | IM Baseline & dynamic BWP | VoIP Baseline & static BWP (20MHz) |
| --- | --- | --- | --- |
| Power consumption (relative unit) | 40.01 | 8.19 | 34.72 |
| Power saving ratio (w.r.t. baseline) | 23.54% | 24.79% | 31.42% |
| Latency (ms) | 62.09 | 150.87 | 13.41 |
| Latency increment (w.r.t. baseline) | 3.98% | 1.91% | -0.07% |
| RU (%) | 32.55 | 1.56 | 0.98 |
| RU increment (w.r.t. baseline) | -0.55% | 1.29% | 0% |

For CSI enhancement for BWP in Rel-16 the following two schemes can be considered:

- Allowing UE to perform periodic wideband measurement:

  - In Rel-15, UE is not expected to measure CSI-RS out of BWP bandwidth. To eliminate the delay of A-CSI reporting after BWP switching, Rel-16 can allow UE to measure periodic CSI-RS covering the union frequency range of all configured BWPs and the initial BWP.

    - Note: If the wideband measurement can cover an effective SSB, there can also eliminate the need of intra-frequency measurement gap for UE to switching from a BWP without SSB to another one with SSB.

- If power-saving signal is configured, allowing it to trigger A-CSI reporting outside DRX OnDuration can also help to eliminate the A-CSI delay to data scheduling.

  - Please refer to Section 6.3.3 for a specific design.

Proposal 2: For CSI enhancement for BWP in Rel-16, the following two schemes can be considered:

- Allowing UE to perform periodic wideband measurement

- Allowing power saving signal to trigger A-CSI reporting outside DRX OnDuration.

## Adaptation Design for Carrier Aggregation

Table 1 shows that significant power saving gain can be realized with dynamic BWP switching for a serving cell. For the case of carrier aggregation (CA), BWP switching can also be applied independently in primary cell (PCell) and secondary cells (SCells). However, there can be more significant power saving by introducing a BWP without PDCCH monitoring in SCell.

LTE Rel-15 introduces a new dormant state for SCell. In the dormant state, UE doesn’t need to monitor PDCCH but keeps periodic measurements and reports. The main benefit is faster recovery time to the activated state, reduced from 24 ms to 8 ms. This feature achieves better balance in UE power saving and SCell efficiency.

For NR, suspending PDCCH monitoring doesn’t require a new SCell state. In fact, switching UE to a BWP without PDCCH monitoring can effectively create the dormant behaviour. Since BWP switching delay is only 2-3 ms, realizing SCell power saving via BWP switching can achieve faster access switching time than LTE. Better UE power saving and SCell efficiency than LTE can therefore be realized.

Proposal 3: Support a BWP without PDCCH monitoring for SCell in Rel-16 to allow UE dormancy and realize faster SCell access switching than LTE by exploiting the short BWP switching time.

From field experience, SCell is mainly utilized to offload large packets on PCell. For the case where data size is small, data will not split to SCell. When dynamic BWP switching is utilized in PCell and small-BW is applied when PCell data size is small, disabling SCell PDCCH monitoring when PCell is in small-BW BWP can eliminate dummy PDCCH monitoring power in SCell. When PCell is in large-BW BWP, enabling SCell offloading can realize faster data delivery. In this way, power saving and system performance can be best balanced.

Although cross-carrier scheduling can be used to switching SCell BWP, there are two drawbacks:

- If SCell data is always scheduled in PCell, PCell control resource will be occupied, and such design is not scalable for larger amount of SCells.

- If network wants to switching back to self-carrier scheduling on SCell after SCell is switched to a normal BWP, it relies on RRC reconfiguration which will cause long latency and break the benefit of fast SCell access switching.

It is therefore suggested that bundled BWP switching between PCell and SCell can be supported in Rel-16 in order to minimize PCell control resource overhead as well as ensuring fast SCell access switching.

More specifically, among all serving cells, some cells are selected as leader cells for adaptation of power consumption configuration; each leader cell is associated with a set of follower cells. The key idea is the active power consumption configuration of a follower cell is determined by that of the associated leader cell, which implies that a switch of power consumption configuration in a leader cell initiates a configuration switch in the associated follower cells. See Figure 2 as an example. In the figure, the leader cell is associated with follower cell 1 and follower cell 2. When UE adapts from BWP 1 to BWP 3 in the leader cell, this adaptation initiates the adaptation in the two follower cells. In follower cell 1, adaptation is from BWP 1 to BWP 3. In follower cell 2, adaptation is from BWP 1 to BWP 2, which implies BWP 1 of the leader cell and BWP 1 of follower cell 2 are bundled; BWP 3 of the leader cell and BWP 2 of follower cell 2 are bundled. The bundling relation can be simply based on the BWP index or a combination of the BWP index and some predefined conditions.

Figure 2: BWPs in leader cell and follower cells are bundled

Proposal 4: In the case of carrier aggregation, some cells are selected as leader cells; each leader cell is associated with a set of follower cells. An adaptation of power consumption configuration in a leader cell initiates a configuration adaptation in the associated follower cells.

Note that either the PCell or the SCell can be a leader cell. The bundling should be flexible so as to reduce the control overhead for the commonly used serving cell settings.

In [1], we evaluate the power saving benefits in SCell for the setting of one PCell and one SCell. Compared with the setting where PCell and SCell apply independent BWP switching, incorporating BWP-based SCell power-saving and bundled BWP switching with PCell can realize 52% and 65% power saving gain for the FTP/Video and IM traffic models mixed with a small-data service in background. This is shown in Table 2 quoted from [1]. Therefore, we have

Observation 3: Bundled BWP switching for BWP-based SCell power saving can realize 47% power saving on SCell by the effective reduction in dummy PDCCH monitoring.

Proposal 5: Bundled BWP switching for carrier aggregation is considered for NR Rel-16 to realize significant power saving gain by incorporating SCell BWP without PDCCH.

Table 2: Evaluation results with BWP-based SCell power saving and bundled BWP switching

| Metric | FTP/Video with background traffic | IM with background traffic |
| --- | --- | --- |
| Power consumption (relative unit) | 19.45 | 5.56 |
| Power saving ratio (w.r.t. SCell baseline) | 51.18% | 67.33% |
| Latency (ms) | 101.56 | 120.65 |
| Latency increment (w.r.t. SCell baseline) | -0.675% | 1.60% |
| RU (%) | 15.31 | 0.83 |
| RU increment (w.r.t. SCell baseline) | -1.23% | 0% |

# Adaptation in Time Domain

For adaptation in time-domain, major consideration lies in comparing cross-slot scheduling and same-slot scheduling. For same-slot scheduling, [2] proposed a Time Domain Resource Allocation (TDRA) design where there is a time gap between PDCCH and PDSCH. The intention is to allow UE to skip PDSCH symbols unless there is data scheduling identified from PDCCH. The time gap is to ensure sufficient transition time for UE to restart receiving PDSCH symbols. When there is no data scheduling, this design can realize same benefit as cross-slot scheduling since UE can skip all PDSCH symbols. When there is data scheduling, the reduced PDSCH volume may cause longer data transmission/reception time and increase power consumption unless the data size is small enough to fit in the reduced PDSCH volume.

In Table 3 quoted from [1], there show the evaluation results with same slot scheduling with 6-symbol time gap between PDCCH and PDSCH and 7 PDSCH symbols. For the case of large data size as in, e.g., FTP/Video traffic model, the reduced PDSCH volume will cause higher power consumption and consume more system resource.

Observation 4: Same-slot scheduling with time gap between PDCCH and PDSCH can realize 15% power saving gain for small-data service, e.g. VoIP. However, it can consume higher power and more system resource for the services with large data size as in, e.g., FTP/Video traffic model.

Table 3: Same slot scheduling with 6-symbol PDCCH-PDSCH gap and 7-symbol PDSCH

| Metric | FTP/Video | IM | VoIP |
| --- | --- | --- | --- |
| Power consumption (relative unit) | 55.44 | 9.02 | 42.78 |
| Power saving ratio (w.r.t. baseline) | -5.94% | 17.17% | 15.50% |
| Latency (ms) | 92.91 | 148.74 | 13.36 |
| Latency increment (w.r.t. baseline) | 55.60% | 0.47% | 0.45% |
| RU (%) | 65.55 | 2.98 | 0.98 |
| RU increment (w.r.t. baseline) | 100.27% | 93.51% | 0% |

On the other hand, applying cross-slot scheduling can provide consistent power saving gains across different services, as shown in Table 4 quoted from [1]. Consequently, we can have

Observation 5: Cross-slot scheduling shows consistent power saving gain, ranging from 13% to 18%, for different traffic models.

Table 4: Evaluation results of cross-slot scheduling

| Metric | FTP/Video | IM | VoIP |
| --- | --- | --- | --- |
| Power consumption (relative unit) | 42.39 | 9.01 | 43.77 |
| Power saving ratio (w.r.t. baseline) | 18.99% | 17.26% | 13.54% |
| Latency (ms) | 60.21 | 148.54 | 13.92 |
| Latency increment (w.r.t. baseline) | 0.84% | 0.34% | 3.72% |
| RU (%) | 32.73 | 1.54 | 0.98 |
| RU increment (w.r.t. baseline) | 0% | 0% | 0% |

# Adaptation in Antenna Domain

When the data packets are small, reducing 4x4 MIMO to 2x2 MIMO for FR1 can save 30% UE power. The system impact can also be confined. When data rate is large, e.g., FTP/Video traffic, UE should support 4x4 MIMO to achieve high spectral efficiency and reduced reception time. How to effectively adapt the MIMO layer number is important to balance UE power consumption and system capacity.

One possibility to realize MIMO layer adaptation is to exploit BWP switching. Assume maximum MIMO layer number is a per-BWP parameter. Then MIMO adaptation can be realized by:

- Switching to the BWP with maximum MIMO layer number equal to 4 whenever there is data arrival

  - Assume UE RX antenna switching can be done within BWP switching delay

- Fall back to default BWP with maximum MIMO layer number no larger than 2 if BWP timer expires

  - Assume BWP timer threshold of 20 ms for the evaluation

Figure 3 illustrates the dynamic adaptation via BWP switching, which can provide the following benefits:

- Minimized control channel resource overhead: When there is no data, there is no need of PDCCH. Even when UE is in standby with only 2 RX antennas, there is no need to send PDCCH of larger aggregation level. The only required PDCCH is for switching BWP, and the resource cost can be minimized if there is a compact dedicated DCI dedicated BWP switching. In this regard, the concern on extra control channel overhead due to reduced UE RX antenna number can be minimized.

- Minimized impact to spectral efficiency: When there is data, UE is indicated to support 4-layer transmissions and there is no spectral efficiency loss. Even considering A-CSI reporting after switching back to 4-layer BWP, the latency impact is only to the first few packets and, in average, the increment can be within 5%. If pipeline transmission is considered, the data rate averaged over a DRX period remains the same, meaning no compromise in the average link throughput.

Figure 3: Illustration of MIMO layer adaptation via BWP switching

In Table 5 quoted from [1], one can see 14% and 15% power saving gain realized for FTP/Video and IM, respectively. For VoIP, we always apply a 2-layer BWP since the service can be identified and a dedicated setting can be applied. As can be seen in the last column of Table 5, 2x2 MIMO for VoIP can save 19% power. Note that the static adaptation can also be realized by fixing a BWP for the UE. Based on the results, we have the following observations and proposals:

Observation 6: Adapting maximum MIMO layer via BWP switching can save NR UE power (14% and 15% for FTP/Video and IM traffic models, respectively).

Observation 7: Restricting 2x2 MIMO to VoIP can save ~20% UE power with no impact to the system efficiency

Proposal 6: BWP-based MIMO adaptation is considered for NR Rel-16

Table 5: Evaluation results of BWP-based adaptation on maximum MIMO layer number

| Metric | FTP/Video Dynamic switching | IM Dynamic switching | VoIP Static 2x2 MIMO |
| --- | --- | --- | --- |
| Power consumption (relative unit) | 44.81 | 9.18 | 40.74 |
| Power saving ratio (w.r.t. baseline) | 14.37% | 15.70% | 19.53% |
| Latency (ms) | 61.72 | 150.69 | 13.18 |
| Latency increment (w.r.t. baseline) | 3.36% | 1.79% | -1.78% |
| RU (%) | 32.59 | 1.57 | 0.94 |
| RU increment (w.r.t. baseline) | -0.43% | 1.95 % | -4.08% |

# Adaptation to DRX Operation with UE Assistance

DRX reduces power consumption by allowing the UE to spend a higher proportion of its time in sleep states. This can be achieved by increasing the DRX cycle duration, or by reducing the active period determined by the OnDuration and Inactivity timers, but each of these adjustments generally results in an increase in latency. This is often acceptable in simple upload or download scenarios, where it appears as additional delay, but for interactive real time applications the increase in round-trip delay can give a perceptibly poorer performance. The optimum setting for the DRX cycle is therefore a compromise that gives the lowest power consumption that is consistent with providing acceptable latency for every application running on the UE.

The network has poor visibility of the mix of applications that are running on each UE, and “acceptable latency” is determined by the user on an application-by application basis, so if optimisation is to occur some form of UE assistance is needed to provide the gNB with sufficient information to allow it to configure a power-efficient DRX cycle.

In a separate study [5], the effect of traffic arrival patterns on the sleep and wakeup patterns is analysed using the reference evaluation scenarios agreed in RAN1 94bis [1], and it is shown that significant power reductions can be obtained by adapting the DRX cycle parameters to the traffic statistics. The characteristics of this adaptation are outlined in the description that follows – more detail is provided in reference [5]

Data arrival for an application is characterised by a mean inter-arrival time. In this description, if data is evenly distributed in time this is defined as the average gap between successive data packets, but if data typically arrives in bursts it is the average gap between successive bursts.

For low power consumption, the active portion of the DRX cycle should occupy a smaller proportion of the cycle than the sleep portion, and this means that most data packets (or bursts) will arrive while the UE is asleep, and must be buffered by the network until the next DRX cycle.

If there is buffered data at the start of a DRX cycle, the OnDuration timer must be active long enough for the network scheduler to transfer at least the first of the buffered packets to prevent unrestricted growth of the buffer size. It should not be longer than this – the inactivity timer will keep the UE awake if there are further packets to transmit, and if there is no data a short OnDuration timer setting allows the UE to return to sleep quickly. In a congested network it may be necessary to increase the OnDuration timer setting to allow additional margin for the network scheduler.

Proposal 7: To ensure good power consumption in periods of low data activity, the OnDuration timer setting should be no more than 5% of the DRX cycle duration

If data arrival is evenly distributed, the Inactivity timer should be short in relation to the mean inter-arrival time of packets to reduce the probability of repeated retriggering. However, it should be at least as long as the OnDuration timer setting to ensure that the network scheduler has time to transmit any outstanding buffered packets from the previous sleep period. If data traffic arrives in bursts, the Inactivity timer setting should be long enough to accommodate internal gaps in the burst and ensure that a complete burst is captured before the UE returns to sleep. This automatically ensures that it is short in relation to the mean inter-arrival time of bursts

Proposal 8: To ensure good power consumption in when data activity consists of infrequent packets or bursts, the Inactivity timer setting should be no more than 10% of the DRX cycle duration

The DRX cycle duration should be aligned to the mean inter-arrival time (packets or bursts, depending on the type of data traffic), in a range between 40% and 100% of the mean inter-arrival time with a target setting of 70% of mean inter-arrival time. This gives an even balance between DRX periods that contain data and those that do not, and means that the cycle has some flexibility to adapt to localised changes in data traffic.

If the expected burst duration is long in relation to the DRX cycle, consideration should be given to implementing a short DRX cycle that will cover the duration of the burst. This will generally provide better power consumption than an inactivity timer that remains active for the entire duration of the burst.

Data traffic statistics are not generally available directly to the UE, so this description presupposes that the UE, or the UE manufacturer, will collect statistics on the active applications and estimate a mean inter-arrival time for each in order to generate a preferred DRX cycle for each application. User preferences can set upper limits on the DRX cycle duration to ensure that latency requirements are satisfied.

When multiple applications are running, the preferred DRX cycles can be combined to give a preferred cycle that meets the most demanding requirements

- The long DRX cycle with the shortest period is selected

- The OnDuration timer is configured by the network

- If no applications require a short DRX cycle, the longest of the Inactivity timer settings is selected

- If there are multiple applications with a short DRX cycle, the shortest cycle duration is selected, and ShortCycleTimer is adjusted to match the duration of the longest timer in the preferred set. If this is not possible (ShortCycleTimer > 16), the ShortCycle is increased until the longest timer requirement can be met.

Figure 4 illustrates the effect of UE assisted DRX adaptation. In Table 6 which is quoted from [1], there show 42% and 70% power saving gains that can be realized for FTP/Video and IM traffic models, respectively. We therefore suggest:

Proposal 9: UE assisted DRX adaptation is considered for NR Rel-16 to exploit UE knowledge of running applications to help the gNB to reduce UE power consumption while keeping application response times within acceptable limits.

Figure 4: PDCCH monitoring reduction with UE assisted DRX adaptation

Table 6: Evaluation results of UE assisted DRX adaptation

| Metric | FTP/Video | IM |
| --- | --- | --- |
| Power consumption (relative unit) | 30.30 | 3.26 |
| Power saving ratio (w.r.t. baseline) | 42.09% | 70.06% |
| Latency (ms) | 77.65 | 626.43 |
| Latency increment (w.r.t. baseline) | 30.04% | 323.14% |

In periods of intense data activity (for example, transferring large files in a lightly loaded network), the Inactivity timer will be continually retriggered so that sleep is not possible. If this scenario persists for long periods, there is a risk that the UE will begin to overheat, and must then reduce its power consumption in order to maintain operation within a safe operating temperature range.

One possible method of achieving this is to limit the active period within each DRX cycle. Under current specifications the Inactivity timer can retrigger indefinitely, so the only method of achieving this would be to set the Inactivity timer to zero, and use the OnDuration timer setting to control the active duty cycle. However, this would enforce a fixed duty cycle even when no data was present.

A preferable alternative for NR Rel-16 would be to add a configurable limit to the active DRX period. The limit would be applied following a UE request for overtemperature assistance, and would terminate the DRX cycle activity at the limiting time if this occurred before Inactivity timer expiry.

Proposal 10: A configurable limit on the maximum active period in a DRX cycle should be considered for NR Rel-16 as a mechanism for limiting UE power consumption in cases where overheating assistance is requested.

# Adaptation to DRX Operation without UE Assistance

In this section, the schemes that depends only on the buffer status for UE data in gNodeB side will be introduced. Compared with UE assisted DRX adaptation, these schemes are to reduce dummy PDCCH monitoring with less traffic statistics information exploited. The power saving gain is less than the UE assisted scheme but there can realize faster adaptation to the change in traffic statistics. UE assisted and non-UE-assisted schemes can collaborate to maximize overall UE power saving benefit.

## Adaptation before gNodeB data delivering

For the case DRX period is not matched to data arrival period, there will induce dummy PDCCH monitoring. Wakeup mechanism can be utilized to indicate whether UE needs to monitor the upcoming DRX OnDuration. When UE data arrives at gNodeB buffer, gNodeB can wake up UE to start monitoring data scheduling. An alternative design is to set a larger PDCCH period and adapt to a short period if there is data arrival. The benefit of the later design is no resource overhead of wakeup signalling. In Table 7 and Table 8 quoted from [1], there show the evaluation results of the above two schemes, and one can see similar power saving gains achieved.

Observation 8: Applying wakeup signalling right before DRX OnDuration and applying a longer period for PDCCH before first data scheduling show similar power saving gains within 5% to 20%. But there is additional signalling overhead for the wakeup signalling.

Table 7: Evaluation results of wakeup signalling right before DRX OnDuration

| Metric | FTP/Video | IM | VoIP |
| --- | --- | --- | --- |
| Power consumption (relative unit) | 49.70 | 8.66 | 46.61 |
| Power saving ratio (w.r.t. baseline) | 5.02% | 20.47% | 7.94% |
| Latency (ms) | 63.54 | 156.28 | 14.39 |
| Latency increment (w.r.t. baseline) | 6.41% | 5.56% | 7.22% |

Table 8: Evaluation results of applying 4-slot PDCCH period before first data scheduling

| Metric | FTP/Video | IM | VoIP |
| --- | --- | --- | --- |
| Power consumption (relative unit) | 49.02 | 9.46 | 47.72 |
| Power saving ratio (w.r.t. baseline) | 6.32% | 13.13% | 5.74% |
| Latency (ms) | 62.01 | 149.57 | 13.49 |
| Latency increment (w.r.t. baseline) | 3.85% | 1.03% | 0.52% |

Further optimization on wakeup monitoring occasion regarding periodic/background activities for, e.g. synchronization, CSI and beam-management, is necessary to improve its power saving gain. In Section 6, there proposes the preparation period design that integrates all background activities and wakeup monitoring occasion so that UE can reduce wakeups and increase deeper sleeps when there is no data activity. In Table 9, one can see ~10% additional power saving gain can be realized, which differentiates the wakeup scheme for DRX adaptation.

Observation 9: By optimizing wakeup monitoring occasion regarding periodic/background activities, ~10% additional power saving gain can be realized. The improved wakeup scheme can now show superior power saving gain than the non-wakeup design with longer initial PDCCH period.

Observation 10: For VoIP, applying a constant longer PDCCH period can realize the best power saving gain of 24% saving without wakeup signalling overhead.

Proposal 11: Wakeup signalling with monitoring occasion aggregated with periodic/background activities around SS burst is supported for NR Rel-16

Proposal 12: For VoIP, apply a constant longer PDCCH monitoring period to realize the best trade-off in power saving gain and system impact.

Table 9: Evaluation results of wakeup signalling around SS burst and periodic activities

| Metric | FTP/Video | IM | VoIP | VoIP (Constant PDCCH Period of 4 ms) |
| --- | --- | --- | --- | --- |
| Power consumption (relative unit) | 47.41 | 7.58 | 42.05 | 38.20 |
| Power saving ratio (w.r.t. baseline) | 9.40% | 30.39% | 16.94% | 24.55% |
| Latency (ms) | 66.96 | 161.07 | 18.56 | 15.02 |
| Latency increment (w.r.t. baseline) | 12.14% | 8.80% | 38.30% | 11.92% |

## Adaptation after gNodeB data delivering

When the data inactivity timer setting is not matched to the traffic statistics, there will also induce dummy PDCCH monitoring. To resolve this issue, gNodeB can send a MAC-CE of go-to-sleep indication along with the transmission of the last TB. According to the indication, UE will go sleep for a short period or a long period after the TB transmission or retransmission is successfully decoded. The overhead of Rel-15 go-to-sleep indication via MAC-CE is low when it is transmitted along a PDSCH.

Observation 11: NR Rel-15 can provide efficient go-to-sleep indication via MAC-CE. There is no need of additional L1 go-to-sleep design for Rel-16.

For the power saving benefits, there show the evaluation results with the Rel-15 solution in Table 10. One can have the following observation.

Observation 12: Go-to-sleep indication can realize significant power saving gain ranging from 42% to 56%. However, for a traffic with shorter data mean arrival time, there can cause significant latency increment since a packet arrival after go-to-sleep indication will be delayed by a DRX period. In particular, there can induce 59% latency increment and 12% UE drop rate for VoIP service.

Proposal 13: Go-to-sleep indication is utilized for services that can tolerate long latency.

Table 10: Evaluation results of Rel-15 go-to-sleep indication via MAC-CE

| Metric | FTP/Video | IM | VoIP |
| --- | --- | --- | --- |
| Power consumption (relative unit) | 23.00 | 5.63 | 29.26 |
| Power saving ratio (w.r.t. baseline) | 56.05% | 48.30% | 42.21% |
| Latency (ms) | 106.01 | 162.60 | 21.39 |
| Latency increment (w.r.t. baseline) | 77.54% | 9.84% | 59.39% |

## Design of Preparation Period

### Preparation Period Design Principles

The aim of the preparation period is to enable the UE to be ready to start monitoring the PDCCH at the next monitoring occasion, by providing the opportunity for the UE/gNodeB to perform essential functionalities for link maintenance such as channel tracking, CSI acquisition and beam tracking. Moving those functionalities outside of the active time, allows to shorten the wake-up time and reduce non-needed PDCCH monitoring.

In order for the preparation period to effectively support the different deployment scenario possible in NR, the preparation period should allow:

CSI-RS acquisition and CSI reporting:

CSI-RS acquisition and CSI reporting are required to enable the UE to measure the CSI –for link adaptation/beam tracking- and report it within the preparation period such that the gNodeB is ready to start data scheduling by the start of the PDCCH monitoring. In Rel-15, both the CSI-RS acquisition and CSI reporting are not allowed outside the active time, therefore the preparation period should allow the UE to perform CSI-RS acquisition and CSI reporting outside of the active time.

SRS transmission:

UL based CSI acquisition and beam tracking using the SRS is an effective alternative to the DL based CSI-RS acquisition/CSI reporting that is especially effective in TDD deployments where channel reciprocity is applicable. In Rel-15 [6], SRS transmission cannot take place outside of active time, therefore the preparation period should be used to allow for SRS transmission outside the active time.

BWP switching/SCell activation:

In cases where the BWP switching/SCell activation is needed, for example in order to allow the UE to handle higher throughput on wake-up, it is more beneficial from power and latency perspective to allow for BWP switching and SCell activation during the preparation period.

Observation 13: The preparation period allows to shorten the wake-up time and eliminate non-need PDCCH monitoring.

Proposal 14: Preparation period should be used to support CSI-RS acquisition, CSI and SRS transmission as well as the BWP switch and SCell activation outside the active time.

The following 2 subsections discuss the use of the preparation period concept to support periodic background activities through the background activity window, as well the support of AP-CSI/SRS transmission during the preparation period after BWP switch/SCell activation.

### Aggregation of Background Activity

The support of background activity is required for a good system behavior as discussed in [1][3]. These background activities include SSB/TRS processing, CSI-RS acquisition, SRS transmission and PUCCH transmission, these activities can be used to enable different functionalities such as RRM, synchronization, beam management, CSI acquisition/feedback and power control.

Currently these activities can be spread in time meaning that the UE may need to wake-up at different instances to perform different activities leading to increased power consumption as can be seen in Figure 5 (B). On the other hand, part of the background activities such CSI-RS acquisition, SRS transmission and PUCCH transmission of CSI reports can only take place during the active time (Figure 5 (A)). Therefore, when power saving signal is used to cancel the on-duration then CSI-RS acquisition, SRS transmission and PUCCH transmission of CSI reports background activities cannot be supported.

Observation 14: In the legacy framework, different background activities may require separate wake-ups therefore increasing power consumption

Observation 15: When power saving signal is used to cancel DRX on duration, background activities such CSI-RS acquisition, SRS transmission and PUCCH-based CSI reports transmission cannot be supported

Introducing a background activity window as in Figure 5 (C) where the UE can perform background activities independently of the power saving signal allows to guarantee the good system behaviour.

Furthermore, the background activity window can be used as a tool to aggregate all the background activities around the SSB and/or the power saving signal. Using the background activity window this way will allow to reduce total wake-up time and hence improves the power saving –see Figure 5 (C)-. An additional advantage of performing background activity before power saving signal, is that gNodeB can adapt the coding therefore improving the reliability of the power saving signal itself and reducing resource usage.

In the case of carrier aggregation, the background activity window could be common among cells to align wake-ups between the PCell and the SCells. This would allow to reduce the number of wake-ups and maximize the power saving. Another advantage, is that the UE can perform background activities for all active serving cells and receive the power saving signal and in case only the PCell is required to wake-up then the UE will have time to reconfigure the RF to receive only the PCell during the light sleep time between the background activity window and the on-duration. If the background activity window wasn’t used the UE would have had to activate the RF for all the serving cells, thus limiting the power saving gain from the monitoring of the PCell only.

Figure 5: Total wake-up time for background activities and different DRX designs: Baseline legacy DRX design (A), introducing power saving signal just before on duration (B) and introducing background activity window aggregating all background activities (C).

Table 11 compares the power consumption saving gains achieved by the use of the background activity window to aggregate all the background activities:

    - (A): Baseline legacy DXR design without power saving signal

    - (B): Use of power saving signal to cancel PDCCH monitoring, however periodic CSI-RS acquisition and PUCCH reporting are still maintained for beam management with period = max(DRX, 160ms) as described in [1]

    - (C): Use of the background activity window to aggregate all the background activities and the power saving signal

The results in Table 11, quoted from [1],  show that the use of the background activity window on the top of the power saving signal allow to achieve an additional power consumption reduction of ~10% for IM and VoIP and ~4% for FTP/Video traffic. These additional gains are of a similar order to the power consumption reduction obtained from introducing the power saving signal.

Table 11: Power saving gains when using power saving signal with and without background activity window

|  | Use of Power Saving Signal (B) | Use of Power Saving Signal and background activity window (C) | Additional gain from the use of the background activity window |
| --- | --- | --- | --- |
| FTP/Video (DRX=160ms) | 5% | 9.4% | 4.4 % |
| IM (DRX=320ms) | 20.5% | 30.4% | 9.9 % |
| VoIP (DRX=40ms) | 7.9% | 16.9% | 9 % |

Observation 16: Use of a window for aggregating all the background activities allows to achieve additional power consumption reduction of ~10%

Proposal 15: Support background activity (BA) window around the SS burst and/or the power saving signal where periodic activities like CSI/BM reporting and SRS transmission can occur

In term of resource overhead, the CSI-RS within the BA window can be shared among UEs meaning that the resource overhead for having multiple UEs sharing the same BA window is very small. Similarly, SRS transmission from different UEs can be code multiplexed therefore sharing the same physical resources with minimal overhead.

As for the PUCCH resource overhead, Table 12 shows that the resource overhead per UE for the support of PUCCH transmission with a long PUCCH is very small ranging from 0.003% for short PUCCH format 2 with 16 PRB to 0.02% for long PUCCH format 3 with 16 PRB. Additionally, assuming a BA window per SSB, with 20ms SSB periodicity and only a single UL slot is used for multiplexing the PUCCH, then a 40MHz bandwidth will allow the support of 784 users if the short PUCCH is used and 112 users if the long PUCCH is used.

Table 12: Resource overhead and multiplexing capacity assuming PUCCH transmission (every 160ms) in the case of 40MHz bandwidth

|  | -Format 2- Short PUCCH 16PRB and 1 symbol | -Format 3- Long PUCCH 16PRB and 7 symbol |
| --- | --- | --- |
| Resource overhead per UE | 0.003 % | 0.02 % |
| #UEs that can be multiplexed assuming 1 UL slot per SSB periodicity (20ms) is used | 784 | 112 |

Observation 17:  The background activity window is a resource efficient mechanism for multiplexing different UEs periodic activities

### Support of CSI Acquisition for BWP switching/SCell activation

While the BA window allows for effective support of Periodic BM, CSI acquisition and SRS transmission, however when a BWP switch/SCell activation is triggered by the power saving signal which only takes effect after the switching delay, the network will still require the triggering of aperiodic CSI reporting and/or SRS transmission to acquire the channel state information. Using the Power Saving signal to trigger the BWP switch/SCell activation allows to reduce wake-up time by allowing the BWP switch/SCell activation to take place during the preparation period (see Figure 6). However, the Aperiodic CSI-RS acquisition and CSI feedback as well the SRS transmission will still need to be triggered through an UL grant during active time which leads to an extra wake-up time and non-needed PDCCH monitoring. There is two use cases possible for the BWP switch/SCell activation and the Aperiodic CSI/SRS triggering:

  - Use case 1: BWP switch/ SCell activation and Aperiodic CSI/SRS triggering prior to data transmission:

    - CSI acquisition prior to data scheduling

  - Use case 2: BWP switch/SCell activation and Aperiodic CSI/SRS triggering for link maintenance:

    - CSI acquisition for link maintenance (CSI, BM, …)

    - Needed regularly to ensure that the network maintain a reliable connection and has enough information to decide what BWP switch/SCell activation to trigger for the next data activity

Figure 6: Support of AP-CSI/SRS transmission during active time under Rel-15 constraints for use case 1 (with data activity) and use case 2 (no data activity)

Observation 18: When BWP switch/SCell activation is triggered, extra wake-up time and non-needed PDCCH monitoring is wasted

Observation 19: AP-CSI/SRS transmission are needed during BWP switch/SCell activation for both the preparation of the data scheduling and the link maintenance scenarios

Allowing the PS signal to trigger AP-CSI/SRS transmission during the preparation period (Figure 7) will help reduce the power consumption, by shortening the wake-up time and eliminating the non-needed PDCCH monitoring. Using such a mechanism will help the UE achieve support of both the use case 1 when data activity is required and the use case 2 when regular AP-CSI reporting/SRS Tx triggering is used.

To support the AP-CSI reporting triggering through the PS signal pre-configured PUSCH/PUCCH resources can be used. Such solution will provide the same resource overhead and multiplexing capacity advantages as discussed in Table 12.

The results in Table 13, show that the support of the AP-CSI reporting during preparation period (Figure 7)  allows to achieve power consumption reduction of ~29% for IM, ~9% for FTP/Video and ~3.5% for VoIP traffic. The evaluation assumes that regular AP-CSI for link maintenance is triggered every period = max(DRX, 160ms).

Table 13: Power consumption (in unit) and power consumption saving when AP-CSI reporting/SRS Tx during preparation period is supported

|  | No support of AP-CSI during preparation period (Figure 6) | Support of AP-CSI during preparation period (Figure 7) | Gain due to the support of AP-CSI during preparation period |
| --- | --- | --- | --- |
| FTP/Video (DRX=160ms) | 52.95 | 48.19 | 9 % |
| IM (DRX=320ms) | 11.28 | 8.03 | 28.8 % |
| VoIP (DRX=40ms) | 45.62 | 44.03 | 3.5 % |

Figure 7: Support of AP-CSI/SRS transmission in the preparation period for use case 1 (with data activity) and use case 2 (no data activity)

Proposal 16: Support PS signal triggering of Aperiodic CSI/SRS transmission during the preparation period with and without PDCCH monitoring

Proposal 17: Support AP- CSI transmission during preparation period can be achieved using pre-configured PUSCH/PUCCH resources

# Adaptation to Reduce PDCCH Monitoring

In Section 6, there introduced wake-up and go-to-sleep schemes to reduce PDCCH monitoring. In addition to them, PDCCH monitoring reduction can be realized by adapting PDCCH monitoring period or skipping PDCCH monitoring for a period of time. When there is no data in gNodeB buffer for a UE, gNodeB can signal the UE to apply a longer PDCCH period or skip PDCCH for a given period of time. These schemes allow gNodeB to resume data scheduling after a period of time and can resolve the latency issue of the go-to-sleep scheme.

Observation 20: In addition to wake-up and go-to-sleep, PDCCH monitoring reduction can be realized by adapting PDCCH monitoring period or skipping PDCCH monitoring for a period of time. These schemes can provide better balance between UE power saving and packet latency increment.

The main difference between the two schemes is the signalling overhead. For adapting PDCCH period, a single DCI indication for BWP switching can be utilized. On the other hand, for the PDCCH-skipping scheme to continue skipping PDCCH during data inactivity, repeated DCI indications will be required.

In Table 14 and Table 15, there compare the two schemes that can reduce 7/8 PDCCH monitoring during data inactivity, and the following observation can be checked:

Observation 21: Adapting PDCCH period or repeatedly skipping PDCCH monitoring during data inactivity can realize significant power saving gain ranging from 22% to 33%. Adapting PDCCH scheme is more preferable for the much smaller DCI/signalling overhead for the adaptation.

Table 14: Evaluation of adapting PDCCH period to 8 slots via DCI signalling during data inactivity

| Metric | FTP/Video | IM | VoIP |
| --- | --- | --- | --- |
| Power consumption (relative unit) | 34.89 | 7.54 | 39.28 |
| Power saving ratio (w.r.t. baseline) | 33.33% | 30.76% | 22.42% |
| Latency (ms) | 68.83 | 149.89 | 14.48 |
| Latency increment (w.r.t. baseline) | 15.27% | 1.25% | 7.90% |
| DCI overhead (w.r.t. data DCI) | 6.03% | 43.78% | -1.02% |

Table 15: Evaluation of skipping PDCCH period for 7 slots via DCI signalling during data inactivity

| Metric | FTP/Video | IM | VoIP |
| --- | --- | --- | --- |
| Power consumption (relative unit) | 34.97 | 7.54 | 39.27 |
| Power saving ratio (w.r.t. baseline) | 33.17% | 30.76% | 22.44% |
| Latency (ms) | 68.44 | 149.93 | 14.47 |
| Latency increment (w.r.t. baseline) | 14.62% | 1.28% | 7.82% |
| DCI overhead (w.r.t. data DCI) | 93.91% | 1250.81% | 286.02% |

If eliminating the DCI overhead for the adaptation is targeted, one can utilize a time out mechanism. For example, PDCCH period is adapted after 10-ms data inactivity. In Table 16, one can see similar power saving gains of ~30% and 28% for FTP/Video and IM, respectively, can be realized without DCI overhead. There is smaller power saving gain for VoIP traffic model since its inactivity timer value is small.

Table 16: Evaluation of adapting PDCCH period to 8 slots via 10-ms time-out during data inactivity

| Metric | FTP/Video | IM | VoIP |
| --- | --- | --- | --- |
| Power consumption (relative unit) | 36.66 | 7.79 | 47.35 |
| Power saving ratio (w.r.t. baseline) | 29.94% | 28.47% | 6.48% |
| Latency (ms) | 68.63 | 149.90 | 13.52 |
| Latency increment (w.r.t. baseline) | 14.94% | 1.26% | 0.75% |
| DCI overhead (w.r.t. data DCI) | 0% | 0% | 0% |

Observation 22: Adapting to a longer PDCCH period after a pre-determined time of data inactivity can achieve power saving gain ranging from 6% to 30% without additional DCI signalling overhead.

Proposal 18: Adapting PDCCH period with DCI indication or time-out mechanism is supported

# Joint Power Saving Adaptation by Extended BWP Framework

From the above evaluations of various power saving adaptations, the following schemes can achieve significant power saving gain with limited latency increment or system impact:

- DCI-based BWP switching and BWP time-out with a 20MHz default BWP

- Wakeup signalling aggregated with periodic/background activities around SS burst

- Adapting PDCCH monitoring period after a specified time of data inactivity

- For VoIP, constantly apply a 20MHz BWP of up to 2 MIMO layers and PDCCH monitoring period of 4 or 8 slots can provide the best balance in power saving gain and latency increment.

To optimize the power saving for NR, the following proposal is suggested:

Proposal 19: Rel-16 UE power saving design should consider an adaptation framework that can jointly realize multiple power saving adaptations as well as service optimized adaptation.

BWP based adaptation is one way to realize the joint adaptation. Optimized adaptation per service can also be realized by optimizing one BWP setting for a targeted service. However, the data interruption and the 2-3 ms BWP switching delay can cause data rate loss if applied during a data burst. In this regard, it is desirable to have:

Proposal 20: For NR Rel-16, the following enhancements to BWP-based adaptation are considered:

Enhancement 1: BWP switching is done before data scheduling so as to avoid data interruption

Enhancement 2: Introduce power profile based switching with no data interruption and short switching delay

For Enhancement 1, allowing power saving signal before DRX on duration to trigger BWP adaptation is a feasible solution. The analysis in [3] the power saving designs that can minimize miss probability subject to an acceptable false alarm rate. Based on those results the following proposal is put forward:

Proposal 21: Power saving signal is utilized to trigger BWP switching in addition to triggering PDCCH monitoring for the upcoming DRX OnDuration.

- Note: The trigger occasion should be earlier than DRX start by at least BWP switching delay.

For Enhancement 2, a power profile is defined on top of a BWP configuration. It specifies a set of selections/restrictions on the BWP parameters where changing their values will not cause data interruption. Examples can include at least search space selection, PDCCH monitoring period selection, restriction on the values of K0, K2 and A-CSI offset. Defining multiple power profiles in a BWP can enable fast adaptation of PDCCH monitoring period without data interruption. Compared with defining multiple BWPs with reduced number of different parameters, power profiles can prevent parameters duplication and exploit compact configuration of a small BWP number. Consequently, the following is suggested:

Proposal 22: Power profile for BWP, defined as a set of selections/restrictions on BWP parameters that do not cause data interruption if their values are changed, is introduced for NR Rel-16 to realize efficient power saving adaptation.

- Note: For each BWP, there is a default power profile which takes effect for the active BWP after BWP switching.

To maximize the adaptation benefit, it is also suggested:

Proposal 23: DCI and timer based power profile switching for the active BWP is introduced for NR Rel-16.

To integrate the useful power saving adaptation designs, Figure 8 shows a suggested extension on BWP framework with power saving signal and power profiles for Rel-16. Note that, by further including maximum MIMO layer number as a BWP parameter, the extended BWP framework can realize joint adaptation on BW, MIMO, PDCCH monitoring, and processing time requirements in a unified way.

Figure 8: BWP extension with power saving signal and power profiles to realize joint adaptation

In Figure 9, there illustrates how multiple power saving adaptations can be incorporated. In particular, power saving signalling is used to trigger wake-up as well as BWP switching before DRX OnDuration. Within DRX OnDuration, profile time-out can be utilized to enlarge PDCCH monitoring period and BWP time-out can bring UE to the default BWP of a small bandwidth. For VoIP, constant wakeup to indicate the optimized BWP can lead to the optimized power consumption.

Figure 9: Adaptations before DRX OnDuration, within DRX OnDuration, and for different services

In Table 17 quoted from [1], there evaluates the joint adaptation illustrated in Figure 9 with profile time-out threshold 10 ms and BWP time-out threshold 20 ms. It can be checked that the best power saving gain subject to limited latency increment can be realized.

Observation 23: The best trade-off in power saving gain and latency increment can be realized via joint adaptation with the extended BWP framework.

Table 17: Evaluation results of joint adaptation on BW and PDCCH monitoring

| Metric | FTP/Video Dynamic BWP switching | IM Dynamic BWP switching | VoIP Fixed BWP |
| --- | --- | --- | --- |
| Power consumption (relative unit) | 35.14 | 5.98 | 32.64 |
| Power saving ratio (w.r.t. baseline) | 32.85% | 45.09% | 35.53% |
| Latency (ms) | 71.99 | 164.41 | 15.02 |
| Latency increment (w.r.t. baseline) | 20.57% | 11.06% | 11.92% |
| RU (%) | 32.39 | 1.56 | 0.93 |
| RU increment (w.r.t. baseline) | -1.04% | 1.30% | -5.10% |

# Power saving signal/channel design and comparison

In the previous sections, power saving signal/channel is suggested to trigger adaptations before DRX OnDuration. In RAN1 AH1901 meeting, there encourage to compare following metrics:

Agreements: - Companies are encouraged to consider the following aspects for the purpose of the power saving signal/channel design, - The multiplexing capability - Include total number of UEs supported - The resource overhead in achieving the power saving - The behaviour when miss detection/false alarm happens - Multiplexing with other signals/channels - Number of information bits - Complexity - Power consumption

Among the metrics of interest, performance and multiplexing capability are most important. In this section, we evaluate the performance of existence detection of power saving signal. Regarding the multiplexing, the detection performances for the signal/channel candidates in cases of single and multiple UEs are also evaluated. Since the penalty of missing the power saving signal for UE wakeup can be large, the miss detection rate should be lower than that of PDCCH BLER at SNR value -6 dB. Hence, in the simulations, the target miss detection rate and false alarm rate at -6 dB are 0.1% and 10%, respectively. The link level evaluation assumptions are provided in Appendix.

## Power saving signal/channel for single UE

The following 3 candidates are considered. All candidates are UE/group-specific.

- TRS: 52 PRBs in frequency domain and 2 symbols in time domain. The total RE number is 312.

- SSS-like: 127 continuous REs in frequency.

- UE-specific PDCCH: the RE number is AL*54, where AL is the configured aggregation level. The CORESET has 1 symbol in time domain and the REG bundle size is 6.

In the evaluations, we use simple correlator as a low-powered receiver for signal detection. When the output of correlator is not smaller than the threshold which is aimed for 10% of false alarm rate, the power saving signal exists. Otherwise, the power saving signal does not exist. For PDCCH, the signal generation at network side is the same as normal PDCCH. However, at UE side, instead of using complicated PDCCH decoding process, e.g. including channel estimation and polar decoding, UE just uses the correlator to detect the signal existence based on the uniqueness of PDCCH sequence.

Figure 10 shows the miss detection rates of different sequences at 10% of false alarm rate. The channel type is TDL-C with delay spread 30ns, and the speed is 3km/hr. It can be observed that the detection performance increases with the increase of available detection resources. That is, the miss detection rate is lower for sequence with longer length. However, the benefits of more detection resources decreases as the increase of resources. For example, for PDCCH sequences, the gain from AL1 to AL2 to meet 0.1% of miss detection rate is around 3dB. While the gain from AL8 to AL16 is about 1dB. Additionally, to have efficient resource usage at network side, it is better to have adjustable resource number of power saving signal. Since the power saving signal is transmitted to CONNECTED UE, the CSI reporting is available, network can adjust the resource number of power saving signal based on channel condition of UE. Based on the analysis, PDCCH sequence is a good candidate of power saving signal. Because the design of R15 NR-PDCCH can be reused and the spec effort for power saving signal will be small.

Figure 10. Miss detection rates for TRS, SSS-like and PDCCH sequences at 10% of false alarm rate

According to the discussions, we have observations and proposals as follows.

Observation 24: The detection rate increases with the increase of available resource number, e.g., sequence length, of detection. For example, to achieve 0.1 % of miss detection rate, the SNR values for AL1 and AL2 are 2.9 dB and -0.4 dB, respectively.

Observation 25: The performance enhancement by further increasing the detection resources to meet the same miss detection rate decreases with the increase of resources. For example, for PDCCH sequences, the gain from AL1 to AL2 to meet 0.1% of miss detection rate is about 3dB. While the gain from AL8 to AL16 is about 1dB.

Proposal 24: Link adaptation should be supported for power saving signal to enable efficient resource usage at network side.

Proposal 25: Further study PDCCH sequence as the candidate of UE-specific power saving signal with more channel types.

### Power saving signal for multiple UEs

In this section, we further discuss the candidates of power saving signal when the gNB should wake up a lot of UEs at a time. The candidates including

- TRS: the link performance is same as in Section 9.1 because sequences for different UEs are multiplexed in time or frequency domain.

- SSS-like: the sequences for different UEs are CDMed. The power is evenly split among the sequences on the same resources.

- UE-specific PDCCH: the link performance is same as in Section 9.1 because sequences for different UEs are multiplexed in time or frequency domain.

- GC-PDCCH: it is group-UE-specific power saving signal. The RE number is AL*54, where AL is the configured aggregation level. We assume 2 bits are needed for each UE, therefore, the payload size is determined by the UE number in the same group.

We suppose there are M UEs need to be waken up at a time. So, the required resources for TRS and UE-specific PDCCH are M times the size of single sequence (note: if link adaptation is not considered). The resource overhead would not be acceptable if M is large. When the required number of power saving signal is large at a time, to save signaling overhead and keep the scheduling flexibility at network side, it is better to have CDMed UE-specific signals or group-UE-specific signal/channel. But if M is not large, as proposed in proposal 25, the PDCCH sequence can be considered.

In the following, we compare the detection performance for SSS-like UE-specific power saving signal and GC-PDCCH-like group-UE-specific power saving signal. Again, the target miss detection rate (or BLER for GC-PDCCH) is 0.001 at SNR value -6dB. Assume M is 6, therefore, the power for each sequence is 1/6 for SSS-like signal and the DCI payload size is 12 for GC-PDCCH.

Figure 11 shows that the detection performance for multiplexing 6 sequences is bad. Its performance is worse than GC-PDCCH with AL2 especially in high SNR region even though it occupies more REs. In high SNR region, the interference dominates its performance, but this is not the case for GC-PDCCH-like power saving signal. Compared with the case without sequence multiplexing, the degradation comes from the power split and interference of co-existed sequences. Figure 11 (b) provides the detection performance for TDL-C RS 300ns. It shows that the performance for SSS-like signal with simple correlator is worse in more frequency-selective channel because the low cross-correlation property is further ruined by the fading channel.

Figure 11. Miss detection rate for SSS-like and GC-PDCCH for (a) TDL-C RS30ns (b) TDL-C RS300ns

Observation 26: Compared with the performance without CDM, the sequence detection performance has significant degradation because the power split and interference from co-existed sequences.

Observation 27: Considering the resource overhead and the detection performance, GC-PDCCH is a good candidate of power saving signal when the gNB needs to wake up a group of UEs at a time.

Proposal 26: Considering both resource overhead and scheduling flexibility at network side, both UE-specific and group-UE-specific power saving signals should be supported.

Proposal 27: Further study GC-PDCCH as the candidate of group-UE-specific power saving signal.

# Power saving procedures

The parts of the text highlighted in red in the following text, are changes to the current TR38.840 draft text to allow for support of the proposals in Section 6:

The power saving signal/channel in triggering UE adaptation to DRX operation for further study is that the power saving signal/channel can be configured along the DRX configuration as the indication for UE to wake up from the sleep state. Additional DL RS resources can be considered to assist UE in performing RRM/CSI measurement and channel time/frequency and/or beam tracking, while UL RS can be considered to assist the gNodeB to perform CSI measurement and/or beam tracking.

The power saving signal/channel candidate in triggering UE to achieve reducing PDCCH monitoring for further study  is that the power saving signal/channel can be used to trigger UE to skip the PDCCH monitoring and/or to go to sleep for a period of time.

The power saving signal/channel in triggering UE frequency domain processing adaptation for further study is the power saving signal/channel can be used to trigger the indication of DL and/or UL RS configuration for channel tracking, CSI measurements with possible associated reporting occasion, and beam management for the additional assistance of dynamic switching of BWP or activation of SCell in achieving the power saving gain.  The power saving signal/channel can be used for BWP switching, activation/deactivation of SCell or adaptation of PDCCH monitoring and/or CORESET/search space of PCell/SCell.

For further study, the power saving signal/channel in triggering adaption to the UE processing, such as MIMO configuration/layers, antenna configuration, UE processing time, and background processing can be used as the indication for UE adaptation.  The power saving signal/channel in triggering UE adaptation to the processing is to allow UE in reducing the power consumption by indication of the processing time, such as PDCCH/PDSCH/PUSCH/PUCCH processing or the essential background processing, such as periodic CSI and RRM measurements.

# UE Energy Efficiency Measures and Joint Optimization

For NR design, network energy efficiency and UE energy efficiency are two targets to optimize [4]. For network energy efficiency, the component metric is defined as:

The traffic per second served by a base station (in bits/s)The power consumed by the base station for the traffic (in Joule/s)

Then weighted average over at least 3 load levels and 2 deployment scenarios (coverage limited and capacity limited) can give a quantitative measure.

For UE energy efficiency, there is no explicit metric defined in [4]. However, analogous metric to the network efficiency can be considered. In particular, we first define the component metric as:

The traffic per second received by a UE (in bits/s)The power consumed by the UE for the traffic (in Joule/s)

Then weighted average over different traffic types (e.g., popular user applications), different deployment scenarios, etc, can provide a quantitative metric for UE energy efficiency. While the analogy is feasible, different UE implementation can bias the metric. Also UE power consumption can include non-modem power. Without an objective power consumption measure, the metric is of no use.

The RAN1 study on NR UE power saving, has established a commonly agreed UE power consumption model. The model can estimate UE power consumption based on link configuration and the scheduling realization/log, which can be carried out by either gNodeB or UE. It is clear that the UE power consumption value is objective and representative of UE modem part of power. In this regard, a quantitative UE energy efficiency is feasible by incorporating RAN1 NR UE power consumption model.

Proposal 28: Define UE energy efficiency by weighted average over the component metric:

The traffic per second received by a UE (in bits/s)The power consumption estimated by RAN1 NR UE power model for the traffic (in Joule/s)

Note that optimization of UE energy efficiency is to improve user experience with NR UEs.

To reach better UE energy efficiency, optimizing the link throughput is one factor. Applying proper link configuration is another important factor. Since the optimization should be done per UE per traffic type, the complexity can be high in a centralized method. A possible distributive solution is to incorporate UE assistance where the UE can suggest link configuration with the corresponding improvement in UE energy efficiency. gNodeB can exploit UE intelligence and make the best decision regarding both network and UE energy efficiency metrics. To realize effective communications, several link configurations can be preconfigured so that the UE and the gNodeB can communicate via a compact index instead of rich dimensions of link parameters. In this regard, BWP index based UE feedback and link adaptation can be considered.

Proposal 29: To optimize UE energy efficiency, UE assistance can offload gNodeB burden and realize optimization per traffic type per UE. For efficient communications, BWP index based UE feedback and link adaptation can be further studied.

# Conclusions

In this contribution, the power saving adaptation designs in various domains are investigated. In particular, we have:

Observation 1: Among the non-data-transmission power states, PDCCH-only and sleep states consume the most power.

Proposal 1: NR power saving improvement should aim for:

- Reducing PDCCH-only power consumption by reducing the monitoring amount and the power consumption per monitoring occasion

- Optimizing the sleep power by reducing the wake-up number and increasing deeper sleeps

Observation 2: The average increment in per-packet latency due to A-CSI reporting after switching to a BWP of wider bandwidth is only within 4% and 2% for FTP/Video and IM traffic models, respectively, since only few front packets are delayed. On the other hand, there can realize significant power saving gain of 23% and 24% accordingly.

Proposal 2: For CSI enhancement for BWP in Rel-16, the following two schemes can be considered:

- Allowing UE to perform periodic wideband measurement

- Allowing power saving signal to trigger A-CSI reporting outside DRX OnDuration.

Proposal 3: Support a BWP without PDCCH monitoring for SCell in Rel-16 to allow UE dormancy and realize faster SCell access switching than LTE by exploiting the short BWP switching time.

Proposal 4: In the case of carrier aggregation, some cells are selected as leader cells; each leader cell is associated with a set of follower cells. An adaptation of power consumption configuration in a leader cell initiates a configuration adaptation in the associated follower cells.

Observation 3: Bundled BWP switching for BWP-based SCell power saving can realize 47% power saving on SCell by the effective reduction in dummy PDCCH monitoring.

Proposal 5: Bundled BWP switching for carrier aggregation is considered for NR Rel-16 to realize significant power saving gain by incorporating SCell BWP without PDCCH.

Observation 4: Same-slot scheduling with time gap between PDCCH and PDSCH can realize 15% power saving gain for small-data service, e.g. VoIP. However, it can consume higher power and more system resource for the services with large data size as in, e.g., FTP/Video traffic model.

Observation 5: Cross-slot scheduling shows consistent power saving gain, ranging from 13% to 18%, for different traffic models.

Observation 6: Adapting maximum MIMO layer via BWP switching can save NR UE power (14% and 15% for FTP/Video and IM traffic models, respectively).

Observation 7: Restricting 2x2 MIMO to VoIP can save ~20% UE power with no impact to the system efficiency

Proposal 6: BWP-based MIMO adaptation is considered for NR Rel-16

Proposal 7: To ensure good power consumption in periods of low data activity, the OnDuration timer setting should be no more than 5% of the DRX cycle duration

Proposal 8: To ensure good power consumption in when data activity consists of infrequent packets or bursts, the Inactivity timer setting should be no more than 10% of the DRX cycle duration

Proposal 9: UE assisted DRX adaptation is considered for NR Rel-16 to exploit UE knowledge of running applications to help the gNB to reduce UE power consumption while keeping application response times within acceptable limits.

Proposal 10: A configurable limit on the maximum active period in a DRX cycle should be considered for NR Rel-16 as a mechanism for limiting UE power consumption in cases where overheating assistance is requested.

Observation 8: Applying wakeup signalling right before DRX OnDuration and applying a longer period for PDCCH before first data scheduling show similar power saving gains within 5% to 20%. But there is additional signalling overhead for the wakeup signalling.

Observation 9: By optimizing wakeup monitoring occasion regarding periodic/background activities, ~10% additional power saving gain can be realized. The improved wakeup scheme can now show superior power saving gain than the non-wakeup design with longer initial PDCCH period.

Observation 10: For VoIP, applying a constant longer PDCCH period can realize the best power saving gain of 24% saving without wakeup signalling overhead.

Proposal 11: Wakeup signalling with monitoring occasion aggregated with periodic/background activities around SS burst is supported for NR Rel-16

Proposal 12: For VoIP, apply a constant longer PDCCH monitoring period to realize the best trade-off in power saving gain and system impact.

Observation 11: NR Rel-15 can provide efficient go-to-sleep indication via MAC-CE. There is no need of additional L1 go-to-sleep design for Rel-16.

Observation 12: Go-to-sleep design can realize significant power saving gain ranging from 42% to 56%. However, for the traffic with shorter data mean arrival time, there can cause significant latency increment since a packet arrival after go-to-sleep indication will be delayed by a DRX period. In particular, there can induce 59% latency increment and 12% UE drop rate for VoIP service.

Proposal 13: Go-to-sleep indication is utilized for services that can tolerate long latency.

Observation 13: The preparation period allows to shorten the wake-up time and eliminate non-need PDCCH monitoring.

Proposal 14: Preparation period should be used to support CSI-RS acquisition, CSI and SRS transmission as well as the BWP switch and SCell activation outside the active time.

Observation 14: In the legacy framework, different background activities may require separate wake-ups therefore increasing power consumption

Observation 15: When power saving signal is used to cancel DRX on duration, background activities such CSI-RS acquisition, SRS transmission and PUCCH-based CSI reports transmission cannot be supported

Observation 16: Use of a window for aggregating all the background activities allows to achieve additional power consumption reduction of ~10%

Proposal 15: Support background activity (BA) window around the SS burst and/or the power saving signal where periodic activities like CSI/BM reporting and SRS transmission can occur

Observation 17:  The background activity window is a resource efficient mechanism for multiplexing different UEs periodic activities

Observation 18: When BWP switch/SCell activation is triggered, extra wake-up time and non-needed PDCCH monitoring is wasted

Observation 19: AP-CSI/SRS transmission are needed during BWP switch/SCell activation for both the preparation of the data scheduling and the link maintenance scenarios

Proposal 16: Support PS signal triggering of Aperiodic CSI/SRS transmission during the preparation period with and without PDCCH monitoring

Proposal 17: Support AP- CSI transmission during preparation period can be achieved using pre-configured PUSCH/PUCCH resources

Observation 20: In addition to wake-up and go-to-sleep, PDCCH monitoring reduction can be realized by adapting PDCCH monitoring period or skipping PDCCH monitoring for a period of time. These schemes can provide better balance between UE power saving and packet latency increment.

Observation 21: Adapting PDCCH period or repeatedly skipping PDCCH monitoring during data inactivity can realize significant power saving gain ranging from 22% to 33%. Adapting PDCCH scheme is more preferable for the much smaller DCI/signalling overhead for the adaptation.

Observation 22: Adapting to a longer PDCCH period after a pre-determined time of data inactivity can achieve power saving gain ranging from 6% to 30% without additional DCI signalling overhead.

Proposal 18: Adapting PDCCH period with DCI indication or time-out mechanism is supported

Proposal 19: Rel-16 UE power saving design should consider an adaptation framework that can jointly realize multiple power saving adaptation as well as service optimized adaptation.

Proposal 20: For NR Rel-16, the following enhancements to BWP-based adaptation are considered:

Enhancement 1: BWP switching is done before data scheduling so as to avoid data interruption

Enhancement 2: Introduce power profile based switching with no data interruption and short switching delay

Proposal 21: Power saving signal is utilized to trigger BWP switching in addition to triggering PDCCH monitoring for the upcoming DRX OnDuration.

- Note: The trigger occasion should be earlier than DRX start by at least BWP switching delay.

Proposal 22: Power profile for BWP, defined as a set of selections/restrictions on BWP parameters that do not cause data interruption if their values are changed, is introduced for NR Rel-16 to realize efficient power saving adaptation.

- Note: For each BWP, there is a default power profile which takes effect for the active BWP after BWP switching.

Proposal 23: DCI and timer based power profile switching for the active BWP is introduced for NR Rel-16.

Observation 23: The best trade-off in power saving gain and latency increment can be realized via joint adaptation with the extended BWP framework.

Observation 24: The detection rate increases with the increase of available resource number, e.g., sequence length, of detection. For example, to achieve 0.1 % of miss detection rate, the SNR values for AL1 and AL2 are 2.9 dB and -0.4 dB, respectively.

Observation 25: The performance enhancement by further increasing the detection resources to meet the same miss detection rate decreases with the increase of resources. For example, for PDCCH sequences, the gain from AL1 to AL2 to meet 0.1% of miss detection rate is about 3dB. While the gain from AL8 to AL16 is about 1dB.

Proposal 24: Link adaptation should be supported for power saving signal to enable efficient resource usage at network side.

Proposal 25: Further study PDCCH sequence as the candidate of UE-specific power saving signal with more channel types.

Observation 26: Compared with the performance without CDM, the sequence detection performance has significant degradation because the power split and interference from co-existed sequences.

Observation 27: Considering the resource overhead and the detection performance, GC-PDCCH is a good candidate of power saving signal when the gNB needs to wake up a group of UEs at a time.

Proposal 26: Considering both resource overhead and scheduling flexibility at network side, both UE-specific and group-UE-specific power saving signals should be supported.

Proposal 27: Further study GC-PDCCH as the candidate of group-UE-specific power saving signal.

Proposal 28: Define UE energy efficiency by weighted average over the component metric:

The traffic per second received by a UE (in bits/s)The power consumption estimated by RAN1 NR UE power model for the traffic (in Joule/s)

Note that optimization of UE energy efficiency is to improve user experience with NR UEs.

Proposal 29: To optimize UE energy efficiency, UE assistance can offload gNodeB burden and realize optimization per traffic type per UE. For efficient communications, BWP index based UE feedback and link adaptation can be further studied.

Finally, the extensive summary tables for evaluations results of the power saving adaptation designs can be referred to the concluding section of [1].

# References

- R1-1901803, “Evaluation results for NR power saving designs”, MediaTek, RAN1 #96

- R1-19000752, “Evaluation of UE power saving techniques”, Apple Inc., RAN1 Ad-Hoc Meeting 1901

- R1-1812362, “Triggering adaptation for UE power saving”, MediaTek, RAN1 #95

- 3GPP TR 38.913, “Study on Scenarios and Requirements for Next Generation Access Technologies”, V14.3.0 (2017-06)

- R1-1901806, “DRX Cycle adaptation for NR UE power saving”, MediaTek, RAN1 #96

- TS 38.321, section 5.7: “Discontinuous Reception (DRX)”

SLS simulation assumptions can refer to [1]. The following lists the LLS simulation assumptions.

Table A.1. Evaluation settings in LLS

| Parameters | SSS-like | TRS | PDCCH |
| --- | --- | --- | --- |
| Simulation time | 10000 slots |  |  |
| Antenna configurations | 2TX2RX |  |  |
| SCS | 15 kHz |  |  |
| Bandwidth | 20 MHz |  |  |
| Speed | 3 km/hr |  |  |
| Fading channel | TDL-C RS 30ns |  |  |
| RE number | Continuous 127 REs | 312 REs | 54 - 864 REs for AL1 to AL16 |
| Frequency error | No |  |  |
| Timing error | No |  |  |
| Target false alarm rate | 10% |  |  |