3GPP TSG RAN WG1 Meeting #96 	R1-1901824

Athens, Greece, February 25th– March 1st, 2019

Agenda Item: 7.2.6.1.3

Source: MediaTek Inc.

Title: Enhancements of NR PUSCH for URLLC

Document for: Discussion and Decision

# Introduction

PUSCH enhancement is one of the SID objectives approved in the RAN#80 as:

- PUSCH Enhancements. Study focus on mini-slot level hopping & retransmission/repetition enhancements.

In Rel-15, multiple mechanisms were specified to improve PUSCH reliability and latency like the new MCS table and slot aggregation for reliability and the support of mini-slot (PUSCH mapping type B) where the length of the PUSCH can be 1 through 14 symbols and the starting symbol can be any position within a slot which is good for latency enhancement. However, there is still room for improvement like supporting the intra-slot and inter-slot mini-slot repetition, reducing the DMRS overhead which are needed for URLLC use-cases to improve latency, reliability and spectral efficiency.

In this contribution, we focus on multiple PUSCH enhancements techniques, like PUSCH repetitions, transmit diversity, DMRS overhead reduction, frequency hopping and resource allocation.

# PUSCH repetitions

In RAN1#95, the following agreement has been made:

Agreements:

Support at least one of the following for one TB:

- One UL grant scheduling two or more PUSCH repetitions that can be in one slot, or across slot boundary in consecutive available slots

- One UL grant scheduling two or more PUSCH repetitions in consecutive available slots, with one repetition in each slot with possibly different starting symbols and/or durations

- N (N>=2) UL grants scheduling N PUSCH repetitions on consecutive available slots, with one repetition in each slot, and the i-th UL grant can be received before the end of the PUSCH transmission scheduled by the (i-1)th UL grant.

- FFS the definition of available slots

And in RAN1#AH1901, the following agreement has been made

Agreements:

- Down-select between “mini-slot based repetitions” and “two-segment transmission”, aiming in RAN1#96

- FFS the option of using separate grants to schedule PUSCH repetitions in consecutive available slots

Three different options for PUSCH repetitions were discussed in the previous RAN1 meeting and they are listed in the above agreement. In RAN1#AH1901 [1], Option 1 and Option 2 were shortlisted as potential candidates and Option 3 was de-prioritized for further study.

| Option 1 | Option 2 |
| --- | --- |

Regarding Option 2, although it provides further flexibility compared to Rel-15 in selecting different starting symbols and durations, it has some limitation such as the SLIV indication for the TD resource allocation where crossing the slot boundary will be needed to accommodate this option.

We also think that option (b) could create some issues from TBS calculation and LDPC encoding perspective.

Using all the TD resources of PUSCH1 and PUSCH2 (signalled in the SLIV) for the TBS calculation will certainly improve the range of possible MCSs to be used. However, if the same TBS is used to encode the data in PUSCH1 and PUSCH2 separately then the difference in the effective coding rate between PUSCH1 and PUSCH2 will be very large, especially if the lengths of PUSCH1 and PUSCH2 are very different (e.g. length PUSCH1 = 2OS, length PUSCH2 = 7 OS). We may even have a coding rate larger than 1 which means the data is not decodable.

Also, regarding the LDPC base graph selection, if the UE determines the base graph to be used based on the coding rate from the signalled MCS, the effective coding rate for the two segments may be totally different and may not be optimal. For example, MCS CR < ¼ means BG2 should be selected. But PUSCH1 CR > ¼ then BG2 is not optimal for the encoding.

As explained in the evaluation provided in Figure 1 (based on the settings in Table 1), based on the MCS coding rate (and TBS) BG2 will be selected for the MCS range 1 to 10. However, the effective coding of the first segment is always above the BG2 CR threshold, which means that using BG2 for this segment is not optimal and performance degradation will be expected. Also, it could be observed that for MCS indexes above 9 (results limited to MCS 15 in Figure 1 for simplicity), the coding rate of the first segment goes above 1 and this means the first segment wouldn’t be self-decodable.

Figure 1 : Code Rate vs. MCS index for option 2

To recall the purpose of having multiple repetitions, first it is to reduce the coding rate and hence having more reliable transmission but also to reduce the latency if the gNB decodes successfully the TB from the initial repetitions (see Figure 2). Unfortunately, this cannot be accommodated using Option 2 because multiple repetitions within the same slot is not allowed. Therefore, option 2 is not suitable from latency perspective as explained in Figure 2. Option 1 however shows better capability in accommodating low latency.

Also, the back-to-back PUSCH repetition within a slot is an essential feature to enhance the operation of the configured-grant scheme. Therefore we believe that the same technique should be also adopted and applied for dynamic grant.

Figure 2 : The low latency of back-to-back repetition

Regarding Option 3, scheduling each PUSCH repetition individually will add a lot of signalling overhead. Also, the number of repetitions per slot is still limited to one repetition per slot and this could be restricting in terms of latency.

Therefore, we find Option 1 is the most suitable and most flexible while still meeting the latency and reliability purposes.

Proposal 1: Adopt Option 1 for UL scheduling of PUSCH repetitions.

# Transmit diversity for URLLC UL

UL coverage is an important topic due to the limited UE transmission power. NR has precoding scheme to enhance the performance of UL data, particularly for channels with short delay spread and low mobility. On the other hand, according to our simulation, a transmit diversity scheme, e.g., CDD, outperforms precoding if the targeted error rate is lower than 10-2. In Figure 3, three possible solutions for PUSCH coverage enhancement are evaluated:

- 1TX + 3dB power boost:

- Power gain

- 2TX + precoding:

- Constructive addition at gNodeB

- 2TX + cyclic delay diversity (CDD)

- TX diversity  multipath diversity

In Figure 3, the BLER curve slope of CDD is much better than the precoding slope and show potential advantage if the BLER is 10-3 or lower. For URLLC applications, we operate at very low BLER targets and it is clear there is still room for improvement for the transmission scheme. With CDD delay confined to ~1/5 CP, we observe that precoding can be better than or comparable to CDD for BLER down to 10-2. The slope of CDD can become steeper with CDD delay increased to ~1/3 CP. Therefore, if the BLER slope is the key for the performance at BLER of 10-5, larger CDD delay can be considered to be explored in this topic.

Observation 1: CDD outperforms precoding if the targeted error rate is lower than 10-2.

Proposal 2: Study the possibility of supporting CDD for URLLC to enhance the UL transmit diversity.

Figure 3: NR precoding vs CDD

# DMRS overhead reduction

Supporting mini-slots repetition in the same slot (or even crossing slot boundaries) could be an important Rel-16 URLLC enhancement if adopted. It allows for multiple transmission opportunities of the same TB in the same slot which will further improve the reliability and the latency and helps meeting the URLLC requirements. However, when mini-slots are used (e.g. 2 OS mini-slots) and when allowing repetitions within the same slot, the repetitions use the same DMRS configuration, hence this leads to a very high DMRS overhead which is sub-optimal and not necessarily needed to meet the targeted performance even for fading channels or high mobility.

Figure 4: DMRS Configuration: Type-1/Type-B mapping/single-symbol.

Observation 2: high DMRS overhead is not needed to meet the targeted URLLC performance.

Therefore, DMRS sharing between the repetitions is a possible solution to reduce the DMRS overhead and it consists of removing or reducing the DMRS from certain repetitions. Figure 5 below is a possible solution for DMRS reduction of the scheme in Figure 4.

Figure 5: DMRS sharing across mini-slots.

If DMRS sharing is disabled, DMRS definition and configuration can continue using the specification given in section 6.4.1 of [TS38.211] and additional DMRS can be transmitted according to the scheduling type and the duration as specified in Table 6.4.1.1.3-3 of [4, TS38.211] for frequency hopping disabled and as specified in Table 6.4.1.1.3-6 of [4, TS38.211] for frequency hopping enabled.

If DMRS sharing is enabled, one possible solution to specify the DMRS density and the DMRS OS positions is to consider a group of repetitions as a single mini-slot. For example, two mini-slots of 2 OS each could be considered as a single mini-slot of 4 OS and we can re-use the same specification given in section 6.4.1 on this basis [TS38.211].

Proposal 3: DMRS sharing should be supported for mini-slot repetition.

Proposal 4: To determine the PUSCH DMRS positions when DMRS sharing is enabled for mini-slot repetition, consider a group of repetitions as a single mini-slot and re-use the same specification given in section 6.4.1 on this basis [TS38.211].

If DMRS sharing is enabled, the number of REs for DM-RS per PRB (NDMRSPRBNDMRSPRB) in the scheduled mini-slot will change from one repetition to the next. This could impact the TB size determination and may lead to a different TB size calculated from one repetition to the next unless the used MCS has also changed from one repetition to the next and the new MCS is signalled to the UE. But if there is only a single DCI scheduling the repetitions, only a single MCS can be signalled to the UE. Therefore, we propose that the UE maintains the same TB size calculated from the DCI for all the repetitions scheduled by the same DCI. The effective code rate could change from one repetition to the other to exploit the REs freed from the DMRS overhead.

Therefore, for mini-slot repetition, two possible options for TBS determination:

- Option 1: the UE maintains the same TB size calculated from the DCI for all the repetitions scheduled by the same DCI. Use first repetition to derive NDMRSPRB.

- Option 2: Assume no DMRS in the TBS calculation NDMRSPRBNDMRSPRB.

# DMRS-sharing vs Frequency Hopping

Frequency hopping makes a lot of sense for the mini-slot repetition and can offer significant frequency diversity gain. Frequency hopping across the mini-slot repetitions should be quite similar to the intra-slot and inter-slot hopping supported already in Rel-15. Some additional specifications may be required to support frequency hopping for the mini-slot repetition. But, if frequency hopping is used across mini-slots, the DMRS sharing cannot be used since each mini-slot will occupy different frequency resources as highlighted in Figure 6.

Figure 6: Issue with DMRS sharing across mini-slots when Frequency Hopping is used

Some possible issues could be faced for the support of the DMRS sharing and frequency hopping simultaneously. Frequency hopping will limit the use of DMRS sharing. A possible solution is to have a trade-off between frequency hopping and DMRS sharing.

Observation 3: Supporting frequency hopping will limit the use of DMRS sharing.

Proposal 5: A study of the trade-off between frequency hopping and DMRS sharing is needed.

One possibility is to enable the frequency hopping based on a group of mini-slots transmissions, and within that group allow for the DMRS sharing to be enabled. For example, assume K is the number of mini-slot repetitions within the slot and H is the number of frequency hops. Let’s H1 = mod(K,H)

- The first H1 groups out of total of H groups include ceil(K/H) mini-slots

- The remaining (H - mod(K,H) ) groups include floor(K/H) mini-slots.

E.g. K = 7 (2 OS mini-slots) and H = 3 frequency hops. Then

- The first group out of total of 3 groups include 3 mini-slots

- The remaining 2 groups include 2 mini-slots.

To allow for a good trade-off between frequency hopping and DMRS sharing, the number of hops H should be selected carefully based on the formula above to avoid groups with only one mini-slot in them. To ensure that, additional checks on the values of ceil(K/H) and floor(K/H) could be included and the number of frequency hops H could be re-adjusted to avoid this to occur.

Proposal 6: Mini-slots grouping should be supported to enable frequency hopping and DMRS sharing simultaneously.

Figure 7: DMRS sharing between mini-slots of the same group.

The number of frequency hops was set to 3 in the example above although only two hops are supported for Rel-15 intra-slot and inter-slot PUSCH frequency hopping. In Rel-16 more hops could be supported with the support of mini-slot repetition to improve URLLC reliability and take advantage of frequency diversity.

Proposal 7: Allow for more than 2 frequency hops when mini-slot repetition is used.

In Rel-15, as explained in [1] and [3], when frequency hopping is enabled, it will borrow 1 or 2 bits from the frequency domain RA bit-field to indicate the hopping position.  2 or 4 hopping positions (2 for BWP less than 50 PRBs and 4 for BWP equal or greater than 50 PRBs) are RRC configured to the UE via the higher layer parameter frequencyHoppingOffsetLists and the selection of the hopping position is done dynamically through the DCI by taking 1 or 2 bits from the FD-RA bit-field. This mechanism could be re-applied for the mini-slot repetition scheme where single DCI scheduling the repetitions is signalling one frequency hopping offset from the list and the same offset is applied to all the repetitions. The position of one repetition relative to the previous one could be incremental or decremental. Alternatively, a hopping pattern P could be defined and RRC signalled to the UE (or specified).

Proposal 8: A frequency hopping pattern could be defined if more than two frequency hops are to be supported.

# Signalling of the aggregation type

In Rel-15, when the UE is configured with aggregationFactorDL > 1, the same symbol and RB allocation is applied across the aggregationFactorDL consecutive slots. The UE may expect that the TB is repeated within each symbol allocation among each of the aggregationFactorDL consecutive slots. Aggregation factor 1, 2, 4 or 8 is semi-statically configured separately (i.e. not part of table). In Rel-15, the number of repetitions for data corresponds to the L1 parameter “aggregation-factor-DL” (see [3]) and when this field is absent the UE applies the value 1, which means the repetition is disabled. The UE is RRC configured with aggregationFactorDL for DL and aggregationFactorUL for UL.

In Rel-16, mini-slot repetition intra-slot and inter-slot could be introduced. This mechanism is different to the Rel-15 mini-slot/slot aggregation. Therefore, the differentiation between these two mechanisms should be captured in the specs and signalled to the UE.

To indicate to the UE the mini-slot repetition, Rel-15 aggregationFactor could be re-used for the mini-slot repetition. A new RRC (or dynamic) parameter could be defined to differentiate between the Rel-15 aggregation and the Rel-16 mini-slot repetition. However, with this option, the new Rel-16 repetition factor and Rel-15 aggregationFactor couldn’t be enabled simultaneously since one single RRC flag is used to switch between them.

Proposal 9: Differentiation between Rel-15 mini-slot/slot aggregation and Rel-16 mini-slot repetition should be specified.

Regarding crossing the slot boundary, the repetitions should be allowed to cross the slot boundary.

# PUSCH Resource Allocation with Frequency Hopping

As explained in 38.212 [1], for Resource Allocation type 1, the RB assignment information indicates to a scheduled UE a set of contiguously allocated localized or distributed virtual RBs within the active carrier bandwidth part of size NBWPsize. A downlink type 1 resource allocation field consists of a Resource Indication Value (RIV) corresponding to a starting virtual resource block RBstart and a length in terms of contiguously allocated resource blocks LRBs.

The maximum size of the UL BWP’s BW is 275RBs and 16 bits RIV are enough to cover all possible starting RBs and RB lengths. log2275*(275+1)/2=16 bits. But when frequency hopping is enabled, then hopping takes 2 bits from frequency domain allocation bits to indicate hopping position. The remaining 14 bits are used to represent the frequency allocation. Then, the maximum RIV using 14 bits is 0x3FFF  16383.

Hence, as shown in Figure 8, when using 14 bits for RIV value and BWP’s BW is 275, we can have allocation sizes: [1, 60] & [218, 275] resource blocks only.

Observation 4: When frequency hopping is enabled and when using 14 bits for RIV value and BWP’s BW is 275, we can have allocation sizes: [1, 60] & [218, 275] resource blocks only.

There is clearly a concern about the FD-RA due to the lost bits. The highlighted issue will limit the size and the position of the allocation when Frequency hopping is enabled. This may not be ideal for URLLC which requires in general large frequency allocation and more flexibility to meet the reliability and the latency requirements.

Figure 8 : BWP’s BW with size > 60RBs and < 218RBs Not possible to allocate

To tackle this issue, we propose to use coarse step sizes for the RB length and the RB start when the frequency hopping is enabled. The step size Ss could be used for the RB start and step size  SL could be used for the RB length. It is desirable that step sizes Ss and SL fulfil certain conditions to avoid holes in the spectrum and inefficient spectrum usage. For example to avoid holes in the spectrum, it is desirable that Ss and SL verify the following equation:

SL = m . SS      where        m∈1,2,3,4⋯  where m is a positive integer.

Proposal 10: Use coarse step sizes for the RB length and the RB start when the frequency hopping is enabled. Step sizes Ss and SL  should fulfil certain conditions to avoid holes in the spectrum and inefficient spectrum usage.

Step size could depend on the number of bits allocated for frequency hopping.  E.g. smaller step for 1 bit DCI FH and coarser step when 2 bits for FH.

# Conclusion

In this contribution, we proposed multiple PUSCH enhancements. Based on the discussions and the analysis, we have the following observations and proposals:

Observation 1: CDD outperforms precoding if the targeted error rate is lower than 10-2.

Observation 2: high DMRS overhead is not needed to meet the targeted URLLC performance.

Observation 3: Supporting frequency hopping will limit the use of DMRS sharing.

Observation 4: When frequency hopping is enabled and when using 14 bits for RIV value and BWP’s BW is 275, we can have allocation sizes: [1, 60] & [218, 275] resource blocks only

Proposal 1: Adopt Option 1 for UL scheduling of PUSCH repetitions.

Proposal 2: Study the possibility of supporting CDD for URLLC to enhance the UL transmit diversity.

Proposal 3: DMRS sharing should be supported for mini-slot repetition.

Proposal 4: To determine the PUSCH DMRS positions when DMRS sharing is enabled for mini-slot repetition, consider a group of repetitions as a single mini-slot and re-use the same specification given in section 6.4.1 on this basis [TS38.211].

Proposal 5: A study of the trade-off between frequency hopping and DMRS sharing is needed.

Proposal 6: Mini-slots grouping should be supported to enable frequency hopping and DMRS sharing simultaneously.

Proposal 7: Allow for more than 2 frequency hops when mini-slot repetition is used.

Proposal 8: A frequency hopping pattern could be defined if more than two frequency hops are to be supported.

Proposal 9: Differentiation between Rel-15 mini-slot/slot aggregation and Rel-16 mini-slot repetition should be specified.

Proposal 10: Use coarse step sizes for the RB length and the RB start when the frequency hopping is enabled. Step sizes Ss and SL  should fulfil certain conditions to avoid holes in the spectrum and inefficient spectrum usage.

# References

- RAN1 #AH1901 chairman notes

- 3GPP TS38.212 v15.2.0. “NR; Multiplexing and channel coding”, 2018-06.

- 3GPP TS 38.213, “NR; Physical layer procedures for control (Release 15)”, V15.0.0.

# Appendix A

| Parameter | Value |
| --- | --- |
| BW size | 50 RBs |
| First Segment Length | 2 OS |
| Second Segment Length | 7 OS |
| Number of REs for DMRS per PRB in the allocated duration of the first Segment | 6 |
| Number of REs for DMRS per PRB in the allocated duration of the first Segment | 6 |
| MCS Table | MCS index table 2 for PUSCH with transform precoding and 64QAM (Table 6.1.4.1-2 [TS 38.214]) |
| Overhead configured by higher layer parameter xOverhead in PUSCH-ServingCellConfig | 0 |
| Number of Layers | 1 |

Table 1 : Settings for TBS determination and Code Rate calculation