3GPP TSG RAN WG1 Meeting #96                                                                 R1-1901837

Athens, Greece, 25th February – 1st March,  2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | Draft | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on DMRS for configured grant PUSCH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-02-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For grant based PUSCH transmission, two higher layer parameters dmrs-UplinkForPUSCH-MappingTypeA and dmrs-UplinkForPUSCH-MappingTypeB are configured for PUSCH mapping type A and type B respectively. Then, DMRS configurations are independent for PUSCH mapping type A and type B. Since PUSCH mapping type A and type B are dynamically indicated by DCI, using two separate higher layer parameters are beneficial if the DMRS requirements of PUSCH mapping type A and type B are different. However, for grant free PUSCH transmission, there is only one higher layer parameter cg-DMRS-Configuration for DMRS configuration. It is unclear whether this parameter is for PUSCH mapping type A or mapping type B. The best way is to introduce one more higher layer parameter since the PUSCH mapping type could be also dynamically changed by the activation DCI for configured grant type 2. Then cg-DMRS-Configuration and the new one can be used for configured grant PUSCH mapping type A and mapping type B respectively. This is more flexible while the solution has impact on RRC signaling. Another solution is to clarify that the higher layer parameter cg-DMRS-Configuration is applicable for both PUSCH mapping type A and mapping type B in 38.214 or 38.331. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that DMRS configuration is determined by the higher layer parameter cg-DMRS-Configuration for both PUSCH mapping type A and PUSCH mapping type B |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | For grant based PUSCH transmission, it is clear that two higher layer parameters dmrs-UplinkForPUSCH-MappingTypeA and dmrs-UplinkForPUSCH-MappingTypeB are configured for PUSCH mapping type A and type B respectively. But for grant free PUSCH transmission, it is unclear that the only higher layer parameter cg-DMRS-Configuration is for PUSCH mapping type A or mapping type B. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

< Unchanged parts are omitted >

6.1.2.3	Resource allocation for uplink transmission with configured grant

When PUSCH resource allocation is semi-statically configured by higher layer parameter configuredGrantConfig in BWP-UplinkDedicated information element, and the PUSCH transmission corresponding to a configured grant, the following higher layer parameters are applied in the transmission:

-	For Type 1 PUSCH transmissions with a configured grant, the following parameters are given in configuredGrantConfig:

-	The higher layer parameter timeDomainAllocation value m provides a row index m+1 pointing to an allocated table, indicating a combination of start symbol and length and PUSCH mapping type, where the table selection follows the rules for the UE specific search space, as defined in sub-clause 6.1.2.1.1;

-	Frequency domain resource allocation is determined by the higher layer parameter frequencyDomainAllocation according to the procedure in Subclause 6.1.2.2 for a given resource allocation type indicated by resourceAllocation;

-	The IMCS is provided by higher layer parameter mcsAndTBS;

-	Number of DM-RS CDM groups, DM-RS ports, SRS resource indication and DM-RS sequence initialization are determined as in Subclause 7.3.1.1 of [5, TS 38.212], and the antenna port value, the bit value for DM-RS sequence initialization, precoding information and number of layers, SRS resource indicator are provided by antennaPort, dmrs-SeqInitialization, precodingAndNumberOfLayers, and srs-ResourceIndicator respectively;

-	When frequency hopping is enabled, the frequency offset between two frequency hops can be configured by higher layer parameter frequencyHoppingOffset.

-	For Type 2 PUSCH transmissions with a configured grant: the resource allocation follows the higher layer configuration according to [10, TS 38.321], and UL grant received on the DCI.

-  DMRS configuration is determined by the higher layer parameter cg-DMRS-Configuration for both PUSCH mapping type A and PUSCH mapping type B.

The UE shall not transmit anything on the resources configured by configuredGrantConfig if the higher layers did not deliver a transport block to transmit on the resources allocated for uplink transmission without grant.

A set of allowed periodicities P are defined in [12, TS 38.331].

< Unchanged parts are omitted >