3GPP TSG RAN WG1 Meeting #96	R1-1901846

Athens, Greece, February 25 - March 1, 2019

Agenda Item:	6.2.4.1

Source:	Shanghai Jiao Tong University

Title:	Performance of Non-Uniform Constellations for LTE-Based 5G Terrestrial Broadcast

Document for:	Discussion

Introduction

In RAN#80, a study item about 5G requirements for eMBMS was approved [1]. In RAN1#94bis, the requirements were analyzed and related simulation assumptions were discussed [2]. In RAN1#95, several preliminary simulation results were discussed, and the simulation assumptions were modified further [3].

In this contribution, we discuss the study of Non-Uniform Constellations (NUCs) which provide better performance than traditional uniform Quadrature Amplitude Modulation (QAM) constellations in both AWGN and TDL channels. We show results of link-level simulations of NUCs in this contribution.

Non-Uniform Constellations

- 2.1 Overview

Uniform 16QAM and 64QAM are shown in Figure 1. As shown in the figure, the constellation symbols in QAM are equally spaced in the I-Q plane. Uniform QAMs with Gray-coded labels are near-optimum constellations for uncoded communications [4] and they also minimize demapping complexity at the receiver. QAM can be demapped by using two Pulse Amplitude Modulation (PAM) demappers.

While coding is applied in the coded communication, better performance can be achieved by NUCs. Since the design process of NUCs will consider the effect of coding, interleaving and label assignment together. NUCs can be divided as one-dimensional NUCs (1D-NUCs) and two-dimensional NUCs (2D-NUCs), as 1D-64UNC and 2D-64NUC shown in Figure 2. 1D-NUCs keep the squared shape while designing distances between the constellation symbols. Thus, the demapping complexity of 1D-NUCs can be reduced by using two Non-uniform PAM demappers. 2D-NUCs are designed without constraint on shape in the I-Q plane, so 2D-NUCs bring more performance gain with higher demapping complexity.

Figure 1: Uniform 16QAM and 64QAM

Figure 2: Non-Uniform Constellations (NUCs): (a)1D and (b)2D

Figure 3: NUCs optimized at (a)(b) 6.75dB, (c)(d) 8.65dB, (e)(f)10.7dB, (g)(h) 12.6dB

- 2.2 Optimization of NUCs

In the designing process of NUCs, the effect of coding, interleaving and label assignment will be taken into account together. The shape of NUC is optimized based on the operating range of the coding. The NUCs used for the simulations in this contribution are optimized at SNR = 6.75dB (Figure3(a) and (b)), 8.65dB (Figure3(c) and (d)), 10.7dB (Figure3(e) and (f)) and 12.6dB (Figure3(g) and (h)). The gain of NUCs in performance over uniform constellations of the same order is known as shapping gain. The shaping gains are, in general expected to increase with increasing order of NUCs.

Performance Evaluations

Performance evaluations were carried out using 1D-NUCs and 2D-NUCs optimized at SNR = 6.75dB (Figure3(a) and (b)), 8.65dB (Figure3(c) and (d)), 10.7dB (Figure3(e) and (f)) and 12.6dB (Figure3(g) and (h)). Simulation parameters are summarized in Table 1.

Table 1: parameters for Link-Level simulation

| Parameter | Value |
| --- | --- |
| System bandwidth | 10 MHz |
| Carrier frequency | 700 MHz |
| CP/Numerology | Extended |
| Channel speed | 3km/h (Indoor) |
| Effective Code Rate (ECR) | 0.4785; 0.6016; 0.4551; 0.5537 |
| Modulation | Uniform 16QAM, 64QAM and 1D-NUCs and 2D-NUCs optimized at 6.75dB, 8.65dB, 10.7dB and 12.6dB |
| FEC | LTE Turbo code and rate matching |
| Channel | TDL-A / AWGN |
| Number of antennas | 1Tx, 1Rx |

Figure 4 compares performance in AWGN for Uniform 64QAM and NUCs (including 1D-NUCs and 2D-NUCs) optimized at 6.75dB, 8.65dB, 10.7dB and 12.6dB.

Figure 4: Performance of NUCs in AWGN

Observation 1: Shaping gain in general increases with the order of the NUC. In specific, 2D-16NUC provides about 0.1dB shaping gain when 2D-64NUC can provide 0.5dB shaping gain.

Observation 2: 2D-NUC provides more shaping gain than 1D-NUC.

Observation 3: There is significant shaping gain (about 0.5dB) for each 2D-64NUC around the operating point for which it was optimized.

Figure 5 compares the performance in TDL-A channel [5] of 64NUCs (including 1D-64NUCs and 2D-64NUCs) optimized at 10.7dB and 12.6dB.

Figure 5: Performance of NUCs in TDL-A channel

Observation 3: 64NUCs maintains a positive shaping gain in TDL channels.

Conclusions

Based on the simulation results, the following observations are provided:

Observation 1: Shaping gain in general increases with the order of the NUC. In specific, 2D-16NUC provides about 0.1dB shaping gain when 2D-64NUC can provide 0.5dB shaping gain.

Observation 2: 2D-NUC provides more shaping gain than 1D-NUC.

Observation 3: There is significant shaping gain (>0.5dB) for each 2D-64NUC around the operating point for which it was optimized.

Observation 4: 64NUCs maintains a positive shaping gain in TDL channels.

From these observations, we would like to propose as follows:

Proposal 1: RAN1 should consider the use of NUC for LTE-Based 5G Terrestrial Broadcast/ LTE ENTV.

- References

RP-181706 (revision of RP-181342), “Revised SID on LTE-based 5G Terrestrial Broadcast,” Qualcomm Incorporated, RAN#81, Gold Coast, Australia, Sep 10- 13, 2018.

RAN1#94bis Chairman notes, Chengdu, China.

R1-1813773, “Chairman's notes of AI 6.2.4 Study on LTE-based 5G Terrestrial Broadcast,” Ad-hoc chair (Ericsson), RAN1#95, Spokane, USA, Nov 12- 16, 2018.

G. Foschini, R. Gitlin, and S. Weinstein, “Optimization of two-dimensional signal constellations in the presence of Gaussian noise,” IEEE Trans. Commun., vol. 22, no. 1, Jan. 1974.

3GPP TR 38.901, “Study on channel model for frequencies from 0.5 to 100 GHz”