3GPP TSG RAN WG1 #96			R1-1901898

Athens, Greece, 25th February – 1st March, 2019

Agenda Item:	7.2.4.1.4

Source:	AT&T

Title:	Resource allocation mechanism

Document for:	Discussion/Approval

# Introduction

In RAN1 Adhoc 1901, the following has been agreed, in this paper, we give our evaluation results on resource allocation mode 2d in addition to the procedures on different aspects of group operation.

- Agreements:

At least for the purpose of evaluation, in Mode-2(d), at least for group operation, a member UE transmits on resources configured by another UE (S-UE) within the same group

  - High layer signaling is assumed between S-UE and a member UE

# Performance evaluation on resource allocation mode 2d.

## Coverage Extension Gain

When network configures UE with resource pool, it can only reach the UE within network’s coverage. However, if S-UE can help to configure resource pool for member UE, effectively the coverage of network is expanded. In this section, we setup 3 scenarios showing percentage of additional coverage resource allocation mode 2d can bring to the cellular network. When a UE observed a geometry higher than -5dB, this UE is in coverage.

| Parameter | Values |
| --- | --- |
| Sidelink Frequency | 6GHz |
| Cellular Frequency | 2GHz |
| Geometry threshold to receive RRC configuration | -5dB |
| Simulation Environment | Urban Macro |
| Cellular Base-station ISD | 1732, 5000 |
| Number of Tx/Rx Antenna elements | 2Tx/4Rx |
| BS transmission Power | 44dBm |
| UE transmission Power | 23dBm |

Baseline: UE receives configuration from gNBs only.

Mode-2d: UE receives configuration either from gNB or from another in-coverage UE.

Scenario-1: Good cellular coverage, Urban Macro with 1732m ISD.

Scenario-2: Limited cellular coverage, Rural Area, Macro with 5000m ISD.

Scenario-3: Minimal cellular coverage, Rural Area, coverage boarder. (Single 3-sectors base-station deployed in a large area).

Table.1 Coverage extension gain from resource allocation mode 2d

| SINR threshold= -5dB | Good coverage | Limited coverage | Minimal coverage |
| --- | --- | --- | --- |
| Coverage gain from 2d. | No gain. (all UEs are covered by cellular) | 17% | 65% |

## Resource Coordination Gain

In this section, we evaluate the performance gain from resource coordination. We setup a platoon in urban scenario and then evaluate the communication quality between two UEs in the platoon. The platoon location is randomly selected. Detail parameters follow the urban grid scenario(6Ghz) in [1].

| Parameter | Values |
| --- | --- |
| Sidelink Frequency | 6GHz |
| Sidelink Bandwidth | 10Mhz |
| Total number of channels | 6. UE select channel using below 3 schemes |
| Simulation Environment | Urban Grid (TR37.885) |
| Number of Tx/Rx Antenna elements | 2 |
| UE transmission Power | 23dBm |

Scheme-1: UE selects resource randomly.

Scheme-2: UE senses all channels in the CC and select the one with least RSSI.

Scheme-3: Dedicated resource assigned to platoon UEs (other UEs in the same cell are prohibiting to use). UEs from neighbor cell may still select this resource.

It is observed that both sensing-based and coordination-based schemes showed obvious gain over random selection. In addition, coordinated resource allocation has better performance than sensing based when the platoon size (distance between Tx and Rx UE in the platoon) is relatively small. When platoon size is large, e.g. more than 200m, it requires larger coordination area (several neighbor cells to participate) otherwise there is no performance gain over sensing-based resource selection (scheme-2).

- Proposal: NR sidelink supports a member UE transmits on resources configured by another UE (S-UE) within the same group

# Conclusion

In this contribution, we showed the performance benefit on the resource allocation 2d. It is observed that resource allocation mode 2d can enable coverage extension as well coordinate the resource utilization in a certain area to ensure the quality of service. Based on the simulation results, we have the following proposal:

- Proposal: NR sidelink supports a member UE transmits on resources configured by another UE (S-UE) within the same group

# Reference

[1]  	RP-181480   	New SID: Study on NR V2X

[2]	R1-1806986	Autonomous topology management, AT&T