Appendix B: TR skeleton for section 8

Evaluation Results of NR Positioning

Editor’s Note: To be determined. Evaluate physical layer design options, measurements, and/or any additional impacts or enhancements, as applicable per technology, for RAT-dependent and RAT-independent positioning systems, including suitable frequencies and signals.

Downlink evaluations

System simulations for Scenario 1 – Indoor Open Office

Results from [R1-1901983]

--------------------- Start TP ------------------------------

The parameters corresponding to the results are listed in  table 1 below.

Table 1 Parameters for Downlink evaluations in Scenario 1

| Parameter | CATT |
| --- | --- |
| Channel model (baseline, otherwise state any modifications) | According to 3GPP TR 38.901 (Indoor Open Office) |
| Carrier frequency | 4GHz |
| Subcarrier spacing | 15KHz |
| Reference Signal Transmission Bandwidth | 5MHz, 50MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | PRS in LTE |
| Reference signal (type of sequence, number of ports, …) | Gold sequence 1 port |
| Number of sites | 57 for total sites, 6 used for OTDOA |
| Number of symbols used per occasion | 8 |
| number of occasions used per positioning estimate | 1 |
| Power-boosting level | 6dB |
| Uplink power control (applied/not applied) | Not applied |
| interference modelling (ideal muting, or other) | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | [R1-1901983], No super resolution, and no interference cancellation |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | Chan's algorithm "A simple and efficient estimator for Hyperbolic location", Y.T. Chan and K.C. Ho, IEEE transactions on signal processing, vol.42, no. 8, August 1994. |
| Network synchronization assumptions | Perfect synchronization. |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Wide Tx beam without beam sweeping is assumed. No Rx beam sweeping. |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Tx: 64 antenna elements Rx: 4 antenna elements codebook: 64*1 precoder used to generate widebeam |
| Additional notes, if any |  |

The results corresponding to the Indoor open office scenario are provided below

Table 2 results for downlink methods evaluations of Scenario 1 – Indoor Open Office (editor’s note: white columns are agreed error percentile, grey columns are examples of additional results)

| Percentile | 10 | 20 | 30 | 40 | 50 | 60 | 67 | 70 | 80 | 90 | 95 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [5Mhz: OTDOA ] |  |  |  |  | 13.1 |  | 17.1 |  | 22.0 | 28.9 |  |
| [50Mhz: OTDOA ] |  |  |  |  | 7.8 |  | 10.5 |  | 13.2 | 17.5 |  |

-------------------- End TP ------------------------------

System simulations for Scenario 2 - Umi

Results from [R1-1901983]

--------------------- Start TP ------------------------------

The parameters corresponding to the results are listed in  table 1 below.

Table 1 Parameters for Downlink evaluations in Scenario 2

| Parameter | CATT |
| --- | --- |
| Channel model (baseline, otherwise state any modifications) | According to 3GPP TR 38.901 (UMi Street Canyon) |
| Carrier frequency | 4GHz |
| Subcarrier spacing | 15KHz |
| Reference Signal Transmission Bandwidth | 5MHz, 50MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | PRS in LTE |
| Reference signal (type of sequence, number of ports, …) | Gold sequence 1 port |
| Number of sites | 57 for total sites, 6 used for OTDOA |
| Number of symbols used per occasion | 8 |
| number of occasions used per positioning estimate | 1 |
| Power-boosting level | 6dB |
| Uplink power control (applied/not applied) | Not applied |
| interference modelling (ideal muting, or other) | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | [R1-1901983], No super resolution, and no interference cancellation |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | Chan's algorithm "A simple and efficient estimator for Hyperbolic location", Y.T. Chan and K.C. Ho, IEEE transactions on signal processing, vol.42, no. 8, August 1994. |
| Network synchronization assumptions | Perfect synchronization. |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Wide Tx beam without beam sweeping is assumed. No Rx beam sweeping. |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Tx: 64 antenna elements Rx: 4 antenna elements codebook: 64*1 precoder used to generate widebeam |
| Additional notes, if any |  |

The results corresponding to the UMi Street Canyon scenario are provided below

Table 2 results for downlink methods evaluations of Scenario 2 –UMi (editor’s note: white columns are agreed error percentile, grey columns are examples of additional results)

| Percentile | 10 | 20 | 30 | 40 | 50 | 60 | 67 | 70 | 80 | 90 | 95 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [5Mhz: OTDOA ] |  |  |  |  | 25.9 |  | 31.9 |  | 38.5 | 49.9 |  |
| [50Mhz: OTDOA ] |  |  |  |  | 13.6 |  | 17.4 |  | 22.5 | 27.5 |  |

-------------------- End TP ------------------------------

System simulations for Scenario 3 - Uma

Results from [R1-1901983]

--------------------- Start TP ------------------------------

The parameters corresponding to the results are listed in  table 1 below.

Table 1 Parameters for Downlink evaluations in Scenario 3

| Parameter | CATT |
| --- | --- |
| Channel model (baseline, otherwise state any modifications) | According to 3GPP TR 38.901 (UMi Street Canyon) |
| Carrier frequency | 4GHz |
| Subcarrier spacing | 15KHz |
| Reference Signal Transmission Bandwidth | 5MHz, 50MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | PRS in LTE |
| Reference signal (type of sequence, number of ports, …) | Gold sequence 1 port |
| Number of sites | 57 for total sites, 6 used for OTDOA |
| Number of symbols used per occasion | 8 |
| number of occasions used per positioning estimate | 1 |
| Power-boosting level | 6dB |
| Uplink power control (applied/not applied) | Not applied |
| interference modelling (ideal muting, or other) | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | [R1-1901983], No super resolution, and no interference cancellation |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | Chan's algorithm "A simple and efficient estimator for Hyperbolic location", Y.T. Chan and K.C. Ho, IEEE transactions on signal processing, vol.42, no. 8, August 1994. |
| Network synchronization assumptions | Perfect synchronization. |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Wide Tx beam without beam sweeping is assumed. No Rx beam sweeping. |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Tx: 64 antenna elements Rx: 4 antenna elements codebook: 64*1 precoder used to generate widebeam |
| Additional notes, if any |  |

The results corresponding to the UMA scenario are provided below

Table 2 results for downlink methods evaluations of Scenario 3 –UMA (editor’s note: white columns are agreed error percentile, grey columns are examples of additional results)

| Percentile | 10 | 20 | 30 | 40 | 50 | 60 | 67 | 70 | 80 | 90 | 95 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [5Mhz: OTDOA ] |  |  |  |  | 46.3 |  | 69.7 |  | 101.6 | 151.2 |  |
| [50Mhz: OTDOA ] |  |  |  |  | 16.6 |  | 27.4 |  | 44.3 | 136.5 |  |

-------------------- End TP ------------------------------

Uplink evaluations

- 8.2.1 System simulations for Scenario 2 - Umi

Results from [R1-1901983]

--------------------- Start TP ------------------------------

The parameters corresponding to the results are listed in  table 3 below.

Table 3 Parameters for Downlink evaluations in Scenario 2

| Parameter | CATT |
| --- | --- |
| Channel model (baseline, otherwise state any modifications) | According to 3GPP TR 38.901 (UMi Street Canyon) |
| Carrier frequency | 4GHz |
| Subcarrier spacing | 15KHz |
| Reference Signal Transmission Bandwidth | 5MHz, 50MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | NR SRS, Comb=4 |
| Reference signal (type of sequence, number of ports, …) | ZC sequence 1 port |
| Number of sites | 57 for total sites, 6 used for UTDOA |
| Number of symbols used per occasion | 4 |
| number of occasions used per positioning estimate | 1 |
| Power-boosting level | 6 dB |
| Uplink power control (applied/not applied) | Not applied |
| interference modelling (ideal muting, or other) | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | [R1-1901983], No super resolution, and no interference cancellation |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | Chan's algorithm "A simple and efficient estimator for Hyperbolic location", Y.T. Chan and K.C. Ho, IEEE transactions on signal processing, vol.42, no. 8, August 1994. |
| Network synchronization assumptions | Perfect synchronization. |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Not applied |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Not applied |
| Additional notes, if any |  |

The results corresponding to the UMi Street Canyon scenario are provided below

Table 2 results for downlink methods evaluations of Scenario 2 –UMi (editor’s note: white columns are agreed error percentile, grey columns are examples of additional results)

| Percentile | 10 | 20 | 30 | 40 | 50 | 60 | 67 | 70 | 80 | 90 | 95 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [5Mhz: UTDOA ] |  |  |  |  | 25.4 |  | 44.8 |  | 63.6 | 82.5 |  |
| [50Mhz: UTDOA ] |  |  |  |  | 20.1 |  | 29.5 |  | 34.2 | 44.1 |  |

-------------------- End TP ------------------------------