3GPP TSG RAN WG1 Meeting #96		     R1-1901983

Athens, Greece, 25th February – 1st March, 2019

Source:	CATT

Title:	Updated System-level Performance Evaluation for NR Positioning

Agenda Item:	7.2.10.1.4

Document for:	Discussion and Decision

Introduction

In this contribution, we provide our updated system-level simulation results for supporting NR downlink positioning (e.g., OTDOA) base on the simulation evaluation scenarios and parameters for NR positioning techniques defined in TR 38.855 [1].

Simulation assumptions

The following three scenarios are defined for RAT-dependent positioning techniques for NR positioning study in TR38.855.

-	Scenario 1. Indoor Office for FR1 and FR2 (Open office and Mixed Office)

-	Scenario 2. UMi street canyon for FR1 and FR2 (ISD 200m)

-	Scenario 3. UMa (ISD 500m, TBD: ISD > 500m) for FR1 only (Macro cell only deployment scenario)

The performance evaluation in this paper will focus mainly on the OTDOA/UTDOA positioning technique following the simulation parameters defined at Section 6.1 in TR 38.855, which are duplicated in the Appendix A for convenience of reference. Additional information of the simulation is provided as follows:

- the simulation evaluation is carried out for FR1 scenarios only

- perfect PRS muting

- for OTDOA,

  - the LTE DL PRS pattern is used, including the PRS sequence and mapping pattern;

  - for each positioning fix, the RSTD measurements are obtained from one single PRS slot (no combining of multiple PRS slots)

- for UTDOA,

  - 4 SRS symbols within a slot and full bandwidth transmission

  - Comb=4

System Level Simulation Results

The OTDOA horizontal positioning error of 50%, 67%, 80%, and 90% for the scenarios are summarized in Table 1. The detailed simulation results for three scenarios can be found in the section 3.1, 3.2 and 3.3, respectively.

Table 1 OTDOA horizontal positioning errors for FR1

| Scenarios | Carrier freq./SCS/BW | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| UMi | 4GHz/15kHz/5MHz | 25.9 | 31.9 | 38.5 | 49.9 |
|  | 4GHz/15kHz/50MHz | 13.6 | 17.4 | 22.5 | 27.5 |
| UMA | 4GHz/15kHz/5MHz | 46.3 | 69.7 | 101.6 | 151.2 |
|  | 4GHz/15kHz/50MHz | 16.6 | 27.4 | 44.3 | 136.5 |
| Indoor | 4GHz/15kHz/5MHz | 13.1 | 17.1 | 22.0 | 28.9 |
|  | 4GHz/15kHz/50MHz | 7.8 | 10.5 | 13.2 | 17.5 |

UTDOA horizontal positioning error of 50%, 67%, 80%, and 90% for  the UMi scenario is summarized in Table 2. The detailed simulation results for the scenario can be found in the section 3.2.

Table 2 UTDOA horizontal positioning errors for FR1

| Scenarios | Carrier freq./SCS/BW | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| UMi | 4GHz/15kHz/5MHz | 25.4 | 44.8 | 63.6 | 82.5 |
|  | 4GHz/15kHz/50MHz | 20.1 | 29.5 | 34.2 | 44.1 |

Scenario 1 – Indoor Open Office

The text proposal capturing our results for scenario 1 is provided in appendix.

Simulation results

For Indoor scenario, the CDF of OTDOA Positioning error is given in Figure 1.

Figure 1: CDF of OTDOA Positioning error of Indoor

.

Summary of results for Scenario 1

For InH in FR1, the OTDOA horizontal positioning errors are 22.0m@80% for 5MHz bandwidth and 13.2m@80% for 50MHz bandwidth respectively, meeting the requirements for the regulatory  use case.

Scenario 2 – Urban micro

The text proposal capturing our results for scenario 2 is provided in appendix.

Simulation results

For UMi scenario, the CDF of OTDOA Positioning error is given in Figure 2.

Figure 2: CDF of OTDOA Positioning error of UMi

For UMi scenario, the CDF of UTDOA Positioning error is given in Figure 3.

Figure 3: CDF of UTDOA Positioning error of UMi

Summary of results for Scenario 2

For UMi in FR1, the OTDOA horizontal positioning errors are 38.5m@80% for 5MHz bandwidth and 22.5m@80% for 50MHz bandwidth respectively, meeting the requirements for the regulatory use case.

For UMi in FR1, the UTDOA horizontal positioning errors are 63.6 m@80% for 5MHz bandwidth, which does not meet the requirements for the regulatory use case, and 34.2 m@80% for 50MHz bandwidth, which meets the requirements for the regulatory  use case.

Scenario 3 – Urban macro

The text proposal capturing our results for scenario 3 is provided in appendix.

Simulation results

For UMA scenario, the CDF of OTDOA Positioning error is given in Figure 4.

Figure 4: CDF of OTDOA Positioning error of UMA

Summary of results for Scenario 3

For UMA in FR1, the OTDOA horizontal positioning error is 101.6m@80% for 5MHz bandwidth, which does not meet the requirements for the regulatory use case; the OTDOA horizontal positioning error is 44.3 m @80% for 50MHz bandwidth respectively, which meets the requirements for the regulatory use case.

Summary

In this contribution, we provide initial system-level simulation evaluation results of OTDOA solution. Based on the discussion and simulation results, we have the following observations:

Observation 1: For UMi in FR1, the OTDOA horizontal positioning errors are 38.5m@80% for 5MHz bandwidth and 22.5m@80% for 50MHz bandwidth respectively, meeting the requirements for the regulatory use case.

Observation 2: For InH in FR1, the OTDOA horizontal positioning errors are 22.0m@80% for 5MHz bandwidth and 13.2m@80% for 50MHz bandwidth respectively, meeting the requirements for the regulatory  use case.

Observation 3: For UMA in FR1, the OTDOA horizontal positioning error is 101.6m@80% for 5MHz bandwidth, which does not meet the requirements for the regulatory use case; the OTDOA horizontal positioning error is 44.3 m @80% for 50MHz bandwidth respectively, which meets the requirements for the regulatory use case.

Observation 4: For UMi in FR1,  the UTDOA horizontal positioning errors are 63.6 m@80% for 5MHz bandwidth, which does not meet the requirements for the regulatory use case,  and 34.2 m@80% for 50MHz bandwidth, which meets the requirements for the regulatory  use case.

Reference

- 3GPP TR 38.855 (v1.0.0), “Technical Specification Group Radio Access Network; Study on NR positioning support (Release 16)

- Draft Report of 3GPP TSG RAN WG1 #94bis v0.1.0, ETSI

- 3GPP TR 38.802, “Study on new radio access technology Physical layer aspects”

- 3GPP TR 38.901, “Study on channel model for frequencies from 0.5 to 100 GHz”

Appendix A

Scenarios and system parameters for positioning evaluations [1]

For evaluating baseline performance, scenarios (with various options/configurations) are defined below for RAT-dependent positioning techniques for NR positioning study:

-	Scenario 1. Indoor Office for FR1 and FR2 (Open office and Mixed Office)

-	Scenario 2. UMi street canyon for FR1 and FR2 (ISD 200m)

-	Scenario 3. UMa (ISD 500m, TBD: ISD > 500m) for FR1 only (Macro cell only deployment scenario)

Table 6.1.1-1: Common scenario parameters applicable for all scenarios

|  | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- |
| Carrier frequency, GHz | 2GHz, 4GHz – Note 1 | 30 GHz – Note 1 |
| Bandwidth, MHz | 5MHz, 50MHz for 2GHz 100MHz for 4GHz | 100MHz, 400MHz |
| Subcarrier spacing, kHz | 15kHz for 5MHz and 50MHz 30kHz for 100MHz | 120kHz |
| gNB model parameters |  |  |
| gNB noise figure, dB | 5dB | 7dB |
| UE max. TX power, dBm | 23dBm – Note 1 | 23dBm – Note 1 EIRP should not exceed 43 dBm. |
| UE model parameters |  |  |
| UE noise figure, dB | 9dB – Note 1 | 13dB – Note 1 |
| UE antenna configuration | Panel model 1 – Note 1 Mg = 1, Ng = 1, P = 2, dH = 0.5λ, (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) | Multi-panel Configuration 1 and Panel Configuration a – Note 1 - Multi-panel Configuration 1: (Mg, Ng) = (1, 2); Θmg,ng=90°; Ω0,1=Ω0,0+180°; (dg,H, dg,V)=(0,0) - Panel Configuration a: - Each antenna array has shape dH=dV=0.5λ - Config a: (M, N, P) = (2, 4, 2), - the polarization angles are 0° and 90° - The antenna elements of the same polarization of the same panel is virtualized into one TXRU - Optional: Provided by company |
| UE antenna radiation pattern | Omni, 0dBi | Antenna model according to Table 6.1.1-2 |
| PHY/link level abstraction | Explicit simulation of all links, individual parameters estimation is applied. Companies to provide description of applied algorithms for estimation of signal location parameters. |  |
| Network synchronization | The network synchronization error, per UE dropping, is defined as a truncated Gaussian distribution of (T1 ns) rms values between an eNB and a timing reference source which is assumed to have perfect timing, subject to a largest timing difference of T2 ns, where T2 = 2*T1 – That is, the range of timing errors is [-T2, T2] – T1: 0ns (perfectly synchronized), 50ns |  |
| Note 1: According to 3GPP TR 38.802 Note 2: According to 3GPP TR 38.901 |  |  |

Table 6.1.1-2: UE antenna radiation pattern model 1 (FR2)

| Parameter | Values |
| --- | --- |
| Antenna element radiation pattern in θ'' dim (dB) |  |
| Antenna element radiation pattern in φ'' dim (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 5dBi |
|  | Note: are in local coordinate system. |

Table 6.1.1-3: Scenario 1. Indoor office scenario parameters

|  | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- |
| Layout | Indoor floor: (12BSs per 120m x 50m), TRP number per floor:12, Inter-gNB distance = 20m - Note 1 |  |
| Total gNB TX power, dBm | 24dBm | 24dBm EIRP should not exceed 58 dBm |
| gNB antenna configuration | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH=dV=0.5λ – Note 1 | (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), dH=dV=0.5λ – Note 1 One TXRU per polarization per panel is assumed |
| gNB antenna radiation pattern | Single sector – Note 1 | 3-sector antenna configuration – Note 1 |
| Channel model | Indoor open office – Note 2 |  |
| Peneteration loss | 0dB |  |
| Number of floors | 1 |  |
| UE drop procedure | 100% indoor, uniformly distributed over the horizontal area |  |
| UE mobility | 3km/h |  |
| UE antenna height | 1.5m |  |
| Min gNB-UE distance (2D), m | 0m |  |
| gNB antenna height | 3m |  |
| Note 1: According to 3GPP TR 38.802 Note 2: According to 3GPP TR 38.901 |  |  |

Table 6.1.1-4: Scenario 2: Urban micro (UMi) scenario parameters

|  | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- |
| Layout | Hexagonal grid, 19 or 7 macro sites, 3 sectors per site, ISD = 200m, Note 1 Wrap-around is applied, Note 2 |  |
| Total gNB TX power, dBm | 44dBm | 37dBm per panel EIRP should not exceed 73dBm |
| gNB antenna configuration | (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1), (dH, dV) = (0.5, 0.8)λ, - Note 1 | (M, N, P, Mg, Ng) = (4, 8, 2, 2, 2), (dH, dV) = (0.5, 0.5)λ, (dg,H,dg,V) = (4.0, 2.0)λ, - Note 1 |
| gNB antenna radiation pattern | Directional, 8dBi – Note 1 Table 6.1.1-5 | Directional, 8dBi – Note 1 Table 6.1.1-5 |
| Channel model | UMi Street Canyon – Note 3 |  |
| Penetration loss | For outdoor UEs: 0dB |  |
| Number of floors | All UEs are on the ground. |  |
| UE drop procedure | 100% outdoor uniformly distributed over the horizontal area |  |
| UE mobility (for modeling Doppler effects) | Outdoor: 3km/h |  |
| UE height, m | 1.5m |  |
| Min. gNB-UE distance (2D), m | 10m |  |
| gNB antenna height | 10 m by default. Companies can bring results with uniform distribution [5-20]m​ |  |
| Note 1: According to 3GPP TR 38.802 Note 2: In case if interference considerations are not properly taken into account for 7 sites companies are encouraged to provide results for 19 sites. Note 3: According to 3GPP TR 38.901 |  |  |

Table 6.1.1-5: gNB antenna patterns for UMi and UMa

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 8dBi |

Table 6.1.1-6: Scenario 3: Outdoor macro (UMa) scenario parameters

|  | FR1 Specific Values |
| --- | --- |
| Layout | Hexagonal grid, 3 sectors per site, 7 or 19 macro sites, ISD = 500m – Note 1 Wrap-around is applied. Note 2 |
| Total gNB TX power, dBm | 49dBm |
| gNB antenna configuration | (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1), (dH, dV) = (0.5, 0.8)λ – Note 1 Applicable for 2GHz and 4 GHz carrier frequency. |
| gNB antenna radiation pattern | Directional, 8dBi – Note 1, Table 6.1.1-5 |
| Channel model | UMa scenario – Note 3 |
| Penetration loss | For outdoor UEs: 0dB For indoor UEs: 20dB+0.5d2D-in – Note 3 |
| Number of floors, (floor height) | 8, (3m) |
| Antenna Height: | Uniformly distributed [20-50] m – Note 4 25m + α, where α~uniform[-5, 25] |
| UE Height | 3(nfl – 1) + 1.5 m – Note 4 where, nfl ~ uniform(1,Nfl) and Nfl = 8 |
| UE dropping procedure | 50% indoor and 50% outdoor uniformly distributed over the horizontal area (separate statistic) |
| Min. gNB-UE distance (2D), m | 35m |
| UE mobility (for modeling Doppler effects) | For indoor UEs: 3km/h For outdoor UEs: 60km/h |
| Note 1: According to 3GPP TR 38.802 Note 2: In case if interference considerations are not properly taken into account for 7 sites companies are encouraged to provide results for 19 sites. Note 3: According to 3GPP TR 38.901 Note 4: According to 3GPP TR 37.857 |  |

The evaluation methodology does not define any baseline reference signals.

The results from the evaluations can potentially be used to select a subset of the reference signals studied as part of the evaluations when considering design options.

The existing channel models as provided in the TR 38.901 are used for NR positioning evaluation as a baseline.

Note: Companies can provide additional results and describe modifications made for NLOS channel model