3GPP TSG RAN WG1 Meeting #96		R1-1901999

Athens, Greece, 25th February – 1st March, 2019

Source:                CATT

Title:	Discussion on coexistence of NR V2X and LTE V2X

Agenda Item:	7.2.4.5

Document for:     Discussion and Decision

Introduction

In RAN #80 meeting [1], the following objective for coexistence in NR V2X study item is as follows:

6: Coexistence [RAN1]: - In-device coexistence: Study the feasibility of the coexistence mechanisms when NR sidelink and LTE sidelink technologies are equipped in the same vehicle for the ‘not co-channel’ scenario: - Advanced V2X services provided by NR sidelink coexisting with V2X service provided by LTE sidelink in different channels (i.e., not co-channel). Not co-channel could include both adjacent channel and channels that are sufficiently far apart. NOTE: It is assumed that any coexistence requirements and mechanisms of NR sidelink with non-3GPP technologies will not be defined by 3GPP.

In RAN1 #94 meeting [2], agreements were achieved for coexistence in NR V2X:

Agreements: For the study of LTE-V2X and NR-V2X sidelink co-existence, at least the following scenarios are considered from the UEs perspective: - LTE sidelink and NR sidelink do not have any coordinated procedures - LTE sidelink and NR sidelink have coordinated procedures and half-duplex constraints are assumed - RAN1 will focus on this scenario in the SI Agreements: RAN1 focus on at least the following potential solutions for coexistence at least until the next meeting: - TDM of LTE V2X and NR V2X sidelink transmissions - FDM of LTE V2X and NR V2X sidelink transmissions

In RAN1 AH #1901 meeting [3], agreements were achieved for coexistence in NR V2X:

Agreements: - For TDM solutions for in-device coexistence between LTE and NR V2X: - Time Alignment - Subframe boundary alignment is required between LTE and NR V2X sidelinks - Both LTE and NR V2X sidelinks are aware of the time resource index (e.g., DFN for LTE) in both carriers Agreements: - For long term time scale TDM solutions for in-device coexistence between LTE and NR V2X: - For a UE with coexistence impact, non-overlapping (in time domain) resource pools are (pre-)configured for NR V2X and LTE V2X sidelinks - No information is exchanged between LTE and NR sidelinks within the UE - Long term time scale TDM solution is feasible from RAN1 point of view - Note: although feasible, it is expected that such a solution may have impact on latency, reliability and data rate requirements for some applications - No additional modifications to LTE specifications are needed Agreements: Assuming SPS scheduling (mode -3 or mode-4) for LTE V2X, for short time scale TDM solutions for in-device coexistence for V2X, - For each occurrence of Tx/Tx overlap, one RAT is prioritized over another - This requires some information exchange between LTE and NR sidelinks within the UE - FFS: whether the information exchange between LTE and NR sidelinks can support this requirement - FFS: if there is impact to RAN1 LTE specification with this agreement - FFS: whether this solution can be up to UE implementation - For each occurrence of Tx/Rx overlap, one RAT is prioritized over another - This requires some information exchange between LTE and NR sidelinks within the UE - FFS: if there is impact to RAN1 LTE specification with this agreement - FFS: whether this solution can be up to UE implementation - FFS: If determination of priority for Rx operation is feasible and whether the information exchange between LTE and NR sidelinks can support this requirement Agreements: - Inter-band FDM Solutions for coexistence - For static power assignment of Pc,max for each carrier - Synchronization is not assumed for inter-band coexistence of NR sidelink and LTE sidelink. - This FDM solution is feasible for resolution of Tx/Tx coexistence conflicts - If the band separation is large enough (based on RAN4 indication), then this FDM solution for coexistence is feasible for Tx/Rx coexistence - If the band separation is NOT large enough, then this FDM solution is not feasible for resolution of Tx/Rx coexistence conflicts - For dynamic power sharing between carriers, - FFS details of FDM solutions and whether they are feasible

In this contribution, we will discuss the issues of coexistence of NR V2X and LTE V2X.

Scenarios of link co-existence

Based on discussion in NR AH #1901 meeting, TDM and FDM solutions would be the potential solutions for coexistence of NR V2X and LTE V2X. Considering the link coexistence, there are four scenarios as follows.

- Scenario 1: NR PC5 + LTE PC5

- Scenario 2: NR PC5 + LTE Uu

- Scenario 3: NR Uu + LTE PC5

- Scenario 4: NR Uu + LTE Uu

For Scenario 1, TDM solutions and FDM solutions are both feasible during the study.

For Scenario 2, the long term TDM solution is the LTE scheduler not to schedule UL transmission on those uplink subframes with time duration allocated for the NR PC5 resource pool.

For Scenario 3, the long term TDM solution would limit the scheduler flexibility in the NR Uu interface. For short term TDM solution, it would select the resource from the resource pool not overlapped with the NR scheduled UL transmission and require both information exchange within the Uu.

For scenario 4, the resource allocation in LTE and NR UL access are scheduled by eNB and gNB respectively with coordination between eNB and gNB.

Issues of TDM solutions for LTE and NR co-existence

For TDM solutions, the assumption is that the LTE and the NR sidelinks are synchronized. The TDM solution is to allocate the resource in different time for LTE and NR sidelink transmission to mitigate the in-device interference.

Long-term coordination in time domain

For the long term TDM coordination, the pattern of resource selection for the LTE and NR sidelink can be statically/quasi-statically configured or pre-configured for each UE. Since the resource allocation between LTE and NR sidelink is UE-specific, the network could allocate the LTE sidelink resource to another UE’s NR sidelink transmission. This would prevent the degradation of resource utilization. The TDM transmission resource configuration would have impacts on latency and reliability, which as illustrated in Figure 1.

Figure 1: TDM solution of LTE V2X and NR V2X sidelink transmissions

As illustrated in Figure 1, each of the two UEs has a UE-specific transmission resource configuration and is to select transmission resources for an arriving packet with latency requirement of 3ms. For UE1, there is no available transmission resource in NR sidelink. While for UE2, only 2 slot resources can be used as candidate resources and the probability of collision would be significantly high.

Observation 1: For the long-term coordination in time domain:

- There would be adverse impacts on the latency and reliability.

Short-term coordination in time domain

For short-term coordination in time domain, there are two potential options as follows:

- Option 1: The resource selection/allocation excludes the resources with potential concurrent transmission/high priority reception indicted by decoded SCI.

- Option 2: Base station dynamically or semi-statically configures TDM resources for LTE and NR V2X transmission.

For option 1, when gNB allocates resources or UE autonomously selects resources, the candidate resources which overlap with the reserved transmission resources or higher priority reception in the other sidelink should be excluded. Either the internal resource allocation information exchange or the network based resource allocation information exchange can achieve this solution. The study of resource allocation mechanism should consider the TDM resource allocation for in-device interference avoidance. The joint resource occupation detection and resource selection of NR and LTE sidelink resources should be supported to minimize the impact of resource utilization.

Observation 2: For the short-term in time domain of the resource allocation/selection by excluding the resources with the potential con-current transmission and the high priority reception indicted by decoded SCI:

- There would be adverse impacts on the latency and reliability.

Proposal 1: Study of resource allocation should consider the procedure of resource selection by excluding the resources with the potential con-current LTE sidelink and NR sidelink transmission and the high priority reception indicted by decoded SCI to achieve the short-term coordination in time domain.

For option 2, base station can dynamically or semi-statically configures UE-specific resources for the TDM transmission at each sidelink, according to the traffic arrival of each sidelink. However, the dynamic allocation by base station would not work for the out-of-coverage case.

Observation 3: For the short-term coordination in time domain with base station dynamically or semi-statically configuring UE-specific resources for the TDM transmission based on the traffic arrival:

- The degradation of spectrum efficiency can be minimized by dynamic allocation of resource in time domain.

- There would be adverse impacts on the latency and reliability.

Issues of FDM solutions

Interference

For inter-band case, the harmonic interference should be studied. The dedicated ITS band would be multiple of other band shared by V2X and cellular communication.  When UE is transmitting LTE V2X in one carrier and receiving NR V2X at the other carrier, the n-th order harmonic interference would occur if NR V2X carrier frequency is n times that of LTE. The inter-modulation interference should also be considered.

Proposal 2: For FDM solutions, considering inter-band coexistence of NR sidelink and LTE sidelink in V2X, harmonic interference should be further studied.

For intra-band case, in the non-co-channel scenario, when one of the two sidelinks is performing transmission, the in-band emission from the transmission would cause the interference to the other carrier’s reception. UE needs to have coherent transmission or reception on both carriers in the same time interval to avoid interference caused by intra-band emission. The resource allocation mechanism needs to address the intra-band interference issues to align the NR V2X transmission and reception with those of LTE.

Proposal 3: For FDM solutions, considering intra-band coexistence of NR sidelink and LTE sidelink in V2X, the resource allocation mechanism needs to address the intra-band interference issues to align the NR V2X transmission and reception with those of LTE.

Issue of con-current transmission

When the packets from different services arrive around the same time, the resource may be selected and overlapped in the time domain. Therefore, the probability of con-current transmissions in NR and LTE sidelink could be high when the LTE and NR sidelink resource allocation are performed independently without any coordination.

In Rel-15 V2X CA, UE capability would have impacts on simultaneous transmissions in multiple carriers, which mainly includes the number of TX chains, power budget sharing capability and TX chain retuning capability.

Figure 2: The non-aligned slot case of con-current transmission

For coexistence of NR and LTE sidelink, because of the introduction of flexible numerology and dynamic slot format in NR, the NR transmission time interval in a slot would be equal to or shorter than LTE transmission time interval in a subframe. For the intra-band case with a single PA structure, the con-current transmission within non-aligned slots as illustrated in Figure 2 would cause performance degradation of both LTE sidelink and NR sidelink. That is because phase discontinuity of transmitted signal would happen in the transition area between overlapped and non-overlapped region slot case [4].

Observation 4: For the con-current transmission in the intra-band non-aligned slot case, a single PA would cause performance degradation of both LTE sidelink and NR sidelink.

While two PAs are supported, the interference, guard band and power sharing should be further considered.

Resource allocation considering con-current transmission in non-aligned slots

Based on the analysis of con-current transmission issue, for single PA UEs, resource allocation should be further studied to achieve the resource exclusion for the candidate resources that can potentially lead to con-current transmission in non-aligned slots, which is illustrated in Figure 3.

Figure 3: Resource exclusion for single PA UE

Proposal 4:  For single PA UEs, resource allocation should be further studied, considering the issue of con-current transmission in non-aligned slots.

Power sharing

Dynamic power sharing has been specified in EN-DC of non-standalone NR. The triggering condition of power sharing would be that the total transmission power of LTE and NR exceeds the available total power of LTE-NR V2X dual connectivity. In NR, the operation of power sharing is to reduce the transmission power of SCG (Secondary Cell Group) or even drop the transmission of SCG when the power exceeds the maximum power of the UE power class.

For coexistence of NR sidelink and LTE sidelink, the following factors should be considered.

- Max TX power of NR sidelink.

In coexistence of NR sidelink and LTE sidelink, the maximum TX power of NR sidelink and LTE sidelink should be individually configured to the UE power class (e.g. 23 dBm) with sum of the maximum Tx power exceeding the UE power class. This would allow dynamic power sharing among NR and LTE carriers. The determination of PC,MAX for NR and LTE would be studied by RAN 4.

Proposal 5: Max TX power of NR sidelink should not be reduced. The determination of PC,MAX for NR and LTE would be studied by RAN 4.

- The solution of power sharing.

In EN-DC of non-standalone NR, the dynamic power sharing is specified [5]. For NR and LTE sidelink, both in-coverage and out-of-coverage cases should be supported. For in-coverage case, the dynamic power sharing and semi-static power allocation could be achieved by the base station. For out-of-coverage case, dynamic power sharing would be performed based on pre-configuration rules. The adaption details should be further studied.

Proposal 6: The dynamic power sharing and semi-static power sharing can be the start points for in-coverage case and the pre-configured power sharing rules should be a start point for out-of-coverage case.

- The adaption details should be further studied.

- The priority rules of power sharing

In RAN 1 AH #1901 meeting, determination of priority between LTE and NR V2X was discussed [6]. There are mainly three options as follows:

- Priority based power allocation

- Prioritize LTE transmissions over NR

- Left to UE implementation

In EN-DC of non-standalone NR, because control signaling is transmitted in LTE link, the LTE transmission is prioritized over NR transmission. While for coexistence of NR V2X sidelink and LTE V2X sidelink, the advanced V2X services are transmitted in NR sidelink and the basic road safety services are transmitted in LTE sidelink. Therefore, it is not suitable to prioritize LTE transmissions over NR. To perform priority based power allocation, the mapping rules between QoS of NR services and PPPP of LTE services should be defined. The advanced V2X services with high priority, low latency and high reliability should be prioritized and mapped to the high priority of PPPP.

Proposal 7: The priority rules of priority based power allocation should be further discussed.

- The mapping rules between QoS of NR V2X services and PPPP of LTE V2X services should be further studied and defined.

- PSD configuration

RAN4 had shown the study results of intermods and reverse intermods on the ability of UEs to meet emission requirements for intra-band EN-DC and sent an LS to RAN1 [7].  The study results showed that power reduction techniques including MPR and A-MPR can be used to minimize the impact of these IMDs and RIMDs.  The MPR and A-MPR with equal PSD between LTE and NR and equal back-off for E-UTRA and NR is assumed in the RAN4 study. The equal PSD assumption is not consistent with RAN1 power control design in LTE and NR.

While for intra-band NR and LTE sidelink, the configuration of PSD needs to be studied, which would impact on the performance of in-band emission. Based on the RAN4 LS, the equal PSD would do well to interference alleviation. However, in LTE V2X, PSCCH transmission has 3dB PSD boosting comparing with PSSCH transmission. Currently, multiplexing PSCCH and PSSCH in NR V2X is studied in AI 7.2.4.1.1. Therefore, the PSD configuration should be further studied based on the study of multiplexing PSCCH and PSSCH. Furthermore, the resource allocation mechanism should consider the solution to avoid PSD imbalance.

Proposal 8:  PSD configuration between NR and LTE sidelink should be further studied based on the study of multiplexing PSCCH and PSSCH and the resource allocation solution to avoid PSD imbalance should be further studied.

Half duplex impact for adjacent channel transmission

For in-device coexistence, if NR sidelink and LTE sidelink work in adjacent channels in one device, half duplex impact should be considered. In Rel-15 LTE V2X carrier aggregation, half duplex impact is evaluated and shown that the system performance would be degraded [8]. Considering coexistence of NR sidelink technology and LTE sidelink technology, more carriers would be used for the advanced services. The more significant half duplex impact would appear in the coexistence scenarios. In addition, considering the new physical layer structure in NR V2X sidelink, e.g. the variable numerology and the dynamic slot format, the half duplex impact to the V2X performance would become more significant as illustrated in Figure 4. Therefore, the half duplex impact and solution for adjacent channel transmission should be further studied. The potential is to reduce the total number of transmission slots, e.g. to centralize the transmission slots of NR carriers and LTE carriers.

Figure 4: The more significant half-duplex impact for coexistence of NR and LTE sidelink

Proposal 9: The issue of half duplex for adjacent channel transmission considering coexistence of NR sidelink and LTE sidelink in V2X should be further studied.

Conclusion

In this contribution, the issues for coexistence of NR sidelink and LTE sidelink in V2X are discussed. Particularly, we have following observations and proposals:

Observation 1: For the long-term coordination in time domain:

- There would be adverse impacts on the latency and reliability.

Observation 2: For the short-term in time domain of the resource allocation/selection by excluding the resources with the potential con-current transmission and the high priority reception indicted by decoded SCI:

- There would be adverse impacts on the latency and reliability.

Proposal 1: Study of resource allocation should consider the procedure of resource selection by excluding the resources with the potential con-current LTE sidelink and NR sidelink transmission and the high priority reception indicted by decoded SCI to achieve the short-term coordination in time domain.

Observation 3: For the short-term coordination in time domain with base station dynamically or semi-statically configuring UE-specific resources for the TDM transmission based on the traffic arrival:

- The degradation of spectrum efficiency can be minimized by dynamic allocation of resource in time domain.

- There would be adverse impacts on the latency and reliability.

Proposal 2: For FDM solutions, considering inter-band coexistence of NR sidelink and LTE sidelink in V2X, harmonic interference should be further studied.

Proposal 3: For FDM solutions, considering intra-band coexistence of NR sidelink and LTE sidelink in V2X, the resource allocation mechanism needs to address the intra-band interference issues to align the NR V2X transmission and reception with those of LTE.

Observation 4: For the con-current transmission in the intra-band non-aligned slot case, a single PA would cause performance degradation of both LTE sidelink and NR sidelink.

Proposal 4:  For single PA UEs, resource allocation should be further studied, considering the issue of con-current transmission in non-aligned slots.

Proposal 5: Max TX power of NR sidelink should not be reduced. The determination of PC,MAX for NR and LTE would be studied by RAN 4.

Proposal 6: The dynamic power sharing and semi-static power sharing can be the start points for in-coverage case and the pre-configured power sharing rules should be a start point for out-of-coverage case.

- The adaption details should be further studied.

Proposal 7: The priority rules of priority based power allocation should be further discussed.

- The mapping rules between QoS of NR V2X services and PPPP of LTE V2X services should be further studied and defined.

Proposal 8:  PSD configuration between NR and LTE sidelink should be further studied based on the study of multiplexing PSCCH and PSSCH and the resource allocation solution to avoid PSD imbalance should be further studied.

Proposal 9: The issue of half duplex for adjacent channel transmission considering coexistence of NR sidelink and LTE sidelink in V2X should be further studied.

References

  - RP-181480, "New SID: Study on NR V2X ", Vodafone, La Jolla, USA, 3GPP RAN #80.

  - 3GPP TSG RAN WG1 Meeting RAN1 #94 chairman notes,  August 2018.

  - 3GPP TSG RAN WG1 Meeting RAN1 NR AH #1901 chairman notes,  January 2019.

  - R1-1900328, "Discussion on coexistence of LTE sidelink and NR sidelink in NR V2X ". CATT, Taipei, Taiwan, 3GPP RAN1 AH #1901.

  - 3GPP TS 38.213 V15.2.0.

  - R1-1901392," Summary of Coexistence Aspects in NR-V2X Study (AI 7.2.4.5)", Qualcomm Incorporated, Taipei, Taiwan, 3GPP RAN1 AH #1901.

  - R1-1807805, " LS on RAN4 agreement on intraband EN-DC A-MPR ", RAN4.

  - R1-1801701, "Discussion on carrier aggregation for mode 4 in V2X phase 2", CATT, Athens, GREECE, 3GPP RAN1 #92.