3GPP TSG RAN WG1 Meeting #96                                                                       R1-1902013

Athens, Greece, 25th February – 1st March, 2019

Source:	CATT

Title:	Self-evaluation on LTE Spectrum Efficiency in Dense Urban-eMBB

Agenda Item:	7.2.7.2

Document for:	Discussion and Decision

Introduction

At RAN#75 meeting, Study Item on Self Evaluation towards IMT-2020 submission was approved [1], the self evaluation will provide the performances towards all the ITU-R IMT-2020 requirements. At RAN#81 meeting, the preliminary self evaluation results of 5G are provided [2] and the evaluation results are captured in TR 37.910[3]. For the spectrum efficiency evaluation in Dense Urban-eMBB, only the NR spectrum efficiency evaluation results are provided. Remaining open issues include [4]:

The evaluation results for eMBB spectral efficiency with ideal channel estimation need to be updated with realistic channel estimation. The LTE part evaluation is to be provided in the test environments not yet evaluated (i.e. other than Rural – eMBB and Urban Macro – mMTC). Further clarification of evaluation assumptions e.g. interference error modeling is needed.

In this document, the evaluation results on the spectral efficiencies in Dense Urban-eMBB based on Rel-15 LTE are provided to evaluate whether it can meet the technical performance requirements.

The key minimum technical performance requirements are provided in Report ITU-R M.2410 [5] for the test environment of Dense Urban-eMBB.

TABLE 1

5th percentile user spectral efficiency

| Test environment | Downlink (bit/s/Hz) | Uplink (bit/s/Hz) |
| --- | --- | --- |
| Dense Urban – eMBB | 0.225 | 0.15 |

TABLE 2

Average spectral efficiency

| Test environment | Downlink (bit/s/Hz/TRxP) | Uplink (bit/s/Hz/TRxP) |
| --- | --- | --- |
| Dense Urban – eMBB (Note 1) | 7.8 | 5.4 |

Evaluation methodology and assumption

System-level simulation with proper link to system interface based on link-level simulation results could be used to evaluate average spectral efficiency and 5th percentile user spectral efficiency.

The considerations of LTE technical parameters and overhead for evaluation refer to our companion contribution [6].

Evaluation results

The preliminary evaluation results for average spectral efficiency and 5th percentile spectral efficiency in Dense Urban-eMBB based on Rel-15 LTE are provided. The detailed evaluation assumptions can be found in Annex.

Downlink evaluation results

The preliminary evaluation results for DL average spectral efficiency and DL 5th percentile spectral efficiency on LTE in Dense Urban -eMBB are provided in Table 1 and Table 2.

Table 1 Initial DL average spectral efficiency evaluation

| Test environment | Duplex scheme | DL CSI measurement | TRxP and UE antenna Config. (M,N,P,Mg,Ng;Mp,Np) at TRxPs | Require- ment | SEavg | Gain (%) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | (bit/s/Hz/TRxP) |  |  |
| Dense Urban-eMBB Config. A, 4GHz UMa-A | FDD | Non-precoded CSI-RS based | 32T/4R, (8,8,2,1,1;2,8) | 7.8 | 8.07 | 3.46% |
|  |  |  | 32T/4R, (16,8,2,1,1;2,8) |  | 8.39 | 7.56% |
|  | TDD DSUUD | SRS and Non-precoded CSI-RS based | 32T/4R, (16,8,2,1,1;2,8) |  | 10.79 | 38.33% |
|  |  | SRS and Precoded CSI-RS based | 32T/4R, (16,8,2,1,1;2,8) |  | 10.82 | 38.72% |
|  |  |  | 64T/4R, (12,8,2,1,1;4,8) |  | 12.84 | 64.62% |
|  |  |  | 64T/4R, (16,8,2,1,1;4,8) |  | 13.18 | 68.97% |

Table 2 Initial DL 5th percentile spectral efficiency evaluation

| Test environment | Duplex scheme | DL CSI measurement | TRxP and UE antenna Config. (M,N,P,Mg,Ng;Mp,Np) at TRxPs | Require- ment | SEuser | Gain (%) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | (bit/s/Hz) |  |  |
| Dense Urban-eMBB Config. A, 4GHz UMa-A | FDD | Non-precoded CSI-RS based | 32T/4R, (8,8,2,1,1;2,8) | 0.225 | 0.254 | 12.89% |
|  |  |  | 32T/4R, (16,8,2,1,1;2,8) |  | 0.264 | 17.33% |
|  | TDD DSUUD | SRS and Non-precoded CSI-RS based | 32T/4R, (16,8,2,1,1;2,8) |  | 0.336 | 49.33% |
|  |  | SRS and Precoded CSI-RS based | 32T/4R, (16,8,2,1,1;2,8) |  | 0.341 | 51.56% |
|  |  |  | 64T/4R, (12,8,2,1,1;4,8) |  | 0.459 | 104.00% |
|  |  |  | 64T/4R, (16,8,2,1,1;4,8) |  | 0.463 | 105.78% |

Observation 1: For sub 6 GHz, LTE can fulfill the DL spectral efficiency requirements for the Dense Urban-eMBB test environment in both TDD and FDD mode.

Uplink evaluation results

The preliminary evaluation results for UL average spectral efficiency and UL 5th percentile spectral efficiency on LTE in Dense Urban -eMBB are provided in Table 3 and Table 4.

Table 3 Initial UL average spectral efficiency evaluation

| Test environment | Duplex scheme | UL CSI measurement | TRxP and UE antenna Config. (M,N,P,Mg,Ng;Mp,Np) at TRxPs | Require- ment | SEavg | Gain (%) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | (bit/s/Hz/TRxP) |  |  |
| Dense Urban-eMBB Config. A, 4GHz UMa-A | FDD | Codebook- based | 4T/32R, (16,8,2,1,1;2,8) | 5.4 | 6.95 | 28.68% |
|  | TDD DSUUD | Codebook-based | 4T/32R, (16,8,2,1,1;2,8) |  | 6.60 | 22.29% |

Table 4 Initial UL 5th percentile spectral efficiency evaluation

| Test environment | Duplex scheme | UL CSI measurement | TRxP and UE antenna Config. (M,N,P,Mg,Ng;Mp,Np) at TRxPs | Require- ment | SEuser | Gain (%) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | (bit/s/Hz) |  |  |
| Dense Urban-eMBB Config. A, 4GHz UMa-A | FDD | Codebook-based | 4T/32R, (16,8,2,1,1;2,8) | 0.15 | 0.389 | 159.46% |
|  | TDD DSUUD | Codebook-based | 4T/32R, (16,8,2,1,1;2,8) |  | 0.302 | 101.31% |

Observation 2: For sub 6 GHz, LTE can fulfill the UL spectral efficiency requirements for the Dense Urban-eMBB test environment in both TDD and FDD mode.

Conclusion

In this contribution, the preliminary LTE evaluation results of Dense Urban-eMBB are provided. We have the following observations:

Observation 1: For sub 6 GHz, LTE can fulfill the DL spectral efficiency requirements for the Dense Urban-eMBB test environment in both TDD and FDD mode.

Observation 2: For sub 6 GHz, LTE can fulfill the UL spectral efficiency requirements for the Dense Urban-eMBB test environment in both TDD and FDD mode.

- References

  - RP-170832, “New Study Item on Self Evaluation towards IMT-2020 submission”, Huawei, Ericsson, Dubrovnik, Croatia, March 6 - 9, 2017.

  - RP-182097, “Draft LTI on 3GPP 5G Preliminary Description Template and Self-Evaluation”, 3GPP TSG RAN, Gold Coast, Australia, September 10 - 13, 2018.

  - 3GPP TR 37.910, “Study on Self Evaluation towards IMT-2020 Submission”, V1.0.0, 2018-09

  - RP-182539，“Status Report to TSG”, Huawei, Sorrento, Italy, December 10-13, 2018

  - Report ITU-R M.2410, “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”, Nov. 2017.

  - R1-1902012, “Self-evaluation on LTE Spectrum Efficiency in Indoor Hotspot-eMBB”, CATT, Athens, Greece, 25th February – 1st March, 2019.

- Annex: Evaluation assumptions

In this contribution, the evaluation assumptions for Dense Urban-eMBB are provided in the following table.

Table A-1 Evaluation assumptions for DL and UL

| Dense Urban - eMBB | Config. A |
| --- | --- |
| Carrier frequency for evaluation | 1 layer (Macro) with 4 GHz |
| Simulation bandwidth | FDD: 10M+10M TDD: 20M |
| Slots structure | FDD: paired spectrum, DL + UL TDD: unpaired spectrum, DSUUD |
| Subcarrier spacing | 15KHz |
| Symbols number per slot | 14 |
| BS antenna height | 25 m |
| Total transmit power per TRxP | FDD: 41 dBm TDD: 44 dBm |
| UE power class | 23 dBm |
| Percentage of high loss and low loss building type | 20% high loss, 80% low loss (applies to Channel model B) |
| Inter-site distance | 200 m |
| Antenna configuration at TRxP | (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1; 2,8) (M,N,P,Mg,Ng; Mp,Np) = (16,8,2,1,1; 2,8) (M,N,P,Mg,Ng; Mp,Np) = (12,8,2,1,1; 4,8) (M,N,P,Mg,Ng; Mp,Np) = (16,8,2,1,1; 4,8) (dH,dV) = (0.5, 0.8)λ +45°, -45° polarization |
| Antenna configuration at UE | (M,N,P,Mg,Ng; Mp,Np) = (1,2,2,1,1; 1,2) (dH,dV) = (0.5, N/A)λ 0°,90° polarization |
| Device deployment | 80% indoor, 20% outdoor (in car) Randomly and uniformly distributed over the area under Macro layer |
| UE speeds of interest | Indoor users: 3km/h Outdoor users (in-car): 30 km/h |
| BS noise figure | 5 dB |
| UE noise figure | 7 dB |
| BS antenna element gain | 8 dBi |
| BS antenna element pattern | See Table 9 in Report ITU-R M.2412 |
| UE antenna element gain | 0 dBi |
| UE antenna element pattern | Omni-directional |
| Thermal noise level | -174 dBm/Hz |
| Traffic model | Full buffer |
| UE density | 10 UEs per TRxP |
| UE antenna height | Outdoor UEs: 1.5 m Indoor UTs: 3(nfl – 1) + 1.5; nfl ~ uniform(1,Nfl) where Nfl ~ uniform(4,8) |
| Channel model | Channel model A |
| TRxP number per site | 3 |
| Mechanic tilt | 90° in GCS |
| Electronic tilt | 102° in LCS |
| UT attachment | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 |
| Wrapping around method | Geographical distance based wrapping |
| Polarized antenna model | Model-2 in TR36.873 |
| TX mode | SU/MU-MIMO |
| scheduler | PF |
| Receiver | MMSE-IRC |
| DL CSI feedback | 10ms period with delay |
| DL CSI method | FDD: LTE Type II codebook based TDD: SRS and non-precoded CSI-RS / precoded CSI-RS based channel reciprocity |
| LTE Type II codebook parameters | 2 beam basis, WB+SB and 4PSK quantification |
| HARQ ACK delay | Refer to TR 36.213 |
| HARQ retransmission delay | The re-transmission delay is according to the UL/DL configurations, the minimum RTT is 8ms. |
| Channel estimation | Non-ideal |
| Interference estimation | Non-ideal |
| UE precoder scheme | Codebook based for FDD and TDD |
| UL CSI derivation | SRS based with delay |
| UL Waveform | OFDM |
| IoT | <10dB |