3GPP TSG RAN WG1 Meeting #96                                                                       R1-1902015

Athens, Greece, 25th February – 1st March, 2019

Source:	CATT

Title:	Self-evaluation on LTE User Experienced Data Rate in Dense Urban-eMBB

Agenda Item:	7.2.7.2

Document for:	Discussion and Decision

Introduction

At RAN#75 meeting, Study Item on Self Evaluation towards IMT-2020 submission was approved [1], the self evaluation will provide the performances towards all the ITU-R IMT-2020 requirements. At RAN#81 meeting, the preliminary self evaluation results of 5G are provided [2] and the evaluation results are captured in TR 37.910[3]. Under Dense Urban-eMBB, user experienced data rate evaluation is provided only for NR. Remaining open issues include [4]:

The evaluation results for eMBB spectral efficiency with ideal channel estimation need to be updated with realistic channel estimation. The LTE part evaluation is to be provided in the test environments not yet evaluated (i.e. other than Rural – eMBB and Urban Macro – mMTC). Further clarification of evaluation assumptions e.g. interference error modeling is needed.

In this document, the evaluation results on the user experienced data rate in Dense Urban-eMBB based on Rel-15 LTE are provided to evaluate whether it can meet the technical performance requirements.

Evaluation methodology and assumption

According to the evaluation methodology of Report ITU-R M.2412 [5], the user experienced data rate can be calculated by 5th percentile user spectral efficiency SEuser through equation in [6] (this KPI is only applied to Dense Urban-eMBB).

Ruser = W × SEuser

According to the discussion in RNA1#94 meeting, for TDD mode, the bandwidth W for the calculation of user experienced data rate is effective bandwidth considering the TDD subframe configuration. For TDD subframe configuration in our evaluation, which is DSUUD, S (11D: 1GP: 2UL), the ratio of DL effective bandwidth to system bandwidth is 0.5643, the ratio of UL effective bandwidth to system bandwidth is 0.4357.

Evaluation results

Considering the various bandwidth at 4GHz, according to the evaluation results of the DL and UL 5th percentile user spectral efficiency of the Dense Urban-eMBB in our companion contribution [7], the preliminary evaluation results of user experienced data rate are provided in Table 1 and Table 2.

In the perspective of RAN1, the maximum supported aggregation bandwidth for LTE is 640MHz, which is aggregated by 32 component carriers with 20MHz bandwidth. So for the preliminary evaluation of user experienced data rate in this contribution, the maximum bandwidth is 640MHz for both DL and UL.

Table 1 DL user experienced data rate for Dense Urban-eMBB

| Test environment | Duplex scheme | DL CSI measurement | TRxP and UE antenna Config. (M,N,P,Mg,Ng;Mp,Np) at TRxPs | Require- ment (Mbit/s) | SEuser (bit/s/Hz) | Band-width [MHz] | Ruser (Mbit/s) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Dense Urban-eMBB Config. A, 4GHz UMa-A | FDD | Non-precoded CSI-RS based | 32T/4R, (8,8,2,1,1;2,8) | 100 | 0.254 | 400 | 101.60 |
|  |  |  | 32T/4R, (16,8,2,1,1;2,8) |  | 0.264 | 380 | 100.32 |
|  | TDD DSUUD | SRS and Non-precoded CSI-RS based | 32T/4R, (16,8,2,1,1;2,8) |  | 0.336 | 540 | 102.39 |
|  |  | SRS and Precoded CSI-RS based | 32T/4R, (16,8,2,1,1;2,8) |  | 0.341 | 520 | 100.06 |
|  |  |  | 64T/4R, (12,8,2,1,1;4,8) |  | 0.459 | 400 | 103.61 |
|  |  |  | 64T/4R, (16,8,2,1,1;4,8) |  | 0.463 | 400 | 104.51 |

Table 2 UL user experienced data rate for Dense Urban-eMBB

| Test environment | Duplex scheme | UL CSI measurement | TRxP and UE antenna Config. (M,N,P,Mg,Ng;Mp,Np) at TRxPs | Require- ment (Mbit/s) | SEuser (bit/s/Hz) | Band-width [MHz] | Ruser (Mbit/s) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Dense Urban-eMBB Config. A, 4GHz UMa-A | FDD | Codebook-based | 4T/32R, (16,8,2,1,1;2,8) | 50 | 0.389 | 140 | 54.46 |
|  | TDD DSUUD | Codebook-based | 4T/32R, (16,8,2,1,1;2,8) |  | 0.302 | 380 | 50.00 |

Observation 1: The ITU requirement on DL and UL user experienced data rate can be met by LTE.

Conclusion

In this contribution, the preliminary LTE evaluation results on user experienced data rate of Dense Urban-eMBB are provided. There is the following observation:

Observation 1: The ITU requirement on DL and UL user experienced data rate can be met by LTE.

- References

  - RP-170832, “New Study Item on Self Evaluation towards IMT-2020 submission”, Huawei, Ericsson, Dubrovnik, Croatia, March 6 - 9, 2017.

  - RP-182097, “Draft LTI on 3GPP 5G Preliminary Description Template and Self-Evaluation”, 3GPP TSG RAN, Gold Coast, Australia, September 10 - 13, 2018.

  - 3GPP TR 37.910, “Study on Self Evaluation towards IMT-2020 Submission”, V1.0.0, 2018-09

  - RP-182539，“Status Report to TSG”, Huawei, Sorrento, Italy, December 10-13, 2018

  - Report ITU-R M.2412, “Guidelines for evaluation of radio interface technologies for IMT-2020”, Nov. 2017.

  - Report ITU-R M.2410, “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”, Nov. 2017.

  - R1-1902013, “Self-evaluation on LTE Spectrum Efficiency in Dense Urban-eMBB”, CATT, Athens, Greece, 25th February – 1st March, 2019.