3GPP TSG-RAN1 Meeting #96	R1-1902120

# Athens, Greece, February 25th – March 1st 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | DRAFT | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to aperiodic CSI-RS triggering with different numerology between PDCCH and CSI-RS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The slot offset of aperiodic CSI-RS relative the triggering PDCCH is ambiguous if the PDCCH and CSI-RS are transmitted with different numerology. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarified that the slot offset X according to higher layer parameter aperiodicTriggeringOffset is given in the numerology of the aperiodic CSI-RS and that the offset is relative to a reference slot n'=n+12μCSIRS2μPDCCH-1, which is the latest slot in the CSI-RS numerology overlapping with slot n of the triggering PDCCH in the PDCCH numerology - Added, for the case when when the PDCCH SCS is smaller than the CSI-RS SCS, a restriction that the UE is not required to process aperiodic CSI-RS if there are less than m2μCSIRS2μPDCCH symbols between the end of the PDCCH containing the triggering DCI and the CSI-RS, where m is the span, in number of OFDM symbols, of the PDCCH monitoring occasion in which the DCI is received. - Added, for the case when the PDCCH SCS is smaller than the CSI-RS SCS, d=m2μCSIRS2μPDCCH symbols to the scheduling offset threshold of aperiodic CSI-RS where the UE should apply a default QCL assumption - Clarified that the restriction that “the CSI-RS triggering offset X is fixed to zero if all the associated trigger states do not have the higher layer parameter qcl-Type set to 'QCL-TypeD' in the corresponding TCI states” only applies when the PDCCH SCS is equal to the CSI-RS SCS. - Added restriction that the CSI-RS triggering offset should be larger than zero if the PDCCH SCS is smaller than the CSI-RS SCS, in order to avoid non-causual triggering of CSI-RS which would require UE to buffer OFDM symbols in the larger SCS in anticipation of possible aperiodic CSI-RS trigger in the smaller SCS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Slot offset of aperiodic CSI-RS with different numerology than that of triggering PDCCH is ambiguous. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated Impact analysis This CR clarifies the behavior when aperiodic CSI-RS is triggered by a PDCCH with different numerology than that of CSI-RS and does not impact the UE behavior when aperiodic CSI-RS and PDCCH have the same numerology. |  |  |  |  |  |  |  |  |

##### 5.2.1.5.1	Aperiodic CSI Reporting/Aperiodic CSI-RS

For CSI-RS resource sets associated with Resource Settings configured with the higher layer parameter resourceType set to 'aperiodic', 'periodic', or 'semi-persistent', trigger states for Reporting Setting(s) (configured with the higher layer parameter reportConfigType set to 'aperiodic') and/or Resource Setting for channel and/or interference measurement on one or more component carriers are configured using the higher layer parameter CSI-AperiodicTriggerStateList. For aperiodic CSI report triggering, a single set of CSI triggering states are higher layer configured, wherein the CSI triggering states can be associated with any candidate DL BWP. A UE is not expected to receive more than one DCI with non-zero CSI request per slot. A UE is not expected to be configured with different TCI-StateId's for the same aperiodic CSI-RS resource ID configured in multiple aperiodic CSI-RS resource sets with the same triggering offset in the same aperiodic trigger state. A UE is not expected to receive more than one aperiodic CSI report request for transmission in a given slot. A UE is not expected to be triggered with a CSI report for a non-active DL BWP. A trigger state is initiated using the CSI request field in DCI.

-	When all the bits of CSI request field in DCI are set to zero, no CSI is requested.

-	When the number of configured CSI triggering states in CSI-AperiodicTriggerStateList is greater than , where  is the number of bits in the DCI CSI request field, the UE receives a selection command [10, TS 38.321] used to map up to  trigger states to the codepoints of the CSI request field in DCI.  is configured by the higher layer parameter reportTriggerSize where . When the HARQ/ACK corresponding to the PDSCH carrying the selection command is transmitted in the slot n, the corresponding action in [10, TS 38.321] and UE assumption on the mapping of the selected CSI trigger state(s) to the codepoint(s) of DCI CSI request field shall be applied starting from slot n+3Nslotsubframe,µ+1.

-	When the number of CSI triggering states in CSI-AperiodicTriggerStateList is less than or equal to , the CSI request field in DCI directly indicates the triggering state.

-	For each aperiodic CSI-RS resource in a CSI-RS resource set associated with each CSI triggering state, the UE is indicated the quasi co-location configuration of quasi co-location RS source(s) and quasi co-location type(s), as described in Subclause 5.1.5, through higher layer signaling of qcl-info which contains a list of references to TCI-State's for the aperiodic CSI-RS resources associated with the CSI triggering state. If a State referred to in the list is configured with a reference to an RS associated with 'QCL-TypeD', that RS may be an SS/PBCH block located in the same or different CC/DL BWP or a CSI-RS resource configured as periodic or semi-persistent located in the same or different CC/DL BWP.

-	If the scheduling offset, in the numerology of the aperiodic CSI-RS, between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info and without the higher layer parameter repetition is smaller than Y+d, where

-	Y is the UE reported threshold beamSwitchTiming, as defined in [13, TS 38.306],

-	d=0 if the PDCCH SCS is equal to or greater than the CSI-RS SCS and d=m2μCSIRS2μPDCCH otherwise, where m is the span, in number of OFDM symbols, of the PDCCH monitoring occasion in which the triggering DCI is received and μCSIRS and μPDCCH are the subcarrier spacing configurations for CSI-RS and PDCCH, respectively,

when the reported value of beamSwitchTiming is one of the values of {14, 28, 48}.

-	if there is any other DL signal with an indicated TCI state in the same symbols as the CSI-RS, the UE applies the QCL assumption of the other DL signal also when receiving the aperiodic CSI-RS. The other DL signal refers to PDSCH scheduled with offset larger than or equal to the threshold timeDurationForQCL, as defined in [13, TS 38.306], aperiodic CSI-RS scheduled with offset larger than or equal to Y+d, when the UE reported threshold beamSwitchTiming is one of the values {14,28,48}, periodic CSI-RS, semi-persistent CSI-RS.

-	If the scheduling offset, in the numerology of the aperiodic CSI-RS, between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources is equal to or greater than Y+d, where

-	Y is the UE reported threshold beamSwitchTiming, as defined in [13, TS 38.306],

-	d=0 if the PDCCH SCS is equal to or greater than the CSI-RS SCS and d=m2μCSIRS2μPDCCH otherwise, where m is the span, in number of OFDM symbols, of the PDCCH monitoring occasion in which the triggering DCI is received and μCSIRS and μPDCCH are the subcarrier spacing configurations for CSI-RS and PDCCH, respectively,

when the reported value of beamSwitchTiming is one of the values of {14,28,48}, the UE is expected to apply the QCL assumptions in the indicated TCI states for the aperiodic CSI-RS resources in the CSI triggering state indicated by the CSI trigger field in DCI.

-	A non-zero codepoint of the CSI request field in the DCI is mapped to a CSI triggering state according to the order of the associated positions of the up to 2NTS-1 trigger states in CSI-AperiodicTriggerStateList with codepoint '1' mapped to the triggering state in the first position.

For a UE configured with the higher layer parameter CSI-AperiodicTriggerStateList, if a Resource Setting linked to a CSI-ReportConfig has multiple aperiodic resource sets, only one of the aperiodic CSI-RS resource sets from the Resource Setting is associated with the trigger state, and the UE is higher layer configured per trigger state per Resource Setting to select the one CSI-IM/NZP CSI-RS resource set from the Resource Setting.

When aperiodic CSI-RS is used with aperiodic CSI reporting, the CSI-RS triggering offset X is configured per resource set by the higher layer parameter aperiodicTriggeringOffset. The CSI-RS triggering offset has the range of 0 to 4 slots. The aperiodic CSI-RS is transmitted in slot n+12μCSIRS2μPDCCH-1+X, where n is the slot with the triggering DCI in the numerology of the PDCCH containing the triggering DCI, X is the CSI-RS triggering offset in the numerology of CSI-RS according to the higher layer parameter aperiodicTriggeringOffset, and μCSIRS and μPDCCH are the subcarrier spacing configurations for CSI-RS and PDCCH, respectively. If all the associated trigger states do not have the higher layer parameter qcl-Type set to 'QCL-TypeD' in the corresponding TCI states and the PDCCH SCS is equal to the CSI-RS SCS, the CSI-RS triggering offset X is fixed to zero. If the PDCCH SCS is smaller than the CSI-RS SCS, the CSI-RS triggering offset X is larger than zero. If the PDCCH SCS is smaller than the CSI-RS SCS, the UE is not required to process aperiodic CSI-RS if there are less than m2μCSIRS2μPDCCH  symbols between the end of the PDCCH containing the triggering DCI and the CSI-RS, where m is the span, in number of OFDM symbols, of the PDCCH monitoring occasion in which the triggering DCI is received. The aperiodic triggering offset of the CSI-IM follows offset of the associated NZP CSI-RS for channel measurement.

The UE does not expect that aperiodic CSI-RS is transmitted before the OFDM symbol(s) carrying its triggering DCI.

If interference measurement is performed on aperiodic NZP CSI-RS, a UE is not expected to be configured with a different aperiodic triggering offset of the NZP CSI-RS for interference measurement from the associated NZP CSI-RS for channel measurement.

If the UE is configured with a single carrier for uplink, the UE is not expected to transmit more than one aperiodic CSI report triggered by different DCIs on overlapping OFDM symbols.