3GPP TSG RAN WG1 Meeting  #96                                                                 	      R1-1902148

Athens, Greece, 25th February – 1st March, 2019

Agenda Item:	6.2.2.1

Source:	Lenovo, Motorola Mobility

Title:	UE-group wake-up signal for Rel-16 NB-IoT

Document for:	Discussion and decision

Introduction

In the RAN#80 plenary meeting, a new work item for Rel-16 NB-IoT is approved. One of the objectives is to specify support for UE-group WUS [1]:

- Improved DL transmission efficiency and/or UE power consumption:

  - Specify support for UE-group wake-up signal (WUS) [RAN1, RAN2, RAN4]

In the RAN1#95 meeting, it was agreed that [2]:

Agreement

UE group ID is used as a parameter to generate WUS UE group sequence(s)

Agreement

One group WUS is designed as a single sequence

Agreement

Further study false detection (cross/auto correlation) performance properties for the following designs:

- legacy WUS + cover codes,

- legacy WUS + shifted scrambling codes,

- legacy WUS + phase shift + cover code + scrambling bits

  - Including combinations of phase shift, cover code, and/or scrambling bits

Other designs are not precluded.

Agreement

The number of UE groups is configurable and broadcasted in SIB.

- FFS: Further details on the number of UE groups. For example, whether it is per PO or per gap configuration of a PO.

Agreement

Multiplexing of Rel-16 UE groups is down-selected among

- Single sequence CDM is supported

- Single sequence CDM is supported and TDM can be additionally applied

  - FFS: Configurability of CDM and/or TDM

Agreement

A common WUS is used to wake up all group WUS UEs monitoring the same WUS (time-frequency) resource if more than one UE group occupies the WUS resource.

- FFS: Whether the above is also applicable for Rel-15 WUS UEs

- FFS: Whether to support waking up a subset of all WUS UE groups

Agreement

For multiplexing between Rel-16 UE-group WUS and Rel-15 legacy WUS within the same carrier, further evaluate and down select among the following options

- TDM

- single-seq. CDM

- single-seq. CDM+TDM

FFS whether legacy WUS is the common WUS for all new UEs or only a part of new UEs.

Agreement

Rel-16 group WUS uses the same gap configurations as for Rel-15 legacy WUS except for differences from possible TDM.

- No new gap higher layer signaling will be introduced for TDM

In this contribution, we present our considerations on UE-group WUS.

Discussion

Backward compatible and multiplexing schemes for Rel-16 WUSs

In Rel-15, cell-specific WUS has been introduced and specified. The Rel-16 UE-group WUS must be backward compatible with the Rel-15 legacy WUS. Candidates for the multiplexing scheme between Rel-16 WUS and legacy WUS, as summarized in [2]:

- TDM

- single-seq. CDM

- single-seq. CDM+TDM

- FFS whether legacy WUS is the common WUS for all new UEs or only a part of new UEs.

For single-sequence CDM, Rel-16 WUS and legacy WUS will share the same physical resource. For a given time (a given PO), one WUS sequence (e.g. either Rel-16 WUS or legacy WUS) will be transmitted if Rel-16 and Rel-15 UEs are paged. Therefore, single-sequence CDM has two drawbacks: 1) legacy WUS detection performance will be influenced by Rel-16 WUS due to sequence correlation and false alarm probability of Rel-15 UE WUS detection will increase significantly. 2) CDMed Rel-16 UE and Rel-15 UE can’t be woken up in the same PO if both UEs groups are paged (e.g. either Rel-16 UE or Rel-15 UE), which will influence the performance of Rel-15 and Rel-16 UEs.

For TDM, Rel-16 WUS and legacy WUS are transmitted on different physical resources. Rel-16 WUS does not affect the performance of legacy WUS. Rel-16 WUSs and legacy WUS occupy two different sets of time resources defined in Rel-15, which is acceptable for resource overhead and time delay.

Single-sequence CDM has been supported as the multiplexing methods between UE-group WUSs. However, configurability of CDM and/or TDM should further studied. Additional TDM with single-sequence CDM can support more UE-groups and better performance. Additional TDM can be considered based on the number of the sequences used for CDM and the number of the UE-group.

Proposal 1: TDM should be supported for Rel-16 WUS and Rel-15 WUS.

The common WUS

In RAN1#95 meeting, common WUS is introduced in NB-IoT as the following agreement:

A common WUS is used to wake up all group WUS UEs monitoring the same WUS (time-frequency) resource if more than one UE group occupies the WUS resource.

- FFS: Whether the above is also applicable for Rel-15 WUS UEs

- FFS: Whether to support waking up a subset of all WUS UE groups

However, there are two FFSs that need to be discussed.

Using the legacy WUS as the common WUS will significantly increase the false alarm for all UEs, when Rel-16 UEs and Rel-15 UEs are monitoring the same WUS resource. Otherwise, Rel-15 UEs will be unware of the new common WUS. Therefore, the common WUS is not applicable for Rel-15 WUS UEs.

Proposal 2:  The common WUS is not applicable for Rel-15 WUS UEs.

There is another issue that whether to design a WUS to wake up a subset of all WUS UE groups.

If UE-group WUSs are transmitted in the same physical resource (single-sequence CDM) and more than one UE-groups are paged, there are only one UE-group woken up in that PO and other UE-groups need to be woken up in next page occasions. Therefore, waking up a subset of all WUS UE groups in CDM is beneficial as shown in Table 1. However, UE needs to detect more sequences, which will increase power consumption and detection complexity of the UEs. It is a tradeoff between false alarm probability, time delay and detection complexity of the UEs to support subset of all WUS UE groups waken up.

Table 1. Support waking up a subset of all WUS UE groups

| sequence 1 | UE-group 1 |
| --- | --- |
| sequence 2 | UE-group 2 |
| sequence 3 | UE-group 3 |
| sequence 4 | UE-group 4 |
| sequence 5 | UE-group 1, 2 |
| sequence 6 | UE-group 1, 3 |
| sequence 7 | UE-group 1, 4 |
| sequence 8 | UE-group 2, 3 |
| sequence 9 | UE-group 2, 4 |
| sequence 10 | UE-group 3, 4 |
| sequence 11 | UE-group 1, 2, 3, 4 |

Observation 1: Supporting waking up a subset of all WUS UE groups in single-sequence CDM is beneficial.

Proposal 3: The characteristic of supporting waking up a subset of all WUS UE groups should consider false alarm probability, time delay and detection complexity of the UEs.

UE grouping

In RAN1#94bis meeting, it has been agreed that UE grouping is based on at least UE ID or some function of UE ID. However, other attributions are not precluded, such as coverage, DRX/eDRX, gap configuration, services.

- Coverage

Grouping UEs with same coverage level will shorten repetition of WUS. However, the coverage of the UE changes as the UE moves, which eNB is not aware. Continuing to use the invalid coverage level to transmit WUS may cause UE to detect WUS incorrectly and miss the paging message. So coverage-based grouping is not preferable.

- DRX/eDRX and gap configuration

eDRX UEs monitor one WUS for every N(N>=1) PO(s). If DRX and eDRX UEs are grouped together, eDRX UEs may detect N POs unnecessarily because the paged DRX UE. Although DRX/eDRX-based grouping is beneficial for eDRX UEs, it will generate two fold number of WUS and not efficient if UE is not massive in the cell. This metric could be further studied.

In Rel-15, there are up to three gaps that eNB can use to determine the start subframes sending WUS, timeoffsetDRX, timeoffset-eDRX-Short and timeoffset-eDRX-Long. Automatically, UEs can be divided into multiple groups by TDM.  It is natural to group UEs with gap configuration in Rel-16.

- Services

NB-IoT and MTC devices belong to different service types. For example, street lamps need to turn on at fixed time each day, but water meters only need to report once a month. It is easy to see that grouping UEs with the same service can significantly reduce the false alarm probability of the UEs that belongs to other service types. However, there are so many service types in the real network, service period or paging probability can be the metric to quantize service type for service-based grouping.

Proposal 4: Service-based grouping should be supported, e.g. service period and paging probability can be the metric to quantize service type.

Sequence design for UE group MWUS

In RAN1#95 meeting, the following agreement about sequence design has been achieved:

Further study false detection (cross/auto correlation) performance properties for the following designs:

- legacy WUS + cover codes,

- legacy WUS + shifted scrambling codes,

- legacy WUS + phase shift + cover code + scrambling bits

  - Including combinations of phase shift, cover code, and/or scrambling bits

Other designs are not precluded.

As can be seen from the above agreement, UE-group ID can be indicated via cover codes, shifted scrambling codes, phase shift or combinations of them. Companies should consider the following factors when design the sequence: 1) False detection (cross/auto correlation) performance properties of the sequences which contain the legacy WUS sequence and new extended sequences. 2) The influence of the Rel-15 WUS 3) Detection complexity and detection delay for UEs.

Conclusions

In this contribution, considerations on UE-group WUS are provided. The following proposals are given.

Observation 1: Supporting waking up a subset of all WUS UE groups in single-sequence CDM is beneficial.

Proposal 1: TDM should be supported for Rel-16 WUS and Rel-15 WUS.

Proposal 2: The common WUS is not applicable for Rel-15 WUS UEs.

Proposal 3: The characteristic of supporting waking up a subset of all WUS UE groups should consider false alarm probability, time delay and detection complexity of the UEs.

Proposal 4: Service-based grouping should be supported, e.g. service period and paging probability can be the metric to quantize service type.

- References

RP-181451, “New WID on Rel-16 enhancements for NB-IoT”, Ericsson Huawei, RAN #80

Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #95, Spokane, USA, November 2018