3GPP TSG RAN WG1 #96		R1-1902177

Athens, Greece, 25th February – 1st March, 2019

Agenda Item:	7.2.6.1.2

Source: 	Sony

Title:	Considerations on UCI enhancements for URLLC

Document for:	Discussion / decision

# Introduction

In RAN1#95, we agreed on the following on UCI enhancements:

Agreements:

- Multiple PUCCHs for HARQ-ACK within a slot should be supported in R16.

In RAN1 Ad-Hoc 1901, we further agreed on the following:

Agreements:

- For a R16 UE, at least two HARQ-ACK codebooks can be simultaneously constructed, intended for supporting different service types for a UE

  - FFS more details (including procedures when applicable)

  - FFS: How to identify a HARQ-ACK codebook

  - FFS applicability to semi-static HARQ-ACK codebook, or dynamic HARQ-ACK codebook, or both

  - FFS more than 2

  - FFS whether or not CBG configuration is supported for Rel-16 URLLC

Agreements:

- Down-select in RAN1#96 for potential A-CSI on PUCCH

  - Opt.1: A-CSI report on PUCCH triggered by DL-scheduling DCI.

    - For measurement source

      - Alt.1: Based on CSI-RS/CSI-IM measurement

      - Alt.2: Based on DMRS/PDSCH/PDCCH measurement

    - For report quantity

      - Alt.1: R15 baseline

      - Alt.2: Delta CQI

      - Alt.3: Delta SINR

    - For report timeline

      - Alt.1: R15 timeline

      - Alt.2: New timeline

  - Opt.2: A-CSI report on PUCCH based on group-common PDCCH (similar to A-SRS triggering in GC-PDCCH in Rel-15) using Rel-15 mechanisms for measurement source, report quatity, and timeline (A-CSI triggered to transmit on PUSCH)

  - Opt.3: No A-CSI on PUCCH due to this SI

This contribution discusses some considerations on the mechanism for realizing multiple HARQ-ACK PUCCH in a slot and the A-CSI enhancements.

# Discussions

## Multiple HARQ-ACK PUCCH per slot

In Rel-15, the size of the HARQ-ACK codebook depends on the set of K1 values (i.e. the cardinal of K1) and the PDSCH opportunities within the time window defined by these K1 values.  An example is shown in Figure 1, where K1 = {1, 2, 3, 4} slots and for a PUCCH in slot n, the codebook would cover acknowledgements for PDSCH within the time window from slot n-4 to slot n-1.  For Type 1 HARQ-ACK codebook, i.e. semi-static codebook, the size of the codebook would enable all PDSCH opportunities that fall within slot n-4 to n-1 to be acknowledged where a NACK is indicated for unscheduled PDSCH opportunities, i.e. in this simplified example there are 2 PDSCH opportunities in each slot thereby giving a codebook size of 8 bits.  For Type 2 HARQ-ACK codebook, i.e. dynamic codebook, the size of the codebook enables all scheduled PDSCH that fall within slot n-4 to n-1 to be acknowledged, i.e. in this example, the codebook size is 2 bits (only 2 PDSCHs are scheduled within the time window).

Figure 1: Rel-15 time window of HARQ-ACK codebook for PUCCH in slot n

The options identified to enable multiple HARQ-ACK PUCCH in a slot [1] are:

- Option 1: Finer K1 granularity: i.e. the time between the start of PUCCH and end of PDSCH is in units of sub-slot (i.e. smaller than a slot)

- Option 2: PDSCH grouping: PDSCHs that points to the same PUCCH resource or group of PUCCH resources in the same PUCCH slot are grouped together, i.e. PDSCH in the group shares the same codebook.

The Rel-15 mechanism for determining the codebook can be reused in Option 1 (finer K1 granularity), where instead of using a slot as the unit, a sub-slot is used [2].  An example is shown Figure 2, where K1 is in units of sub-slot (7 symbols) and K1 = {1, 2, 3, 4} sub-slots.  The time window for multiplexing HARQ-ARQ in the PUCCH in sub-slot m starts from sub-slot m-4 to sub-slot m-1.  The Rel-15 procedure for Type 1 (semi-static) and Type 2 (dynamic) codebook can therefore be reused.

Figure 2: Time window for codebook at Sub-slot m using finer K1 granularity

The drawback noted in using a finer granularity is that it would reduce the time window that can be addressed by the codebook for a PUCCH [1], [3].  For example, for the same range of K1 values, the PUCCH in slot n using finer K1 granularity in Figure 2 can address HARQ-ACK for PDSCH only for slot n-2 and slot n-1 compared to the Rel-15 slot based K1 in Figure 1, which can address HARQ-ACK for PDSCH from slot n-4 to slot n-1.  In order for Option 1 (finer K1 granularity) to address the same time window as Rel-15, more bits are required in the DCI, i.e. in this example the finer K1 values are {1, 2, 3, 4, 5, 6, 7, 8} to address HARQ-ACK for PDSCH from slot n-4 to slot n-1.  It should be appreciated that the whole point of having multiple PUCCH for HARQ-ACK in a slot is to enable URLLC PDSCH to be acknowledged quickly and hence we do not see the point of extending the time range to be the same level as that for Rel-15.  Hence, the same number of bits for slot based K1 can be used for sub-slot based K1.

Observation 1: The time range that can be addressed by a finer K1 granularity is expected to be smaller than that in Rel-15 since the whole objective of using a finer K1 granularity is to reduce the timing between PDSCH and the PUCCH carrying HARQ-ACK.

In Option 2, K1 maintains a granularity of a slot as per Rel-15 but the PDSCHs are grouped according to which PUCCH resource or PUCCH resource group they use for HARQ-ACK.  The PUCCH resource can be grouped for example according to their time resources such that PUCCH with overlapping time resources are grouped [4].  For example in Figure 3, 4 PUCCH resource sets are configured where Set#1 and Set#2 overlap in time and have the same starting symbol and are therefore grouped into Group 1 and Set#3 and Set#4 are grouped into Group 2.  PDSCH that use the PUCCH within a group share the same codebook. For example scheduled PDSCH in slot n-4 has PRI=2 i.e. HARQ-ACK uses PUCCH Set#2 and the scheduled PDSCH in slot n-2 has PRI=1, i.e. HARQ-ACK uses PUCCH Set#1.  Since Set#1 and Set#2 belong to Group 1, these scheduled PDSCH using these PUCCH resources are therefore grouped together and share the same HARQ-ACK codebook and PUCCH resource indicated by the last PDSCH in the group (i.e. PDSCH in slot n-2) will be used to carry HARQ-ACK.

Figure 3: Grouping of PDSCH to PUCCH groups

For Type 1 (semi-static) HARQ-ACK codebook, Option 2 would require that each PUCCH group has a codebook size that can address all PDSCH opportunities defined in the set K1.  In the example in Figure 3, the codebook carried by PUCCH in Group 1 and Group 2 would both have 8 bits, where Group 1 would indicate NACK for the PDSCH in slot n-1 since this PDSCH is not addressed by PUCCH in Group 1, whilst Group 2 would indicate NACK for PDSCH in slot n-4 and slot n-2 since these PDSCH are being addressed by Group 2 PUCCH.  That is the codebook size for PUCCH groups in the same slot are inefficiently used.

Observation 2: For Type 1 HARQ-ACK codebook, grouping of PDSCH to PUCCH resource requires each PUCCH resource in the slot to have the same codebook size since they each address the same time window, which can lead to inefficient use of PUCCH resources.

The PUCCH resource is determined by PRI and may also be dependent upon the CCE index carrying the DL grant for PDSCH.  Hence, the scheduler would need to take into account the HARQ-ARQ codebook to use when scheduling the PDCCH.  Alternatively, instead of using the PRI, a new explicit field can be used to group the PDSCH [3].  However this would introduce additional overhead to the DCI.

Observation 3: Since the CCE index for the DL grant may also be used to determine the PUCCH resource, scheduler would have to take into account the HARQ-ARQ codebook when assigning resources to the PDCCH.

Option 1 of using finer K1 granularity is a cleaner and more straightforward method and hence we have a preference for this option.

Proposal 1: Use finer K1 granularity where the granularity is in units of sub-slot to support multiple HARQ-ACK PUCCH in a slot.

The K1 granularity is in units of sub-slot and the sub-slot size can be configurable, e.g. 2 OFDM symbols or 7 OFDM symbols.  The number of HARQ-ACK PUCCH in a slot is therefore dependent upon this sub-slot size, i.e. 14SSub-slot, where SSub-slot is the size of the K1 sub-slot in number of OFDM symbols.

Proposal 2: The K1 sub-slot granularity size is configurable

Proposal 3: The number of HARQ-ACK PUCCH in a slot is dependent upon the size of the K1 sub-slot size, i.e. 14SSub-slot.

For UE that is configured for eMBB and URLLC services, one of the proposals is to have separate HARQ-ACK codebooks for eMBB and URLLC.  This allows a codebook based on sub-slot K1 granularity to be used for URLLC and the Rel-15 slot based K1 for eMBB.  Different PUCCH resources can also be used for eMBB and URLLC thereby allowing PUCCH with different reliability to be used for eMBB & URLLC.

Proposal 4: Separate HARQ-ACK codebooks are used for eMBB and URLLC services.

In order to run two separate codebooks, the UE needs to distinguish which codebook and PUCCH resource a PDSCH should use.  It is expected that URLLC may use a different DCI format (e.g. compact DCI) to that used for eMBB and hence the codebook and PUCCH resource can therefore be linked to the DCI format.  Alternatively, a different RNTI can be used in scheduling eMBB and URLLC and so the codebook and PUCCH resources are linked to the RNTI.

Proposal 5: The HARQ-ACK codebook and PUCCH resources for carrying HARQ-ACK for eMBB and URLLC PDSCH are distinguished by using different DCI formats or different RNTI.

If the PUCCH carrying HARQ-ACK for eMBB PDSCH and PUCCH carrying HARQ-ACK for URLLC are scheduled in the same slot then both PUCCH are transmitted if they do not overlap in time.  If the PUCCHs overlap in time then the UE should drop the PUCCH carrying HARQ-ACK for eMBB PDSCH.

Proposal 6:  When PUCCH carrying HARQ-ACK for eMBB and PUCCH carrying HARQ-ACK for URLLC occurs in the same slot, transmit both PUCCHs if they do not overlap in time.  Otherwise if they overlap in time drop the PUCCH carrying HARQ-ACK for eMBB PDSCH.

## A-CSI on PUCCH

In Rel-15 A-CSI is triggered by UL grant and carried by PUSCH.  However, it was proposed by some companies [5] [6] that an A-CSI is triggered by a DL grant and carried by a PUCCH and the argument is to enable the UE to provide a CSI report more quickly to the network.  The motivation for doing so under the context of URLLC was not clear.

Firstly we assume that the DL grant that triggers the A-CSI would schedule the PDSCH first followed by the PUCCH carrying the A-CSI, for example, in Figure 4, the DL grant at time t0 would schedule a PDSCH to start at time t1 and the PUCCH carrying the A-CSI which can also carry the HARQ-ACK would be scheduled at time t2, which is after the transmission of the PDSCH.   It is therefore unclear clear why an A-CSI would be useful for the scheduling of a URLLC PDSCH when the A-CSI arrives AFTER the PDSCH has been scheduled.

Figure 4: Triggering of A-CSI on PUCCH from DL grant

Observation 4: The DL grant triggering an A-CSI on PUCCH would likely schedule the PDSCH to arrive before the start of the PUCCH and hence the A-CSI does not provide information for the scheduled PDSCH.

An alternative is to trigger A-CSI using GC-DCI.  The same argument applies, that is the A-CSI must arrive BEFORE the URLLC PDSCH is scheduled otherwise the benefit for that PDSCH is unclear.  However, waiting for the A-CSI to be transmitted on PUCCH before the PDSCH is scheduled would lead to latency which is undesirable in URLLC.

Observation 5: Using GC-DCI to trigger A-CSI on PUCCH is only beneficial if the A-CSI reaches the gNB prior to the scheduling of the PDSCH.  However, this would introduce latency to the scheduling of PDSCH which is undesirable in URLLC.

It has been argued that the A-CSI on PUCCH can be beneficial for the retransmission of PDSCH.  However, under the URLLC, we expect the PDSCH to be highly reliable with BLER of 10-5 or 10-6.  Even if we do not rely on a single shot PDSCH transmission to achieve this reliability, the BLER would be still be very low at 10-3 to 10-4, that is only 1 in a 1000 or 10,000 PDSCH would be retransmitted and can benefit from this A-CSI.  Hence we do not see the benefit of a DL grant triggered A-CSI on PUCCH.  It can be considered in other SI/WI where latency is not an issue.

Observation 6: Since URLLC PDSCH is expected to have very low BLER, the transmission of A-CSI on PUCCH for the retransmission of PDSCH is unlikely to beneficial.

Proposal 7: A-CSI on PUCCH triggered by DL grant or GC-DCI is not considered under eURLLC SI.

# Conclusion

In this contribution we discuss some Layer 1 enhancement on UCI to support URLLC.  We observe the following:

Observation 1: The time range that can be addressed by a finer K1 granularity is expected to be smaller than that in Rel-15 since the whole objective of using a finer K1 granularity is to reduce the timing between PDSCH and the PUCCH carrying HARQ-ACK.

Observation 2: For Type 1 HARQ-ACK codebook, grouping of PDSCH to PUCCH resource requires each PUCCH resource in the slot to have the same codebook size since they each address the same time window, which can lead to inefficient use of PUCCH resources.

Observation 3: Since the CCE index for the DL grant may also be used to determine the PUCCH resource, scheduler would have to take into account the HARQ-ARQ codebook when assigning resources to the PDCCH.

Observation 4: The DL grant triggering an A-CSI on PUCCH would likely schedule the PDSCH to arrive before the start of the PUCCH and hence the A-CSI does not provide information for the scheduled PDSCH.

Observation 5: Using GC-DCI to trigger A-CSI on PUCCH is only beneficial if the A-CSI reaches the gNB prior to the scheduling of the PDSCH.  However, this would introduce latency to the scheduling of PDSCH which is undesirable in URLLC.

Observation 6: Since URLLC PDSCH is expected to have very low BLER, the transmission of A-CSI on PUCCH for the retransmission of PDSCH is unlikely to beneficial.

We therefore propose the following:

Proposal 1: Use finer K1 granularity where the granularity is in units of sub-slot to support multiple HARQ-ACK PUCCH in a slot.

Proposal 2: The K1 sub-slot granularity size is configurable

Proposal 3: The number of HARQ-ACK PUCCH in a slot is dependent upon the size of the K1 sub-slot size, i.e. 14SSub-slot.

Proposal 4: Separate HARQ-ACK codebooks are used for eMBB and URLLC services.

Proposal 5: The HARQ-ACK codebook and PUCCH resources for carrying HARQ-ACK for eMBB and URLLC PDSCH are distinguished by using different DCI formats or different RNTI.

Proposal 6:  When PUCCH carrying HARQ-ACK for eMBB and PUCCH carrying HARQ-ACK for URLLC occurs in the same slot, transmit both PUCCHs if they do not overlap in time.  Otherwise if they overlap in time drop the PUCCH carrying HARQ-ACK for eMBB PDSCH.

Proposal 7: A-CSI on PUCCH triggered by DL grant or GC-DCI is not considered under eURLLC SI.

# References

[1] R1-1901287, “Summary of RAN1#AH1901 Tdocs on  UCI enhancements for URLLC,” OPPO, RAN1 Ad-Hoc 1901

[2] R1-1900897, “UCI Enhancements for eURLLC,” Qualcomm, RAN1 Ad-Hoc 1901

[3] R1-1900070, “UL control enhancements for URLLC,” ZTE, RAN1 Ad-Hoc 1901

[4] R1-1900332, “UL control enhancements for URLLC,” CATT, RAN1 Ad-Hoc 1901

[5] R1-1900044, “UCI enhancements for URLLC,” Huawei, HiSilicon, RAN1 Ad-Hoc 1901

[6] R1-1900159, “UCI Enhancements for NR URLLC,” Ericsson, RAN1 Ad-Hoc 1901