3GPP TSG RAN WG1 #96	R1-1902188

Athens, Greece, 25th February – 1st March, 2019

Agenda Item:	7.2.10.1.1

Source: 	Sony

Title:	Considerations on Downlink based Positioning in NR

Document for:	Discussion / Decision

# Introduction

A new study item “Study on NR positioning support” was approved In RAN plenary #80 [1] . The objective of this study item is to evaluate potential solutions to address NR positioning requirements as defined in TR 38.913, TS 22.261, TR 22.872 and TR 22.804 while considering E911 requirements by analyzing positioning accuracy (including latitude, longitude and altitude), availability, reliability, latency, network synchronization requirements and/or UE/gNB complexity to perform positioning, and taking into account a preference to maximize synergy where possible with existing positioning support for E-UTRAN.

In RAN1 adhoc meeting 1901, the following agreements on DL-based positioning were made [2] :

Agreement:

- NR should support timing (DL-TDOA) based DL-only positioning techniques in FR1 and FR2

- NR should support angle (DL-AoD) based DL-only positioning techniques with at least beam sweeping at least at the gNB in FR1 and FR2

  - Note: This does not necessarily need different reference signals from the timing based DL-only positioning techniques

Agreement:

Enhancements to performance obtainable based on existing NR DL reference signals (e.g., based on extensions to current reference signals or with new reference signals) are necessary to meet accuracy requirements at least in some scenarios

Agreement:

NR DL PRS design for FR1 and FR2 supports:

- Configurable NR DL PRS signal bandwidth

  - FFS granularity of configuration, relationship with BWPs, whether the configuration is cell and/or UE specific

- Configurable NR DL PRS signal numerology (SCS)

  - FFS configurability of CP for NR DL PRS

- Configurable NR DL PRS frequency and time allocation

- Use of DL beam sweeping / alignment

  - i.e. beam alignment of gNB DL PRS transmission and UE reception of DL PRS

- Localized in time NR DL PRS transmissions with periodic and/or on-demand resource allocation

  - FFS signaling details

- Dedicated NR DL PRS resources - time-frequency grid at resource block level

  - PRS transmitted in one cell may or may not collide with PRS transmissions in other cell

    - e.g. frequency vShift/comb-offset is the same or different for two different PRSs in the same RB

  - There is no data/control transmission in time-frequency grid of dedicated NR-DL PRS resources

  - FDM multiplexing with other signals at RE level inside of PRS time-frequency grid is precluded

Agreement:

- A PRS resource should have a PRS resource ID and a PRS sequence should have a PRS sequence ID

Agreement:

- NR DL PRS resource is defined as a set of resource elements used for NR DL PRS transmission that can span multiple PRBs within N (1 or more) consecutive symbol(s) within a slot, where N is FFS.

  - In any OFDM symbol, PRS resource occupies consecutive PRBs

  - FFS if multiple symbols can be non-consecutive

  - FFS on PRS Resource Set

- FFS: whether support of PRS frequency hopping is needed

Agreement:

- NR supports RSTD measurements for NR DL PRS

  - FFS: RSTD b/w different configured PRS resources, beams, TRPs, cells, etc.

- NR supports RSRP measurement for NR DL PRS

- FFS if NR supports RSTD measurement quality metric for NR DL PRS

- FFS other measurements

# In this contribution, we discuss our view on downlink-based positioning solutions in NR, particularly on PRS design and configuration to maintain the beam alignment.

# PRS Design and Configuration

In principle, the properties/characteristics of PRS in LTE could be re-used. However, the substantial NR properties, such as beam operation makes PRS in NR may have a unique design. In LTE, the PRS is transmitted under the assumption the eNB has an omni-directional or sector antenna, at least there is no indication of the beam aspect of the antenna being used in transmitting the PRS. Similarly, the UE is expected to use an omni-directional or relatively wide beam antenna for receiving the beam carrying the reference signal.

In NR, particularly in FR2, the operation with beam forming, with beams transmitted in multiple directions (facilitated by MIMO antenna scheme), is commonly used. Each beam may have a narrow beam pattern with high gain in specific directions. For example, the existing NR rel-15 specifications can accommodate up to 64 beams for synchronization signal block (SSB) transmission. A similar principle can also be applied in NR PRS transmission. As illustrated in Figure 2, a UE needs to receive multiple beam-forming signals from gNBs or TRPs for the triangulation in OTDOA. In addition, the UE is expected to receive a beam-forming signal that can indicate a certain direction. This can be further exploited in order to improve the positioning accuracy. This operation can be facilitated by introducing beam-identity on each beam-formed carrying PRS signal. Considering the positioning measurement report is the legacy RSTD information and the beam information, the positioning measurement error can be minimized. In addition, the UE positioning estimation by LS can be performed with less than three gNBs. This is particularly the case when the positioning measurements quality of one or two gNBs are significantly better than the others.

Figure 1: Illustration of OTDOA with beam-forming signal from gNBs.

In LTE, Positioning reference signal (PRS) has been carefully designed. Thus, some properties and configuration can be re-used in NR. This should be part of the study as well. However, we can consider at least to use the same sequence that is a length-31 gold sequence, PRS muting operation, and the concept of PRS occasion and PRS periodicity. PRS muting can be enabled by the gNB to minimize any potential interference. PRS occasion is the duration of consecutive subframe/slot carrying PRS. The parameter may be the same but the actual values may not be the same due to the nature of NR transmission.

As illustrated in Figure 1, PRS transmission is transmitted in a certain beam direction. A gNB typically need to cover multiple directions. For example, in NR rel.15 initial access, the transmission of SSBs in an SSB burst set that can cover multiple directions. Similarly, the gNB can perform beam-sweeping for PRS transmission as agreed in the previous meeting. Here, we need to define a multiplexing (resource mapping) of beam-based PRS transmission. A set of PRS resources (e.g. resource elements) can be configured to represent a certain beam direction as illustrated in Figure 2 below. Here, the multiplexing of beam-based PRS transmission from multiple gNBs is also illustrated in a dedicated NR DL PRS resources.

Proposal 1: A dedicated NR DL PRS resources (e.g. one or multiple slot) is designed to accommodate multi-beam transmission (e.g. beam-sweeping) from one or more gNBs.

Proposal 2: A set of PRS resources represent a beam direction of PRS transmission.

There may be a gap between set of resources to accommodate beam-switching operation. The transmission of PRS in all beam directions should be confined within a known duration of time. For example, within a certain burst duration. The PRS-burst can be consecutively repeated or periodically transmitted with a certain interval. The UE knows the PRS configuration. Based on the received PRS resources the UE can identify the gNB beam direction of the associated positioning measurement (RSTD).

Figure 2: Illustration of beam-based PRS multiplexing

There may also be a need to minimize UE positioning measurement time. This can be facilitated by multiplexing of beam-based PRS transmission from a gNB in frequency domain and thus, minimize the PRS burst duration. This is illustrated in Figure 3. Here, a PRS transmission with repetition is also illustrated. As comparison, the illustration in Figure 2 represents time-domain multiplexing operation. In practice, depending on the PRS bandwidth size and the number of beams, the beam-based PRS transmission can be multiplexed in both frequency and time domain. On the arrangement, it can be frequency domain first then followed by time-domain multiplexing.

Note that the PRS transmission with frequency domain multiplexing may also subject to gNB limitations. The frequency domain multiplexing with the need to cover multiple beam directions is typically supported with digital beam-forming operation.

Figure 3: Beam based PRS transmission with frequency domain multiplexing

Proposal 3: A set of PRS resources represents a beam direction can be multiplexed in either time-domain and/or frequency-domain depending on the gNB capability.

In order to facilitate the positioning measurement with multi-beam operation and to assist the local server to utilize the beam information for the positioning estimation, LS collects the gNB beam configuration. Furthermore, UE is informed of this configuration by LS via LPP protocol. The detailed mechanism on LPP protocol will be handled by RAN2. Furthermore, UE may optionally inform LS on its beam configuration. This is to facilitate an LS to obtain a positioning measurement with a pair of angle of departure (AoD) and angle of arrival (AoA).

Proposal 4: On supporting positioning measurement with multi-beam operation, location server collects the gNB beam configuration for the PRS transmissions and optionally UE beam configuration.

# UE measurement and report

Assuming the configuration of PRS transmission, including the beam configuration, is known by the UE, A UE can perform positioning measurement based on the received PRS signal. A UE may attempt to perform measurement based on each set of PRS resources representing a beam direction. The UE then performs selection among multiple positioning measurement results typically by selecting the best positioning measurement. Here, the UE may also identify the set of PRS resources which provide the best positioning measurement and indirectly known the associated beam information of that PRS resources. Thus, the UE reports the positioning measurement results (e.g. RSTD) and the associated beam information related to the set of PRS resources of that measurement. Assuming gNB introduces an index to each set of PRS resources then the beam information can be in a form of index information. This is basically an implicit information of PRS angle-of departure (AoD) from the gNB.

Furthermore, a UE uses a certain UE RX beam for the PRS signal reception. The UE can receive the PRS signal on multiple set of PRS resources with an RX beam or multiple RX beams. If the best positioning measurement is obtained with a UE RX beam facing to a certain direction. The UE may also report this beam information. We would expect this is an optional feature as some UE may not support this operation depending on the UE implementation.

Proposal 5: The UE reports RSTD and the associated beam information related to the set of PRS resources of that measurement (e.g. implicit information of PRS angle-of departure (AoD)).

Proposal 6: Optionally the UE can also report the associated UE RX beam information related to the set of PRS resources of that measurement (e.g. implicit information of PRS angle-of arrival (AoA)).

## Adaptive PRS transmission

In our previous contribution, we have summarized NR positioning requirements. The requirements are targeting numerous use-cases/scenarios. There are multiple dimensions of performance metric (horizontal accuracy, vertical accuracy, availability, latency, etc) and various requirement levels (sub-1m, 10m, 50m, etc).

It may be difficult to identify one technical solution that fits to all use-cases and scenarios. In OTDOA, an accurate positioning result typically requires wide bandwidth and many reference signal (e.g. PRS) subframes. Many reference signal subframes may increase the latency. Yet, some use-cases require high accuracy and short latency. In practice, we consider the majority of use-cases would not require high accuracy and only require reasonable latency. Nevertheless, the solution should also cover the more demanding use cases with very high accuracy requirement, e.g. sub 1m.

Based on the above observation, an adaptive/flexible approach to positioning techniques may be beneficial. A flexible approach can include the combination of RAT dependent and RAT independent techniques. Another consideration is the ability to flexibly configure OTDOA positioning with a goal of achieving the positioning requirements. It has been agreed that the flexibility can be achieved by a possibility allocating of on-demand NR DL PRS resource allocation. By doing so, the UE may receive more reference signals in order to obtain higher positioning accuracy. A location server is assumed to know the required positioning accuracy. It can trigger (e.g. via LPPa protocol) the selected gNBs to allocate on demand resource allocation for NR DL PRS transmission.

Proposal 7: On demand resource allocation for NR DL PRS transmissions is triggered by the required positioning accuracy.

Proposal 8: A location server can trigger (e.g. via LPPa protocol) the selected gNBs to allocate on demand resource allocation for NR DL PRS transmission.

# Conclusion

In this contribution, we discussed our view on downlink-based positioning particularly on the operation of multi-beam in both gNB and UE. We have made the following proposals:

Proposal 1: A dedicated NR DL PRS resources (e.g. one or multiple slot) is designed to accommodate multi-beam transmission (e.g. beam-sweeping) from one or more gNBs.

Proposal 2: A set of PRS resources represent a beam direction of PRS transmission.

Proposal 3: A set of PRS resources represents a beam direction can be multiplexed in either time-domain and/or frequency-domain depending on the gNB capability.

Proposal 4: On supporting positioning measurement with multi-beam operation, location server collects the gNB beam configuration for the PRS transmissions and optionally UE beam configuration.

Proposal 5: The UE reports RSTD and the associated beam information related to the set of PRS resources of that measurement (e.g. implicit information of PRS angle-of departure (AoD)).

Proposal 6: Optionally the UE can also report the associated UE RX beam information related to the set of PRS resources of that measurement (e.g. implicit information of PRS angle-of arrival (AoA)).

Proposal 7: On demand resource allocation for NR DL PRS transmissions is triggered by the required positioning accuracy.

Proposal 8: A location server can trigger (e.g. via LPPa protocol) the selected gNBs to allocate on demand resource allocation for NR DL PRS transmission.

# References

- RP-181399, “New SID: Study on NR positioning support”

- Chairman’s notes RAN1 meeting 95, Spokane USA, November 2018