3GPP TSG RAN WG1 Meeting #96	R1-1902191

Athens, Greece, 25th February – 1st March, 2019

Agenda Item:	6.2.1.1

Source: 	Sony

Title:	UE-group wake-up signal for MTC

Document for:	Discussion and Decision

# Introduction

In RAN1 meeting #95 [1], the following related agreements on UE-group wake up signal for MTC were made:

- For multiplexing between Rel-16 UE-group WUS and Rel-15 WUS, further evaluate and down select among the following options

- TDM

- FDM

- single-seq CDM

- single-seq CDM+TDM

- single-seq CDM+FDM

- FFS whether legacy WUS is the common WUS for all new UEs or only a part of new UEs.

- For multiplexing between different Rel-16 UE-group WUS, further evaluate and down select among the following options

- single-seq CDM

- FDM

- single-seq CDM+TDM

- single-seq CDM+FDM

Note: At least the maximum number of UE groups should be considered.

- The number of UE groups is configurable and broadcasted in SIB.

- FFS: Further details on the number of UE groups. For example, whether it is per PO or per gap configuration of a PO

In this contribution, we discuss some aspects on the operation of UE-group wake-up signal for MTC (MWUS).

# Discussion

Subgrouping of UEs within a MWUS allows only a subgroup of UEs to be woken up by the MWUS for a paging occasion (PO). The UE subgrouping operation will reduce the number of UEs having to wake up unnecessarily to detect MPDCCH and decode PDSCH. Thus, it reduces UE power consumption for such UEs.

# Multiplexing of MWUS

UE-group MWUS in Rel-16 should consider coexistence with MWUS operation introduced in Rel-15 (legacy). One of the easiest solutions is to allocate different WUS physical resources for UE-group MWUS and legacy MWUS. Moreover, MWUS occupies 2 PRBs and essentially an eNB can configure 3 MWUS resources in a narrowband (FDM-ed approach). Three UE-groups can be introduced for MTC UE Rel-16 and MTC UE Rel-15 can be assigned to any of these groups, as MTC Rel-15 UEs are not aware of UE grouping.

CDM and TDM have also been considered for the WUS multiplexing scheme. The advantage/disadvantage of each scheme is discussed in [2]. In our understanding CDM represented with multiple sequences exposes many issues (UE complexity, largest delay, performance issues). The main issue with TDM is a potentially larger delay. The main issue with FDM is the limited number of groups (up to 3).

Proposal 1: Support FDM for multiplexing between legacy Rel-15 MWUS and Rel-16 MWUS.

Proposal 2: Support single-seq CDM+FDM for multiplexing between different Rel-16 UE-group MWUS

There was a discussion during the online session in RAN1#94 on the introduction of a super group MWUS or common MWUS. In such case, basically there is a dedicated MWUS to wake up all UEs assigned to the corresponding paging occasion. Super-group is in addition to the individual UE group MWUS. The benefit of common MWUS is that the eNB only needs to transmit the common MWUS in case the eNB needs to wake up all UEs. However, from the UE point of view, this could increase UE power consumption and complexity as the UE needs to detect both common MWUS and group specific MWUS, especially if the UE needs to detect multiple MWUS resources. This will make MWUS Rel-16 worse than MWUS Rel-15 in terms of UE power consumption and complexity.

Proposal 3: MTC Rel-16 does not support common MWUS operation which requires the UE to receive and detect MWUS in multiple MWUS resources.

A challenging case is when a cell requires more than 3 UE-groups, considering the FDM resources are limited. Introducing more than 3 UE-groups can be beneficial in a dense network. In a legacy LTE system, the probability of UEs having the same paging frame as a function of the number of paging frames is given in Table 1. While a reduction of the occasions of many UEs being paged at the same time can be achieved by allocating many PFs, this solution might not be the optimal one as more resources are being used to carry paging information.

Table 1: Number of PF versus Probability UEs with same PF

| N | UE_ID mod N | % UEs have same paging frame |
| --- | --- | --- |
| 1 | 0 | 100% |
| 2 | 0,1 | 50% |
| 4 | 0,1,2,3 | 25% |
| 8 | 0,1,2,…,7 | 12.5% |
| 16 | 0,1,2,…,15 | 6.75% |

In scenarios where the number of paging frames is limited or can be configured to be a small number, introducing more than 3 UE-groups would be clearly beneficial. The advantage is that the network can have more flexibility and the number of UEs being paged unnecessarily can be further reduced. In terms of the maximum supported number of UE-groups, we propose to maximize the benefit of the UE-group mechanism. In order to limit the probability of overhearing to below 15% we propose the maximum number of UE groups supported is at least 6.

Proposal 4: The introduction of UE-group should be designed to maximize the benefit of UE-group mechanism. We propose the maximum number of UE groups supported is at least 6.

The usage of UE-group ID may support the operation where the eNB pages a group, some groups, or all groups. This information should be carried out in UE-group ID indication embedded in MWUS for the case of MWUS with single-seq CDM. The UE needs to decode the group ID information in order to decide whether to continue with paging reception or go to sleep. The UE-group ID can be included in a form of bitmap operation as illustrated in Table 2. Here, an example is given when the supported number of groups is 4. It can be observed that the table size is significantly bigger in proportional to the maximum number of UE groups. Furthermore, most of the possible operation/combination is to page some of the groups. The number of operations/combinations will be significantly less in case MWUS with single-seq CDM for some of the groups is not supported. However, it may have some potential issues if MWUS group-ID is designed to page a group and all groups. This can be further investigated. We consider RAN1 needs to decide whether MWUS with single-seq CDM supports paging for some groups or not. This will affect the UE-group MWUS design and UE receiver to detect the MWUS.

Proposal 5: RAN1 needs to decide whether MWUS with single-seq CDM supports paging for some groups or not.

Table 2: UE-group ID with bitmap operation

| No. | Bitmap | UE-Group ID being paged |
| --- | --- | --- |
| 1 | 1111 | 1,2,3,4 |
| 2 | 1110 | 2,3,4 |
| 3 | 1101 | 1,3,4 |
| 4 | 1100 | 1,2,4 |
| 5 | 1011 | 1,2,3 |
| 6 | 1010 | 3,4 |
| 7 | 1001 | 2,4 |
| 8 | 1000 | 2,3 |
| 9 | 0111 | 1,4 |
| 10 | 0110 | 1,3 |
| 11 | 0101 | 1,2 |
| 12 | 0100 | 4 |
| 13 | 0011 | 3 |
| 14 | 0010 | 2 |
| 15 | 0001 | 1 |

# 3	Conclusions

In this contribution, the following proposals are made:

Proposal 1: Support FDM for multiplexing between legacy Rel-15 MWUS and Rel-16 MWUS.

Proposal 2: Support single-seq CDM+FDM for multiplexing between different Rel-16 UE-group MWUS

Proposal 3: MTC Rel-16 does not support common MWUS operation which requires the UE to receive and detect MWUS in multiple MWUS resources.

Proposal 4: The introduction of UE-group should be designed to maximize the benefit of UE-group mechanism. We propose the maximum number of UE groups supported is at least 6.

Proposal 5: RAN1 needs to decide whether MWUS with single-seq CDM supports paging for some groups or not.

# References

- RAN1 #95 Chairman notes, Spokane, USA November 2018

- R1-1809521, “Summary of 6.2.1.1 UE group WUS” Qualcomm, Göteborg, August 2018