3GPP TSG RAN WG1 #96		R1-1902204

Athens, Greece, 25th February – 1st March, 2019

Source:	Panasonic

Title: 	Discussion on sidelink resource allocation in mode 1 for NR V2X

Agenda Item:		7.2.4.3

Document for:	Discussion, Decision

Introduction

Following was agreed in last meeting,

Agreements:

- When NR Uu schedules NR SL mode 1, both type 1 and type 2 configured grants are supported for NR SL

Agreements:

- LTE Uu to schedule NR sidelink mode 1 is supported:

  - The support is done based on type 1 configured grant with configuration restricted to time/frequency resources & periodicity, with the condition that no additional function/procedure is to be introduced for LTE Uu

  - Both DCI based scheduling and type 2 configured grant scheduling are not supported for scheduling NR sidelink mode 1

- Send an LS to RAN2 – Phillipe (HW), R1-1901410, which is approved with the final LS in R1-1901445

This contribution is resubmission of R1-1900645 and to continue the discussion on sidelink resource allocation in mode 1 for NR V2X. The focus here is to use NR Uu to schedule NR sidelink resources.

Discussion

PSFCH is supported for unicast and groupcast communication based on previous agreements. The straightforward procedure for unicast/groupcast communication based on LTE-V2X principle would have four steps, as shown in Figure 1 below. The gNB transmits sidelink DCI to Tx UE who transmits PSCCH/PSSCH to Rx UE. After reception of PSCCH/PSSCH, Rx UE feedbacks PSFCH to Tx UE who feedbacks HARQ-ACK to gNB via PUCCH (or UCI in PUSCH). It causes large overhead and latency although it has merit on supporting any scenario regardless of Rx UE’s coverage status. When PSSCH’s data size is small, control overhead (sidelink DCI/PSCCH/PSFCH/PUCCH) can be larger than data.

Figure 1 Straightforward procedure for unicast/groupcast communication

Observation 1: Straightforward procedure for unicast/groupcast communication in mode 1 based on LTE-V2X principle causes large overhead.

In case Tx UE and Rx UE are both in the same cell, some simplified behaviors would be possible i.e. 1) The gNB transmits sidelink DCI to both Tx UE and Rx UE while Tx UE only transmits PSSCH to Rx UE and 2) Rx UE directly feedback HARQ-ACK to gNB. For 1), the example is shown in Figure 2 (option 1). For 2), the example is shown in Figure 3 (option 2). By either approach, the signaling overhead and latency are reduced. Please note in option 1, gNB may directly transmit PSCCH to both Tx UE and Rx UE as well.

Above discussion is the procedure on unicast/groupcast communication in mode 1. For broadcast, the same proposal like option 1 could also be applied. The difference is just there is no HARQ-ACK feedback from Rx UE to gNB, as shown in Figure 4.

The optimization is mainly based on the assumption that both Tx UE and Rx UE are in the same cell. In case Rx UE is out of coverage, it seems straightforward solution shown in Figure 1 would be more reasonable approach. On the other hand, this case should be simply covered by mode 2 which can reduce overall complexity of the system design. Assuming gNB coverage is sufficiently large and the universal usage of mode 2, mode 1 is optimized only for the case both Tx and Rx UEs in the same cell would be reasonable design.

Proposal 1: Simplified behaviors are considered for communication procedure in mode 1, i.e. 1) The gNB directly transmits sidelink DCI or SCI to both Tx UE and Rx UE and 2) Rx UE directly feedbacks HARQ-ACK to gNB.

Figure 2 Simplified communication procedure for unicast/groupcast (option 1)

Figure 3 Simplified communication procedure for unicast/groupcast (option 2)

Figure 4 Simplified communication procedure for broadcast (option 3)

In order to show an institutive impression on latency value in mode 1, taking straightforward solution with 30 kHz as an example and the total average latency of user plane is 2.35ms as shown in Table 1 in appendix part. Based on option 1, the average latency of user plane could be reduced to 2.3 ms, which is around 2% improvement on latency performance. The HARQ RTT latency of straightforward solution is 3.5ms while HARQ RTT latency of option 1 is 3ms, which has 14% latency improvement.

Regarding HARQ-ACK resource determination, fixed or pre-configured timing (e.g., from PSCCH/PSSCH to PSFCH) is desirable in our view, as what is discussed in our companion contribution [1] for mode 2. This approach can be applied to any of above mentioned solutions, for example in straightforward solution, PSFCH resource is not indicated in sidelink DCI/PSCCH. Instead, PSFCH’s resource is implicitly determined by associated PSCCH/PSSCH transmitted by Tx UE. By such approach, the signalling overhead is saved.

Proposal 2: HARQ-ACK resource for unicast/groupcast is implicitly determined by associated PDCCH or PSCCH/PSSCH.

Conclusion

In this contribution we discussed resource allocation mechanism for mode 1. Based on the discussions, followings are observed and proposed,

Observation 1: Straightforward procedure for unicast/groupcast communication in mode 1 based on LTE-V2X principle causes large overhead.

Proposal 1: Simplified behaviors are considered for communication procedure in mode 1, i.e. 1) The gNB directly transmits sidelink DCI or SCI to both Tx UE and Rx UE and 2) Rx UE directly feedbacks HARQ-ACK to gNB.

Proposal 2: HARQ-ACK resource for unicast/groupcast is implicitly determined by associated PDCCH or PSCCH/PSSCH.

Reference

[1] R1-1902202, “Discussion on PSFCH channel design in NR V2X”, Panasonic

[2] 3GPP TR 37.910 V1.0.0 (2018-09), “Study on Self Evaluation towards IMT-2020 Submission”

- Appendix

30 kHz SCS case with some simplified calculation carried out by the following.

Table 1 Latency calculation for straightforward solution and option 1

|  |  |  | Straightforward solution | Option 1 |
| --- | --- | --- | --- | --- |
| ID | Component | Notations | Value | Value |
| 0 | SR/BSR report | Ignored here as no difference between two solutions | Not considered | Not considered |
| 1 | DL control channel transfer | T1 = (tBS,tx + tFA,DL) + tDL_duration + tUE,rx | 1 ms | 1 ms |
| 1.1 | BS processing delay | tBS,tx Sidelink DCI is prepared | 0.5ms of the combination of tBS,tx and tFA,DL | 0.5ms of the combination of tBS,tx and tFA,DL |
| 1.2 | DL Frame alignment (transmission alignment) | tFA,DL It includes frame alignment time, and the waiting time for next available DL slot |  |  |
| 1.3 | TTI for sidelink DCI transmission | tDL_duration | 0.5ms of the combination of tDL_duration and tUE,rx | 0.5ms of the combination of tDL_duration and tUE,rx |
| 1.4 | Tx UE processing delay | tUE,rx The time that the sidelink DCI is decoded and PSCCH/PSSCH is prepared |  |  |
| 2 | Sidelink data transfer from Tx UE | T2 = (t2.1 + t2.2) | 1 ms | 1 ms |
| 2.1 | TTI for PSCCH/PSSCH transmission | t2.1 | 0.5ms | 0.5ms Note PSCCH is not sent by Tx UE |
| 2.2 | Rx UE processing delay | t2.2 | 0.5ms | 0.5ms |
| 3 | PSFCH transfer from Rx UE | T3= (t3.1 + t3.2) | 0.5ms | 0 ms |
| 3.1 | TTI for PSFCH transmission | t3.1 | 2 symbols (0.07ms) | 0 ms |
| 3.2 | Tx UE processing delay | t3.2 | 14 symbols (0.43ms) | 0 ms |
| 4 | PUCCH transmission | T4 = (tUE,tx + tFA,UL) + tUL_duration + tBS,rx | 1 ms | 1 ms |
| 4.1 | UE processing delay | tUE,tx The time PUCCH is prepared | 0.5ms of the combination of tUE,tx and tFA,UL | 0.5ms of the combination of tUE,tx and tFA,UL |
| 4.2 | UL frame alignment (transmission alignment) | tFA,UL It includes frame alignment time, and the waiting time for the next available UL slot |  |  |
| 4.3 | TTI for ACK/NACK transmission | tUL_duration | 0.5ms of the combination of tUL_duration and tBS,rx | 0.5ms of the combination of tUL_duration and tBS,rx |
| 4.4 | BS processing delay | tBS,rx The time interval between the ACK is received and the ACK is decoded. |  |  |
| - | Total one way user plane latency for DL | TUP= T1 + T2 + p×THARQ where p is the probability of re-transmissions and 0.1 THARQ = T1 + T2 + T3 + T4 | TUP = 2.35 ms THARQ = 3.5ms | TUP = 2.3 ms THARQ = 3 ms |