3GPP TSG RAN WG1 Meeting #96 R1-1902237 Athens, Greece, February 25th – March 1st, 2019 CR-Form-v11.2. [DRAFT] CHANGE REQUEST. 38.213; CR; rev; Current version:; 15.4.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; X; Core Network. Title:; Draft CR on PUCCH Repetitions. Source to WG:; Samsung. Source to TSG:. Work item code:; NR_newRAT-Core; Date:; 2019-02-25. Category:; F; Release:; Rel-15. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16). Reason for change:; Number of PUCCH repetitions does not account for number of UCI bits.. Summary of change:; Introduce scaling of number of PUCCH repetitions by number of UCI bits.. Consequences if not approved:; Number of repetitions is unnecessarily increased for ‘small’ UCI payloads (or BLER targets cannot be met for ‘large’ UCI payloads). This increases UL overhead and latency for when a UE considers HARQ-ACK transmission complete which in turn further decreases data rates and UE throughput due to PUCCH repetitions.. Clauses affected:; 9.2.6. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:; Isolated impact analysis: The CR affects only how a UE and a gNB determine a number PUCCH repetitions. If the gNB implements the CR and the UE does not, the UE may transmit the PUCCH with a different number of repetitions than expected by the gNB. If the UE implements the CR and the gNB does not, the gNB may receive the PUCCH with different number of repetitions than transmitted by the UE..

### 9.2.6	PUCCH repetition procedure

For PUCCH formats 1, 3, or 4, a UE can be configured a number of slots, , for repetitions of a PUCCH transmission by respective nrofSlots. For PUCCH format 1, . For PUCCH formats 3 and 4, .

*** Unchanged text is omitted ***