3GPP TSG RAN WG1 Meeting #96	R1-1902258

Athens, Greece, Feburary 25th – March 1st, 2019

Agenda item:	7.2.2.2.2

Source:	Samsung

Title:	Enhancements to Initial Access Procedure for NR-U

Document for:	Discussion and Decision

# Introduction

Agreement:

- It is considered beneficial to enhance paging opportunities using one or more of the following mechanisms:

  - Increased time-domain paging occasions or paging monitoring occasions

    - This can enable additional paging occasions outside of DRS

- Note: Parts or all of the above enhancement may fall under RAN2 purview and may not require any further study in RAN1

Agreement:

- It is considered beneficial to report a metric to represent channel occupancy or medium contention in addition to RSSI.

  - The exact definition of the metric(s) is left for the WI

  - Note: The above is a confirmation of RAN2’s recommendation for the same

Agreement:

- For RLM, the following recommendations are considered beneficial for further design in the WI:

  - Identifying a set of RLM-RS, e.g., DRS, SS/PBCH blocks, CSI-RS

    - Transmission of the RS in a COT may be subject to LBT

  - Identifying which set(s) of RLM-RS are used for in-sync and out-of-sync evaluations

    - For example, determining which RLM-RS within or outside the DMTC for RLM can be utilized for in-sync and out-of-sync evaluations

  - Potential definition of a metric, e.g., Rel-15 out-of-sync indication or new metric, to accurately identify instances of unsuccessful detection of RLM-RS. Whether/how to report such a metric to higher layers is to be further studied.

Agreement:

- It is considered beneficial for the time-domain measurement windows for RRM measurements and RLM to be different

In RAN1 #95 meeting [1], the following agreements regarding the initial access and mobility procedure were made:

This contribution discusses the details of the agreed study points and other aspects related to the initial access and mobility procedure for NR-U:

- Enhancement to SS/PBCH Block Transmission

- Enhancement to Random Access Procedure

- Enhancement to RRM Measurement

- Enhancement to RLM Procedure

# Enhancement to SS/PBCH Block Transmission

In NR, the transmission of a SS/PBCH burst set is confined within a half frame and starts from the slot of the half frame. For example, as illustrated in Figure 1, for SCS of SS/PBCH block as 30 kHz, the transmission of SS/PBCH burst set consisting from maximum 8 SS/PBCH blocks can be confined with the first 2 ms of the half frame (e.g. 1 slot with 0.5 ms duration can contain 2 SS/PBCH blocks). A UE can determine the timing information from the reception of a SS/PBCH block by getting the SS/PBCH block index from the DMRS of PBCH (and potentially from PBCH content as well for above 6 GHz), and getting the SFN and half frame indicator from the PBCH content (half frame indicator can be also from the DMRS of PBCH as well for below 3 GHz). SS/PBCH blocks with the same SS/PBCH block index in different burst sets are assumed to be QCLed, the UE can assume to perform combining of the corresponding signals and channels in the QCLed SS/PBCH blocks to improve performance on synchronization, cell detection, and system information delivery.

Figure 1 Illustration of SSB transmission in a NR licensed spectrum.

In NR-U, the transmission of SS/PBCH blocks can be subject to the sensing result of Listen-Before-Talk (LBT), such that the UE cannot always expect to receive the SS/PBCH blocks periodically, which challenges the receiver implementation on combining, even the QCL assumption still holds. Moreover, due to the uncertainty of channel access from LBT, UE may not be able to receive any SS/PBCH block for a long time duration and the delay of initial cell search may be remarkably enlarged, hence, enhancement of the channel access of SS/PBCH block needs to be studied in NR-U, to address the loss in channel access opportunities subject to failed LBT.

Rel-13 LTE-LAA can be a starting point for studying the channel access scheme of SS/PBCH blocks in NR-U, by assuming the same NR SS/PBCH burst set mapping pattern (e.g. Figure 1) to address potential multi-beam operation as a baseline. Hence, for transmission of SS/PBCH blocks in NR-U DRS, a transmission timing configuration (short for DTTC) method can be considered to be beneficial for NR-U, wherein the configuration contains a window periodicity, a window duration, and a window offset.

For a given frequency layer, at least one DTTC shall be supported and considered as a default configuration from a UE’s perspective for initial access purpose, and other configurations, e.g. at least the window periodicity, can be supported to address the flexibility from a network’s perspective. It would be beneficial to align the periodicity of DRS with the periodicity of SS/PBCH block, such that UE assumes a received SS/PBCH block is always located in a DRS window. .

The window duration shall not exceed the measurement gap, e.g. 6 ms for FR1, and it shall be further restricted to 5 ms due the minimum configurable periodicity of SS/PBCH block is 5 ms, such that there is confusion of SS/PBCH block in the previous half frame transmitted late due to LBT and the one in the next half frame. Furthermore, to align with the proposal of cyclically wrapping SS/PBCH blocks dropped due to LBT, it is beneficial to determine the default window duration as 4 ms, such that the SS/PBCH blocks with different SS/PBCH block index has the same channel access opportunities.

DTTC is mainly for rate matching purpose, and a UE can utilize configurations in DTTC, combined with the indication of actual transmitted SS/PBCH blocks to perform rate matching for serving cell. In this sense, DTTC can be separately specified and configured from DMTC, wherein the later one is mainly for measurement purpose. Meantime, DTTC can also be utilized for UE to perform DRS based RLM, e.g. the DTTC window is the same as RLM-DMTC.

The transmission of the SS/PBCH burst set is assumed to be confined within the window configured by the DTTC, and there can be multiple predefined starting locations for the transmission in order to compensate the loss on channel access opportunities due to LBT. In the last meeting, it was identified that it is beneficial to expand the maximum number of candidate SS/PBCH block locations within a DRS window, and the actually transmitted SS/PBCH blocks are selected from the candidate SS/PBCH block locations. There are still remaining details left, for example the maximum number of candidate locations, maximum of SS/PBCH blocks in a DRS window, the granularity between the grouped SS/PBCH blocks, duration of DRS window, timing information acquisition, and QCL assumption acquisition.

In [2], we proposed a scheme of periodically wrapping around the truncated SS/PBCH blocks to the end of the actually transmitted SS/PBCH blocks, and Figure 2 illustrates an example of this scheme for FR1 unlicensed. The example is also summarized in the terminology from the agreement of last meeting for clarification:

- DRS transmission window duration: 4 ms;

- DRS subcarrier spacing: 30 kHz;

- Maximum number of candidate SS/PBCH blocks within a DRS window: Y = 16;

- Shift granularity between group of SS/PBCH blocks: 0.5 ms (e.g. 1 slot of 30 kHz SCS);

- Maximum number of transmitted SS/PBCH blocks: X = 8;

- Duration of the transmitted DRS within the window: up to 2 ms (depending on the configuration of actually transmitted SS/PBCH blocks);

- Mapping between candidate SS/PBCH block location index i (e.g. i = 0, 1, …, Y-1) to SS/PBCH block index i_SSB: i_SSB = i mod X (e.g. periodically wrapping around);

- Timing information acquisition: the timing offset o_SSB = floor(i/X) is indicated to the UE by the DMRS sequence of PBCH in the corresponding SS/PBCH block;

- QCL information acquisition: RSs in SS/PBCH blocks with the same SS/PBCH block index are assumed to be QCLed (e.g. same as NR Rel-15), and no need to specify the QCL assumption within a DTTC window.

Figure 2 Illustration of DTTC window and cyclically wrap-around transmission of SS/PBCH blocks.

Due to LBT, there can be a potential timing offset between the start of DTTC window and the start of transmission of SS/PBCH blocks, and this timing offset should be indicated to the UE for determining the timing info within the DTTC window in initial access. We observe that comparing to an increasing order of the SS/PBCH block indexes, the wrap-around order of the SS/PBCH block indexes can save bits in the indication. Still taking Figure 2 for an example, by using an increasing order of the SS/PBCH block indexes, the number of bits for indicating the timing offset is 3 (e.g. indicating all 5 possible starting locations); while by using a wrap-around order of the SS/PBCH block indexes, the number of bits for indicating the timing offset is 1 (e.g. indicating o_SSB = 0, or 1).

Ideally, the acquiring of timing offset should be before the decoding of PBCH, in order to avoid reading PBCH content in determining the timing of a neighboring cell in the handover. Hence, the number of DMRS sequences of PBCH for NR-U shall be increased to 16, in order to indicate the potential SS/PBCH block locations in a DRS window, or equivalently, the combination of offset o_SSB and SS/PBCH block index i_SSB.

Based on the above observations and discussions, we have the following proposals for the transmission of SS/PBCH blocks for NR-U.

Proposal 1: NR-U shall support DTTC-based transmission in order to compensate the limitations on channel access due to LBT.

- The periodicity of a DTTC window shall be aligned with the periodicity of SS/PBCH block, and configurable from {5, 10, 20, 40, 80, 160} ms.

- One default periodicity of a DTTC window is used for initial access purpose, e.g. 20 ms.

- The default duration of a DTTC window is 4 ms.

- The DTTC window is the same as RLM-DMTC window.

Proposal 2: NR-U shall support cyclically wrapping SS/PBCH blocks dropped due to LBT failure, for SS/PBCH blocks transmitted in DRS.

- The timing information of the potential SS/PBCH block location in a DRS window shall be carried by the DMRS of PBCH in the corresponding SS/PBCH block.

- The number of DMRS sequences of PBCH increases from 8 to 16 for NR-U.

- RSs in SS/PBCH blocks with the same SS/PBCH block index are assumed to be QCLed.

# Enhancement to Random Access Procedure

NR-U supports the architectural scenarios including standalone NR-U, and carrier aggregation/dual-connectivity between NR-U and licensed NR/LTE carrier. Therefore, in addition to supporting the contention-free random access (RA), NR-U also supports the 4-step contention-based random access. NR-U needs to satisfy unlicensed spectrum regulations such as the LBT, occupied channel bandwidth (OCB) regulation, and power spectral density (PSD) regulation. Due to the LBT requirement, any failure of LBT during the 4-step random access procedure can lead to a significantly increased random access delay and resource overhead. Therefore, NR-U random access needs enhancements from the baseline procedure to decrease the expected random access delay caused by potential LBT failures. Specifically, the enhancements to decrease the expected random access delay of NR-U are detailed in this session. Other aspects for the enhancement to PRACH design in NR are also discussed in this section.

## NR-U PRACH configuration enhancement

In Rel-15 NR, PRACH can be transmitted in the time resources given by the higher-layer parameter PRACHConfigurationIndex, based on which the RACH occasions can be derived from the PRACH configuration table. In particular, the RO(s) within a RACH slot are allocated consecutively in Rel-15 NR. Given that LBT is required before PRACH transmission, extra time-domain resource overhead may potentially be incurred for LBT. As a result, NR-U needs to incorporate the time-domain overhead for performing LBT on the available time-domain RACH occasions within RACH slot(s).

One option is for the ROs to be consecutively allocated as in Rel-15 NR, and LBT is performed before each RO to grant the transmission of PRACH. Despite the PRACH configuration table from Rel-15 NR can be reused to support this option, LBT for UEs assigned to a RO can be blocked by PRACH from earlier RO(s) due to the TA difference, which may severely impact the PRACH performance for NR-U.

A second option is to allow the neighboring ROs within the same RACH slot to be non-consecutive, with a gap duration introduced between two neighboring ROs, wherein the gap can be utilized for incorporating the LBT resource overhead in transmitting PRACH in the RO that comes after this gap. An example of introducing a gap duration between two neighboring ROs is provided in Figure 3.

Figure 3. An example of gap between neighboring ROs for PRACH LBT overhead.

The second option can avoid the LBT blocking issue between neighboring ROs, which happens with consecutively allocated ROs. However, enhancements are needed to configure ROs and the gap duration for this option. In one option, the gap duration between neighboring ROs can be achieved by configuring UEs to only utilize the RACH occasions with even indexes 2n (e.g., the 0-th, 2-nd, 4-th, 6-th) within the RACH slot for PRACH transmissions, while the RACH occasions with odd indexes 2n-1 (e.g., the 1-st, 3-rd, 5-th) can be used for the LBT operation of UEs that will utilize RO with index 2n. This option can directly use the existing PRACH configuration table from Rel-15 and thus requires less changes.

In another option, the gap duration between neighboring ROs can be configured through enhancing the Rel-15 PRACH configuration table and/or adding new higher layer parameters. However, such enhancements will depend on the LBT type for PRACH. Specifically, with CAT-2 LBT of 25 µs, the LBT gap will require 1 OFDM symbol with 15 kHz or 30 kHz SCS for PRACH. By contrast, with CAT-4 LBT, the LBT duration non-deterministic, which will be at least 34 µs (iCCA/eCCA duration) and can even require much longer duration than the PRACH itself. Therefore, CAT-4 LBT for PRACH requires much more significant changes to the existing Rel-15 PRACH configuration method as opposed to CAT-2 LBT.

As detailed in our companion contribution [3], a CAT-2 LBT can be the preferred LBT option for PRACH as opposed to CAT-4 LBT, and the corresponding enhancements for PRACH occasion configurations shall be further investigated based on PRACH LBT type.

Proposal 3: NR-U shall support non-consecutive ROs within the same RACH slot, with a gap duration introduced between two neighboring ROs for the PRACH LBT resource overhead.

## Time/frequency-domain enhancements for NR-U RACH resources

Agreement:

Following options have been identified for potential RACH resource enhancements in NR-U beyond the flexibility already available in Rel-15:

- Frequency-domain enhancement

  - Multiple PRACH resources across multiple LBT sub-bands/carriers for both contention-free and contention-based RA

- Time-domain enhancements

  - For connected mode UE, scheduling of PRACH resources via DCI.

    - Triggered PRACH within TXOP can use a new resource

  - For idle mode UE, scheduling of PRACH resources via paging

    - Note: potential inefficiency in network resource due to paging across multiple cells

  - Additional, new RACH resources are used immediately following detection of DRS transmission

  - Multiple PRACH transmissions before Msg2 reception in RAR window for initial access

    - Number of allowed transmissions is pre-defined or indicated, e.g., in RMSI

    - FFS: How to handle potential multiple RARs to same UE

  - Group wise SSB-to-RO mapping by frequency first-time second manner, where grouping is in time domain

In RAN1 #94bis [4], several frequency-domain and time-domain potential NR-U RACH resource enhancements have been identified.

Among the identified enhancement options, time-domain enhancement option (d) and option (e) wherein multiple PRACH transmission and time domain grouping method can be utilized as the most general approaches in compensating the potential PRACH LBT failures for both contention-based and contention-free random access.

By contrast, each of the remaining options would have certain constraints. Specifically, for the frequency-domain enhancement in [4], since it has already been agreed that the initial active UL BWP is (approximately) 20 MHz and that LBT will be performed in units of 20 MHz for NR-U, performing LBT on a per sub-band basis for NR-U PRACH will have minimal channel access benefit for initial access UEs.

As discussed in our companion tdoc [3], CAT-2 LBT is the more reasonable LBT option for standalone PRACH transmission instead of CAT-4 LBT, thus time-domain enhancement option (a) has minimal channel access benefit. In addition, the PRACH performance of option (a) is not guaranteed, since the DCI-scheduled RO is non-repeatable and is also subject to at least a CAT-2 LBT, thus enhancements such as option (d) or option (e) would still be necessary to increase the PRACH LBT success chance. Using paging to configure the PRACH resource with time-domain option (b) could be a huge resource waste as already noted in the agreement, this often causes the PRACH resource reserved in multiple cells and expected such UE could utilize only one of them. For the time-domain enhancement option (c), the RACH resources that can be utilized immediately following detection of a DRS are very limited or even unavailable, when the DRS is subject to CAT-4 LBT with lowest priority class value, and/or when the number of SS/PBCH blocks within DRS is high (e.g., 8). Therefore, UEs utilizing such RACH resources (if any) could be subject to high collision probability. In addition,

As a result, the time-domain enhancement option (d) and (e) can be the prioritized RACH enhancement options for NR-U; while the other enhancement options require further study to see if there is any benefits. More detailed discussion on the enhancements could be found in our companion contribution [5] and more related discussion on multiple PRACH transmissions could be found in our companion contribution [6].

Proposal 4: NR-U shall prioritize time-domain enhancement option (d) and (e) for PRACH.

## Enhancement to increase transmit opportunities for Msg1 in 4-step RACH

Due to the LBT requirement for transmitting PRACH, the failure of PRACH LBT can cause significant delay to the overall RA procedure. As a result, enhancements to increase the transmission opportunities of Msg1 in 4-step RACH can be used to decrease the PRACH transmission delay.

As a first enhancement, NR-U can try to inherit the SSB-RO association design in the NR Rel-15, and add an associated LBT before each RO to enhance the transmit opportunities for Msg1 as discussed in Section 3.1. For example, NR Rel-15 supports configuring one or more than one SSB for each RO, and it may end up with multiple ROs, associated with the same SSB, not consecutive to each other in time domain. Then, if the LBT for an earlier RO fails, Msg1 still has opportunities to be transmitted in later configured ROs within the association period, subject to the LBT results of the later ROs. An example of this enhancement is illustrated in Figure 4.

Figure 4 Illustration of the first enhancement method.

As another enhancement, NR-U can configure multiple consecutive ROs associated with the same SSB, and an associated LBT can be performed before each RO (there can be gap symbol(s) between neighbouring ROs among the consecutive ROs to account for LBT overhead). As a result, a UE can be configured with multiple consecutive time-domain ROs for RACH attempt, which can also increase the transmit opportunities for Msg1. Equivalently, this example can be considered as configuring an associated RO window to each SSB, wherein multiple consecutive time-domain ROs can be configured within the window. In order to support configuring multiple time-domain ROs to be associated with SSBs in a consecutive manner, the SSB-RO mapping can be enhanced from NR Rel-15, e.g, time-domain first mapping; especially when the enhancement can be combined with FDMed multiple ROs. Figure 5 gives such an example where each SSB is configured with 4 consecutive ROs in the time domain (with LBTs associated to each RACH occasion), and two sets of FDMed ROs in the frequency domain. Note the PRACH LBT shall be performed over the initial active UL BWP, so the LBTs associated with FDM’ed ROs in Figure 5 may actually overlap in frequency domain.

Figure 5 Illustration of the second enhancement method.

Proposal 5: NR-U shall configure a UE with multiple Msg1 transmission opportunities within an SSB-RO association period to decrease the PRACH transmission delay.

## Valid RO determination

In a NR TDD band, the system information includes the semi-static UL/DL configuration to provide the duration of the DL part, UL part as well as flexible part. Meantime, due to complex situation of possible semi-static UL/DL configuration (variable period, variable durations of DL/UL), the validation rule is finally promoted to apply to the PRACH configuration indicated via the PRACH configuration index of a table. In short, the RO indicated by the PRACH configuration index will be invalid if it is collided with DL part or a part before the end of the last SSB in the flexible part. Such rule in addition to the gap definition after the last symbol of DL part/SSB helps the UE to determine the actual valid RO within the PRACH configuration period. Note that each PRACH configuration period may have different valid ROs, thus it produces the different consistence of the SSB-RO association period, i.e., different number of PRACH configuration period in one SSB-RO association period. For example, it could be 4 PRACH configuration periods at one time but becomes 2 PRACH configuration periods after. Furthermore, to ease the burden at the UE side, instead of the always (re)-calculating the association period, NR Rel-15 defines the association pattern period which consists of one or multiple SSB-RO association period(s), in which the mapping pattern between the association pattern periods will be purely repeated.

For NR-U, the uncertainty of the LBT outcome will add the variance to the (actually transmitted) SSB positions, as well as other DL signal transmission, making the situation of channel utilization more complicated. Method of effective determination of valid RO should be further studied, using PRACH configuration design from NR Rel-15 as a baseline.

Proposal 6: Validation rule of RO shall be studied for NR-U.

## Power ramping behavior

In last meeting, RAN1 has agreed that the preamble power ramping is not performed and that the preamble transmission counter is not incremented if preamble transmissions are dropped due to LBT failure. However, one thing is not very clear is that such rule shall apply to LBT failures of all the ROs regardless the timing period, such as PRACH configuration period, or it shall apply to the LBT failures of these the ROs in a given time period. Different consequences could be obtained according to above two different understandings.

In LTE/NR licensed system, power ramping is a useful tool to give a certain level of priority to a UE who spends more time (experiences longer delay) in random access compared with another UE who spends less time (experiences shorter delay). In TS38.321,

PREAMBLE_RECEIVED_TARGET_POWER = preambleReceivedTargetPower + DELTA_PREAMBLE + (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP.

In general, UE who fails more, the PREAMBLE_POWER_RAMPING_COUNTER will be larger (incremented more times), thus it will have larger power comparing to these who fails less. For example, in Figure 6, two UEs are trying random access in the cell, UE 1 already tries 4 times, but UE 2 first try of PRACH, so the power ramping operation will give UE 1 more priority compared to UE 2 when these two UEs hold contention to each other, i.e., UE 1 will transmit with P+3*P_step (i.e., the power ramping step) while UE 2 will transmit with P, which will give UE 1 a higher possibility to win the contention and access to system. In LTE/NR, the main reason to keep UE trying in random access procedure is due to previous random access failure, thus power ramping will happen after the failure of previous RACH transmission. For NR-U, LBT is another reason to keep UE still trying random access. The extra delay caused by the LBT failures should also be considered in power ramping behavior.

Figure 6 Illustration of the power difference created by power ramping operation in LTE/NR.

Based on the different interpretation of the previous agreements, following two options could be derived:

Option 1: the power ramping unchanged rule applies to all ROs regardless of RACH configuration period;

Option 2: the power ramping unchanged rule applies to the ROs in the same given RACH configuration period;

Figure 7 Illustration of the different interpretation of the power ramping agreement.

As shown in Figure 7, the consequences of these two different understandings are illustrated. By option 1, the UE 1 who experienced longer delay will have no priority comparing to a newly joined random access UE 2, i.e., UE 1 is transmitting the same power as the UE 2; while option 2 could still provide ability to create power difference among the random access UEs who experienced different delay like in the LTE/NR system. Note that in NR, the PRACH configuration period could be 10, 20, 40, 80, 160ms, the larger the PRACH configuration period, the longer delay UE may experience by the LBT failures. From this perspective, the understanding of option 2 could better represent the purpose of power ramping. So it’s recommended to consider the power ramping unchanged rule applies to the ROs in the same given RACH configuration period only, while the LBT failures of ROs in different PRACH configurations could be considered as a new RACH attempt, thus the increment of power ramping counter can be considered.

Proposal 7: If preamble transmissions are dropped due to LBT failure of ROs in the same PRACH configuration period, then it is recommended that preamble power ramping is not performed; if preamble transmissions are dropped due to LBT failure of ROs across different PRACH configuration periods, then the increment of power ramping counter can be considered.

# Enhancement to RRM Measurement

In Rel-13 LTE-LAA, RSSI measurement is to identify low interference carriers in unlicensed band. Specifically, RSSI measurement serves two purposes including the indication of the average channel load/usage and the detection of hidden nodes. Therefore, channel occupancy, which is the percentage of time that RSSI was above a threshold, is always reported together with the RSSI to reflect the congestion status of a given CC is reported. With the same motivation, NR-U shall support similar RSSI + channel occupancy reporting scheme as in LTE-LAA.

In NR phase-1, although RSSI is defined as a measurement quantity supported for RRM measurement purpose, RSSI measurement result (e.g NR carrier RSSI or CSI-RSSI) is not separately reported, and has to be inferred from RSRP and RSRQ, respectively. However, in NR-U, the reference signals for measuring RSRP and RSRQ may not be always transmitted due to LBT, purely relying on the indirect way of acquiring RSSI measurement may not be sufficient, especially considering RSSI is an important metric for hidden node detection in the unlicensed band. Moreover, if RSSI measurement is restricted to the resources containing reference signals for RSRP/RSRQ measurement, the interference would be over-estimated because such reference signals are more likely to be transmitted than normal data transmission with more aggressive LBT. Furthermore, in some scenarios, it would be beneficial to enable UE and gNB to perform the RSSI measurement on exactly the same resources for hidden node detection. Therefore, similar to LTE-LAA, the timing configuration for RSSI measurement (RMTC) with a measurement periodicity and a measurement window should be configured separately from RSRP/RSRQ measurement resource, and the frequency resources can be confined with a given bandwidth, e.g., 20MHz.

Proposal 8: NR-U should support RSSI measurement and reporting together with channel occupancy .

- LTE-LAA RMTC can be considered as a starting point.

# Enhancement to RLM Procedure

For radio link monitoring, the downlink radio link quality of the primary NR or NR-U cell shall be monitored by a UE for the purpose of indicating out-of-sync/in-sync status to higher layers. In NR, the UE can be configured for a cell with a set of resources (e.g. SS/PBCH block and/or CSI-RS) for radio link monitoring by higher layers. The physical layer in the UE shall indicate out-of-sync to higher layers when the radio link quality is worse than the threshold Qout for all resources in the set of resources for radio link monitoring. When the radio link quality is better than the threshold Qin for any resource in the set of resources for radio link monitoring, the physical layer in the UE shall indicate in-sync to higher layers. In order to operate NR-U without any assistance from licensed carrier, the radio link monitoring is also performed by a NR-U cell. In NR-U, it is possible that gNB fails to access the channel so that RLM-RS cannot be transmitted in a set of slots designated for RLM at a UE side. In this case, the UE may or may not measure the link in a slot where gNB fails to access the channel, by UE implementation. Therefore, further discussions may be required whether existing radio link monitoring procedure can be reused for NR-U. For instance, whether/how to distinguish cases of out-of-sync, restriction of RLM to inside of COT or RLM-RS transmission window, etc. Since it is difficult for UE to distinguish out-of-sync which is due to LBT failure at gNB or low channel condition at UE, it would be beneficial to perform DRS-based RLM in the DRS transmission window (e.g. DTTC) if SS/PBCH block and CSI-RS in NR-U DRS are more likely to be transmitted than other transmission with more aggressive LBT.

Proposal 9: DRS-based RLM shall be performed inside DRS transmission window, i.e., the DTTC window is the same as RLM-DMTC window.

# Conclusion

The proposals made in this contribution are summarized below:

Proposal 1: NR-U shall support DTTC-based transmission in order to compensate the limitations on channel access due to LBT.

- The periodicity of a DTTC window shall be aligned with the periodicity of SS/PBCH block, and configurable from {5, 10, 20, 40, 80, 160} ms.

- One default periodicity of a DTTC window is used for initial access purpose, e.g. 20 ms.

- The default duration of a DTTC window is 4 ms.

- The DTTC window is the same as RLM-DMTC window.

Proposal 2: NR-U shall support cyclically wrapping SS/PBCH blocks dropped due to LBT failure, for SS/PBCH blocks transmitted in DRS.

- The timing information of the potential SS/PBCH block location in a DRS window shall be carried by the DMRS of PBCH in the corresponding SS/PBCH block.

- The number of DMRS sequences of PBCH increases from 8 to 16 for NR-U.

- RSs in SS/PBCH blocks with the same SS/PBCH block index are assumed to be QCLed.

Proposal 3: NR-U shall support non-consecutive ROs within the same RACH slot, with a gap duration introduced between two neighboring ROs for the PRACH LBT resource overhead.

Proposal 4: NR-U shall prioritize time-domain enhancement option (d) and (e) for PRACH, and time-domain enhancement option (a) can also be supported.

Proposal 5: It is identified beneficial to configure a UE with multiple Msg1 transmission opportunities within an SSB-RO association period to decrease the PRACH transmission delay.

Proposal 6: Validation rule of RO shall be studied for NR-U.

Proposal 7: If preamble transmissions are dropped due to LBT failure of ROs in the same PRACH configuration period, then it is recommended that preamble power ramping is not performed; if preamble transmissions are dropped due to LBT failure of ROs across different PRACH configuration periods, then the increment of power ramping counter can be considered.

Proposal 8: NR-U should support RSSI measurement and reporting together with channel occupancy .

- LTE-LAA RMTC can be considered as a starting point.

Proposal 9: DRS-based RLM shall be performed inside DRS transmission window, i.e., the DTTC window is the same as RLM-DMTC window.

# References

[1] RAN1 Chairman’s Note, 3GPP TSG RAN WG1 Meeting #95, Spokane, US, November 12th-16th, 2018.

[2] R1-1810861, Initial Access and Mobility Procedure for NR-U, Samsung

[3] R1-1902257, Channel access procedures for NR-U, Samsung

[4] RAN1 Chairman’s Note, 3GPP TSG RAN WG1 Meeting #94bis, Chengdu, China, October 8th –12th, 2018.

[5] R1-1902266, Discussion on time/freq-domain enhancements for RACH resources, Samsung

[6] R1-1902267, Discussion on multiple Msg. 1 transmission procedure, Samsung