3GPP TSG RAN WG1 #96		  	   R1-1902809

Athens, Greece, 25st February – 1st March, 2019

Source:	NTT DOCOMO, INC.

Title:	Enhanced UL transmission with configured grant for URLLC

Agenda Item:	7.2.6.3

Document for: 	Discussion and Decision

- Introduction

In this contribution, we present our views on enhanced UL transmission with configured grant for URLLC.

- Multiple active configured grant configurations for a given BWP of a serving cell

It was agreed to support multiple active configured grant configurations for a given BWP of a serving cell. In the following, we will discuss the enhancements for higher layer configurations and physical layer signalling to support the multiple active configured grant configurations for a given BWP based on Rel.15 design.

Higher layer configurations

For the multiple active configured grant configurations, the maximum number of active configured grant configurations for a given BWP should be determined such that the use case of supporting different services/traffic types and the use case of reducing the latency while ensuring the reliability are well covered. Considering the maximum number of configured SR resources per BWP of a cell is 8 and the maximum number of repetition factor is 8, 8 can be the maximum number of configured grant configurations for a given BWP of a serving cell.

Currently, there are two Types of configured grant transmission, i.e., Type 1 and Type 2. Type 1 is useful when the traffic profile or resource usage is not much changed and the PDCCH overhead is a bottleneck of the system capacity, while Type 2 is advantageous in other cases. Therefore, Rel.16 enhancements should be applicable to both Type 1 and Type 2 configured grant configurations, and multiple active configured grant configurations should be available for both Type 1 and Type 2 for a given BWP of a serving cell.

When multiple configured grant configurations are configured/active, introducing a configuration index for each configured grant configuration is a straightforward way to manage and differentiate different configured grant configurations. For Type 2 configured grant, a configuration index can also be used to distinguish which configured grant configuration is activated/deactivated by an activation/deactivation DCI. Based on above, following are proposed:

Proposal 1:

- A configuration index should be introduced for each configured grant configuration.

  - The configuration index is used to manage and distinguish the configurations.

- Design multiple configured grant configurations such that the maximum number of configured grant configurations for a given BWP of a serving cell in the specification to be at most 8.

Note that in Rel.15, some parameters are independently defined for ConfiguredGrantConfig and PUSCH-Config, while some other parameters are not provided in ConfiguredGrantConfig, in which case parameters in PUSCH-Config are used, for example: parameters of dataScramblingIdentityPUSCH, txConfig, codebookSubset, maxRank, scaling of UCI-OnPUSCH and tp-pi2BPSK are not in the ConfiguredGrantConfig. At the end of Rel.15 WI, it was extensively discussed how these parameters can be flexibly configured, taking into account total DCI payload size, fixed NDI position to differentiate between activation/deactivation signalling and re-scheduling signalling, etc. Rel.15 tight timeline requires rushing agreements and hence the design was not much clean. In Rel.16, due to this URLLC SI and for other WIs/SIs, PUSCH would be enhanced and additional parameters will be defined for PUSCH configuration. The signalling design for configured grant should be designed to be cleaner, universal, and efficient.

To support different services/traffic types running simultaneously at the UE side, it is natural that most of the parameters should be configurable among multiple configured grant configurations separately since different services/traffic types having different requirements and traffic patterns including the traffic arrival time/periodicity, packet size, target BLER, etc.

To ensure K repetitions while reduce latency by multiple configurations, it is not always necessary to allow full configurability. For example, the parameters such as waveform, MCS table, periodicity, repetition factor, RV sequence, transmission duration, etc, can be common across configurations for K repetitions. It is very useful to study the RRC signaling overhead reduction considering the oversized RRC signaling for NR [1]. Other parameters for different configurations for K repetitions can be separately configured or have some regularity among the configurations such as the starting offset difference among configurations, DMRS configuration/sequence, and HARQ ID offset if it is introduced/defined. By this, it is possible to treat the multiple configurations as one configuration set or one configuration group. Fig.1 gives an example, in which with the same configured grant configuration group, most parameters can be common for all configurations; across the configured grant configuration group, all parameters can be separately configured to accommodate different service/traffic types.

Fig. 1	Concept of configured grant configuration group.

Proposal 2:

- Introduce a concept of configured grant configuration group.

  - UE can be configured with one or more configured grant configuration group.

  - Within a group,

    - UE can be configured with one or more configured grant configurations.

    - Use of multiple configured grant configurations is to ensure K repetitions with reduced latency.

    - Some parameters should be common across configured grant configurations.

    - Some parameters should be independent across configured grant configurations.

  - Across groups,

    - Some parameters should be common across the groups.

    - Some parameters should be independent across the groups.

  - Use of multiple configured grant configuration groups is to support various service/traffic types.

Physical layer aspects

For Type 2 configured grant configuration, L1 signalling is needed to activate/deactivate the configured grant. In Rel.15, DCI format 0_0 and 0_1 with CRC scrambled by CS-RNTI can be used as activation signalling, while only DCI format 0_0 can be used for deactivation signalling. Following special fields are defined to validate and differentiate between activation and deactivation signalling.

Table 10.2-1: Special fields for DL SPS and UL grant Type 2 scheduling activation PDCCH validation

|  | DCI format 0_0/0_1 | DCI format 1_0 | DCI format 1_1 |
| --- | --- | --- | --- |
| HARQ process number | set to all '0's | set to all '0's | set to all '0's |
| Redundancy version | set to '00' | set to '00' | For the enabled transport block: set to '00' |

Table 10.2-2: Special fields for DL SPS and UL grant Type 2 scheduling release PDCCH validation

|  | DCI format 0_0 | DCI format 1_0 |
| --- | --- | --- |
| HARQ process number | set to all '0's | set to all '0's |
| Redundancy version | set to '00' | set to '00' |
| Modulation and coding scheme | set to all '1's | set to all '1's |
| Frequency domain resource assignment | set to all '1's | set to all '1's |

For multiple active configured grant configurations, it is not clear yet whether an activation/deactivation DCI is per configured grant configuration or common for multiple configured grant configurations. Individual DCI for activating/deactivating each configured grant configuration is flexible while increases overhead. The suitable option depends on whether the multiple configured grant configurations are for different service/traffic types or for reducing latency while keeping reliability. Therefore, supporting both approaches is preferable. This can be realized by introducing explicit new field in the DCI or by re-using/interpreting the existing field in the DCI to indicate which configured grant configuration(s) is/are activated/deactivated. For re-using/interpreting the existing field, RV field and/or HPN filed could be one possibility, since they are not used for differentiation the activation and deactivation signalling.

In addition, in Rel.15, only DCI format 0_0 and 1_0 can be used for deactivation signalling to reduce the downlink overhead. However, it cannot support the feature of cross-carrier scheduling. Therefore, for Rel.16 configured grant, both activation and deactivation DCI should be designed to support the feature of the cross-carrier scheduling. Detailed DCI formats depends on the DCI format/size design for URLLC [2].

Proposal 3:

- An activation/deactivation DCI indicates one or multiple configured grant configuration(s) to be activated or deactivated

  - A field(s) in the activation/deactivation DCI indicates which configured grant configuration(s) to be activated/deactivated

    - Consider having new field or re-using the existing field for this purpose, where RV field and/or HPN field can be the re-use since they are not used for differentiation of activation and deactivation signalling.

UE and gNB should have common understanding on which HARQ process ID is assigned to a configured grant PUSCH. In Rel.15 configured grant, the HARQ Process ID associated with the first symbol of a UL transmission is derived from the following equation:

HARQ Process ID = [floor(CURRENT_symbol/periodicity)] modulo nrofHARQ-Processes

where CURRENT_symbol=(SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot + slot number in the frame × numberOfSymbolsPerSlot + symbol number in the slot), and numberOfSlotsPerFrame and numberOfSymbolsPerSlot refer to the number of consecutive slots per frame and the number of consecutive symbols per slot [3]. For Rel.16 multiple active configured grant configurations, UE may transmit a PUSCH based on one of the configured grant configurations. Even if the PUSCH resource and/or DMRS sequence is unique for the configured grant configuration, if the HARQ process ID pool is shared among the configured grant configurations, HARQ process ID can be unclear depending on the CURRENT_symbol, periodicity, and nrofHARQ-Processes. In LTE HRLLC, this was resolved by defining nrofHARQ-Processes and HARQ ID offset per configured grant configuration, and modify the above equation as following:

HARQ Process ID = 
[floor(CURRENT_symbol/periodicity)] modulo nrofHARQ-Processes + harq-procID-offset,

where harq-procID-offset is an integer to provide a fixed offset for ID determination for the given configured grant configuration. nrofHARQ-Processes is also per configured grant configuration. By these change, HARQ process ID for each configured grant configuration can be chosen from a sub-pool of HARQ process IDs for each configured grant configuration.

Proposal 4:

- nrofHARQ-Processes should be configured per configured grant configuration.

- A HARQ Process ID offset should be introduced per configured grant configuration.

- Mechanisms to ensure K repetitions

In Rel.15, UE is allowed to start a PUSCH transmission for a configured grant configuration when the transmission occasion of the K repetitions is associated with RV=0, except for the following two cases:

Case 1: If the configured RV sequence is {0, 2, 3, 1}, the UE is allowed to start transmission only at the first transmission occasion of the K repetitions

Case 2: If the configured RV sequence is {0, 0, 0, 0} and K=8, the UE is not allowed to start transmission at the last transmission occasion

From the above, it is understood that K repetitions can be ensured by RV sequence {0, 2, 3, 1} for K=2, 4, 8, RV sequence {0, 3, 0, 3} for K=2. Multiple configured grant configurations having timing offsets can realize ensuring K repetitions while reducing latency, by appropriate setting of RV sequence for the given K value.

Observation 1:

- In Rel.15, K repetitions for a configured grant configuration can be ensured by setting RV sequence {0, 2, 3, 1} for K=2, 4, 8 and RV sequence {0, 3, 0, 3} for K=2.

  - Combined with multiple active configured grant configurations having time offsets, lower latency is achievable while keeping ensuring K repetitions

Enhancements can be considered on top of the combination of Rel.15 mechanism and multiple configured grant configurations; ensuring K repetitions for RV sequence {0, 3, 0, 3} with K=4, 8, and for RV sequence {0, 0, 0, 0} with K=2, 4, 8. Example of the problematic case is illustrated in Fig. 2, where RV sequence {0, 3, 0, 3} and K=4 with periodicity 4 slots are assumed.

Fig. 2	Example case where K repetitions are not ensured.

There are in general two approaches:

Option 1: UE does not start transmission if the K repetitions cannot be within a period P for a given configuration.

Option 2: UE shall continue K repetitions once starts, even if K repetitions cannot be within a period P for a given configuration.

With option 1, the transmission opportunity is reduced for a given configured grant configuration. However, by configuring multiple configured grant configurations with time offsets, the reduction of the transmission opportunity can be complimented (see Fig.3 (a)). With option 2, the transmission opportunity can be increased for the given configured grant configuration (see Fig.3 (b). However, several issues are identified if repetitions can be beyond the period P, e.g., HARQ ID determination.

(a) Option 1

(b) Option 2

Fig. 3	Options to ensure K repetitions.

Considering that multiple configured grant configurations is already agreed to enable reliability enhancement and latency reduction, and option 2 would require more specification work, we propose to adopt Option 1.

Proposal 5:

- Support restriction of transmission occasion of a configured grant configuration such that K repetitions are ensured within a period P.

  - I.e., UE does not start transmission if the K repetitions cannot be within a period P.

- Mini-slot repetitions

As for PUSCH enhancements, there is a still discussion on whether to support mini-slot repetition or multi-segment transmission. We believe mini-slot repetition should be adopted. Once adopted, the mini-slot repetition solution should be common for dynamic grant and configured grant. As discussed in our companion contribution [4], following is proposed:

Proposal 6:

- Support mini-slot repetition for configured grant for Rel.16 URLLC.

  - Time-domain resource allocation for the first repetition is the re-use of Rel.15 SLIV.

  - Time-domain resource allocation for the remaining repetitions are following.

    - Back-to-back unless UL/DL direction conflict or cross slot boundary occurs.

      - If UL/DL direction conflict or cross slot boundary occurs, consider to drop the concerned repetition as the baselinePostpone can be further considered case by case.

  - Number of repetitions for a mini-slot repetition is indicated by the activation DCI.

  - Number of hops is no more than 2.

  - For inter-PUSCH-repetition hopping,

    - Even repetitions start from RBstart, and;

    - Odd repetitions start from (RBstart + RBoffset) mod NBWP.

  - For inter-slot hopping,

    - Repetitions in slot 2n start from RBstart, and;

    - Repetitions in slot 2n+1 start from (RBstart + RBoffset) mod NBWP.

  - DMRS sharing is not required.

  - TBS determination can be based on the first repetition.

  - Different repetitions convey different RVs of a transport block, where the RV sequence is configured from {0 2 3 1}, {0 3 0 3}, or {0 0 0 0}.

- Conclusion

In this contribution, we discussed the details for enhanced UL transmission with configured grant for URLLC, the proposals are summarized below:

Proposal 1:

- A configuration index should be introduced for each configured grant configuration.

  - The configuration index is used to manage and distinguish the configurations.

- Design multiple configured grant configurations such that the maximum number of configured grant configurations for a given BWP of a serving cell in the specification to be at most 8.

Proposal 2:

- Introduce a concept of configured grant configuration group.

  - UE can be configured with one or more configured grant configuration group.

  - Within a group,

    - UE can be configured with one or more configured grant configurations.

    - Use of multiple configured grant configurations is to ensure K repetitions with reduced latency.

    - Some parameters should be common across configured grant configurations.

    - Some parameters should be independent across configured grant configurations.

  - Across groups,

    - Some parameters should be common across the groups.

    - Some parameters should be independent across the groups.

  - Use of multiple configured grant configuration groups is to support various service/traffic types.

Proposal 3:

- An activation/deactivation DCI indicates one or multiple configured grant configuration(s) to be activated or deactivated

  - A field(s) in the activation/deactivation DCI indicates which configured grant configuration(s) to be activated/deactivated

    - Consider having new field or re-using the existing field for this purpose, where RV field and/or HPN field can be the re-use since they are not used for differentiation of activation and deactivation signalling.

Proposal 4:

- nrofHARQ-Processes should be configured per configured grant configuration.

- A HARQ Process ID offset should be introduced per configured grant configuration.

Observation 1:

- In Rel.15, K repetitions for a configured grant configuration can be ensured by setting RV sequence {0, 2, 3, 1} for K=2, 4, 8 and RV sequence {0, 3, 0, 3} for K=2.

  - Combined with multiple active configured grant configurations having time offsets, lower latency is achievable while keeping ensuring K repetitions

Proposal 5:

- Support restriction of transmission occasion of a configured grant configuration such that K repetitions are ensured within a period P.

  - I.e., UE does not start transmission if the K repetitions cannot be within a period P.

Proposal 6:

- Support mini-slot repetition for configured grant for Rel.16 URLLC.

  - Time-domain resource allocation for the first repetition is the re-use of Rel.15 SLIV.

  - Time-domain resource allocation for the remaining repetitions are following.

    - Back-to-back unless UL/DL direction conflict or cross slot boundary occurs.

      - If UL/DL direction conflict or cross slot boundary occurs, consider to drop the concerned repetition as the baselinePostpone can be further considered case by case.

  - Number of repetitions for a mini-slot repetition is indicated by the activation DCI.

  - Number of hops is no more than 2.

  - For inter-PUSCH-repetition hopping,

    - Even repetitions start from RBstart, and;

    - Odd repetitions start from (RBstart + RBoffset) mod NBWP.

  - For inter-slot hopping,

    - Repetitions in slot 2n start from RBstart, and;

    - Repetitions in slot 2n+1 start from (RBstart + RBoffset) mod NBWP.

  - DMRS sharing is not required.

  - TBS determination can be based on the first repetition.

  - Different repetitions convey different RVs of a transport block, where the RV sequence is configured from {0 2 3 1}, {0 3 0 3}, or {0 0 0 0}.

References

- R1-1812108, ‘LS on the total RRC Configuration size,’ RAN2

- R1-1902804, ‘Potential enhancements to PDCCH,’ NTT DOCOMO, INC.

- 3GPP TS 38.321 V15.3.0 (2018-09), ‘NR Medium Access Control (MAC) protocol specification’

- R1-1902806, ‘PUSCH enhancements for URLLC,’ NTT DOCOMO, INC.