3GPP TSG-RAN WG1 Meeting #96	R1-1902830

# Athens, Greece, February 25 – March 1, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR |  | rev |  | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: |  |  | UICC apps |  |  |  |  | ME | x | Radio Access Network |  |  |  |  | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Title: | Correction on SRS Power Scaling |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  |  |  |  | Date: |  |  | 2019-02-14 |  |  |  |
| Category: | F |  |  |  |  |  |  |  |  |  | Release: |  |  | Rel-15 |  |  |  |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |  |  |  |
| Reason for change: |  | SRS power scaling behavior is ambiguous. SRS power is split equally among “configured antenna ports for SRS”. However, “configured antenna ports for SRS” can have multiple interpretations, for example: - Number of SRS ports in SRS resource set qs - Number of SRS ports in the corresponding SRS resource - Number of SRS ports transmitted in a given OFDM symbol - Number of SRS ports transmitted in transmission occasion i The power transmitted by the UE on SRS ports can vary with these interpretations, and therefore different UEs may divide transmit power between SRS ports differently for a given SRS configuration. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Antenna ports within an SRS resource set transmit at the same power and at most - Pcmax for non-codebook based transmission - Pcmax/Ntx for codebook based transmission, beam management, and antenna switching, where Ntx is the maximum number of ports supported by the UE in an SRS resource - Transmissions on multiple SRS resource sets in a transmission occasion i are scaled down when they exceed Pcmax |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Different UEs may divide transmit power between SRS ports differently for the same SRS configuration, resulting in degraded uplink CSI, downlink CSI from reciprocity, and link adaptation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | First sentence of 7.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Y |  | N |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  |  |  | X | Other core specifications |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |
| affected: |  |  |  | X | Test specifications |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |
| (show related CRs) |  |  |  | X | O&M Specifications |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## 7.3	Sounding reference signals

For SRS, the antenna ports of SRS resource set qs in transmission period i on active UL BWP  of carrier  of serving cell  are transmitted with the power power α(i)/β(i,qs)PSRS,b,ci,qs,l per antenna port, where

- PSRS,b,f,ci,qs,l is the linear value of the transmit power PSRS,b,f,ci,qs,l

- For SRS resource set qs with usage set to ‘nonCodebook’, β(i,qs) is the maximum number of antenna ports transmitted in one of the OFDM symbols corresponding to SRS resource set qs in transmission occasion i

- For SRS resource set qs with usage set to ‘codebook’, ‘beamManagement’, or ‘antennaSwitching’,

  - When the number of SRS ports in any SRS resource of the set is 1, β(i,qs)=1

  - Otherwise, β(i,qs)=Nsrs_max, where Nsrs_max is the maximum number of SRS ports supported by the UE in one SRS resource.

- αi=minnPcmax,f,ciPtotn,i,1 where n denotes the OFDM symbol index in transmission period i and

  - Ptotn,i=qsdqs,n,iPSRS,b,f,ci,qs,l with

    - dqs,n,i=1 if a transmission corresponding to SRS resource set qs is transmitted in symbol n in transmission occasion i, and

    - dqs,n,i=0 if no transmission corresponding to SRS resource set qs is transmitted in symbol n in transmission occasion i.