3GPP TSG RAN WG1 Meeting #96	R1-1903102

Athens, Greece, 25th February - 1st March, 2019

Agenda Item:	7.2.8.6

Source:	Huawei, HiSilicon

Title:	Evaluation results of sequence design for Pi/2-BPSK DFT-S-OFDM

Document for:	Discussion and Decision

Introduction

For the DMRS sequence for DFT-s-OFDM, it has been agreed that:

Agreement Metrics to consider for new sequence design for the pi/2 BPSK DMRS are the gNB receiver complexity, PAPR relative to data and link level throughput/BLER performance considering frequency domain flatness and autocorrelation properties, interference considering cross correlation properties and when applicable (e.g. PUCCH), orthogonality of sequences Agreement - For length 12, 18 and 24 respectively, NR Rel-16 supports the binary CGS in the Table C1, C2, and C3 respectively followed by pi/2 BPSK modulation followed by DFT as DMRS sequence for π π/2 BPSK modulation for both PUSCH and PUCCH. - The above is applicable to single-symbol DMRS configuration - FFS: CGS for two-symbol DMRS configuration - Table C1, C2, and C3 can be found in R1-1901362 Agreement - For length 6 CGS; 8-PSK is used - Decide the associated sequences in the next RAN1 meeting

Based on the agreed metrics and principles, we provided proposed sequence and related simulation results in this contribution.

Simulation results on DM-RS sequence for Pi/2-BPSK DFT-S-OFDM

For length 6, due to one sequence set for the different combs is difficult to guarantee the performance of PAPR, auto-correlation and cross-correlation as discussed in our companion contribution, so we proposed that two separate groups of sequence design for port 0 occupying even subcarrier s(0) and port 2 occupying odd subcarrier s(1), so that the sequences can be optimized by each comb.Table 1 is the sequences and related performance of the sequences. Table 2 to 4 are the sequences and related performance with the common sequences for different comb proposed in [2] to [4].

Table 1.  Proposed sequences and corresponding performance

| u | Sequence for Port 0 |  |  |  |  |  | PAPR | AC | Sequence for Port 2 |  |  |  |  |  | PAPR | AC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 3 | 1 | -5 | 1 | 7 | 2.0792 | 0.2357 | 1 | 5 | 1 | -5 | 3 | 3 | 2.1005 | 0.235702 |
| 1 | 1 | -3 | 3 | 1 | 7 | -7 | 2.1155 | 0.1804 | 1 | -5 | 1 | 3 | -3 | 7 | 1.7244 | 0.235702 |
| 2 | 1 | -5 | 5 | 5 | -5 | 1 | 2.0359 | 0.13807 | 1 | 7 | 1 | 7 | -3 | -5 | 1.7714 | 0.235702 |
| 3 | 1 | 7 | 1 | -1 | 1 | -5 | 2.0793 | 0.2357 | 1 | 5 | 5 | -5 | 3 | -1 | 2.0083 | 0.235702 |
| 4 | 1 | 7 | 1 | -1 | -7 | -1 | 1.8853 | 0.2357 | 1 | 7 | 1 | 1 | -3 | 5 | 1.8877 | 0.235702 |
| 5 | 1 | 5 | 1 | -7 | -3 | -5 | 1.9572 | 0.2357 | 1 | 7 | 1 | -1 | 5 | -5 | 1.7244 | 0.235702 |
| 6 | 1 | 7 | 1 | -5 | -3 | 3 | 1.7901 | 0.2357 | 1 | 7 | 1 | -5 | -3 | -1 | 1.7715 | 0.235702 |
| 7 | 1 | 5 | 1 | -1 | 3 | -7 | 2.1070 | 0.2357 | 1 | -1 | 5 | -7 | -1 | -1 | 1.9848 | 0.180399 |
| 8 | 1 | 5 | 1 | -5 | 7 | -1 | 1.7675 | 0.2357 | 1 | 7 | 1 | -5 | -3 | 7 | 1.9289 | 0.235702 |
| 9 | 1 | 3 | 1 | 7 | -3 | -7 | 2.0214 | 0.1804 | 1 | -3 | 1 | 1 | -5 | 3 | 1.8619 | 0.235702 |
| 10 | 1 | 5 | 1 | -1 | 3 | -3 | 1.7817 | 0.097631 | 1 | 1 | 7 | -7 | 3 | -1 | 1.9849 | 0.180399 |
| 11 | 1 | -3 | 1 | 5 | -1 | 3 | 1.7817 | 0.097631 | 1 | 5 | 1 | 1 | 7 | -1 | 1.8619 | 0.235702 |
| 12 | 1 | -5 | 1 | 3 | -7 | 7 | 1.8864 | 0.2357 | 1 | -5 | 1 | 7 | 5 | -5 | 1.9289 | 0.235702 |
| 13 | 1 | -3 | 1 | -7 | 7 | -5 | 1.9665 | 0.2357 | 1 | -5 | 1 | 7 | -3 | -5 | 1.9701 | 0.235702 |
| 14 | 1 | -3 | 5 | -7 | -5 | 5 | 2.0290 | 0.2357 | 1 | 7 | 3 | -1 | 5 | 5 | 2.0881 | 0.097631 |
| 15 | 1 | 5 | 1 | -5 | -1 | -3 | 1.4418 | 0.097631 | 1 | 5 | 1 | 3 | -1 | 5 | 2.0755 | 0.235702 |
| 16 | 1 | 7 | 5 | -1 | -7 | -5 | 1.7657 | 0.2357 | 1 | -3 | 1 | -5 | 3 | -7 | 1.8619 | 0.235702 |
| 17 | 1 | -3 | 1 | 5 | 3 | -7 | 2.0932 | 0.2357 | 1 | -7 | 5 | -1 | 3 | -7 | 1.9151 | 0.235702 |
| 18 | 1 | -5 | 5 | 3 | -7 | -1 | 1.7657 | 0.2357 | 1 | 5 | 1 | 7 | -1 | -7 | 1.8619 | 0.235702 |
| 19 | 1 | 5 | 1 | 5 | -5 | -7 | 1.6489 | 0.2357 | 1 | 5 | 1 | -5 | -5 | 3 | 2.1006 | 0.235702 |
| 20 | 1 | 3 | 1 | -5 | 5 | -7 | 1.9297 | 0.1804 | 1 | -5 | 1 | -1 | 5 | -5 | 1.7245 | 0.235702 |
| 21 | 1 | 5 | 1 | -3 | 1 | 5 | 1.5333 | 5.55E-17 | 1 | -5 | 1 | 3 | -3 | -1 | 1.9289 | 0.235702 |
| 22 | 1 | 7 | 1 | -5 | -7 | -1 | 1.8863 | 0.2357 | 1 | -3 | 1 | 5 | -1 | -5 | 2.0887 | 0.235702 |
| 23 | 1 | 5 | 1 | 5 | -5 | 5 | 1.5200 | 0.2357 | 1 | -3 | 1 | -1 | 3 | -3 | 2.0755 | 0.235702 |
| 24 | 1 | 5 | 1 | -5 | -1 | 3 | 1.8358 | 0.097631 | 1 | 7 | 1 | -5 | 5 | 7 | 1.9701 | 0.235702 |
| 25 | 1 | -1 | 1 | -7 | -3 | 7 | 1.9298 | 0.1804 | 1 | 7 | 1 | 3 | 5 | -1 | 1.7715 | 0.235702 |
| 26 | 1 | -3 | 1 | 5 | -7 | 7 | 1.9853 | 0.097631 | 1 | 7 | 3 | -1 | -1 | 5 | 1.8384 | 0.097631 |
| 27 | 1 | 5 | 1 | 7 | -1 | -3 | 2.1115 | 0.2357 | 1 | 7 | 1 | 7 | 5 | 3 | 1.7714 | 0.235702 |
| 28 | 1 | -3 | 1 | -5 | -1 | 5 | 1.8003 | 0.2357 | 1 | 5 | 1 | -3 | 3 | 7 | 2.0887 | 0.235702 |
| 29 | 1 | -7 | 5 | -1 | -5 | -3 | 1.8788 | 0.2357 | 1 | -5 | 3 | 7 | -3 | -3 | 2.084 | 0.235702 |

Table 2. Length 6 sequences proposed in [2]

| u | Sequence |  |  |  |  |  | PAPR-p0 | PAPR-p1 | PAPR-p2 | PAPR-p3 | AC-p0 | AC-p1 | AC-p2 | AC-p3 | XC-p0p1 | XC-p2p3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | -7 | -3 | 5 | 1 | -7 | -3 | 1.1529 | 1.3284 | 1.5844 | 1.9575 | 0.3333 | 0.7453 | 0.4714 | 0.4714 | 0.12922 | 0.3333 |
| 1 | -7 | 1 | 5 | 1 | 5 | -3 | 1.1529 | 1.32843 | 1.95752 | 1.58447 | 0.33333 | 0.74536 | 0.4714 | 0.4714 | 0.12922 | 0.3333 |
| 2 | -7 | -1 | 5 | 1 | 7 | -5 | 1.4302 | 0.91935 | 0.83713 | 1.61621 | 0.33333 | 0.93264 | 0.6666 | 0.6159 | 0.25512 | 0.3333 |
| 3 | -7 | -1 | 3 | 1 | 7 | -3 | 1.4302 | 0.91935 | 1.61621 | 0.83713 | 0.33333 | 0.93264 | 0.6159 | 0.6666 | 0.25512 | 0.3333 |
| 4 | -7 | -3 | 7 | -5 | 5 | -7 | 1.4426 | 1.29018 | 1.81957 | 1.66156 | 0.27317 | 0.40825 | 0.4814 | 0.097631 | 0.2357 | 0.3014 |
| 5 | -7 | 3 | 7 | 7 | -5 | 5 | 1.4426 | 1.29018 | 1.66156 | 1.81957 | 0.27317 | 0.40825 | 0.0976 | 0.481408 | 0.2357 | 0.3014 |
| 6 | -7 | -3 | -7 | 5 | -7 | -3 | 1.5333 | 1.17656 | 0.77782 | 0.49979 | 0 | 0.66667 | 0.333333 | 0.333333 | 0.33333 | 0.6666 |
| 7 | -7 | -3 | 1 | -3 | -7 | -3 | 1.5332 | 1.17665 | 0.77788 | 0.49991 | 0 | 0.66667 | 0.333333 | 0.333333 | 0.33333 | 0.6659 |
| 8 | -7 | 5 | -7 | 5 | -7 | -3 | 1.5332 | 1.17665 | 0.49991 | 0.77788 | 0 | 0.66667 | 0.333333 | 0.333333 | 0.33333 | 0.6666 |
| 9 | -7 | -3 | 7 | -7 | 3 | 7 | 1.5917 | 1.38324 | 1.79509 | 2.01539 | 0.33333 | 0.49121 | 0.255122 | 0.255122 | 0.20947 | 0.3333 |
| 10 | -7 | 3 | 7 | -7 | -3 | 7 | 1.5917 | 1.38324 | 2.01539 | 1.79509 | 0.33333 | 0.49121 | 0.255122 | 0.255122 | 0.20947 | 0.3333 |
| 11 | -7 | 3 | -3 | 1 | -5 | 7 | 1.6128 | 0.87717 | 0.83713 | 1.61621 | 0.33333 | 0.93264 | 0.666667 | 0.61592 | 0.25512 | 0.3333 |
| 12 | -7 | 3 | -1 | 1 | -5 | 5 | 1.6128 | 0.87717 | 1.61621 | 0.83713 | 0.33333 | 0.93264 | 0.61592 | 0.666667 | 0.25512 | 0.3333 |
| 13 | -7 | 3 | -3 | -1 | -7 | 5 | 1.6131 | 0.9783 | 1.57994 | 0.89008 | 0.33333 | 0.93264 | 0.61592 | 0.666667 | 0.24657 | 0.3333 |
| 14 | -7 | 3 | -1 | 3 | -3 | 7 | 1.6131 | 0.9783 | 0.89008 | 1.57994 | 0.33333 | 0.93264 | 0.666667 | 0.61592 | 0.24657 | 0.3333 |
| 15 | -7 | -3 | 5 | 1 | 5 | -3 | 1.6467 | 0.78949 | 1.77005 | 1.77005 | 0.33333 | 0.74536 | 0.471405 | 0.471405 | 5.801E-17 | 0.3333 |
| 16 | -7 | 1 | 5 | 1 | -7 | -3 | 1.6466 | 0.78949 | 1.77012 | 1.77012 | 0.33333 | 0.74536 | 0.471405 | 0.471405 | 3.182E-17 | 0.3333 |
| 17 | -7 | -3 | -7 | -3 | -7 | -3 | 1.8257 | 0.78378 | 0.49979 | 0.49979 | 0 | 0 | 0.333333 | 0.333333 | 0.33333 | 0.2729 |
| 18 | -7 | 3 | -7 | -3 | 7 | -5 | 1.8315 | 1.38001 | 1.13712 | 1.85724 | 0.25512 | 0.4714 | 0.5773 | 0.333333 | 0.34386 | 0.2531 |
| 19 | -7 | 3 | 7 | 5 | -1 | 5 | 1.8315 | 1.38001 | 1.85724 | 1.13712 | 0.25512 | 0.4714 | 0.333333 | 0.57735 | 0.34386 | 0.2551 |
| 20 | -7 | -1 | -7 | 7 | -5 | 5 | 1.8315 | 1.37992 | 1.85725 | 1.13703 | 0.25512 | 0.4714 | 0.333333 | 0.57735 | 0.34386 | 0.2531 |
| 21 | -7 | -3 | 7 | -5 | 1 | -5 | 1.8315 | 1.37992 | 1.13703 | 1.85725 | 0.25512 | 0.4714 | 0.57735 | 0.333333 | 0.34386 | 0.2551 |
| 22 | -7 | -1 | -7 | 7 | 1 | 7 | 1.8853 | 0.90086 | 1.0982 | 1.0982 | 0.2357 | 0.70711 | 0.527046 | 0.527046 | 0.2357 | 0.2357 |
| 23 | -7 | 3 | -7 | -5 | 1 | -5 | 1.8853 | 0.90086 | 1.0982 | 1.0982 | 0.2357 | 0.70711 | 0.527046 | 0.527046 | 0.2357 | 0.2357 |
| 24 | -7 | -1 | 3 | -1 | -7 | -3 | 1.9665 | 0.85409 | 1.57855 | 1.3698 | 0.2357 | 0.70711 | 0.408248 | 0.408248 | 0.2357 | 0.5626 |
| 25 | -7 | 3 | 7 | 3 | -7 | -3 | 1.9665 | 0.85409 | 1.3698 | 1.57855 | 0.2357 | 0.70711 | 0.408248 | 0.408248 | 0.2357 | 0.5690 |
| 26 | -7 | -3 | 7 | 3 | 7 | -3 | 1.9666 | 0.854 | 1.57855 | 1.36967 | 0.2357 | 0.70711 | 0.408248 | 0.408248 | 0.2357 | 0.5690 |
| 27 | -7 | -3 | 3 | -1 | 3 | -3 | 1.9666 | 0.854 | 1.36967 | 1.57855 | 0.2357 | 0.70711 | 0.4082 | 0.4082 | 0.2357 | 0.5626 |
| 28 | -7 | -1 | -7 | -5 | 3 | -5 | 2.0649 | 0.64116 | 1.40959 | 1.40949 | 0.33333 | 0.80474 | 0.61592 | 0.6159 | 0.25512 | 0.3333 |
| 29 | -7 | 1 | -7 | 7 | -3 | 7 | 2.0649 | 0.64116 | 1.40949 | 1.40959 | 0.33333 | 0.80474 | 0.61592 | 0.6159 | 0.25512 | 0.3329 |

Table 3. Length 6 sequences proposed in [3]

| u | Sequence |  |  |  |  |  | PAPR-p0 | PAPR-p1 | PAPR-p2 | PAPR-p3 | AC-p0 | AC-p1 | AC-p2 | AC-p3 | XC-p0p1 | XC-p2p3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 2 | 1 | 2 | 7 | 4 | 7 | 2.0824 | 2.0824 | 2.6511 | 2.6511 | 0.2357 | 0.2357 | 0.2357 | 0.2357 | 0.7071 | 0.7071 |
| 1 | 6 | 5 | 2 | 5 | 2 | 5 | 2.1654 | 2.1654 | 2.4796 | 2.4796 | 0.2357 | 0.2357 | 0.2357 | 0.2357 | 0.3710 | 0.3710 |
| 2 | 2 | 5 | 2 | 1 | 2 | 5 | 2.1654 | 2.1654 | 2.4796 | 2.4796 | 0.2357 | 0.2357 | 0.2357 | 0.2357 | 0.3710 | 0.3710 |
| 3 | 2 | 5 | 6 | 5 | 8 | 5 | 2.113 | 2.113 | 2.2577 | 2.2577 | 0.2357 | 0.2357 | 0.2357 | 0.2357 | 0.4714 | 0.4714 |
| 4 | 8 | 1 | 6 | 1 | 4 | 1 | 2.0793 | 2.0793 | 2.3653 | 2.3653 | 0.2357 | 0.2357 | 0.2357 | 0.2357 | 0.5684 | 0.5684 |
| 5 | 6 | 3 | 4 | 3 | 8 | 3 | 2.0793 | 2.0793 | 2.3653 | 2.3653 | 0.2357 | 0.2357 | 0.2357 | 0.2357 | 0.5684 | 0.5684 |
| 6 | 2 | 7 | 4 | 5 | 4 | 7 | 2.0823 | 2.0823 | 2.651 | 2.651 | 0.2357 | 0.2357 | 0.2357 | 0.2357 | 0.7071 | 0.7071 |
| 7 | 2 | 3 | 6 | 3 | 8 | 3 | 2.1131 | 2.1131 | 2.2577 | 2.2577 | 0.2357 | 0.2357 | 0.2357 | 0.2357 | 0.4714 | 0.4714 |
| 8 | 2 | 3 | 2 | 5 | 8 | 5 | 2.0824 | 2.0824 | 2.6511 | 2.6511 | 0.2357 | 0.2357 | 0.2357 | 0.2357 | 0.7071 | 0.7071 |
| 9 | 2 | 5 | 8 | 7 | 8 | 5 | 2.0823 | 2.0823 | 2.651 | 2.651 | 0.2357 | 0.2357 | 0.2357 | 0.2357 | 0.7071 | 0.7071 |
| 10 | 2 | 1 | 2 | 5 | 2 | 5 | 2.1654 | 2.1654 | 2.4796 | 2.4796 | 0.2357 | 0.2357 | 0.2357 | 0.2357 | 0.3710 | 0.3710 |
| 11 | 4 | 1 | 8 | 1 | 6 | 1 | 2.0792 | 2.0792 | 2.3653 | 2.3653 | 0.2357 | 0.2357 | 0.2357 | 0.2357 | 0.5684 | 0.5684 |
| 12 | 4 | 1 | 4 | 3 | 4 | 7 | 2.1131 | 2.1131 | 2.2577 | 2.2577 | 0.2357 | 0.2357 | 0.2357 | 0.2357 | 0.4714 | 0.4714 |
| 13 | 4 | 3 | 6 | 3 | 8 | 3 | 2.1131 | 2.1131 | 2.2577 | 2.2577 | 0.2357 | 0.2357 | 0.2357 | 0.2357 | 0.4714 | 0.4714 |
| 14 | 2 | 5 | 6 | 5 | 2 | 5 | 2.1654 | 2.1654 | 2.4796 | 2.4796 | 0.2357 | 0.2357 | 0.2357 | 0.2357 | 0.3710 | 0.3710 |
| 15 | 2 | 1 | 6 | 1 | 4 | 1 | 2.0793 | 2.0793 | 2.3653 | 2.3653 | 0.2357 | 0.2357 | 0.2357 | 0.2357 | 0.5684 | 0.5684 |
| 16 | 2 | 5 | 2 | 3 | 2 | 7 | 2.0793 | 2.0793 | 2.3653 | 2.3653 | 0.2357 | 0.2357 | 0.2357 | 0.2357 | 0.5684 | 0.5684 |
| 17 | 4 | 7 | 4 | 3 | 4 | 1 | 2.0793 | 2.0793 | 2.3653 | 2.3653 | 0.2357 | 0.2357 | 0.2357 | 0.2357 | 0.5684 | 0.5684 |
| 18 | 2 | 3 | 2 | 7 | 2 | 7 | 2.1652 | 2.1652 | 2.4794 | 2.4794 | 0.2357 | 0.2357 | 0.2357 | 0.2357 | 0.3710 | 0.3710 |
| 19 | 2 | 3 | 2 | 5 | 2 | 7 | 2.1131 | 2.1131 | 2.2578 | 2.2578 | 0.2357 | 0.2357 | 0.2357 | 0.2357 | 0.4714 | 0.4714 |
| 20 | 2 | 3 | 8 | 5 | 8 | 3 | 2.0823 | 2.0823 | 2.651 | 2.651 | 0.2357 | 0.2357 | 0.2357 | 0.2357 | 0.7071 | 0.7071 |
| 21 | 2 | 7 | 8 | 7 | 2 | 5 | 2.0824 | 2.0824 | 2.6511 | 2.6511 | 0.2357 | 0.2357 | 0.2357 | 0.2357 | 0.7071 | 0.7071 |
| 22 | 4 | 7 | 6 | 7 | 4 | 1 | 2.0824 | 2.0824 | 2.6511 | 2.6511 | 0.2357 | 0.2357 | 0.2357 | 0.2357 | 0.7071 | 0.7071 |
| 23 | 2 | 1 | 2 | 5 | 2 | 7 | 2.113 | 2.113 | 2.2577 | 2.2577 | 0.2357 | 0.2357 | 0.2357 | 0.2357 | 0.4714 | 0.4714 |
| 24 | 4 | 1 | 8 | 1 | 4 | 1 | 2.1652 | 2.1652 | 2.4794 | 2.4794 | 0.2357 | 0.2357 | 0.2357 | 0.2357 | 0.3710 | 0.3710 |
| 25 | 2 | 1 | 4 | 7 | 4 | 1 | 2.0823 | 2.0823 | 2.651 | 2.651 | 0.2357 | 0.2357 | 0.2357 | 0.2357 | 0.7071 | 0.7071 |
| 26 | 8 | 7 | 8 | 5 | 8 | 3 | 2.0793 | 2.0793 | 2.3653 | 2.3653 | 0.2357 | 0.2357 | 0.2357 | 0.2357 | 0.5684 | 0.5684 |
| 27 | 2 | 3 | 2 | 7 | 2 | 5 | 2.0792 | 2.0792 | 2.3653 | 2.3653 | 0.2357 | 0.2357 | 0.2357 | 0.2357 | 0.5684 | 0.5684 |
| 28 | 2 | 7 | 2 | 3 | 2 | 5 | 2.1131 | 2.1131 | 2.2577 | 2.2577 | 0.2357 | 0.2357 | 0.2357 | 0.2357 | 0.4714 | 0.4714 |
| 29 | 2 | 5 | 4 | 5 | 8 | 5 | 2.1131 | 2.1131 | 2.2578 | 2.2578 | 0.2357 | 0.2357 | 0.2357 | 0.2357 | 0.4714 | 0.4714 |

Table 4. Length 6 sequences proposed in [4]

| u | Sequence |  |  |  |  |  | PAPR-p0 | PAPR-p1 | PAPR-p2 | PAPR-p3 | AC-p0 | AC-p1 | AC-p2 | AC-p3 | XC-p0p1 | XC-p2p3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | -5 | -1 | 3 | -1 | 3 | 1.6341 | 2.12958 | 1.8823 | 2.7257 | 0.097631 | 0.235702 | 0.40825 | 0.471405 | 0.43311 | 0.24117 |
| 1 | 5 | -3 | -1 | -5 | 1 | 7 | 2.0315 | 1.51547 | 1.8296 | 2.4001 | 0.1804 | 0.754344 | 0.50474 | 0.471405 | 0.29614 | 0.50474 |
| 2 | 7 | 1 | -5 | -1 | -3 | 5 | 2.2559 | 2.09132 | 1.8297 | 2.4002 | 0.1804 | 0.754344 | 0.50474 | 0.61592 | 0.31246 | 0.49116 |
| 3 | -1 | 5 | 5 | -7 | 3 | -3 | 2.209 | 2.00131 | 1.7166 | 2.5835 | 0.1804 | 0.504743 | 0.31246 | 0.666667 | 0.49863 | 0.75434 |
| 4 | 7 | 1 | -5 | -1 | -1 | 5 | 2.2089 | 2.00146 | 1.7166 | 2.5835 | 0.1804 | 0.504743 | 0.31246 | 0.097631 | 0.50474 | 0.74358 |
| 5 | 1 | 5 | -3 | -1 | -7 | 3 | 1.9506 | 0.69723 | 1.4085 | 1.7852 | 0.1804 | 0.641795 | 0.43552 | 0.481408 | 0.64179 | 0.50474 |
| 6 | 5 | 1 | 5 | -7 | -1 | -7 | 2.0434 | 2.55953 | 1.5515 | 1.5515 | 0.2357 | 0.235702 | 0.40825 | 0.333333 | 0.40825 | 0.32759 |
| 7 | -1 | -5 | -7 | 5 | -7 | -5 | 2.2435 | 2.06293 | 1.8695 | 1.8695 | 0.2357 | 0.235702 | 0.40825 | 0.333333 | 0.92751 | 0.67597 |
| 8 | -7 | 3 | -1 | 7 | -5 | -7 | 2.2721 | 3.05002 | 1.9462 | 4.5145 | 0.2357 | 0.235702 | 0.40825 | 0.333333 | 0.70444 | 0.44256 |
| 9 | 5 | -5 | 1 | -5 | 5 | 3 | 1.9919 | 1.99322 | 1.1292 | 2.6671 | 0.2357 | 0.235702 | 0.52705 | 0.255122 | 0.69476 | 0.52705 |
| 10 | -3 | 5 | 3 | -7 | -3 | -7 | 1.6489 | 4.16245 | 2.1837 | 3.4622 | 0.2357 | 0.408248 | 0.2357 | 0.255122 | 0.33617 | 0.39839 |
| 11 | -1 | -3 | 1 | 5 | 1 | 5 | 1.4624 | 2.66941 | 1.9606 | 2.7257 | 0.097631 | 0.235702 | 0.40825 | 0.61592 | 0.40825 | 0.26149 |
| 12 | 1 | -7 | 5 | -1 | 3 | -1 | 2.2285 | 2.96668 | 1.3545 | 3.6871 | 0.2357 | 0.235702 | 0.56904 | 0.666667 | 0.56904 | 0.23765 |
| 13 | 3 | 7 | 3 | 7 | -5 | 5 | 1.6341 | 2.12944 | 1.8824 | 2.7257 | 0.097631 | 0.235702 | 0.40825 | 0.666667 | 0.43311 | 0.24117 |
| 14 | 1 | -5 | -3 | -7 | -3 | 1 | 2.0481 | 2.78126 | 2.1408 | 3.0748 | 0.2357 | 0.527046 | 0.2357 | 0.61592 | 0.52705 | 0.58162 |
| 15 | -3 | 7 | 3 | -7 | -3 | -5 | 1.9943 | 3.27711 | 1.7276 | 3.5402 | 7.40E-17 | 0.471405 | 0.33333 | 0.471405 | 0.66195 | 0.61298 |
| 16 | -5 | -7 | -3 | 5 | 1 | -5 | 2.2721 | 3.04996 | 1.9462 | 4.5143 | 0.2357 | 0.235702 | 0.40825 | 0.471405 | 0.70444 | 0.44256 |
| 17 | 5 | 1 | -3 | 1 | 5 | 7 | 2.2493 | 2.41905 | 2.243 | 1.896 | 0.2357 | 0.097631 | 0.2357 | 0.333333 | 0.92983 | 0.74788 |
| 18 | -1 | -5 | 5 | -5 | -1 | 3 | 2.0434 | 1.98531 | 1.9301 | 2.3795 | 0.2357 | 0.097631 | 0.40825 | 0.333333 | 0.8978 | 0.71135 |
| 19 | -7 | -5 | 5 | 1 | -7 | -3 | 1.8771 | 1.76324 | 1.9562 | 2.6898 | 0.2357 | 0.527046 | 0.2357 | 0.57735 | 0.31961 | 0.52705 |
| 20 | 1 | 5 | -7 | 5 | 1 | -1 | 2.2493 | 2.37637 | 2.243 | 1.896 | 0.2357 | 0.097631 | 0.2357 | 0.57735 | 0.90297 | 0.74788 |
| 21 | 7 | 5 | -7 | 1 | -3 | 5 | 2.1101 | 3.97415 | 1.9716 | 4.3685 | 0.097631 | 0.235702 | 0.40825 | 0.333333 | 0.66206 | 0.40825 |
| 22 | 5 | -1 | -7 | -5 | 3 | 7 | 1.9507 | 0.69723 | 1.4083 | 1.7852 | 0.1804 | 0.641795 | 0.43552 | 0.527046 | 0.62739 | 0.49231 |
| 23 | -1 | -5 | -1 | 5 | 3 | -5 | 1.6488 | 4.16278 | 2.1837 | 3.4621 | 0.2357 | 0.408248 | 0.2357 | 0.527046 | 0.33617 | 0.40825 |
| 24 | 1 | -7 | 7 | -5 | 5 | -1 | 2.256 | 2.09135 | 1.8296 | 2.4001 | 0.1804 | 0.754344 | 0.50474 | 0.408248 | 0.30025 | 0.50474 |
| 25 | -7 | 1 | -3 | 5 | -7 | 7 | 2.1099 | 3.97401 | 1.9717 | 4.3682 | 0.097631 | 0.235702 | 0.40825 | 0.408248 | 0.66206 | 0.39843 |
| 26 | 3 | -7 | -3 | -7 | 1 | 5 | 1.9572 | 1.14805 | 1.6835 | 2.3256 | 0.2357 | 0.838546 | 0.56904 | 0.408248 | 0.23207 | 0.40025 |
| 27 | -5 | -7 | -3 | 3 | -1 | -7 | 1.9942 | 3.27731 | 1.7278 | 3.5404 | 4.62E-17 | 0.471405 | 0.33333 | 0.408248 | 0.66195 | 0.61592 |
| 28 | -1 | 7 | 3 | -7 | -1 | -3 | 2.0214 | 3.29481 | 1.5018 | 3.4847 | 0.1804 | 0.435521 | 0.43552 | 0.61592 | 0.43552 | 0.42742 |
| 29 | 1 | -5 | -1 | 3 | -1 | 3 | 1.6341 | 2.12958 | 1.8823 | 2.7257 | 0.097631 | 0.235702 | 0.40825 | 0.471405 | 0.43311 | 0.24117 |

Figure 1 provide the PAPR performance of the sequence proposed in [3] for port 2/3 and the sequence proposed in [4] for port 1/3. We can see that the sequences for port 1 and 3 from [4] has worse PAPR property, even worse than Rel-15 CGS sequences, and the proposed sequences have better PAPR performance than the sequences proposed in [3] and [4].

Figure 1. PAPR of some sequences proposed in [3] and [4]

Figure 2 provide the cross-correlation in different sequences, where port 0 is used in the evaluation of the sequences in [2], [3] and [4].  We can see that the cross-correlation is problematic in the sequences from [2] and [3], and the proposed sequences are better than other proposed ones.

Figure 2. Cross-correlation of 30 sequences in different proposals

Figure 3 provides the BLER when using the proposed sequences discussed above, where the BLER is averaged from different ports and all 30 sequences. As shown in the figure, the proposed sequences are with better performance in BLER than the sequences proposed in [2], [3], and [4].

Figure 3. Average BLER for all ports and sequences in each of the proposed sequence sets

From the above performance compared with different sequences, we have the following observations

Observation 1: The maximum auto correlation is up to 0.93264 for port 1 and up to 0.666667 for port 3, and the maximum cross correlation between port2&port3 is up to 0.666667 in [2].

Observation 2: PAPR proposed in [3] and [4] cannot fulfill the low PAPR requirement for both of the two CDM groups.

Observation 3: PAPR and auto correlation proposed in [4] cannot fulfill the low PAPR and auto correlation requirement for both of the two CDM groups.

Observation 4: Our proposed sequences with low auto-correlation achieve better channel estimation performance than the sequences proposed in [2], [3] and [4].

Conclusions

According to the provided evaluation, we have the following observations:

Observation 1: The maximum auto correlation is up to 0.93264 for port 1 and up to 0.666667 for port 3, and the maximum cross correlation between port2&port3 is up to 0.666667 in [2].

Observation 2: PAPR proposed in [3] and [4] cannot fulfill the low PAPR requirement for both of the two CDM groups.

Observation 3: PAPR and auto correlation proposed in [4] cannot fulfill the low PAPR and auto correlation requirement for both of the two CDM groups.

Observation 4: Our proposed sequences with low auto-correlation achieve better channel estimation performance than the sequences proposed in [2], [3] and [4].

- References

R1-1901570, “Discussion on DMRS sequence design for low PAPR”, Huawei, HiSilicon.

R1-1813445, “Lower PAPR reference signals”, Qualcomm.

R1-1812510, “Low PAPR Reference Signals”, Intel.

R1-1812259, “Discussion on Low PAPR RS”, Ericsson.

Appendix: Simulation assumptions

| System bandwidth | 20 MHz |
| --- | --- |
| Numerology | 30 KHz SCS |
| Channel | CDL-A 1000ns |
| Number of Antennas | UE Tx=1; gNB Rx =2 |
| Number of UEs | 1 UE |
| PUSCH duration | 12 OFDM symbols with one front loaded DMRS symbol |
| # RBs for PUSCH | 1,2,3,4 RBs |
| Carrier Frequency | 4 GHz |
| Coding rate | 0.2 |
| UE mobility | 3 Km/h |
| Receiver | Practical Time Domain Channel Estimation Ideal Noise Estimation |