3GPP TSG-TSG RAN WG1 Meeting # 96	R1-1903267

# Athens, Greece,  25th Feb - 1st March, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 1215 | rev |  | Current version: | 14.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for VoLTE enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_VoLTE_ViLTE_enh-Core |  |  |  |  |  | Date: |  |  | 2019-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Some specification errors originating from copy/paste the same eMTC clauses: (E)PDCCH does not have repetitions, and that RVID may be obtained from DCI 0C. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In subclause 8.0, remove “last” from “last subframe in which the PDCCH/EPDCCH is received”, since (E)PDCCH is not repeated. - In subclause 8.6.1, clarify that RVID is obtained from DCI format 0C |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification is inconsistent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.0, 8.6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 8.0	UE procedure for transmitting the physical uplink shared channel

<Unchanged parts are omitted>

A UE configured with parameter pusch-EnhancementsConfig shall upon detection on a given serving cell of an PDCCH/EPDCCH with DCI Format 0C intended for the UE, perform a corresponding PUSCH transmission in subframe(s) n+ki if a transport block corresponding to the HARQ process of the PUSCH transmission is generated as described in [8] with i = 0, 1, …, N-1 according to the PDCCH/EPDCCH, where

-	subframe n is the subframe in which the PDCCH/EPDCCH is transmitted; and

-	x≤k0<k1<…,kN-1 and the value of N is given by Table 8-2k based on the repetition number field in the corresponding DCI Format 0C; and

-	in case N>1, subframe(s) n+ki with i=0,1,…,N-1 are N consecutive UL subframe(s) starting with subframe n+x, and in case N=1, k0=x;

-	for FDD, x = 4;

-	for TDD UL/DL configurations 1-6, the value of x is given as the value of k in Table 8-2 for the corresponding TDD UL/DL configuration; If the value x is not given in Table 8-2 for subframe n, denote subframe n' as the first downlink/special subframe which has a value in Table 8-2 after subframe n, and substitute n with n' in the above procedure for performing the PUSCH transmission.

-	for TDD UL/DL configuration 0 and N=1, if the MSB of the UL index in the PDCCH with DCI format 0C is set to 1, the value of x is given as the value of k in Table 8-2 for the corresponding TDD UL/DL configuration; if the LSB of the UL index in the PDCCH with DCI format 0C is set to 1, x = 7. The UE is not expected to receive DCI format 0C with both the MSB and LSB of the UL index set to 1 when N>1. In case both the MSB and LSB of the UL index are set to 1, the HARQ process number of the PUSCH corresponding the MSB of the UL index is  and the HARQ process number of the PUSCH corresponding the LSB of the UL index is , where  is determined according to the HARQ process number field in DCI format 0C

<Unchanged parts are omitted>

### 8.6.1	Modulation order and redundancy version determination

<Unchanged parts are omitted>

For BL/CE UEs or for UEs configured with higher layer parameter PUSCH-EnhancementsConfig, the same redundancy version is applied to PUSCH transmitted in a given block of  consecutive subframes. The subframe number of the first subframe in each block of consecutive subframes, denoted as , satisfies . Denote  as the subframe number of the first uplink subframe intended for PUSCH. For BL/CE UEs, the PUSCH transmission spans  consecutive subframes including non-BL/CE subframes where the PUSCH transmission is postponed. For the  block of consecutive subframes within the set of  subframes, the redundancy version (rvidx) is determined according to Table 7.1.7.1-2 using , where , and . The  blocks of subframes are sequential in time, starting with  to which subframe belongs. For a BL/CE UE configured in CEModeA,  and  is determined by the 'Redundancy version' field in DCI format 6-0A. For a BL/CE UE configured with CEModeB,  for FDD and  for TDD, and . For a UE configured with higher layer parameter PUSCH-EnhancementsConfig,  and  is determined by the 'Redundancy version' field in DCI format 0C. For UEs configured with higher layer parameter PUSCH-EnhancementsConfig, NabsPUSCH=NrepPUSCH.

<Unchanged parts are omitted>