3GPP TSG RAN WG1 Meeting #96												R1-1903559

Athens, Greece, February 25th – March 1st, 2019

Source:	vivo

Title:	Summary#3 of UE power Consumption Reduction in RRM Measurements

Agenda Item:	7.2.9.3

Document for:	Discussion and Decision`

Introduction

This contribution is a summary of the contributions of RAN1#96 in Athens.

UE adpating/relaxing RRM measurement and power consumption characteristics

This section includes UE adpating/relaxing RRM measurement techniques from the following aspects,

- Adapting/Relaxing RRM measurement in time domain

- Adapting/Relaxing intra-frequency measurements

- Additional resources for RRM measurement

- Adapting/Relaxing inter-frequency measurements

- Adapting/relaxing the number of measured beams by considering spatial information aspects

- Other techniques

Adapting/Relaxing RRM measurement in time domain

Performance of power reduction

<Please check and filling the results in the following sheet>

Table 1 Evaluation of relaxed RRM measurement in time-domain

|  | (0) | (1) | (2) |  | (3) | (4) | (5) | (6) | (7) | (8) | (9) | (10) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (0) Power saving schemes description (1) Average power consumption (2) Power reduction compared to baseline [%] |  |  |  | (3) RRC Idle/RRC Inactive (i) or RRC Connected (c) (4)Measurement period [ms] (5)number of samples(e.g., SSB bursts) in each MP (6) UE speed and channel model (7) system impact, e.g., overhead (8) Performance impact (9)Additional assumption, e.g., PO and SSB offset, |  |  |  |  |  |  | Note |
| Source 1 Huawei R1-1903430 | If the RSRP of the serving cell is less than X, the MPs of serving cell and neighbour cells are according to RAN4’s requirement; otherwise the MPs are N times of RAN4’s requirement. | DRX cycle length = 0.32s 7.22(baseline) 5.21(N=4) 4.87(N=8) | 27.9% (N=4) 32.5% (N=8) |  | IDLE | - | 1 | 3km/h and 30km/h NLOS | - | Little impact on RSRP accuracy, which can be seen in Fig. 1 in R1-1903430. RSRP difference increases, which can be seen in Fig. 2 in R1-1903430. | The periodicity of SSB is 20ms, and the gap length between PO and SSB are randomly distributed | FR1 8 neighbor cells for intra-frequency measurement; 8 neighbor cells in each layer for inter-frequency measurement and totally 3 layers. Cell search is performed every three measurements. Group paging rate is 10%, and paging rate is 0%. 1 SSB burst set with 2 SSB is used for loop convergence, with periodicity of max (160ms, DRX cycle). |
|  |  |  |  | DRX cycle length = 0.64s 4.11(baseline) 3.11(N=4) 2.94(N=8) |  |  |  | 24.75% (N=4) 28.6 (N=8) |  |  |  | IDLE |  | - |  | 1 |  |  | - |  |  |  |  |  |
|  |  |  |  | DRX cycle length = 1.28s 2.56(baseline) 2.05(N=4) 1.97(N=8) |  |  |  | 19.7% (N=4) 23.0% (N=8) |  |  |  | IDLE |  | - |  | 1 |  |  | - |  |  |  |  |  |
|  |  |  |  | DRX cycle length = 2.56s 1.78(baseline) 1.53(N=4) 1.48(N=8) |  |  |  | 14.2% (N=4) 16.5% (N=8) |  |  |  | IDLE |  | - |  | 1 |  |  | - |  |  |  |  |  |
|  |  |  |  | DRX cycle length = 0.32s 3.35(baseline) 2.21(N=4) 2.02(N=8) |  |  |  | 34.0% (N=4) 39.7 (N=8) |  |  |  | IDLE |  | - |  | 1 | 3km/h and 30km/h LOS |  | - |  |  |  |  |  |
|  |  |  |  | DRX cycle length = 0.64s 2.17(baseline) 1.60(N=4) 1.51(N=8) |  |  |  | 26.2% (N=4) 30.6% (N=8) |  |  |  | IDLE |  | - |  | 1 |  |  | - |  |  |  |  |  |
|  |  | DRX cycle length = 1.28s 1.59(baseline) 1.30(N=4) 1.25(N=8) | 17.9% (N=4) 20.9% (N=8) |  | IDLE | - | 1 |  | - |  |  |  |
|  |  | DRX cycle length = 2.56s 1.29(baseline) 1.15(N=4) 1.13(N=8) | 11.0% (N=4) 12.8% (N=8) |  | IDLE | - | 1 |  | - |  |  |  |
| Source 2 Vivo R1-1901711 | Only SSB is used for RRM (2 samples per DRX cycle) | 4.17 | Baseline1 |  | Idle | 320 | 2 | TDL-C 3km/h 30km/h | Low | RSRP accuracy impact are captured in Figure 2-3 in R1-1901711 | The power saving gain is average gain of [0-10] ms SSB/CSI-RS and PO offset. Additional assumptions: 1. The duration of SSB/CSI-RS measurement occasion per paging cycle is 2ms. 2. Periodicity of SSB is 20ms and SSB is transmitted in the 1st slot of each 20ms period. 3. Periodicity of CSI-RS is 10ms and CSI-RS is transmitted in the 6th slot of each radio frame. 4. Paging cycle (DRX cycle) is 320ms, and the paging is monitored in one slot in first 2 radio frames of each paging cycle by different UEs with equal probability (random paging occasion). 5. Assuming no cell reselection. Thus no SIB1 energy consumption is included. 6. Assuming no additional resource for AGC tuning. The slot duration is based on 30 kHz SCS for FR1 and 120kHz for FR2. | FR1 |
|  | The number of measurement samples is reduced to 1 per paging cycle in idle state. Measurement is still based on SSB | 2.84 | 31.9% |  | Idle | 320 | 1 |  | Low |  |  |  |
|  | The measurement periodicity is extended to 2 paging cycle, 2 measurements sample per paging cycle. | 3.02 | 27.6% |  | Idle | 640 | 2 |  | Low |  |  |  |
|  | Only SSB is used for RRM (2 samples per DRX cycle) | 2.97 | Baseline2 |  | Idle | 320 | 2 |  | Low |  |  | FR2 |
|  | The number of measurement samples is reduced to 1 per paging cycle in idle state. Measurement is still based on SSB | 2.173 | 26.8% |  | Idle | 320 | 1 |  | Low |  |  |  |
|  | The measurement periodicity is extended to 2 paging cycle, 2 measurements sample per paging cycle. | 2.11 | 29% |  | Idle | 640 | 2 |  | Low |  |  |  |
|  | Reduce RRM measurement in RRC connected state. with short DRX InactivityTimer | 40.45 | Baseline1 |  | Con | 200 | 5 |  | Low |  | Long DRXCycle = 160ms, onDurationTimer = 8ms, data_rate=300Mbps FTP Model 3, packet size =0.5MByte Periodic activities is not modeled, e.g., AGC | FR1, 200 ms measurement period is considered as the baseline. DRX InactivityTimer = 40ms |
|  |  | 35.80 | 11.5% |  | Con | 400 | 5 |  | Low |  |  |  |
|  |  | 31.38 | 22.4% |  | Con | 800 | 5 |  | Low |  |  |  |
|  | Reduce RRM measurement in RRC connected state. With long DRX InactivityTimer | 73.65 | Baseline2 |  | Con | 200 | 5 |  | Low |  |  | FR1, 200 ms measurement period is considered as the baseline. DRX InactivityTimer = 100ms |
|  |  | 69.98 | 5.2% |  | Con | 400 | 5 |  | Low |  |  |  |
|  |  | 69.15 | 6.2% |  | Con | 800 | 5 |  | Low |  |  |  |
| Source 3 MediaTek R1-1903354 | UE adapts the measurement period to be 2 times default measurement period when PCell RSRP is 2dB better than the best neighbor cell and 4 times default measurement period when PCell RSRP is 4dB better than the best neighbour cell | 28.28 (baseline is 35.95 according to Table II) | 21.34% (55.64% RRM measurement is saved) |  | Con | 200/400/800 | 5 | 30km/h, 3D UMa with 200m ISD | Near negligible, only simple comparison is needed | handover fail rate 2.08% 2.81% | CDRX-cycle=40ms DRX-on=4ms SMTC window=2ms SMTC period=20ms, Light sleep time for Case 2=8ms FR1 | Periodic activities, e.g. time/frequency, channel or beam tracking, and loop convergence, are performed every 160ms during the SMTC window. Periodic activities will align with RRM measurement if possible. |
|  | UE adapts the measurement period to be 2 times default measurement period when PCell RSRP is 2dB better than the best neighbor cell and 4 times default measurement period when PCell RSRP is 4dB better than the best neighbour cell. UE keeps default measurement period when the variation between two serving cell RSRP measurements is larger than 0.3dB. | 29.45 (baseline is 35.95 according to Table II) | 18.08% (47.42% RRM measurement is saved) |  | Con | 200/400/800 | 5 | 30km/h, 3D UMa with 200m ISD | Near negligible, only simple comparison is needed | handover fail rate 2.08% 2.43% | CDRX-cycle=40ms DRX-on=4ms SMTC window=2ms SMTC period=20ms, Light sleep time for Case 2=8ms FR1 | Periodic activities, e.g. time/frequency, channel or beam tracking, and loop convergence, are performed every 160ms during the SMTC window. Periodic activities will align with RRM measurement if possible. |
|  | UE adapts the measurement period to be 2 times default measurement period when PCell RSRP is 2dB better than the best neighbor cell and 4 times default measurement period when PCell RSRP is 4dB better than the best neighbour cell | 29.49 (baseline is 35.95 according to Table II) | 17.97% (46.76% RRM measurement is saved) |  | Con | 200/400/800 | 2 | 120km/h, 3D UMa with 500m ISD | Near negligible, only simple comparison is needed | handover fail rate 9.03% 10.07% | CDRX-cycle=40ms DRX-on=4ms SMTC window=2ms SMTC period=20ms, Light sleep time for Case 2=8ms FR1 | Periodic activities, e.g. time/frequency, channel or beam tracking, and loop convergence, are performed every 160ms during the SMTC window. Periodic activities will align with RRM measurement if possible. |
|  | UE adapts the measurement period to be 2 times default measurement period when PCell RSRP is 2dB better than the best neighbor cell and 4 times default measurement period when PCell RSRP is 4dB better than the best neighbour cell. UE keeps default measurement period when the variation between two serving cell RSRP measurements is larger than 1dB. | 30.78 (baseline is 35.95 according to Table II) | 14.38% (37.38% RRM measurement is saved) |  | Con | 200/400/800 | 2 | 120km/h, 3D UMa with 500m ISD | Near negligible, only simple comparison is needed | handover fail rate 9.03% 9.59% | CDRX-cycle=40ms DRX-on=4ms SMTC window=2ms SMTC period=20ms, Light sleep time for Case 2=8ms FR1 | Periodic activities, e.g. time/frequency, channel or beam tracking, and loop convergence, are performed every 160ms during the SMTC window. Periodic activities will align with RRM measurement if possible. |
| Source 4 CATT R1-1902026 | UE performs 5 measurement in one measurement period. | 30.24 | baseline |  | Con | 200 | 5 | 3km/h | - | - Fulfilling the RSRP accuracy requirement in RAN4 RSRP accuracy impact are captured in R1-1902026 | DRX-cycle=40ms, DRX-on=4ms, SMTC period = 20ms | Relaxing samples in MP |
|  | UE reduces the number of measurement samples in one measurement period from 5 to 3. | 27 | 10.71% |  | Con | 200 | 3 | 3km/h | - |  | DRX-cycle=40ms, DRX-on=4ms, SMTC period = 20ms |  |
|  | UE performs 5 measurement in one measurement period. | 10.08 | baseline |  | Con | 800 | 5 |  | - |  | DRX-cycle=160ms, DRX-on=8ms, SMTC period = 20ms |  |
|  | UE reduces the number of measurement samples in one measurement period from 5 to 3. | 9.55 | 5.26% |  | Con | 800 | 3 |  | - |  | DRX-cycle=160ms, DRX-on=8ms, SMTC period = 20ms |  |
|  | UE performs 5 measurement in one measurement period. | 5.88 | baseline |  | Con | 1600 | 5 |  | - |  | DRX-cycle=320ms, DRX-on=10ms, SMTC period = 20ms |  |
|  | UE reduces the number of measurement samples in one measurement period from 5 to 3. | 5.35 | 2.57% |  | Con | 1600 | 3 |  | - |  | DRX-cycle=320ms, DRX-on=10ms, SMTC period = 20ms |  |
|  | The measurement period is 800ms. | 11.5 | baseline |  | RRC Connected | 800 | 5 | - | - |  | DRX-cycle=160ms, DRX-on=8ms, SMTC period = 40ms | Relaxing measurement period |
|  | The measurement period is extended from 800ms to 1600ms | 10.13 | 11.91% |  | RRC Connected | 1600 | 5 | - | - |  | DRX-cycle=160ms, DRX-on=8ms, SMTC period = 40ms |  |
|  | The measurement period is extended from 800ms to 3200ms | 9.45 | 17.83% |  | RRC Connected | 3200 | 5 |  | - |  | DRX-cycle=160ms, DRX-on=8ms, SMTC period = 40ms |  |
| Source 5 Sony R1-1902187 | Reduced number of samples per measurement period | 32.28 | 24% |  | Conn | 80 | 2/slot | N/A | None | - |  | Baseline power is 42.4 units |
|  | Reduced number of samples per measurement period | 27.21 | 36% |  | Conn | 160 | 2/slot | N/A | none | - |  |  |
| Source 6 DoCoMo R1-1903236 | Normal case | 31 | baseline |  | Conn | 320 | 5 | 3km/h |  |  | FR1 | C-DRX cycle = 40ms, C-DRX on duration = 4ms 3km/h pedestrian, PDCCH-only monitoring during active time |
|  | Relaxed RRM in time domain | 26.63 | 14.1% |  | Conn | 640 | 5 |  |  |  | FR1 |  |
|  |  | 25.75 | 16.9% |  | Conn | 800 | 5 |  |  |  | FR1 |  |
|  |  | 24.44 | 21.2% |  | Conn | 1280 | 5 |  |  |  | FR1 |  |
|  |  | 24 | 22.6% |  | Conn | 1600 | 5 |  |  |  | FR1 |  |
|  |  | 23.34 | 24.7% |  | Conn | 2560 | 5 |  |  |  | FR1 |  |
| Source 7 Qualcomm R1-1903017 | L = 1, R = 1/2 | 2.06 | baseline |  | Idle/Inactive | 1280 | 2 | stationary |  |  | FR1 | - the considered adaptation skips the measurement in one or more I-DRX cycles which is defined by variable R (e.g., R = 1/2 means RRM measurement is skipped once every 2 DRXs). - L denotes the number of SSBs in a SSB set Notes: (3) I-DRX/paging cycle (4) The number of SSB burst sets are used for RRM and AGC/T-F tracking loops (5) UE speed (km/h) or stationary |
|  |  | 2.04 | 0.71% |  |  | 1280 | 2 |  |  |  |  |  |
|  | L = 4, R = 1/2 | 2.11 | baseline |  |  | 1280 | 2 |  |  |  | FR1 |  |
|  |  | 2.07 | 1.85% |  |  | 1280 | 2 |  |  |  |  |  |
|  | L = 8, R = 1/2 | 2.19 | baseline |  |  | 1280 | 2 |  |  |  | FR1 |  |
|  |  | 2.11 | 3.57% |  |  | 1280 | 2 |  |  |  |  |  |
|  | L = 1, R = 1/4 | 2.06 | baseline |  |  | 1280 | 2 |  |  |  | FR1 |  |
|  |  | 2.04 | 1.07% |  |  | 1280 | 2 |  |  |  |  |  |
|  | L = 4, R = 1/4 | 2.11 | baseline |  |  | 1280 | 2 |  |  |  | FR1 |  |
|  |  | 2.05 | 2.78% |  |  | 1280 | 2 |  |  |  |  |  |
|  | L = 8, R = 1/4 | 2.19 | baseline |  |  | 1280 | 2 |  |  |  | FR1 |  |
|  |  | 2.07 | 5.36% |  |  | 1280 | 2 |  |  |  |  |  |
|  | L = 1, R = 1/2 | 2.48 | baseline |  |  | 1280 | 3 |  |  |  | FR1 |  |
|  |  | 2.47 | 0.59% |  |  | 1280 | 3 |  |  |  |  |  |
|  | L = 4, R = 1/2 | 2.53 | baseline |  |  | 1280 | 3 |  |  |  | FR1 |  |
|  |  | 2.49 | 1.54% |  |  | 1280 | 3 |  |  |  |  |  |
|  | L = 8, R = 1/2 | 2.61 | baseline |  |  | 1280 | 3 |  |  |  | FR1 |  |
|  |  | 2.53 | 3.00% |  |  | 1280 | 3 |  |  |  |  |  |
|  | L = 1, R = 1/4 | 2.48 | baseline |  |  | 1280 | 3 |  |  |  | FR1 |  |
|  |  | 2.46 | 0.89% |  |  | 1280 | 3 |  |  |  |  |  |
|  | L = 4, R = 1/4 | 2.53 | baseline |  |  | 1280 | 3 |  |  |  | FR1 |  |
|  |  | 2.47 | 2.32% |  |  | 1280 | 3 |  |  |  |  |  |
|  | L = 8, R = 1/4 | 2.61 | baseline |  |  | 1280 | 3 |  |  |  | FR1 |  |
|  |  | 2.49 | 4.49% |  |  | 1280 | 3 |  |  |  |  |  |
| Source 8 ZTE R1-1902030 R1-1902032 | The measurement period is 800ms. | 20.28 | baseline |  | Connect | 800 | 5 | N/A | N/A | N/A | DRX:160-40-10 (cycle-inactivity timer-ON duration) offset：U(0, measurement period) | Cell number to be measurement is 8. |
|  | The measurement period is extended from 800ms to 1600ms | 18.78 | 7.4% |  | Connect | 1600 | 5 | N/A | N/A | N/A |  |  |
|  | The measurement period is extended from 800ms to 3200ms | 18.03 | 11.1% |  | Connect | 3200 | 5 | N/A | N/A | N/A |  |  |
|  | The measurement period is 200ms. | 31.15 | baseline |  | Connect | 200 | 5 | N/A | N/A | N/A | DRX:40-10-4 (cycle-inactivity timer-ON duration) offset：U(0, measurement period) |  |
|  | The measurement period is extended from 200ms to 400ms | 25.61 | 17.8% |  | Connect | 400 | 5 | N/A | N/A | N/A |  |  |
|  | The measurement period is extended from 200ms to 800ms | 22.85 | 26.6% |  | Connect | 800 | 5 | N/A | N/A | N/A |  |  |
|  | The offset between SMTC window and DRX ON duration is subject to uniform distribution U(0, measurement period). | 6.53 | baseline |  | Idle/Inactive | 320 | 1 | N/A | N/A | N/A | DRX cycle is 320ms , PO：10% | Cell number to be measurement is 8. |
|  | The offset between SMTC window and DRX ON duration is 0 | 5.65 | 13.5% |  | Idle/Inactive | 320 | 1 | N/A | N/A | N/A | DRX cycle is 320ms , PO：10% |  |
|  | The offset between SMTC window and DRX ON duration is subject to uniform distribution U(0, measurement period). | 2.42 | baseline |  | Idle/Inactive | 1280 | 1 | N/A | N/A | N/A | DRX cycle is 1280ms , PO：10% |  |
|  | The offset between SMTC window and DRX ON duration is 0 | 2.19 | 9.5% |  | Idle/Inactive | 1280 | 1 | N/A | N/A | N/A | DRX cycle is 1280ms , PO：10% |  |
|  | The offset between SMTC window and DRX ON duration is subject to uniform distribution U(0, measurement period). | 20.28 | baseline |  | Connect | 800 | 5 | N/A | N/A | N/A | DRX:160-40-10 (cycle-inactivity timer-ON duration) Baseline offset：U(0, measurement period) |  |
|  | The offset between SMTC window and DRX ON duration is 0 | 18.83 | 7.1% |  | Connect | 800 | 5 | N/A | N/A | N/A | DRX:160-40-10 (cycle-inactivity timer-ON duration) Baseline offset：U(0, measurement period) |  |
| Source 9 Nokia R1-1903136 | System level evaluation of the impact of increasing measurement period. No power consumption analysis. | - | - |  | c | (a) 200ms (b) 400ms (c) 800ms (d) 1600ms (e) 3200ms | 5 samples | 3 km/h | N/A | Handover failure rate (%) - 0 - 0 - 0,26 - 0,14 - 0,49 | System level assumptions given in R1-1903136 |  |
|  |  |  |  |  |  |  |  | 30 km/h | N/A | Handover failure rate (%) - 0 - 0,08 - 0,41 - 1,57 - 11,33 |  |  |
|  |  |  |  |  |  |  |  | 60 km/h | N/A | Handover failure rate (%) - 0 - 0,33 - 1,00 - 3,68 - 25,89 |  |  |
|  |  |  |  |  |  |  |  | 120 km/h | N/A | Handover failure rate (%) - 0,17 - 0,86 - 3,14 - 15,59 - 56,74 |  |  |

In a short summary of the results

Relaxing measurement period

|  | (1) IDLE/INACTIVE | (2)CONNECTED |  | (3)Intra-/inter-frequency measurement included? | (4)Periodic activities includeded? | Note |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 2x |  | 4x |  |  |  |  |  |  |  |
| Source 1 Huawei R1-1903430 |  | 11%-39.7% |  |  |  |  |  | Y |  | y |  |  |  |
| Source 2 Vivo R1-1901711 |  | 29% |  | 11.5% |  |  | 22.4% | N |  | n |  | With FTP model 3 |  |
| Source 3 MediaTek R1-1903354 |  | 14.38% - 21.34% |  | Y | y |  |
| Source 4 CATT R1-1902026 |  |  |  | 11.91% |  |  | 17.83% | N |  | n |  |  |  |
| Source 6 DoCoMo R1-1903236 |  |  |  | 14.1% |  |  | 21.2% | N |  | n |  |  |  |
| Source 7 Qualcomm R1-1903017 |  | 0.71% - 5.36% |  |  |  |  |  | N |  | y |  |  |  |
| Source 8 ZTE R1-1902030 R1-1902032 |  |  |  | 7.4% - 17.8% |  |  | 11.1% - 26.6% | Y |  | n |  |  |  |

Based on the above results, it can be observed that,

For adapting/relaxing RRM measurement in time domain, the followings are observed from simulation results,

[8 sources] provide the results for  increasing measuremenrt period.

For RRC CONNECTED state,

- [4 sources] show power saving gain by increasing measurement period. Among the results, [11.1% - 26.6%] power saving gain is shown for extending measurement period  4 times, and [7.4% - 17.8%] power saving gain is shown for extending measurement period  2 times.

For RRC IDLE/INACTIVE state,

- If period of RRM activities is small or equal to that of periodic activities, [2 sources] show [11.91% - 29%] power saving gain by increasing measurement period,

- Otherwise, if intra-/inter-frequency measurement is also included, [1 source] show [11%-39.7%] power saving gain by increasing the measurement period,else [1 source] shows [0.71% - 5.36%] power saving gain by increasing the measurement period,

While at the same time, for stationary or low mobility case, [3 sources] provide results show the mobility performance impact of  increasing measuremenrt period  (e.g., handover failure rate, RSRP difference between two adjacent samples).

- [2 sources] provides results show that the handover failure rate does not exceed 0.5% for 3km/h and 3% for 30km/h with RRM relaxing in time-domain.

- [2 sources] provide results show the  (5%-tile, 95%-tile) of the RSRP measurement error is within the range of (XX, YY)

Relaxing number of samples within measurement period

|  | (1)IDLE/INACTIVE | (2)CONNECTED | (3)Intra-/inter-frequency measurement included? | (4)Periodic activities includeded? | Note |
| --- | --- | --- | --- | --- | --- |
| Source 4 CATT R1-1902026 |  | 2.57% -10.71% | n | n |  |
| Source 5 Sony R1-1902187 |  | 24% - 36% | n | n |  |
| Source 2 Vivo R1-1901711 | 26.8% - 31.9% |  | n | n |  |

[3 sources] show that relaxing the number of samples within measurement period provides [2.57% - 36%] power saging gain for RRC CONNECTED. For stationary or low mobility case, [1 source] results show the  (5%-tile, 95%-tile) of the RSRP measurement error is within the range of ([-2dB], [3dB]) for both 3km/h and 30km/h .

Impact to sampled channel power variance

R1-1903430	Power consumption reduction in RRM measurements		Huawei, HiSilicon

| (a) AWGN 3km/h RSRP difference | (b) AWGN 30km/h RSRP difference |
| --- | --- |
| (c) CDL-C 3km/h RSRP difference | (d) CDL-C 30km/h RSRP difference |

[R1-1903430]Fig. 2 RSRP difference between two adjacent samples

Impact to measurement accuracy

R1-1903430	Power consumption reduction in RRM measurements	Huawei, HiSilicon

| (a) AWGN 3km/h RSRP accuracy | (b) AWGN 30km/h RSRP accuracy |
| --- | --- |
| (c) CDL-C 3km/h RSRP accuracy | (d) CDL-C 30km/h RSRP accuracy |

[R1-1903430]Fig. 1 L3 filtered RSRP accuracy performance

R1-1901711	UE power saving in RRM Measurements	vivo

| (1) L1 RSRP accuracy | (2) L3 filtered RSRP accuracy |
| --- | --- |

Figure 2: RSRP accuracy per evaluation period-TDLC 3km/h

| (1) L1 RSRP accuracy | (2) L3 filtered RSRP accuracy |
| --- | --- |

Figure 3: RSRP accuracy per evaluation period-TDLC 30km/h

* Note 1: Measurement period (MP) is equal to the DRX cycle (320ms).

* Note 2: The RSRP measured on SSB is averaged over the RSRP measured on SSS and PBCH DMRS.

R1-1902026	UE Power saving scheme for RRM measurements		CATT

- UE mobile speed=3km/h                              b) UE mobile speed=30km/h

Figure 1:  RSRP measurement accuracy in different sampling number

Impact to mobility performance

R1-1903136	UE Power Consumption Reduction in RRM Measurements	Nokia, Nokia Shanghai Bell

Table 2. Handover failure rate as function of UE speed and measurement period

| Handover Failure Rate (%) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Speed (km/h)/Meas. Period (ms) | 200 | 400 | 800 | 1600 | 3200 |
| 3 | 0 | 0 | 0,26 | 0,14 | 0,49 |
| 30 | 0 | 0,08 | 0,41 | 1,57 | 11,33 |
| 60 | 0 | 0,33 | 1,00 | 3,68 | 25,89 |
| 120 | 0,17 | 0,86 | 3,14 | 15,59 | 56,74 |

Observation 1: For low mobility/low speed UE, increasing the measurement period has no severe impact on mobility performance.

Observation 2: On high/moderate UE speeds, using too long measurement period has non-negligible on mobility performance.

R1-1901805	NR RRM UE power saving	MediaTek Inc.

Procedures/Triggering

a) For UE autonomous RRM measurement adaptation in time-domain with gNB controlled threshold, in the last meeting the followings is agreed that,

Agreements:

For UE autonomous RRM measurement adaptation in time-domain with gNB controlled threshold, the following thresholds and corresponding adaptation schemes can be considered,

- A RSRP threshold for UE adapting RRM measurement period,

- A RSRP threshold for UE adapting RRM number of samples within a measurement period,

- A RSRP threshold and a RSRP variation threshold within a period of time, and based on that, UE can adapt the measurement or report period

- A RSRP variation threshold within a period of time, and based on that, UE can adapt the measurement or report period

- A threshold for UE adapting RRM measurement period and the threshold can be at least one of the followings,

  - The amount of time during which the UE stays with a specific cell or beam (for RRM measurement)

  - UE’s active TCI state for PDCCH does not change for specific time period.

  - The number of handovers/reselections for certain period.

- A threshold which includes UE mobility status, serving cell quality, and based on that, UE can adapt the SMTC window, number of CSI-RS resources sets per target cell, periodicity of CSI-RS resources.

Discuss whether any update is needed or not?

b) For gNB controlled RRM measurement operation with/without UE assistant information, the gNB can indicate to the UE the following information in order to adapt/relax RRM measurement in time-domain,

- gNB indicates a relaxed measurement periods/samples in time domain which enables UE to perform RRM measurement less frequently,

  - CATT, LGE, MediaTek, Qualcomm, Vivo, ZTE

Discuss the possible proposals/agreements for this part

Proposals from companies’ contributions

DoCoMo

Proposal 3: Relaxing the number of RRM measurement attempts in time domain based on the conditions regarding measurement result should be studied for UE power consumption reduction in RRM. - NW can fully control whether/how the above conditions can be applied for relaxing the number of RRM measurement attempts in time domain. Proposal 5: The measurement quantity which triggers RRM measurement relaxation should be same as that for triggering the measurement reporting. - Section 5.3.1 in TR 38.840, i.e., “Adapting/Relaxing RRM measurement in time domain,” should include RSRQ and SINR as measurement quantity of the threshold for RRM measurement relaxation/adaptation.

Intel

Proposal 1: - To define a new "stationary-mobility state" for NR UEs (i.e. UEs with no mobility or very low mobility). - UE reports its mobility speed status or request approximate measurement configurations based on the evaluated mobility status. Proposal 2: - To support the activated or deactivated of a configured measurement objects (MO) based on UE’s mobility state.

Qualcomm

Proposal 5: RAN1 considers adaptive RRM measurement configuration in RRC-CONNECTED i.e., - Multiple RRM measurement configurations (e.g., measurement duration, periodicity and measurement gap) can be configured via RRC - A suitable measurement configuration can be dynamically indicated to UE via DCI or MAC-CE.

Huawei

Proposal 2: The threshold based mechanism for RRM relaxation in time domain can reduce power consumption while having little impact on the measurement accuracy.

MediaTek

Proposal 1: The measurement period can be extended when PCell RSRP is offset better than the neighbor cell with best RSRP and return to default value when the condition is no longer satisfied. Proposal 2: An additional serving cell RSRP variation threshold can be used on top of the threshold between serving cell and neighbor cell to guarantee UE does not extend measurement period when the serving cell RSRP variation is large.

Nokia

Proposal 1: Consider enhancements to RSRP based thresholds based mechanism such as S-measure for adapting RRM measurements that consider additional conditions such as UE location in a cell and UE mobility state. Proposal 2: Before considering any further changing the UE measurement samples, it should be clarified with RAN4 if they have intent to introduce such requirements that would allow this kind of behaviour to be verified. Proposal 3: Define the conditions when UE cannot anymore apply the adaptation to RRM measurements.

LGE

Proposal 2: Consider configuring multiple sets of measurement behaviours/configurations to a UE where each set corresponds to different level of RRM requirements and measurement periodicities.

OPPO

Proposal 5: The following can be studied for autonomous adaptation of the RRM measurements and reporting. - The configured threshold may be the UE’s velocity or measured RSRP - Adaptation of RRM measurement and reporting periodicity, measured cell lists, measured SSB positions, measurement slots etc. based on the thresholds.

ZTE

Proposal 3: Different configurations can be applied for different states for RRM measurement and UE movement speed.

Samsung

Proposal 1: Autonomous reduction in number of measurement samples dynamically for RRM measurement based on SNR and/or mobility state of the UE is supported. Proposal 2: It is supported that gNB facilitates the operation with dynamically adjusting the density of measurement resources based on UE inputs.

vivo

Proposal 2: UE is allowed to take one sample for RRM measurement per paging DRX cycle. Proposal 3: The L3 RRM measurement periodicity can be relaxed to multiple paging DRX cycles.

Conclusions

Adapting/Relaxing intra-frequency measurements

Descriptions from TR38.840 It is observed that the following mechanisms can be beneficial in achieving the UE power saving, - Reducing the number of cells for intra-frequency measurement can be beneficial for UE power saving, - Assuming that UE can limit the processing for measurement within a constrained time period and/or with reduced complexity. - Assuming number of neighbouring cells to be measured is reduced. - For UE power saving perspective, reducing the need in neighbour cell intra-frequency measurement can be beneficial.

Performance of power reduction

<Please check and filling the results in the following sheet>

Table 2 Evaluation of Adapting/Relaxing intra-frequency measurements

|  | (0) | (1) | (2) |  |  | (3) | (4) | (5) | (6) | (7) | (8) | (9) | (10) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (0) Power saving schemes description (1) Average power consumption (2) Power reduction compared to baseline [%] |  |  |  |  | (3) RRC Idle/RRC Inactive (i) or RRC Connected (c) (4)Measurement period [ms] (5)number of samples(e.g., SSB bursts) in each MP (6) UE speed and channel model (7) system impact, e.g., overhead (8) Performance impact (9)Additional assumption, e.g., PO and SSB offset, |  |  |  |  |  |  | Note |
| Source 1 Huawei R1-1903430 | Intra-cell measurement: With full cell search, the measured cell number can be reduced from 8 to 4. The power can be reduced from M to N. M is taken as baseline. The power in the bracket is power unit per millisecond. FR2 | M=821860(1.64635) N(a)=763360(1.52917) N(b)= 604630(1.2112) |  | (a)7.12% (b)26.43% |  | IDLE | 1280 | 1 | - | Additional signalling overhead from network | possible mobility impact | FR1 30kHz Sync | DRX cycle length = 1.28s; Total considered time is 390 DRX cycle, i.e., 499.2s; the gap length between PO and SSB is assumed as 0; Cell search is performed every three measurements. Group paging rate is 10%, and paging rate is 0%. 1 SSB burst set with 2SSB is used for loop convergence, with periodicity of max (160ms, DRX cycle). (a) measurement duration = SMTC duration(5ms) (b) measurement duration = 2ms randomly located in SMTC window |
|  |  | M= 860860(1.72448) N(a)= 802360(1.60729) N(b)= 620230(1.24245) |  | (a)6.80% (b)27.95% |  | IDLE | 1280 | 1 | - |  | possible mobility impact | FR1 30kHz Async |  |
|  |  | M=822055(1.64674) N(a)= 763555(1.52956) N(b)= 604825(1.21159) |  | (a)7.12% (b)26.43% |  | IDLE | 1280 | 1 | - |  | possible mobility impact | FR1 60kHz Sync |  |
|  |  | M= 861055(1.72487) N(a)= 802555(1.60768) N(b)= 620425(1.24284) |  | (a)6.79% (b)27.95% |  | IDLE | 1280 | 1 | - |  | possible mobility impact | FR1 60kHz Async |  |
|  |  | M=997555(1.99831) N(a)= 939055(1.88112) N(b)= 675025(1.35221) |  | (a)5.86% (b)32.33% |  | IDLE | 1280 | 1 | - |  | possible mobility impact | FR2 60kHz Sync |  |
|  |  | M= 1114555(2.23268) N(a)= 1056055(2.11569) N(b)= 721825(1.44596) |  | (a)5.25% (b)35.24% |  | IDLE | 1280 | 1 | - |  | possible mobility impact | FR2 60kHz Async |  |
|  |  | M=997652.5(1.9985) N(a)= 939152.5(1.88132) N(b)= 675122.5(1.35241) |  | (a)5.86% (b)32.33% |  | IDLE | 1280 | 1 | - |  | possible mobility impact | FR2 120kHz Sync |  |
|  |  | M= 1114653(2.23288) N(a)= 1056153(2.11569) N(b)= 721922.5(1.44616) |  | (a)5.25% (b)35.23% |  | IDLE | 1280 | 1 | - |  | possible mobility impact | FR2 120kHz Async |  |
|  | Intra-cell measurement: With full cell search, the measured cell number can be reduced from 16 to 4. The power can be reduced from M to N. M is taken as baseline. The power in the bracket is power unit per millisecond. FR1 | M=938860(1.88073) N=763360(1.52917) |  | 18.69% |  | IDLE | 1280 | 1 | - |  | possible mobility impact | FR1 30kHz Sync | DRX cycle length = 1.28s; Total considered time is 390 DRX cycle, i.e., 499.2s; measurement duration = SMTC duration(5ms) the gap length between PO and SSB is assumed as 0; Cell search is performed every three measurements. Group paging rate is 10%, and paging rate is 0%. 1 SSB burst set with 2SSB is used for loop convergence, with periodicity of max (160ms, DRX cycle). |
|  |  | M= 977860(1.95885) N= 802360(1.60729) |  | 17.95% |  | IDLE | 1280 | 1 | - |  | possible mobility impact | FR1 30kHz Async |  |
|  |  | M=939055(1.88112) N= 763555(1.52956) |  | 18.69% |  | IDLE | 1280 | 1 | - |  | possible mobility impact | FR1 60kHz Sync |  |
|  |  | M= 978055(1.95924) N= 802555(1.60768) |  | 17.94% |  | IDLE | 1280 | 1 | - |  | possible mobility impact | FR1 60kHz Async |  |
|  |  | M=1114555(2.23568) N= 939055(1.88112) |  | 15.75% |  | IDLE | 1280 | 1 | - |  | possible mobility impact | FR2 60kHz Sync |  |
|  |  | M= 1231555(2.46706) N= 1056055(2.11569) |  | 14.25% |  | IDLE | 1280 | 1 | - |  | possible mobility impact | FR2 60kHz Async |  |
|  |  | M=1114652.5(2.23288) N= 939152.5 (1.88132) |  | 15.75% |  | IDLE | 1280 | 1 | - |  | possible mobility impact | FR2 120kHz Sync |  |
|  |  | M= 1231652.5(2.46725) N= 1056153(2.11569) |  | 14.25% |  | IDLE | 1280 | 1 | - |  | possible mobility impact | FR2 120kHz Async |  |
| Source 2 Vivo R1-1901711 | Energy for full measurement (8 cells) | 4.597 |  | Baseline1 |  | IDLE | 320 | 2 | -- | No | Low | The measurement duration in SMTC is 2ms | FR1, Cell search is assumed to be performed in 1/3 of the SMTC measurement occasions. |
|  | Reduce neighbour cell measurement( SCS is 30 kHz, Synchronous case, 4 cells) | 4.229 |  | 8% |  | IDLE | 320 | 2 | -- | No | Low | The measurement duration in SMTC is 2ms |  |
|  | Reduce neighbour cell measurement( SCS is 30 kHz, Asynchronous case, 4 cells) | 4.474 |  | 37.54% |  | IDLE | 320 | 2 | -- | No | Low | The measurement duration in SMTC duration is 2ms and 4ms for 4 cells and 8 cells. |  |
|  | Energy for full measurement (8 cells) | 7.77 |  | Baseline2 |  | IDLE | 320 | 2 | -- | No | Low | The measurement duration in SMTC is 2ms | FR2, Cell search is assumed to be performed in 1/3 of the SMTC measurement occasions. |
|  | Reduce neighbour cell measurement( SCS is 30 kHz, Synchronous case, 4 cells) | 7.02 |  | 9.6% |  | IDLE | 320 | 2 | -- | No | Low | The measurement duration in SMTC is 4ms |  |
|  | Additional threshold is considered to reduce the number of intra-frequency measurements based on S-measures enhancement. Details in section 4.2 R1-1901711 | - |  | Baseline3 |  | IDLE | 320 | 2 | -- | No | Low |  | System level analysis. All the UEs in the cell is considered and there are 70% UEs which can actually reduce the number of cells from more than 8 cells to 4 cells, in dense urban single layer. |
|  |  | - |  | 18% |  | IDLE | 320 | 2 | -- | No | Low | SintraSearchP(2) is around -110 ~ -102dBm, when SIntraSearchP =-90.5dB. |  |

| Source 3 ZTE R1-1902030 R1-1902032 | The number to be measured is 8, and UE performs intra-frequency measurement for each DRX cycle. | 6.53 | baseline |  | Idle | 320 | 1 | N/A | N/A | N/A | DRX cycle is 320ms PO：10% Offset：U(0, measurement period) | The SMTC duration is 2ms |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | The measured cell number is 8, but UE carries out intra frequency measurement every four DRX cycles, and only serving cell measurement is performed for the rest of the cycle. | 6.22 | 4.7% |  | Idle | 320 | 1 | N/A | N/A | N/A | DRX cycle is 320ms PO：10% Offset：U(0, measurement period) | The SMTC duration is 2ms |
|  | There are only serving cell to be measured. | 5.66 | 13.3% |  | Idle | 320 | 1 | N/A | N/A | N/A | DRX cycle is 320ms PO：10% Offset：U(0, measurement period) | The SMTC duration is 0.5ms |
|  | The number to be measured is 8 | 20.28 | baseline |  | Connect | 800 | 5 | N/A | N/A | N/A | DRX:160-40-10 (cycle-inactivity timer-ON duration) offset：U(0, measurement period) | The SMTC duration is 2ms |
|  | Reduce the measurement cell from 8 to 4 | 19.91 | 1.8% |  | Connect | 800 | 5 | N/A | N/A | N/A | DRX:160-40-10 (cycle-inactivity timer-ON duration) offset：U(0, measurement period) | The SMTC duration is 2ms |
|  | Reduce the measurement cell from 8 to 1 | 18.59 | 8.3% |  | Connect | 800 | 5 | N/A | N/A | N/A | DRX:160-40-10 (cycle-inactivity timer-ON duration) offset：U(0, measurement period) | The SMTC duration is 0.5ms |
|  | The number to be measured is 8 | 31.15 | baseline |  | Connect | 200 | 5 | N/A | N/A | N/A | DRX:40-10-4 (cycle-inactivity timer-ON duration) offset：U(0, measurement period) | The SMTC duration is 2ms |
|  | Reduce the measurement cell from 8 to 4 | 29.68 | 4.7% |  | Connect | 200 | 5 | N/A | N/A | N/A | DRX:40-10-4 (cycle-inactivity timer-ON duration) offset：U(0, measurement period) | The SMTC duration is 2ms |
|  | Reduce the measurement cell from 8 to 1 | 24.51 | 21.3% |  | Connect | 200 | 5 | N/A | N/A | N/A | DRX:40-10-4 (cycle-inactivity timer-ON duration) offset：U(0, measurement period) | The SMTC duration is 0.5ms |

Other performance impact

Procedures/Triggering

a) For UE autonomous RRM measurement for reducing the intra-frequency measurement with gNB controlled threshold, the following thresholds and corresponding adaptation schemes can be considered,

- Using RSRP/RSRQ threshold(s) for UE adapting  the number of measured non-serving cells,

b) For gNB controlled RRM measurement operation with/without UE assistant information, the following information in order to adapt/relax RRM measurement for intra-frequency provide by gNb can be studied,

- Spatial information from SSB and CSI-RS,

  - Huawei (directional information),

- Temporarily omit search for additional unknown cells if the NW can establish that all relevant candidate cells have been detected.

  - Ericsson

- Historical data and/or gNB assistance information (e.g. time offsets –absolute/indexed values)

  - Samsung

Proposals’ from contributions

DoCoMo

Proposal 4: Relaxing the number of measured cells/frequencies for neighbour cells based on the conditions regarding measurement result should be studied for UE power consumption reduction in RRM. - NW can fully control whether/how the above conditions can be applied for relaxing the number of measured cells/frequencies for neighbour cells.

Ericsson

Proposal 5: Prioritize use of UE mobility state info and serving cell RSRP as criteria for configuration switching. Proposal 7: NW may provide assistance info to UEs regarding maximal inter-cell T/F misalignment the UE should assume in the current deployment. Proposal 9: NW may indicate to the UE to temporarily omit search for additional unknown cells if UE has detected all candidate cells that are relevant at its location.

Huawei

Proposal 5: Additional information (e.g., directional information of reference signals) can be considered to reduce the number of neighbor cells and neighbor frequencies for RRM measurements without degrading the mobility performance.

LGE

Proposal 1: Investigate further an enhancement to the existing threshold based measurement relaxation (e.g., enhancement of s-Measure mechanism) to reduce UE power consumption, e.g. - Multi-level s-Measure mechanism, e.g., 1st level for skipping intra-frequency non-serving cell measurement, 2nd level for skipping all non-serving cell measurements

ZTE

Proposal 2: The RRM measurement requirements could be optimized for UE power saving.

Samsung

Proposal 4: Dynamic selection of beams and/or cells by UE is allowed with the help of historical data and/or gNB assistance information (e.g. time offsets –absolute/indexed values) through L1 signaling/MAC CE signaling.

Vivo

Proposal 6: Reducing number of measured intra-frequency cells should be supported for NR UE power saving. - Additional threshold can be considered to reduce the number of intra-frequency measurements based on S-measures enhancement.

Conclusions

Possible Proposal in RAN1#96:

- Capture Table 2 in R1-19xxxxx to TR38.840 section 5.3.2

  - With additional information provided and updated

    - baseline assumptions for measurement activities, e.g., loop convegernce, synchronization and etc.

Possible Proposal in RAN1#96: The followings are proposed to be captured in TR38.840 section 5.3.2

For adapting/relaxing RRM measurement for intra-frequency measurement, the followings are observed from simulation results,

[3 sources] show the results for adapting/relaxing RRM measurement for intra-frequency measurement by reducing the number of measured cells.

By assuming number of neighbouring cells to be measured is reduced, [3 source] shows that about [4.7% ~ 7.1%] power saving gain can be observed if reducing the number of measured cells for IDLE state. [1 source] shows that about [1.8% 21.3%] power saving gain can be observed if reducing the number of measured cells for CONNECTED state.

In additional to that, by also assuming that UE can limit the processing for measurement within a constrained time period and/or with reduced complexity, [2 sources] provide results show [26.43% - 37.5%] power saving gain if reducing the number of measured cells.

Additional resources for RRM measurement

Performance of power reduction

Table 3 Evaluation of additional resources for RRM power saving

|  | (0) | (1) | (2) |  | (3) | (4) |  | (5) | (6) | (7) |  | (8) | (9) | (10) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (0) Power saving schemes description (1) Average power consumption (2) Power reduction compared to baseline [%] |  |  |  | (3) RRC Idle/RRC Inactive (i) or RRC Connected (c) (4)Measurement period [ms] (5)number of samples(e.g., SSB bursts) in each MP (6) UE speed and channel model (7) system impact, e.g., overhead (8) Performance impact (9)Additional assumption, e.g., PO and SSB offset, |  |  |  |  |  |  |  |  | Note |
| Source 1 vivo R1-1901711 | Only SSB is used for RRM (2 samples per DRX cycle) | 4.17 | Baseline1 |  | RRC Idle |  | 320 | 2 | TDL-C 3km/h 30km/h |  | Low |  | The power saving gain is average gain of [0-10] ms SSB/CSI-RS and PO offset. | FR1 |
|  | The number of measurement samples is reduced to 1 per paging cycle in idle state. Measurement is still based on CSI-RS | 2.84 | 31.9% |  | RRC Idle |  | 320 | 1 | TDL-C 3km/h 30km/h |  | Low | No impact for RSRP accuracy, RSRP accuracy analysis in R1-1901711 | The power saving gain is average gain of [0-10] ms SSB/CSI-RS and PO offset. |  |
|  | The number of measurement samples is reduced to 1 per paging cycle in idle state. Measurement is still based on either SSB or CSI-RS depending on which is closer to PO in time. | 2.72 | 34.8% |  | RRC Idle |  | 320 | 1 | TDL-C 3km/h 30km/h |  | Low |  | The power saving gain is average gain of [0-10] ms SSB/CSI-RS and PO offset. The periodicity of CSI-RS is assumed to be 10ms. |  |
|  | Only SSB is used for RRM (2 samples per DRX cycle) | 2.97 | Baseline2 |  | RRC Idle |  | 320 | 2 | TDL-C 3km/h 30km/h |  | Low |  | The power saving gain is average gain of [0-10] ms SSB/CSI-RS and PO offset. | FR2 |
|  | The number of measurement samples is reduced to 1 per paging cycle in idle state. Measurement is still based on CSI-RS | 2.173 | 26.8% |  | RRC Idle |  | 320 | 1 | TDL-C 3km/h 30km/h |  | Low | No impact for RSRP accuracy, RSRP accuracy analysis in R1-1901711 | The power saving gain is average gain of [0-10] ms SSB/CSI-RS and PO offset. |  |
|  | The number of measurement samples is reduced to 1 per paging cycle in idle state. Measurement is still based on either SSB or CSI-RS depending on which is closer to PO in time. | 2.07 | 30.3% |  | RRC Idle |  | 320 | 1 | TDL-C 3km/h 30km/h |  | Low |  | The power saving gain is average gain of [0-10] ms SSB/CSI-RS and PO offset. The periodicity of CSI-RS is assumed to be 10ms. |  |
| Source 2 MediaTek R1-1903354 | Using SSS-only signal as additional RS before SSB for paging indication and RRM assistance. | 1.634 (baseline is 2.089, values are calculated in Appendix in R1-1903354) | 21.78% |  | Idle |  | One IDRX cycle (1280ms) | 2 | General |  | One symbol of SSS for SMTC window |  | IDRX-cycle=1280ms DRX on duration = 4ms SMTC window = 2ms SMTC period =20ms FR1 | Periodic activities, e.g. time/frequency, channel or beam tracking, and loop convergence, are performed every IDRX cycle during the SMTC window. Periodic activities will align with RRM measurement if possible. The SSS-only signal can be multiplexed in a CDMed way to accommodate more paging indications for different paging groups. |
| Source 3 CATT R1-1902026 | Normal RRM measurement according to SMTC configuration | 12.2 | baseline |  | connected |  | 800 | 5 | - |  | - | - | DRX-cycle=160ms, DRX-on=8ms, SMTC= 80ms | on-demand RS for RRM measurement is that UE could perform multiple RRM measurements (e.g. 5 measurements in a consecutive period) when UE is triggered to wake up at a given DRX-ON period. Then UE can do L1 filtering for multiple RRM measurements at one DRX-ON period |
|  | On-demand RS assistant RRM measurement during DRX-on duration | 9.2 | 24.6% |  | connected |  | 800 | 5 | - |  | - | - | DRX-cycle=160ms, DRX-on=8ms, SMTC= 80ms |  |
|  | Normal RRM measurement according to SMTC configuration | 66.34 | 0% |  | connected |  | 800 | 5 | - |  | - | - | DRX-cycle=160ms, DRX-on=8ms, Inactivity timer =100ms | on-demand RS is introduced to assist UE measurement during the data transmission, UE would go to sleep after data reception, which can reduce a part of UE power consumption. |
|  | On-demand RS assistant RRM measurement during data transmiton | 12.42 | 81.28% |  | connected |  | 800 | 5 | - |  | - | - | DRX-cycle=160ms, DRX-on=8ms, Inactivity timer =100ms |  |
| Source 4 Qualcomm R1-1903017 | L=1 | 2.06 |  |  | Idle/Inactive |  | 1.28 | 2 | stationary |  |  |  | FR1 | Notes: - (3) I-DRX/paging cycle - (4) The number of SSB burst sets are used for RRM and AGC/T-F tracking loops - (5) UE speed (km/h) or stationary L denotes the number of SSBs in a SSB set |
|  |  | 1.67 | 18.86% |  |  |  |  |  |  |  |  |  |  |  |
|  | L=4 | 2.11 |  |  | Idle/Inactive |  | 1.28 | 2 | stationary |  |  |  | FR1 |  |
|  |  | 1.67 | 20.74% |  |  |  |  |  |  |  |  |  |  |  |
|  | L=8 | 2.19 |  |  | Idle/Inactive |  | 1.28 | 2 | stationary |  |  |  | FR1 |  |
|  |  | 1.67 | 23.58% |  |  |  |  |  |  |  |  |  |  |  |
|  | L=1 | 2.17 |  |  | Idle/Inactive |  | 1.28 | 2 | 3 |  |  |  | FR1 |  |
|  |  | 1.68 | 22.36% |  |  |  |  |  |  |  |  |  |  |  |
|  | L=4 | 2.21 |  |  | Idle/Inactive |  | 1.28 | 2 | 3 |  |  |  | FR1 |  |
|  |  | 1.68 | 24.08% |  |  |  |  |  |  |  |  |  |  |  |
|  | L=8 | 2.29 |  |  | Idle/Inactive |  | 1.28 | 2 | 3 |  |  |  | FR1 |  |
|  |  | 1.68 | 26.66% |  |  |  |  |  |  |  |  |  |  |  |
|  | L=1 | 2.17 |  |  | Idle/Inactive |  | 1.28 | 2 | 30 |  |  |  | FR1 |  |
|  |  | 1.68 | 22.67% |  |  |  |  |  |  |  |  |  |  |  |
|  | L=4 | 2.22 |  |  | Idle/Inactive |  | 1.28 | 2 | 30 |  |  |  | FR1 |  |
|  |  | 1.68 | 24.37% |  |  |  |  |  |  |  |  |  |  |  |
|  | L=8 | 2.30 |  |  | Idle/Inactive |  | 1.28 | 2 | 30 |  |  |  | FR1 |  |
|  |  | 1.68 | 26.94% |  |  |  |  |  |  |  |  |  |  |  |
|  | L=1 | 2.48 |  |  | Idle/Inactive |  | 1.28 | 3 | stationary |  |  |  | FR1 |  |
|  |  | 1.67 | 32.66% |  |  |  |  |  |  |  |  |  |  |  |
|  | L=4 | 2.53 |  |  | Idle/Inactive |  | 1.28 | 3 | stationary |  |  |  | FR1 |  |
|  |  | 1.67 | 33.96% |  |  |  |  |  |  |  |  |  |  |  |
|  | L=8 | 2.61 |  |  | Idle/Inactive |  | 1.28 | 3 | stationary |  |  |  | FR1 |  |
|  |  | 1.67 | 35.94% |  |  |  |  |  |  |  |  |  |  |  |
|  | L=1 | 2.59 |  |  | Idle/Inactive |  | 1.28 | 3 | 3 |  |  |  | FR1 |  |
|  |  | 1.68 | 35.02% |  |  |  |  |  |  |  |  |  |  |  |
|  | L=4 | 2.64 |  |  | Idle/Inactive |  | 1.28 | 3 | 3 |  |  |  | FR1 |  |
|  |  | 1.68 | 36.22% |  |  |  |  |  |  |  |  |  |  |  |
|  | L=8 | 2.71 |  |  | Idle/Inactive |  | 1.28 | 3 | 3 |  |  |  | FR1 |  |
|  |  | 1.68 | 38.06% |  |  |  |  |  |  |  |  |  |  |  |
|  | L=1 | 2.60 |  |  | Idle/Inactive |  | 1.28 | 3 | 30 |  |  |  | FR1 |  |
|  |  | 1.68 | 35.24% |  |  |  |  |  |  |  |  |  |  |  |
|  | L=4 | 2.65 |  |  | Idle/Inactive |  | 1.28 | 3 | 30 |  |  |  | FR1 |  |
|  |  | 1.68 | 36.43% |  |  |  |  |  |  |  |  |  |  |  |
|  | L=8 | 2.72 |  |  | Idle/Inactive |  | 1.28 | 3 | 30 |  |  |  | FR1 |  |
|  |  | 1.68 | 38.26% |  |  |  |  |  |  |  |  |  |  |  |
|  | L=1 | 5.24 |  |  | Idle/Inactive |  | 0.32 | 2 | stationary |  |  |  | FR1 |  |
|  |  | 3.68 | 29.67% |  |  |  |  |  |  |  |  |  |  |  |
|  | L=4 | 5.43 |  |  | Idle/Inactive |  | 0.32 | 2 | stationary |  |  |  | FR1 |  |
|  |  | 3.68 | 32.20% |  |  |  |  |  |  |  |  |  |  |  |
|  | L=8 | 5.75 |  |  | Idle/Inactive |  | 0.32 | 2 | stationary |  |  |  | FR1 |  |
|  |  | 3.68 | 35.89% |  |  |  |  |  |  |  |  |  |  |  |
|  | L=1 | 5.66 |  |  | Idle/Inactive |  | 0.32 | 2 | 3 |  |  |  | FR1 |  |
|  |  | 3.73 | 34.21% |  |  |  |  |  |  |  |  |  |  |  |
|  | L=4 | 5.86 |  |  | Idle/Inactive |  | 0.32 | 2 | 3 |  |  |  | FR1 |  |
|  |  | 3.73 | 36.40% |  |  |  |  |  |  |  |  |  |  |  |
|  | L=8 | 6.17 |  |  | Idle/Inactive |  | 0.32 | 2 | 3 |  |  |  | FR1 |  |
|  |  | 3.73 | 39.62% |  |  |  |  |  |  |  |  |  |  |  |
|  | L=1 | 5.70 |  |  | Idle/Inactive |  | 0.32 | 2 | 30 |  |  |  | FR1 |  |
|  |  | 3.73 | 34.61% |  |  |  |  |  |  |  |  |  |  |  |
|  | L=4 | 5.89 |  |  | Idle/Inactive |  | 0.32 | 2 | 30 |  |  |  | FR1 |  |
|  |  | 3.73 | 36.78% |  |  |  |  |  |  |  |  |  |  |  |
|  | L=8 | 6.21 |  |  | Idle/Inactive |  | 0.32 | 2 | 30 |  |  |  | FR1 |  |
|  |  | 3.73 | 39.96% |  |  |  |  |  |  |  |  |  |  |  |
|  | L=1 | 6.93 |  |  | Idle/Inactive |  | 0.32 | 3 | stationary |  |  |  | FR1 |  |
|  |  | 3.68 | 46.81% |  |  |  |  |  |  |  |  |  |  |  |
|  | L=4 | 7.12 |  |  | Idle/Inactive |  | 0.32 | 3 | stationary |  |  |  | FR1 |  |
|  |  | 3.68 | 48.27% |  |  |  |  |  |  |  |  |  |  |  |
|  | L=8 | 7.43 |  |  | Idle/Inactive |  | 0.32 | 3 | stationary |  |  |  | FR1 |  |
|  |  | 3.68 | 50.44% |  |  |  |  |  |  |  |  |  |  |  |
|  | L=1 | 7.35 |  |  | Idle/Inactive |  | 0.32 | 3 | 3 |  |  |  | FR1 |  |
|  |  | 3.73 | 49.31% |  |  |  |  |  |  |  |  |  |  |  |
|  | L=4 | 7.55 |  |  | Idle/Inactive |  | 0.32 | 3 | 3 |  |  |  | FR1 |  |
|  |  | 3.73 | 50.63% |  |  |  |  |  |  |  |  |  |  |  |
|  | L=8 | 7.86 |  |  | Idle/Inactive |  | 0.32 | 3 | 3 |  |  |  | FR1 |  |
|  |  | 3.73 | 52.59% |  |  |  |  |  |  |  |  |  |  |  |
|  | L=1 | 7.39 |  |  | Idle/Inactive |  | 0.32 | 3 | 30 |  |  |  | FR1 |  |
|  |  | 3.73 | 49.55% |  |  |  |  |  |  |  |  |  |  |  |
|  | L=4 | 7.58 |  |  | Idle/Inactive |  | 0.32 | 3 | 30 |  |  |  | FR1 |  |
|  |  | 3.73 | 50.85% |  |  |  |  |  |  |  |  |  |  |  |
|  | L=8 | 7.89 |  |  | Idle/Inactive |  | 0.32 | 3 | 30 |  |  |  | FR1 |  |
|  |  | 3.73 | 52.80% |  |  |  |  |  |  |  |  |  |  |  |
|  | CSI-RS for mobility, w/o repetition (baseline) | 36.9 | baseline |  | Connected |  | 40ms | 16slots | Stationary |  |  |  | the power model for CSI-RS for mobility and the scaling rule with respect to the number of CSI-RS symbols in a slot are described in R1-1903017 | Repetition of CSI-RS for mobility in FR2 |
|  | CSI-RS for mobility, with 2 repetition | 32.5 | 11.8% |  | Connected |  | 80ms | 16slots | Stationary |  |  | Refined Rx beam |  |  |
|  | CSI-RS for mobility, with 4 repetition | 29.2 | 20.7% |  | Connected |  | 160ms | 16slots | Stationary |  |  | Refined Rx beam |  |  |
| Source 5 Ericsson R1-1902936 | UE performs AGC on previous SSB, measures on next SSB | 2826/1280 = 2.20 | - |  | Idle |  | 1280 |  |  |  |  |  |  |  |
|  | One additional SSB sweep for AGC is provided 4 ms ahead of the measured SSB sweep | 2522/1280 =1.97 | 11.8 |  | Idle |  | 1280 |  |  |  |  |  |  |  |
|  | UE performs AGC on previous SSB, then monitors ON-duration | 3032/1280 =2.36 | - |  | Idle |  | 1280 |  |  |  |  |  |  |  |
|  | An additional on-demand RS for AGC purposes is provided 5 ms before the start of ON-duration | 2918/1280 =2.28 | 3.8 |  | Idle |  | 1280 |  |  |  |  |  |  |  |
|  | UE performs AGC on previous SSB, measures on next SSB, followed by monitoring ON-duration | 3746/1280 =2.93 | - |  | Idle |  | 1280 |  |  |  |  |  |  |  |
|  | One additional SSB sweep for AGC is provided 4 ms ahead of the measured SSB sweep, measures on next SSB, followed by monitoring ON-duration | 3442/1280 =2.69 | 8.1 |  | Idle |  | 1280 |  |  |  |  |  |  |  |

Impact to measurement accuracy

R1-1901711	UE power saving in RRM Measurements	vivo

| (1) L1 RSRP accuracy | (2) L3 filtered RSRP accuracy |
| --- | --- |

Figure 2: RSRP accuracy per evaluation period-TDLC 3km/h

| (1) L1 RSRP accuracy | (2) L3 filtered RSRP accuracy |
| --- | --- |

Figure 3: RSRP accuracy per evaluation period-TDLC 30km/h

* Note 1: Measurement period (MP) is equal to the DRX cycle.

* Note 2: The RSRP measured on SSB is averaged over the RSRP measured on SSS and PBCH DMRS.

R1-1902026	UE Power saving scheme for RRM measurements		CATT

Figure 3. The RSRP performances of SSB-based RRM measurement and on-demand RS based RRM measurement

Procedures/Triggering

Additional resource for RRM measurement

- Supporting: MediaTek, vivo, Qualcomm, ASUSTek, OPPO, Samsung

- Not support: Nokia, Ericsson

Propose to further discuss during this week.

Agreemnts from last meeting

- The followings can be considered as the usage of additional resource for RRM measurement,

  - CSI-RS, including TRS

  - SSS only

  - SSB

  - PSS, SSS and wake-up signaling/paging

    - E.g., including transmitted in SFN (single frequency network) manner

  - DMRS for RMSI PDCCH/PDSCH for standalone

  - Additional new RS/signal (e.g, configuring additional RS next to SSB) in addition to existing RS/signal in Rel-15

  - Note: Existing structure/waveform of Rel-15 signals/channels is recommended to be studied for additional resource

  - Note: companies report the purpose of additional resource for RRM measurement, e.g., measurement, cell search and etc.

- Companies report in which of the following RRC state(s) the additional resource can be used,

  - RRC IDLE

  - RRC INACTIVE

  - RRC CONNECTED

Proposals’ from contributions

Qualcomm

Proposal 2: Capture the following procedure in TR when using PSS, SSS and wake-up signaling as additional resource for RRM for FR1 in RRC-IDLE/RRC-INACTIVE: - Cells within an area (e.g., RNA or RAN area) periodically broadcast PSS, SSS and wake-up signaling in an SFN manner over a single beam PSS and SSS are not transmitted on a sync frequency raster - UE performs SSS RSRP/RSRQ measurement of a camping area and neighbor areas if triggered - UE tracks wake-up signaling to detect whether UE is paged or not If UE is paged, UE shall determine a suitable cell to acquire paging. Otherwise, UE goes to sleep. Proposal 3: Capture the following procedure in TR when using PSS, SSS and paging as additional resource for RRM for FR1 in RRC-IDLE/RRC-INACTIVE: - Cells within an area (e.g., RNA or RAN area) periodically broadcast PSS, SSS and paging in an SFN manner over a single beam PSS and SSS are not transmitted on a sync frequency raster - UE performs SSS RSRP/RSRQ measurement of a camping area and neighbor areas if triggered - UE decodes paging for the page. If UE is paged, UE shall determine a suitable cell to transit into RRC-CONNECTED. Otherwise, UE goes to sleep. Proposal 6: RAN1 considers repetition of CSI-RS resources for mobility within a resource set.

MediaTek

Proposal 3: SSS-only signal is used as additional RS before the SSB as wake-up (power-saving) signal for paging indication and RRM assistance in RRC_IDLE/ RRC_INACTIVE. Proposal 4: Multiplexed SSS-only wake-up (power-saving) signals can be used before the SSB for paging indication. When no UE is to be paged, normal SSS is transmitted for enhancement of sync, RRM, and AGC tuning. Proposal 5: In 38.840 [2], 5.2.3, the following sub-bullet can be added Multiplexed SSS-only signal as additional RS before SSB (measurement occasion) for paging indication can be considered for power-saving and RRM/sync assistance.

Nokia

Proposal 4: Do not introduce additional signal for UE power saving in RRM measurements.

Ericsson

Proposal 4 :Additional RS before measurement or paging occasion are not supported.

Spreadtrum

Proposal 3: The additional resource can have functionalities of RRM measurement. Proposal 4: Additional resource for RRM measurement and additional RS as the aid of power saving signal can share the same resource. Proposal 5: Whether to support using channel in additional resource needs to consider UE complexity.

ASUSTek

Proposal 2: CSI-RS is used to perform RRM measurement in additional resource. Proposal 3: Adapting CSI-RS to fit the needs of power saving could be considered, e.g. transmission bandwidth adaptation of CSI-RS.

OPPO

Proposal 1: option 1(CSI-RS/TRS) and option 2(PBCH) as listed above can be considered as the additional RRM measurement reference signals.

Samsung

Proposal 3: Support RRM measurement triggered by CSI-RS based power saving signal outside C-DRX on duration.

vivo

Observation 5: Use of CSI-RS for idle state RRM measurement does not cause issues from network overhead or UE complexity perspective. Proposal 1: CSI-RS is introduced as measurement RS for UE in idle/inactive state.

Conclusion

Possible Proposal in RAN1#96:

- Capture Table 3 in R1-19xxxxx to TR38.840 section 5.3.3

  - With additional information provided and updated

- FFS: How to capture the results in section 2.2.2, 2.2.3

Based on the inputs, it is observed that,

Observation 1: With the results from 4 contributions, it shows that using additional resource for the purpose of RRM is useful,

Observation 2: with the results from 4 supporting companies, the proposals for additional resources are from different angles. It is listed as follows,

- Qualcommprovides results based on SFN reference signals which as additional resources for measurement and shows benefit reducing UE power saving due to reducing the neighboring cell measurement.

- MediaTek has simulated that with Rel-15 SSB structure, in some channel conditions, say 30km/h in-car dense urban, UE will require more time for cell/beam detection in case of bad AGC setting due to large measurement period and inter-frequency measurement, which increases RRM power consumption. Therefore, it is recommended to allocate extra resource for AGC tuning before PSS. The extra resource should lie in the same beam and locate before PSS. For this purpose, there can be assistant resource before the measurement occasion to assist AGC tuning. For example, NW can intentionally transmit resource in the control region of the same beam as the corresponding SSB to assist AGC tuning.

- Vivo and CATT’s proposal is trying to reduce the active time for measurement but from different angles.

  - Vivo proposes to have CSI-RS is introduced as measurement RS for UE in idle/inactive state and provide results shows about 35% power saving gain can be achieved if CSI-RS can be introduced as an additional measurement RS, based on which the measurement samples can be reduced and the measurement accuracy can still be guaranteed.

  - CATT proposes on-demand RS based RRM measurement can be beneficial for UE power saving, in which UE would finish measurement during DRX-ON period and go to sleep after all. The main idea of on-demand RS for RRM measurement is that UE could perform multiple RRM measurements (e.g. 5 measurements in a consecutive period) when UE is triggered to wake up at a given DRX-ON period. Then UE can do L1 filtering for multiple RRM measurements at one DRX-ON period. As a result, UE would finish RRM measurement in an active period with data arrival and continue sleeping after all.

Therefore, the following is proposed.

Possible Proposal in RAN1#96: The followings are proposed to be captured in TR38.840 section 5.3.3

For additional resources for RRM measurement , the followings are observed from simulation results,

[4] sources show that additional resources is beneficial for power saving perspective. The power saving gain is about [21%~81%]. For these schemes,

1 source evaluated CSI-RS based RRM for RRC IDLE state by reducing the active time for measurement.

1 source evaluated allowing UE performs multiple RRM measurements (e.g. 5 measurements in a consecutive period) when UE is triggered to wake up at a given DRX-ON period.

1 source evaluated multiplexed SSS-only signal as additional RS before each SSB (measurement occasion) for paging indication which can assist sync, RRM, and AGC tuning for RRC IDLE/INACTIVE state.

1 source provides results based on PSS, SSS and wake-up singalling/paging transmitted in single frequency network as additional resources for measurement and shows benefits reducing UE power saving due to reducing the number of UE wake-up occasions for measurement, loop convergence and paging monitoring, reducing the frequency of performing neighbor cell search, and reducing the neighboring cell measurement , and reducing the paging monitoring.

For the impact of the system overhead, reusing existing transmitted RS (e.g, TRS, CSI-RS) for IDLE UEs does not increase system overhead, increasing the period of the additional RS can reduce the system overhead.

Adapting/Relaxing inter-frequency measurements

Possible agreements in RAN1 Adhoc 1901 In order to reduce the inter-frequency measurement, the following directions are for further studies, - The number of measured frequencies could be reduced in intra-band RRM measurements to reduce UE power consumption. - The candidate frequencies in a group (i.e., grouping different frequencies within a band, since they have similar measurement results.) can be configured to UE, e.g. in SIB-x for measurement in idle mode. UE may measure only one frequency among the associated frequencies instead of measuring all the candidate frequencies. - The measurement probability for each measured frequencies can be configured by gNB. UE generates a random number and selects the cell to camp on or handover according to the measurement probability. - Providing time-frequency alignment information for improving inter-frequency measurement - Reducing inter-frequency measurement in case different serving beams have different neighbor frequency information.

| Company | Comments |
| --- | --- |
| Company A | comments |

Performance of power reduction

Table 4 Evaluation of Adapting/Relaxing inter-frequency measurements

|  | (0) | (1) | (2) |  | (3) | (4) | (5) | (6) | (7) | (8) | (9) | (10) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (0) Power saving schemes description (1) Average power consumption (2) Power reduction compared to baseline [%] |  |  |  | (3) RRC Idle/RRC Inactive (i) or RRC Connected (c) (4)Measurement period [ms] (5)number of samples(e.g., SSB bursts) in each MP (6) UE speed and channel model (7) system impact, e.g., overhead (8) Performance impact (9)Additional assumption, e.g., PO and SSB offset, |  |  |  |  |  |  | Note |
| Source 1 Huawei R1-1903430 | Inter-frequency measurement: With full cell search in each layer, the measured frequency number can be reduced from 6 to 3. The power can be reduced from M to N. M is taken as baseline. The power in the bracket is power unit per millisecond. | M(a)= 2820220(5.64948) M(b)= 2124460(4.25573) N= 1825330(3.65651) | (a)35.28% (b)14.08% |  | IDLE | 1280 | 1 | - | Additional signalling overhead from network | possible mobility impact | FR1 30kHz Sync | DRX cycle length = 1.28s; Total considered time is 390 DRX cycle, i.e., 499.2s; measured duration in each layer equals to SMTC duration = 5ms; switching time for FR1 equals 0.5ms, for FR2 equals 0.25ms; the gap length between PO and SSB is assumed as 0; 8 neighbor cells for intra-frequency measurement; 8 neighbor cells in each layer for inter-frequency measurement. Cell search is performed every three measurements. Group paging rate is 10%, and paging rate is 0%. 1 SSB burst set with 2SSB is used for loop convergence, with periodicity of max (160ms, DRX cycle); (a) totally indicated layer number= 6 and every layer has 8 neighbor cells. (b) totally indicated layer number= 6 and only 3 layers have neighbour cells with 8 for each. |
|  |  | M(a)= 3093220(6.19635) M(b)= 2280460(4.56823) N= 1981330(3.96901) | (a)35.95% (b)13.12% |  | IDLE | 1280 | 1 | - |  | possible mobility impact | FR1 30kHz Async |  |
|  |  | M(a)= 2820415(5.64987) M(b)= 2124655(4.25612) N= 1825525(3.6569) | (a)35.28% (b)14.08% |  | IDLE | 1280 | 1 | - |  | possible mobility impact | FR1 60kHz Sync |  |
|  |  | M(a)= 3093415(6.19635) M(b)= 2280655(4.56862) N= 1981525(3.9694) | (a)35.94% (b)13.12% |  | IDLE | 1280 | 1 | - |  | possible mobility impact | FR1 60kHz Async |  |
|  |  | M(a)= 4018885(8.05065) M(b)= 3039205(6.08815) N= 2510365(5.02878) | (a)37.54% (b)17.40% |  | IDLE | 1280 | 1 | - |  | possible mobility impact | FR2 60kHz Sync |  |
|  |  | M(a)= 4837885(9.69128) M(b)= 3507205(7.02565) N= 2978365(5.96628) | (a)38.44% (b)15.08% |  | IDLE | 1280 | 1 | - |  | possible mobility impact | FR2 60kHz Async |  |
|  |  | M(a)= 4018982.5(8.05085) M(b)= 3039302.5(6.08835) N= 2510462.5(5.02897) | (a)37.54% (b)17.4% |  | IDLE | 1280 | 1 | - |  | possible mobility impact | FR2 120kHz Sync |  |
|  |  | M(a)= 4837982.5(9.69147) M(b)= 3507302.5(7.02585) N= 2978462.5(5.96647) | (a)38.44% (b)15.08% |  | IDLE | 1280 | 1 | - |  | possible mobility impact | FR2 120kHz Async |  |
|  | CSI-RS resource in the same frequency layer are configured in a measurement window. | Number of configured CSI-RS resources = 4 5.03(baseline) 4.27(concentrated CSI-RS configuration) | 15.10% |  | CONNECTED | 200 | 5 | 3km/h NLOS | - | Ref to the figures | For baseline, all CSI-RS resource are randomly placed in the time domain (uniform distribution) | UE is assumed to retune its RF to target frequency to perform CSI-RS based inter-frequency measurement once there is a CSI-RS resource to measure, and it will immediately switch RF back to its serving frequency to continue Tx/Rx unless there is no enough time (i.e. time interval between two CSI-RS resources is shorter than 1ms). Number of configured CSI-RS resources are the allowed maximum number of CSI-RS resource for mobility |
|  |  | Number of configured CSI-RS resources = 16 7.32(baseline) 4.54(concentrated CSI-RS configuration) | 37.97% |  | CONNECTED | 200 | 5 | 3km/h NLOS | - | Ref to the figures |  |  |
|  |  | Number of configured CSI-RS resources = 32 8.69(baseline) 4.90(concentrated CSI-RS configuration) | 43.61% |  | CONNECTED | 200 | 5 | 3km/h NLOS | - | Ref to the figures |  |  |
|  |  | Number of configured CSI-RS resources = 48 9.04(baseline) 5.25(concentrated CSI-RS configuration) | 41.92% |  | CONNECTED | 200 | 5 | 3km/h NLOS | - | Ref to the figures |  |  |
|  |  | Number of configured CSI-RS resources = 64 8.97(baseline) 5.61(concentrated CSI-RS configuration) | 37.46% |  | CONNECTED | 200 | 5 | 3km/h NLOS | - | Ref to the figures |  |  |
|  |  | Number of configured CSI-RS resources = 80 8.75(baseline) 5.97(concentrated CSI-RS configuration) | 31.77% |  | CONNECTED | 200 | 5 | 3km/h NLOS | - | Ref to the figures |  |  |
|  |  | Number of configured CSI-RS resources = 96 8.52(baseline) 6.32(concentrated CSI-RS configuration) | 25.82% |  | CONNECTED | 200 | 5 | 3km/h NLOS | - | Ref to the figures |  |  |
| Source 2 Sony R1-1902187 | Reduced number of inter-freq layers measured (1 layer vs 2) | 58.4: 1 layer 73.9: 2 layer | 21% |  | Conn | 40ms | 2/slot | N/A | See note | - |  | Note: more non-gap resource is available to UE with fewer measurement gaps Power consumption averaged over DRX cycle and is also applicable to full measurement period |
|  | Reduced number of inter-freq layers measured (2 layer vs 4) | 73.9: 2 layer 99.6: 4 layer | 26% |  | Conn | 40 ms | 2/slot | N/A | See note | - |  |  |

Procedures/Triggering

The  followings are proposed, further discussion is needed.

In order to reduce the inter-frequency RRMmeasurement, the followings can be considered, - Reducing the measurement by grouping different frequencies within a band, UE measures only one frequency within the group. - CMCC, Samsung, - Providing time-frequency alignment information for improving inter-frequency measurement - Ericsson - Additional information (e.g., directional information of reference signals) can be considered to reduce the number of neighbor cells and neighbor frequencies for RRM measurements without degrading the mobility performance. - Huawei - CSI-RS transmission benefits from a measurement window configuration to avoid UE performing measurement outside the window. - Huawei

Proposals’ from contributions

CMCC

Proposal 1. For inter-frequency measurement, the number of intra-band measured frequencies could be reduced to realize UE power saving. Proposal 2. The measured frequencies or frequency groups and the association relationships between measured frequencies or measured frequency groups can be indicated to UE in measurement configuration information. UE measures one frequency or frequency group among the associated frequencies or frequency groups to be measured, and the measurement results are applied to all the associated frequencies or frequency groups. Proposal 3. The measurement probability for each measured frequencies can be configured by gNB. UE generates a random number and selects the cell to camp on or handover according to the measurement probability.

Huawei

Proposal 5: Additional information (e.g., directional information of reference signals) can be considered to reduce the number of neighbor cells and neighbor frequencies for RRM measurements without degrading the mobility performance.

Samsung

Proposal 5: Support cell-group based measurement based on gNB assistance information.

Ericsson

Proposal 7 : NW may provide assistance info to UEs regarding maximal inter-cell T/F misalignment the UE should assume in the current deployment.

Conclusion

Possible Proposal in RAN1#96:

- Add new section in TR38.840 “5.3.4 Adapting/relaxing RRM measurement for inter-frequency measurement”

- Capture Table 4 in R1-19xxxxx to TR38.840 section 5.3.4.

Possible Proposal in RAN1#96: The followings are proposed to be captured in TR38.840 section 5.3.4

For adapting/relaxing RRM measurement for inter-frequency measurement, 2 sources show that reducing the number of measured inter-frequency layers can provide 21~38% power saving gainfor all RRC states.

Therefore, considering the outcome of studies, it is recommended to specify the adapting/relaxing RRM measurement for inter-frequency measurement during the work item.

Reducing the number of measured beams by considering spatial information aspects

Performance of power reduction

Table 5 Evaluation of relaxed the number of measured beams by considering spatial information aspects

|  | (0) | (1) | (2) |  | (3) | (4) | (5) | (6) | (7) | (8) | (9) | (10) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (0) Power saving schemes description (1) Average power consumption (2) Power reduction compared to baseline [%] |  |  |  | (3) RRC Idle/RRC Inactive (i) or RRC Connected (c) (4)Measurement period [ms] (5)number of samples(e.g., SSB bursts) in each MP (6) UE speed and channel model (7) system impact, e.g., overhead (8) Performance impact (9)Additional assumption, e.g., PO and SSB offset, |  |  |  |  |  |  | Note |
| Source 1 Huawei R1-1903430 | Adjacent directional beams of each SSB can be provided by gNB to reduce SSB number of UE RRM measurement and loop convergence. A: UE measures all the beams B: all the adjacent beams and serving beam is used for RRM measurement and loop convergence. | A(baseline):6.09 B:3 | 50.7% |  | IDLE | DRX cycle length = 0.32s | 1 | - | Determined by signalling periodicity | Ref to the Fig. 4 | The periodicity of SSB is 20ms, and the gap length between PO and SSB are randomly distributed | - IDLE mode; - 1 sample in each DRX; - The periodicity of SSB is 20ms, and the gap length between PO and SSB are randomly distributed; - PDCCH only rate is 10%, and paging rate is 0%; - SSB number in a SSB burst set is 64 and 32 - The UE only performs the serving cell measurement and doesn’t do the neighbor cell measurement - The total simulation time is 499.2s 1 SSB burst set with 64 SSBs is used for loop convergence in baseline and with averaged adjacent SSBs for loop convergence in optimal schemes. |
|  |  | A(baseline):4.64 B:2 | 56.9% |  | IDLE | DRX cycle length = 0.64s | 1 | - |  |  |  |  |
|  |  | A(baseline): 3.91 B:1.5 | 61.7% |  | IDLE | DRX cycle length = 1.28 s | 1 | - |  |  |  |  |
|  |  | A(baseline): 3.55 B:1.25 | 64.7% |  | IDLE | DRX cycle length = 2.56 s | 1 | - |  |  |  |  |
|  |  | A(baseline): 3.9 B:2.47 | 36.6% |  | IDLE | DRX cycle length = 0.32s | 1 | - |  |  |  |  |
|  |  | A(baseline):3 B:1.73 | 42.1% |  | IDLE | DRX cycle length = 0.64s | 1 | - |  |  |  |  |
|  |  | A(baseline): 2.54 B:1.37 | 46.25% |  | IDLE | DRX cycle length = 1.28 s | 1 | - |  |  |  |  |
|  |  | A(baseline): 2.32 B:1.18 | 48.9% |  | IDLE | DRX cycle length = 2.56 s | 1 | - |  |  |  |  |

Fig. 4 RSRP accuracy performance evaluation for different RSRP levels of UE[Huawei, R1-1903430]

Procedures/Triggering

Propose to further discuss during this week.

Proposals’ from contributions

Huawei

Proposal 3: Information on adjacent directional beams of each SSB is provided by gNB to UE in order to reduce the number of SSBs for UE RRM measurements.

Sony

Observation 4: A UE switching from RRC CONNECTED state to RRC IDLE/INACTIVE state will have knowledge of a suitable receive beam configuration. Proposal 1: In certain conditions the UE can relax RRM measurements by reusing previously known beam configurations, and avoiding unnecessary measurements in other beam directions. Proposal 2: Previously known beam configurations can be used for up to a set time duration, subject to measurement results not degrading below an RSRP threshold.

Samsung

Proposal 4: Dynamic selection of beams and/or cells by UE is allowed with the help of historical data and/or gNB assistance information (e.g. time offsets –absolute/indexed values) through L1 signaling/MAC CE signaling.

Vivo

Proposal 5: Reducing the number of measured beams is beneficial for power saving and should be supported. - The concept of S criterion in cell re-selection can be used to reduce the number of measured beams of serving cell

Conclusion

Possible Proposal in RAN1#96:

- Add new section in TR38.840 “5.3.5 Adapting/relaxing the number of measured beams by considering spatial information aspects”

- Capture Table 5 in R1-19xxxxx to TR38.840 section 5.3.5

Possible Proposal in RAN1#96: The followings are proposed to be captured in TR38.840 section 5.3.5

For adapting/relaxing the number of measured beams, the followings are observed from simulation results,

For IDLE state, 1 source shows 36%~64% power saving gain for reducing RRM measurement for SSBs by considering adjacent directional beams of each SSB can be provided by gNB to reduce SSB number of UE RRM measurement and loop convergence. It also shows that the it has no severe impact on RSRP difference between two adjacent samples.

Others

Proposals’ from contributions

Qualcomm

Proposal 8: At least for FR2, RAN1 studies multi-panel RRM measurement options for UE power savings so that the link to the serving cell may not be interrupted during the RRM measurement.

Huawei

Proposal 6：Introducing of CSI-RS measurement window should be discussed in Rel-16.

OPPO

Proposal 2: Study the adaptation of the measurement and RRM measurement report based on UE’s actual states. Proposal 3: Study the methods on how to get the UE’s moving state, including gNB sensing/UE measurement and reporting. Table 2 power saving gain of RRM report adaptation Frequent RRM report; Sparse RRM report; Power saving gain. FTP (UL power =23dbm); 32.95; 32.39; 1.7%. FTP(UL power =0dbm); 32.49; 32.29; 0.62%. Instant message (UL power =23dbm); 7.01; 6.82; 2.71%. Instant message (UL power =0dbm); 6.78; 6.71; 1.03%.

Summary of the proposals

- Incorporated updated results from Huawei, Nokia for table 1 in section 2.1.1 (time-domain relaxation), update the description of the scheme part of Qualcomm, adding note 1,2,3 at the bottom of Table 1 which is to address the performance impact.

- Added Ericsson results for additional resource in section 2.3.1.

- Other updates in table 2 and table 3 in section 2.2.1 and 2.3.1.

Possible offline proposals

- Update Table 1 in R1-190XXXX to TR38.840 section 5.3.1

  - For RSRP accuracy, capture Figure 1 in R1-1903430, Figure 2 in R1-1901711 and Figure 1 in R1-1902026 I TR38.840 section 5.3.1

  - Note: the handover failure rate is already embedded in the table.

- Update Table 2 in R1-190XXXX to TR38.840 section 5.3.2

- Update Table 3 in R1-190XXXX to TR38.840 section 5.3.3

For adapting/Relaxing RRM measurement in time domain

Relaxing measurement period

|  | (1) IDLE/INACTIVE | (2)CONNECTED |  | (3)Intra-/inter-frequency measurement included? | (4)Periodic activities includeded? | Note |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 2x | 4x |  |  |  |
| Source 1 Huawei R1-1903430 | 11%-39.7% |  |  | Y | y |  |
| Source 2 Vivo R1-1901711 | 29% | 11.5% | 22.4% | N | n | With FTP model 3 |
| Source 3 MediaTek R1-1903354 |  | 14.38% - 21.34% |  | Y | y |  |
| Source 4 CATT R1-1902026 |  | 11.91% | 17.83% | N | n |  |
| Source 6 DoCoMo R1-1903236 |  | 14.1% | 21.2% | N | n |  |
| Source 7 Qualcomm R1-1903017 | 0.71% - 5.36% |  |  | N | y |  |
| Source 8 ZTE R1-1902030 R1-1902032 |  | 7.4% - 17.8% | 11.1% - 26.6% | Y | n |  |

Relaxing number of samples within measurement period

|  | (1)IDLE/INACTIVE | (2)CONNECTED | (3)Intra-/inter-frequency measurement included? | (4)Periodic activities includeded? | Note |
| --- | --- | --- | --- | --- | --- |
| Source 4 CATT R1-1902026 |  | 2.57% -10.71% | n | n |  |
| Source 5 Sony R1-1902187 |  | 24% - 36% | n | n |  |
| Source 2 Vivo R1-1901711 | 26.8% - 31.9% |  | n | n |  |

Based on the above results, it can be observed that,

The followings are proposed to be captured in TR38.840 section 5.3.1

For adapting/relaxing RRM measurement in time domain, the followings are observed from simulation results,

For RRC CONNECTED state,

- Among the results, [11.1% - 26.6%] power saving gain is shown for extending measurement period  4 times, and [7.4% - 17.8%] power saving gain is shown for extending measurement period  2 times.

For RRC IDLE/INACTIVE state,

By increasing measurement period, the UE power saving gain at least depends on the number of I-DRX/paging cycles to be skipped for measurement and the number of SSB burst sets to be used for measurement and periodic activities. When measurement is performed once every 4 paging cycles, the gain is [17.9%-19.7%]  (one SSB burst set for measurement and periodic activities), [0.89%-5.36%] (two or three SSB burst sets for measurement and periodic activities).

While at the same time, for stationary or low mobility case, the mobility performance impact of  increasing measuremenrt period  (e.g., handover failure rate, RSRP difference between two adjacent samples) is shown as follow,

- The handover failure rate for RRC CONNECTED state changes from [0% - 0.26%] for 3km/h and [0% - 2.43%] for 30km/h with 4 times relaxing the measurement period.

- the  (5%-tile, 95%-tile) of the RSRP measurement error is within the range of ([-4dB, 4dB]) .

It is shown that relaxing the number of samples within measurement period provides [2.57% - 36%] power saging gain for RRC CONNECTED. For stationary or low mobility case, 1 source results show the  (5%-tile, 95%-tile) of the RSRP measurement error is within the range of ([-2dB], [3dB]) for both 3km/h and 30km/h .

For adapting/Relaxing intra-frequency measurements

The followings are proposed to be captured in TR38.840 section 5.3.2

For adapting/relaxing RRM measurement for intra-frequency measurement by reducing the number of measured cells, the followings are observed from simulation results,

By assuming number of neighbouring cells to be measured is reduced, it is shown that about [4.7% - 7.1%] power saving gain can be observed if reducing the number of measured cells for IDLE state. 1 source shows that about [1.8% - 21.3%] power saving gain can be observed if reducing the number of measured cells for CONNECTED state.

In additional to that, by also assuming that UE can limit the processing for measurement within a constrained time period and/or with reduced complexity,  [26.43% - 37.5%] power saving gain is shown.

For additional resources for RRM measurement

The followings are proposed to be captured in TR38.840 section 5.3.3

For additional resources for RRM measurement , the followings are observed from simulation results,

[4 sources] shown that additional resources provide [21%~81%] power saving gain by skipping measurement/paging.  [1 source] shows [3.8%-11.8%] power saving gain by assuming additional RS to reduce the offset between DRX ON and SSB measurement to increase the opportunities for UE deep sleep instead of light sleep.

For these schemes,

1 source evaluated CSI-RS based RRM for RRC IDLE state by reducing the active time for measurement.

1 source evaluated allowing UE performs multiple RRM measurements (e.g. 5 measurements in a consecutive period) when UE is triggered to wake up at a given DRX-ON period.

1 source evaluated multiplexed SSS-only signal as additional RS before each SSB (measurement occasion) for paging indication which can assist sync, RRM, and AGC tuning for RRC IDLE/INACTIVE state.

1 source provides results based on PSS, SSS and wake-up singalling/paging transmitted in single frequency network as additional resources for measurement and shows benefits reducing UE power saving due to reducing the number of UE wake-up occasions for measurement, loop convergence and paging monitoring, reducing the frequency of performing neighbor cell search, and reducing the neighboring cell measurement , and reducing the paging monitoring.

1 source evaluate additional RS to reduce the offset between DRX ON and SSB measurement.

System overhead  may or may not be increased a lot by additional resource depending on the specific additional resourcedesign. Reusing existing transmitted RS (e.g, TRS, CSI-RS) for IDLE UEs does not increase system overhead, increasing the period of the additional RS can reduce the system overhead.

For Adapting/Relaxing inter-frequency measurements

Possible Proposal in RAN1#96:

- Add new section in TR38.840 “5.3.4 Adapting/relaxing RRM measurement for inter-frequency measurement”

- Capture Table 4 in R1-19xxxxx to TR38.840 section 5.3.4.

The followings are proposed to be captured in TR38.840 section 5.3.4

For adapting/relaxing RRM measurement for inter-frequency measurement,it is shown that reducing the number of measured inter-frequency layers can provide [21~38%] power saving gainfor all RRC states.

For Adapting/relaxing the number of measured beams by considering spatial information aspects

Possible Proposal in RAN1#96:

- Add new section in TR38.840 “5.3.5 Adapting/relaxing the number of measured beams by considering spatial information aspects”

- Capture Table 5 in R1-19xxxxx to TR38.840 section 5.3.5

The followings are proposed to be captured in TR38.840 section 5.3.5

For adapting/relaxing the number of measured beams, the followings are observed from simulation results,

For IDLE state, 1 source shows [36%~64%] power saving gain for reducing RRM measurement for SSBs by considering adjacent directional beams of each SSB can be provided by gNB to reduce SSB number of UE RRM measurement and loop convergence. It also shows that the it has no severe impact on RSRP difference between two adjacent samples.

For Procedurs/triggerings

Agreements(last meeting)

For UE autonomous RRM measurement adaptation in time-domain with gNB controlled threshold, the following thresholds and corresponding adaptation schemes can be considered,

- A RSRP threshold for UE adapting RRM measurement period,

- A RSRP threshold for UE adapting RRM number of samples within a measurement period,

- A RSRP threshold and a RSRP variation threshold within a period of time, and based on that, UE can adapt the measurement or report period

- A RSRP variation threshold within a period of time, and based on that, UE can adapt the measurement or report period

- A threshold for UE adapting RRM measurement period and the threshold can be at least one of the followings,

  - The amount of time during which the UE stays with a specific cell or beam (for RRM measurement)

  - UE’s active TCI state for PDCCH does not change for specific time period.

  - The number of handovers/reselections for certain period.

- A threshold which includes UE mobility status, serving cell quality, and based on that, UE can adapt the SMTC window, number of CSI-RS resources sets per target cell, periodicity of CSI-RS resources.

Possible Proposal in RAN1#96

The followings are proposed to be captured in TR38.840 section 5.3.1

For gNB controlled RRM measurement operation with/without UE assistant information, the gNB can indicate to the UE the following information in order to adapt/relax RRM measurement in time-domain,

- gNB indicates a relaxed measurement periods/samples in time domain which enables UE to perform RRM measurement less frequently,

The followings are proposed to be captured in TR38.840 section 5.3.2

For UE autonomous RRM measurement for reducing the intra-frequency measurement with gNB controlled threshold, the following thresholds and corresponding adaptation schemes can be considered,

- Using RSRP/RSRQ threshold(s) for UE adapting  the number of measured non-serving cells,

For gNB controlled RRM measurement operation with/without UE assistant information, the following information in order to adapt/relax RRM measurement for intra-frequency provide by gNb can be studied,

- Spatial information from SSB and CSI-RS,

- Temporarily omit search for additional unknown cells if the NW can establish that all relevant candidate cells have been detected.

- Historical data and/or gNB assistance information (e.g. time offsets –absolute/indexed values)

The followings are proposed to be captured in TR38.840 section 5.3.4

In order to reduce the inter-frequency RRMmeasurement, the followings can be considered,

- Reducing the measurement by grouping different frequencies within a band, UE measures only one frequency within the group.

- Providing time-frequency alignment information for improving inter-frequency measurement

- Additional information (e.g., directional information of reference signals) can be considered to reduce the number of neighbor cells and neighbor frequencies for RRM measurements without degrading the mobility performance.

- CSI-RS transmission benefits from a measurement window configuration to avoid UE performing measurement outside the window.

Proposals from companies’ contributions

R1-1903430 Power consumption reduction in RRM measurements Huawei, HiSilicon

Proposal 1: Capture the simulation results of Table 1, Fig. 1 and Fig. 2 into TR 38.840. Proposal 2: The threshold based mechanism for RRM relaxation in time domain can reduce power consumption while having little impact on the measurement accuracy. Observation 1: Most UEs in a cell only perform serving cell measurement and do not perform neighbor cell measurement in the evaluated cases. Proposal 3: Information on adjacent directional beams of each SSB is provided by gNB to UE in order to reduce the number of SSBs for UE RRM measurements. Proposal 4: Both reduction in number of neighbor cells and neighbor frequencies for RRM measurements are considered for power saving. Observation 2: There exists the possibility to reduce the measurements without degrading the mobility performance. Proposal 5: Additional information (e.g., directional information of reference signals) can be considered to reduce the number of neighbor cells and neighbor frequencies for RRM measurements without degrading the mobility performance. Proposal 6：Introducing of CSI-RS measurement window should be discussed in Rel-16.

R1-1901711 UE power saving in RRM Measurements vivo

Observation 1: In idle/inactive state, the power consumption caused by RRM measurements takes a great proportion of the overall UE power consumption. Observation 2: Reducing measurement activities should be targeted for UE power saving in RRM. Observation 3: For UE in idle state, about 30% power saving gain can be achieved if the number of measurement samples is reduced to 1 per paging cycle, or the measurement periodicity is extended to multiple paging cycle. Observation 4: For UE in idle state, more than 30% power saving gain can be achieved if CSI-RS can be introduced as an additional measurement RS, based on which the measurement samples can be reduced and the measurement accuracy can still be guaranteed. Observation 5: Use of CSI-RS for idle state RRM measurement does not cause issues from network overhead or UE complexity perspective. Observation 6: Reducing number of measured intra-frequency cells provide power saving benefit, especially when SSBs from neighbor cells are located at different slots within the SMTC. Proposal 1: CSI-RS is introduced as measurement RS for UE in idle/inactive state. Proposal 2: UE is allowed to take one sample for RRM measurement per paging DRX cycle. Proposal 3: The L3 RRM measurement periodicity can be relaxed to multiple paging DRX cycles. Proposal 4: Extending RRM measurement periodicity is beneficial for power saving when C-DRX is configured. Proposal 5: Reducing the number of measured beams is beneficial for power saving and should be supported. - The concept of S criterion in cell re-selection can be used to reduce the number of measured beams of serving cell Proposal 6: Reducing number of measured intra-frequency cells should be supported for NR UE power saving. - Additional threshold can be considered to reduce the number of intra-frequency measurements based on S-measures enhancement. Proposal 7: Capture the power saving gain of each power saving scheme for RRM measurements in Table 4-6 into TR.

R1-1901805 NR RRM UE power saving MediaTek Inc.

Observation 1: The non-alignment of SMTC window and DRX on-duration can bring 46% power overhead to NR RRM power consumption. Observation 2: The power saving gain of RRM measurement adaptation in time-domain is more evident when the SMTC window and DRX ON duration is not aligned. Increasing the measurement period can bring up to 28.8% power saving gain. Observation 3: For A3 RSRP measurement event, the handover is not triggered when PCell RSRP is better than all neighbor cells. Proposal 1: The measurement period can be extended when PCell RSRP is offset better than the neighbor cell with best RSRP and return to default value when the condition is no longer satisfied. Proposal 2: An additional serving cell RSRP variation threshold can be used on top of the threshold between serving cell and neighbor cell to guarantee UE does not extend measurement period when the serving cell RSRP variation is large. Observation 4: The proposed method of RRM measurement adaptation in time-domain can save 14% to 21% UE power (37%~55% RRM measurement power). Observation 5: With the proposed additional serving cell RSRP variation threshold, the power saving percentage is reduced by 3% while the mobility performance penalty is about halved. Observation 6: Putting a wake-up (power-saving) signal before the DRX ON for a fixed time gap cannot prevent from another wake up during the SMTC window to perform periodic activities and RRM. Proposal 3: SSS-only signal is used as additional RS before the SSB as wake-up (power-saving) signal for paging indication and RRM assistance in RRC_IDLE/ RRC_INACTIVE. Proposal 4: Multiplexed SSS-only wake-up (power-saving) signals can be used before the SSB for paging indication. When no UE is to be paged, normal SSS is transmitted for enhancement of sync, RRM, and AGC tuning. Observation 7: Using multiplexed SSS-only signal as additional RS before each SSB (measurement occasion) for paging indication can achieve 21.78% power saving in idle/inactive mode and assist sync, RRM, and AGC tuning. Proposal 5: In 38.840 [2], 5.2.3, the following sub-bullet can be added - Multiplexed SSS-only signal as additional RS before SSB (measurement occasion) for paging indication can be considered for power-saving and RRM/sync assistance.

R1-1902026 UE Power saving scheme for RRM measurements CATT

Proposal 1: For some concern conditions, the number of RSRP measurement samples for a measurement period can be relaxed with negligible impact on accuracy. Proposal 2: For some concern conditions, increasing measurement period can be efficient for UE power saving. Proposal 3: For some concern conditions, reducing the number of cells for intra-frequency measurement can cause significant for UE power saving. Proposal 4: On-demand RS based RRM measurement can be beneficial for UE power saving, in which UE would finish measurement during DRX-ON period and go to sleep after all. Proposal 5: The on-demand RS assistant RRM measurement during data transmission can obtain obvious power saving gain.

R1-1902032 On UE Power Saving for RRM Measurement ZTE Corporation

Proposal 1: The UE power saving techniques in RRM measurement should not affect the network performances and additional power consumed by gNB should keep low. Proposal 2: The RRM measurement requirements could be optimized for UE power saving. Proposal 3: Different configurations can be applied for different states for RRM measurement and UE movement speed.

R1-1902054 Discussion on UE power consumption reduction in RRM measurements LG Electronics

Proposal 1: Investigate further an enhancement to the existing threshold based measurement relaxation (e.g., enhancement of s-Measure mechanism) to reduce UE power consumption, e.g. - Multi-level s-Measure mechanism, e.g., 1st level for skipping intra-frequency non-serving cell measurement, 2nd level for skipping all non-serving cell measurements Proposal 2: Consider configuring multiple sets of measurement behaviours/configurations to a UE where each set corresponds to different level of RRM requirements and measurement periodicities. Proposal 3: It is necessary to investigate the negative impact of RRM measurement relaxation (e.g. reduction of measured frequency layers and/or measured cell and/or SSB slots) together with UE power consumption reduction. Proposal 4: Fall-back operation (to normal measurement) should be considered to mitigate negative impact of measurement relaxation, e.g., - Different RSRP criteria for fall back to normal measurement from that for measurement relaxation - Timer based fall back to normal measurement

R1-1902187 Power consumption reduction in RRM measurements Sony

The following observations and proposals are made: Observation 1: Reducing the frequency of measurement sampling can reduce power consumption by between 14 and 36%. Observation 2: Our power consumption analysis methodology in consistent with other companies in RAN1. Observation 3: Reducing the number of inter-frequency layers from 2 to 1 reduces the power consumption by 21%. Reducing from 4 layers to 2 reduces the power consumption by 26%. Observation 4: A UE switching from RRC CONNECTED state to RRC IDLE/INACTIVE state will have knowledge of a suitable receive beam configuration. Proposal 1: In certain conditions the UE can relax RRM measurements by reusing previously known beam configurations, and avoiding unnecessary measurements in other beam directions. Proposal 2: Previously known beam configurations can be used for up to a set time duration, subject to measurement results not degrading below an RSRP threshold. 错误!未找到引用源。 provides a summary of results from this Tdoc formatted for direct inclusion in TR38.840.

R1-1902319 UE power consumption reduction in RRM measurement Samsung

Proposal 1: Autonomous reduction in number of measurement samples dynamically for RRM measurement based on SNR and/or mobility state of the UE is supported. Proposal 2: It is supported that gNB facilitates the operation with dynamically adjusting the density of measurement resources based on UE inputs. Proposal 3: Support RRM measurement triggered by CSI-RS based power saving signal outside C-DRX on duration. Proposal 4: Dynamic selection of beams and/or cells by UE is allowed with the help of historical data and/or gNB assistance information (e.g. time offsets –absolute/indexed values) through L1 signaling/MAC CE signaling. Proposal 5: Support cell-group based measurement based on gNB assistance information.

R1-1902341 Discussion on power saving in intra-band RRM measurements CMCC

Proposal 1. For inter-frequency measurement, the number of intra-band measured frequencies could be reduced to realize UE power saving. Proposal 2. The measured frequencies or frequency groups and the association relationships between measured frequencies or measured frequency groups can be indicated to UE in measurement configuration information. UE measures one frequency or frequency group among the associated frequencies or frequency groups to be measured, and the measurement results are applied to all the associated frequencies or frequency groups. Proposal 3. The measurement probability for each measured frequencies can be configured by gNB. UE generates a random number and selects the cell to camp on or handover according to the measurement probability.

R1-1902509 On RRM aspects of NR UE power saving Intel Corporation

Proposal 1: - To define a new "stationary-mobility state" for NR UEs (i.e. UEs with no mobility or very low mobility). - UE reports its mobility speed status or request approximate measurement configurations based on the evaluated mobility status. Proposal 2: - To support the activated or deactivated of a configured measurement objects (MO) based on UE’s mobility state. Proposal 3: - Study mechanism to enable efficient UE's transmission from RRC_CONNECTED into RRC_INACTIVE, and potentially even into RRC_IDLE.

R1-1902731 Consideration on NR RRM for UE power saving Spreadtrum Communications

Proposal 1: For procedure/triggering for reducing RRM measurement activity in time domain, the threshold should be carefully studied to avoid complicated calculation in UE autonomous behaviour. Proposal 2: For procedure/triggering for reducing the intra-frequency measurement, the threshold should be carefully studied to avoid complicated calculation in UE autonomous behaviour. Proposal 3: The additional resource can have functionalities of RRM measurement. Proposal 4: Additional resource for RRM measurement and additional RS as the aid of power saving signal can share the same resource. Proposal 5: Whether to support using channel in additional resource needs to consider UE complexity.

R1-1903351 UE power Consumption Reduction in RRM Measurements OPPO

Proposal 1: option 1(CSI-RS/TRS) and option 2(PBCH) as listed above can be considered as the additional RRM measurement reference signals. Proposal 2: Study the adaptation of the measurement and RRM measurement report based on UE’s actual states. Proposal 3: Study the methods on how to get the UE’s moving state, including gNB sensing/UE measurement and reporting. Proposal 4: study new metric to reflect the UE’s moving states. Proposal 5: The following can be studied for Autonomous adaptation of the RRM measurements and reporting. - The configured threshold may be the UE’s velocity or measured RSRP - Adaptation of RRM measurement and reporting periodicity, measured cell lists, measured SSB positions, measurement slots etc. based on the thresholds.

R1-1902752 Additional resource for RRM measurement in power saving aspect ASUSTEK COMPUTER (SHANGHAI)

Observation 1: To achieve power saving, addition resource is better to be in proximity to other event UE needs to wake up such as PDCCH monitoring. Observation 2: There is tradeoff between time domain resource and frequency domain of additional resource in view of power saving. Proposal 1: RAN1 should further investigate ways to adapt time domain and frequency domain of additional resource. Proposal 2: CSI-RS is used to perform RRM measurement in additional resource. Proposal 3: Adapting CSI-RS to fit the needs of power saving could be considered, e.g. transmission bandwidth adaptation of CSI-RS.

R1-1903236 Discussion on UE power Consumption Reduction in RRM Measurements NTT DOCOMO, INC.

Observation 1: Relaxing non-essential RRM measurement in appropriate conditions would not affect to the mobility management and system performance. Proposal 1: Appropriate condition for relaxing non-essential RRM measurement should be focused on so that a degradation of the mobility and system performances can be avoided. Proposal 2: Relaxing RRM measurement should be fully controllable by NW. Proposal 3: Relaxing the number of RRM measurement attempts in time domain based on the conditions regarding measurement result should be studied for UE power consumption reduction in RRM. - NW can fully control whether/how the above conditions can be applied for relaxing the number of RRM measurement attempts in time domain. Proposal 4: Relaxing the number of measured cells/frequencies for neighbour cells based on the conditions regarding measurement result should be studied for UE power consumption reduction in RRM. - NW can fully control whether/how the above conditions can be applied for relaxing the number of measured cells/frequencies for neighbour cells. Proposal 5: The measurement quantity which triggers RRM measurement relaxation should be same as that for triggering the measurement reporting. - Section 5.3.1 in TR 38.840, i.e., “Adapting/Relaxing RRM measurement in time domain,” should include RSRQ and SINR as measurement quantity of the threshold for RRM measurement relaxation/adaptation. Observation 2: Based on the Table 1 and 2, power saving gain by increasing measurement period is at least 16.9% on the condition that the impact on mobility performance would be marginal, in case that SMTC window and DRX on duration are not aligned and the RSRP threshold for relaxing RRM measurement is 6dB less than that for triggering the measurement reporting.

R1-1902936 RRM aspects of NR UE power saving Ericsson

Observation 1 Reduction of L1 measurement sample collection does not necessarily reduce the frequency of UE wake-up in DRX/CDRX. Observation 2 Additional cell-specific RSs introduce additional static Resource Utilization overhead. Observation 3 Static additional RS transmission has a strong adverse impact on NW Energy Efficiency. Observation 4 Additional UE-specific SSB transmission before POs has a NW RU impact that scales with the number of UEs in idle/inactive mode and is expected to have a CN impact. Observation 5 Activation/deactivation of additional RS for idle/inactive UEs incurs a SI update signaling overhead. Observation 6 Achieving SFN gains to replace beamforming gains is not feasible without a major NW redesign and replacing gNB HW. Proposal 1 Changes to UE measurement behavior must not lead to degrading cell detection and mobility performance compared to RAN4 requirements defined in Rel-15. Proposal 2 Changes to UE measurement behavior for UE PS purposes that are not fully transparent to the NW must be under NW control. Proposal 3 Decision whether to introduce a new stationary UE class should be handled by RAN2. Proposal 4 Additional RS before measurement or paging occasion are not supported. Proposal 5 Prioritize use of UE mobility state info and serving cell RSRP as criteria for configuration switching. Proposal 6 Parameter estimation quality for UE assistance info for NW-controlled adaptation or UE-autonomous adaptation of RRM measurement configurations must be covered by performance requirements. Proposal 7 NW may provide assistance info to UEs regarding maximal inter-cell T/F misalignment the UE should assume in the current deployment. Proposal 8 Limiting the set of cells to measure should be NW-controlled and should prioritize scenarios where this results in reducing the SMTC window. Proposal 9 NW may indicate to the UE to temporarily omit search for additional unknown cells if UE has detected all candidate cells that are relevant at its location. Proposal 10 SFN transmission of paging is not supported.

R1-1903017 UE Power Consumption Reduction in RRM Measurements Qualcomm Incorporated

Observation 1: Rel-15 RRM might require some scheduling restrictions due to the use of different Rx beamforming and/or different sub-carrier spacings of RRM reference signals and/or measurement gap for intra/inter frequency measurements. Observation 2: Although RRM measurement might be skipped in one or more I-DRX cycles in RRC-IDLE/INACTIVE, the UE still has to perform AGC/time-frequency tracking loops and monitor paging every I-DRX. Observation 3: The specifications do not specify how many RSRP measurement samples UE shall perform within one DRX cycle (in RRC-IDLE/RRC-INACTIVE) or within one measurement period (in RRC-CONNECTED) i.e., it is up to UE implementation as long as RAN4 test cases are satisfied. Proposal 1: The power for AGC loop and time-frequency tracking loop is the same as the power for serving cell SSB processing. Proposal 2: Capture the following procedure in TR when using PSS, SSS and wake-up signaling as additional resource for RRM for FR1 in RRC-IDLE/RRC-INACTIVE: - Cells within an area (e.g., RNA or RAN area) periodically broadcast PSS, SSS and wake-up signaling in an SFN manner over a single beam - PSS and SSS are not transmitted on a sync frequency raster - UE performs SSS RSRP/RSRQ measurement of a camping area and neighbor areas if triggered - UE tracks wake-up signaling to detect whether UE is paged or not - If UE is paged, UE shall determine a suitable cell to acquire paging. Otherwise, UE goes to sleep. Proposal 3: Capture the following procedure in TR when using PSS, SSS and paging as additional resource for RRM for FR1 in RRC-IDLE/RRC-INACTIVE: - Cells within an area (e.g., RNA or RAN area) periodically broadcast PSS, SSS and paging in an SFN manner over a single beam - PSS and SSS are not transmitted on a sync frequency raster - UE performs SSS RSRP/RSRQ measurement of a camping area and neighbor areas if triggered - UE decodes paging for the page. - If UE is paged, UE shall determine a suitable cell to transit into RRC-CONNECTED. Otherwise, UE goes to sleep. Proposal 4: Capture in TR the UE power savings gain in 错误!未找到引用源。 when using PSS, SSS and wake-up signaling/paging as additional resource for FR1 RRM in RRC-IDLE/RRC-INACTIVE. Proposal 5: RAN1 considers adaptive RRM measurement configuration in RRC-CONNECTED i.e., - Multiple RRM measurement configurations (e.g., measurement duration, periodicity and measurement gap) can be configured via RRC - A suitable measurement configuration can be dynamically indicated to UE via DCI or MAC-CE. Proposal 6: RAN1 considers repetition of CSI-RS resources for mobility within a resource set. Proposal 7: Capture in TR the UE power savings gain in 错误!未找到引用源。 when using repetition of CSI-RS resources for Rx beam refinement for FR2 RRM in RRC-CONNECTED. Proposal 8: At least for FR2, RAN1 studies multi-panel RRM measurement options for UE power savings so that the link to the serving cell may not be interrupted during the RRM measurement. Proposal 9: Capture in TR the UE power savings gain in 错误!未找到引用源。 for RRM measurement adaptation for FR1 RRM in RRC-IDLE/RRC-INACTIVE.

R1-1903136 UE Power Consumption Reduction in RRM Measurements Nokia, Nokia Shanghai Bell

Based on the simulation results presented in the document we observe the following: Observation 1: For low mobility/low speed UE, increasing the measurement period has no severe impact on mobility performance. Observation 2: On high/moderate UE speeds, using too long measurement period has non-negligible on mobility performance. On introducing RSRP threshold for adapting RRM measurements we observed that: Observation 3: Using cell quality (RSRP) as metric to determine when UE can adapt RRM measurement period does not take into account the UE mobility Observation 4: S-measure can be configured for UE to reduce RRM measurements when the current serving cell is in good condition but single threshold value does not take into account UE specific conditions Observation 5: For RRM measurements, in RAN4 specifications, there is no requirement set for number of measurements samples or sampling rate within a measurement period. Observation 6: For reported measurements, UE has to fulfil the measurement accuracy requirement (as specified in 38.133) and the number of samples to reach the accuracy is left for the UE implementation. And propose following: Proposal 1: Consider enhancements to RSRP based thresholds based mechanism such as S-measure for adapting RRM measurements that consider additional conditions such as UE location in a cell and UE mobility state. Proposal 2: Before considering any further changing the UE measurement samples, it should be clarified with RAN4 if they have intent to introduce such requirements that would allow this kind of behaviour to be verified. On the measurement adaptation based on UE mobility: Observation 7: In addition to determine when UE is in low/stationary mobility or in more general view when UE is in a state where it can apply adaptation to measurements, it should be defined what are the conditions where the adaptation cannot anymore be applied. Proposal 3: Define the conditions when UE cannot anymore apply the adaptation to RRM measurements. And finally we discuss the use of additional signal for power saving for RRM measurements: Observation 8:Additional signals would enable additional or alternative measurement occasions/opportunities for serving cell but would not be feasible for L3 mobility measurements due to configuration issues. Observation 9:The use gains of using additional The power saving gain from additional signals for RRM measurements from RRM measurement perspective are unclear. Proposal 4: Do not introduce additional signal for UE power saving in RRM measurements.

Summary of the previous agreements

RAN WG1#94bis, Chengdu

Agreements: - RRM power saving study should consider RRC IDLE, INACTIVE and CONNECTED states. Agreements: - Further study on relaxing/adapting RRM measurement for UE power saving, - Relaxing/adapting the number of RRM measurements in time domain for serving cell and neighbor cells based on certain conditions, e.g., number of SSB/CSI-RS/measurement occasions. - Relaxing/adapting the number of measured cells/frequencies for neighbor cells based on different conditions - Schemes considered should disclose the mechanism used to determine the mobility/speed assumed - FFS conditions, e.g., channel condition, UE location, UE speed, measurement configuration, measurement results, cell type, etc. - FFS impact on measurement/mobility performance - Relaxed/adapting measurement report (if any) - Other solutions are not precluded. Agreements: - Further study on the benefit/impact for additional resource for RRM measurements for power saving for mobility performance not inferior to that of Rel-15 - The schemes proposed should at least include the following aspects: - The signals/channels used as additional resource - Procedure on how to use additional resources. - Overhead from network perspective should be considered. Agreements: - Focus on stationary (e.g., 0km/h), pedestrian (e.g., 3km/h) and vehicular (e.g., 30km/h) scenarios for further studies when considering UE power saving techniques for RRM measurements, - Further studies UE power saving for RRM measurement by considering the following issues - Multi-beam/multi-cell related RRM measurement issues. - Possible misalignment between SSB(s)/SMTC and DRX ON duration

RAN WG1#95, Spokane

Agreements: - Observation: for certain conditions (e.g., low mobility deployment/UE speed/favorable RSRP conditions), the number of RSRP measurement samples for a given duration (e.g., measurement period / evaluation period) can be relaxed with negligible impact on accuracy achieved by existing Rel-15 measurement. Agreements: - Observation: - For certain conditions (e.g., favorable RSRP conditions, etc.), reducing RRM measurement activities (e.g., measurement, reporting) for a given time period is beneficial from UE power saving perspective for RRC IDLE/INACTIVE/CONNECTED states. Agreements: - For intra frequency and/or inter frequency measurement, the following approaches are to be studied for UE power saving in time domain, including impact on mobility performance - Increasing measurement period - Reducing number of samples (e.g., OFDM symbols / slots) within a measurement period (e.g., SMTC window) - Confining RRM measurements within a measurement window and increasing the periodicity of the measurement window for intra frequency and/or inter frequency measurement - Other approaches are not precluded - Note: this does not necessarily mean that the techniques studied will have spec impact Agreements: To further study the following adaption mechanism of RRM measurement activities for UE power saving: - gNB controlled RRM measurement operation with UE assistance information reported to gNB, e.g., - mobility related information (e.g., mobility state, history of mobility state, UE's visited cells and cells not reselected due to the ping-pong effect, the number of handovers for certain period, etc.) - channel condition (e.g. change in serving RS/signal) - gNB controlled RRM measurement operation without UE assistance information reported to gNB based on certain conditions, e.g., - subscription, capabilities (e.g. at AS or NAS level), device type - Doppler estimation for RRC CONNECTED states - cell type (e.g., small cell/macro cell) - gNB controlled threshold to support UE autonomous RRM measurement adaptation based on e.g., - signal measurements (e.g., RSRP) - UE mobility state (e.g., low/medium/high mobility) - UE location in the cell (e.g., cell-center/cell-edge) - S-measure enhancement (e.g. S-measure for SCell, CSI-RS) - Other mechanisms/approaches are not precluded Agreements: Observation: - Reducing the number of cells for intra-frequency measurement can be beneficial for UE power saving due to at least the followings, - Assuming that UE can limit the processing for measurement within a constrained time period and/or with reduced complexity. - Assuming number of neighbouring cells to be measured is reduced. - For UE power saving perspective, reducing the need in neighbour cell intra-frequency measurement can be beneficial. Agreements: Observation: Under certain conditions and certain deployment scenarios, additional resource for RRM measurement can be beneficial for UE power saving, including at least the following aspects: - Minimizing/reducing the timing gap between measurement (e.g., SSB) and DRX ON duration (e.g., paging monitoring occasion/reception, data reception, etc.) - Additional resource around the measurement occasion, e.g., for AGC assistance - Reducing measurement activities by providing additional resource may provide sufficient measurement/T-F accuracy.

RAN WG1 Adhoc 1901, Taipei

Agreements: - For IDLE/INACTIVE state, at least the following power components are recommended to be considered for RRM measurement power saving evaluation - Loop convergence (AGC, TTL & FTL) / time-frequency tracking - How many SSB bursts are used for Loop convergence, with consideration of being potentially confined in the same SSB burst or different SSB bursts for serving cells and neighboring cells measurement/ time-frequency tracing - FFS: The power value for loop convergence /time-frequency tracking is as the same as SSB processing. - Paging - SIB1 decoding (PDCCH+PDSCH) - Neighbor cell search (within SMTC), if any - SSB measurement (serving cell only / severing cell and neighboring cells, if any) - Sleep - For CONNECTED state, at least the following power components are recommended to be considered for RRM measurement power saving evaluation, - Loop convergence (AGC, TTL & FTL) / time-frequency tracking - How many SSB bursts are used for Loop convergence, with consideration of being potentially confined in the same SSB burst or different SSB bursts for serving cells and neighboring cells measurement/ time-frequency tracing - Based on SSB and/or TRS(if configured) - PDCCH-only monitoring during active time - SSB measurement (serving cell only / severing cell and neighboring cells, if any) - Neighbor cell search (within SMTC), if any - Sleep Agreements: - Update the table format as follow for capturing the results in TR38.840 section 5.3 “Power consumption reduction in RRM measurements” for - Adapting/Relaxing RRM measurement in time domain - Adapting/Relaxing intra-frequency measurements - Additional resources for RRM measurement - Other categories of techniques is not precluded and can be further incorporated if available. - Note: for RRC CONNECTED, the ‘measurement period’ in the table and agreements refers to the measurement period in TS38.133 - Companies are encouraged to report the following additional assumption if applicable for each scheme, - DRX configuration - Offset between paging occasion and SSB/CSI-RS - For evaluation of adapting/relaxing RRM measurement in time domain, the following is recommended to be provided - Impact to measurement accuracy, e.g., RSRP accuracy - Impact to mobility performance, e.g., handover failure rate - For evaluation of reducing intra-frequency measurement - Number of cells for measurement - For evaluation of additional resource - Resource type, e.g., CSI-RS, SSB or others - Others are not precluded - Note: companies are encouraged to report assumptions for synchronous or asynchronous deployments/case (0); (1); (2); Evaluation assumptions; (6); (7); Note. (1); (2); (3); (4); (5). (0) Power saving schemes description (1) Average power consumption per slot (2) Power reduction compared to baseline [%]; (1) RRC Idle / connected? RRC Idle/RRC Inactive or RRC Connected (2)Measurement period [ms] (3)number of samples(e.g., SSB bursts) in each MP (4) UE speed and channel model (5) system impact, e.g., overhead (6) Performance impact, e.g., RSRP accuracy, channel power variation (7)Additional assumption, e.g., PO and SSB offset,. Source xxx. Agreements: For UE autonomous RRM measurement adaptation in time-domain with gNB controlled threshold, the following thresholds and corresponding adaptation schemes can be considered, - A RSRP threshold for UE adapting RRM measurement period, - A RSRP threshold for UE adapting RRM number of samples within a measurement period, - A RSRP threshold and a RSRP variation threshold within a period of time, and based on that, UE can adapt the measurement or report period - A RSRP variation threshold within a period of time, and based on that, UE can adapt the measurement or report period - A threshold for UE adapting RRM measurement period and the threshold can be at least one of the followings, - The amount of time during which the UE stays with a specific cell or beam (for RRM measurement) - UE’s active TCI state for PDCCH does not change for specific time period. - The number of handovers/reselections for certain period. - A threshold which includes UE mobility status, serving cell quality, and based on that, UE can adapt the SMTC window, number of CSI-RS resources sets per target cell, periodicity of CSI-RS resources. Agreements: - The followings can be considered as the usage of additional resource for RRM measurement, - CSI-RS, including TRS - SSS only - SSB - PSS, SSS and wake-up signaling/paging - E.g., including transmitted in SFN (single frequency network) manner - DMRS for RMSI PDCCH/PDSCH for standalone - Additional new RS/signal (e.g, configuring additional RS next to SSB) in addition to existing RS/signal in Rel-15 - Note: Existing structure/waveform of Rel-15 signals/channels is recommended to be studied for additional resource - Note: companies report the purpose of additional resource for RRM measurement, e.g., measurement, cell search and etc. - Companies report in which of the following RRC state(s) the additional resource can be used, - RRC IDLE - RRC INACTIVE - RRC CONNECTED

Study Item Description

FS_NR-PowerSaving; SID in RP-181463. The objectives are as follows

The objective is to study UE power saving framework taking into consideration of latency and performance in NR as well as network impact. The objective of the UE power saving study includes the following, - Identify techniques for UE power saving study with focus in RRC_CONNECTED mode [RAN1, RAN2] - Study UE adaptation to the traffic and UE power consumption characteristics in frequency, time, antenna domains, DRX configuration, and UE processing timeline for UE power saving (Note: existing UE capabilities are assumed for UE processing timeline) - Network and/or UE assistance information - Include mechanism in reducing PDCCH monitoring, taking into account current DRX scheme - Study the power saving signal/channel/procedure for triggering adaptation of UE power consumption characteristics - Study the UE power consumption reduction in RRM measurements in synchronous and asynchronous network deployment [RAN1/2] - Study the enhancement of higher layer procedures for UE power saving [RAN2] - Study the enhancement of UE paging procedure based on the additional power saving signal/channel/procedure - Study the enhancement of UE power saving procedure in supporting efficient transition from RRC_CONNECTED to RRC_IDLE/RRC_INACTIVE mode

Reference

The following contributions are submitted in RAN1#96,

R1-1903430	Power consumption reduction in RRM measurements	Huawei, HiSilicon

R1-1901711	UE power saving in RRM Measurements	vivo

R1-1901805	NR RRM UE power saving	MediaTek Inc.

R1-1902026	UE Power saving scheme for RRM measurements		CATT

R1-1902030	Evaluation results of UE power consumption	ZTE Corporation

R1-1902032	On UE Power Saving for RRM Measurement	ZTE Corporation

R1-1902054	Discussion on UE power consumption reduction in RRM measurements	LG Electronics

R1-1902187	Power consumption reduction in RRM measurements	Sony

R1-1902319	UE power consumption reduction in RRM measurement	Samsung

R1-1902341	Discussion on power saving in intra-band RRM measurements	CMCC

R1-1902509	On RRM aspects of NR UE power saving	Intel Corporation

R1-1902731	Consideration on NR RRM for UE power saving	Spreadtrum Communications

R1-1903351	UE power Consumption Reduction in RRM Measurements	OPPO (revised from R1-1902746)

R1-1902752	Additional resource for RRM measurement in power saving aspect	ASUSTEK COMPUTER (SHANGHAI)

R1-1903236	Discussion on UE power Consumption Reduction in RRM Measurements	NTT DOCOMO, INC. (revised from R1-1902816)

R1-1902936	RRM aspects of NR UE power saving	Ericsson

R1-1903017	UE Power Consumption Reduction in RRM Measurements	Qualcomm Incorporated

R1-1903136	UE Power Consumption Reduction in RRM Measurements	Nokia, Nokia Shanghai Bell

R1- 1903133	…	Nokia

History

List of the previous summaries

R1-1811914	Summary of UE power Consumption Reduction in RRM Measurements, Chengdu, RAN1#94bis

R1-1811962	Offline outcome of UE power Consumption Reduction in RRM Measurements, Chengdu, RAN1#94bis

R1-1814099	Summary of UE power Consumption Reduction in RRM Measurements, Spokane, RAN1#95 Wednesday

R1-1814028	Summary of UE power Consumption Reduction in RRM Measurements, Spokane, RAN1#95, Thusday

R1-1814286	Summary of UE power Consumption Reduction in RRM Measurements, Spokane, RAN1#95, Friday

R1-1901339	Summary of UE power Consumption Reduction in RRM Measurements, Taipei, RAN1 Adhoc#1901, Tuesday

R1-1901384	Summary#2 of UE power Consumption Reduction in RRM Measurements, Taipei, RAN1 Adhoc#1901, Wednesday

R1-1901409	Summary#3 of UE power Consumption Reduction in RRM Measurements, Taipei, RAN1 Adhoc#1901, Thusday

R1-1901454	Summary#4 of UE power Consumption Reduction in RRM Measurements, Taipei, RAN1 Adhoc#1901, Thusday

R1-1901466	Summary#5 of UE power Consumption Reduction in RRM Measurements, Taipei, RAN1 Adhoc#1901, Friday

R1-1903424	Summary of UE power Consumption Reduction in RRM Measurements, Athens, RAN1#96, Monday

R1-1903424	Summary#2 of UE power Consumption Reduction in RRM Measurements, Athens, RAN1#96, Tuesday