3GPP TSG RAN WG1 Meeting #96    	               R1-1903640

Athens, Greece, February 25th – March 1st, 2019

Source:	NTT DOCOMO, INC.

Title:	Corrections for PUSCH with configured grant

Agenda Item:	7.1.3

Document for: 	Discussion and Decision

- Corrections to configured grant

  - R1-1903616 (Correction on PUSCH with configured grant, Huawei)

Reason for change:

- The determination of DMRS related procedure for PUSCH transmission with configured grant is not complete.

Summary of change:

- Correct TS 38.214 Section 6.2.2 as following:

< Unchanged parts are omitted > ### 6.2.2 UE DM-RS transmission procedure When transmitted PUSCH is neither scheduled by PDCCH format 0_1 with CRC scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI, nor triggered by activation PDCCH format 0_0 or 0_1 with CRC scrambled by CS-RNTI, nor configured by configured grant Type 1, the UE shall use single symbol front-loaded DM-RS of configuration type 1 on DM-RS port 0 and the remaining REs not used for DM-RS in the symbols are not used for any PUSCH transmission except for PUSCH with allocation duration of 2 or less OFDM symbols with transform precoding disabled, additional DM-RS can be transmitted according to the scheduling type and the PUSCH duration as specified in Table 6.4.1.1.3-3 of [4, TS38.211] for frequency hopping disabled and as specified in Table 6.4.1.1.3-6 of [4, TS38.211] for frequency hopping enabled, and If frequency hopping is disabled: - The UE shall assume dmrs-AdditionalPosition equals to 'pos2' and up to two additional DM-RS can be transmitted according to PUSCH duration, or If frequency hopping is enabled: - The UE shall assume dmrs-AdditionalPosition equals to 'pos1' and up to one additional DM-RS can be transmitted according to PUSCH duration. When transmitted PUSCH is triggered by activation PDCCH format 0_0 with CRC scrambled by CS-RNTI, UE shall follow the same procedure for DM-RS transmission as described above, except for the dmrs-Type and dmrs-AdditionalPosition is determined by cg-DMRS-Configuration in configuredGrantConfig, respectively. For the UE-specific reference signals generation as defined in Subclause 6.4.1.1 of [4, TS 38.211], a UE can be configured by higher layers with one or two scrambling identity(s), nIDDMRS,i i = 0,1 which are the same for both PUSCH mapping Type A and Type B. When transmitted PUSCH is scheduled by PDCCH format 0_1 with CRC scrambled by C-RNTI, CS-RNTI or MCS-RNTI, or triggered by activation PDCCH format 0_1 with CRC scrambled by CS-RNTI, or configured by configured grant Type 1, - the UE may be configured with higher layer parameter dmrs-Type in DMRS-UplinkConfig, and the configured DM-RS configuration type is used for transmitting PUSCH in as defined in Subclause 6.4.1.1 of [4, TS 38.211]. - the UE may be configured with the maximum number of front-loaded DM-RS symbols for PUSCH by higher layer parameter maxLength in DMRS-UplinkConfig. - if maxLength is not configured, single-symbol DM-RS can be scheduled for the UE by DCI or configured by configured grant Type 1, and the UE can be configured with a number of additional DM-RS for PUSCH by higher layer parameter dmrs-AdditionalPosition, which can be 'pos0', 'pos1', 'pos2', 'pos3'. - if maxLength is configured, either single-symbol DM-RS or double symbol DM-RS can be scheduled for the UE by DCI or configured by configured grant Type 1, and the UE can be configured with a number of additional DM-RS for PUSCH by higher layer parameter dmrs-AdditionalPosition, which can be 'pos0' or 'pos1'. - and, the UE shall transmit a number of additional DM-RS as specified in Table 6.4.1.1.3-3 and Table 6.4.1.1.3-4 in -Subclause 6.4.1.1.3 of [4, TS 38.211]. If a UE transmitting PUSCH is configured with the higher layer parameter phaseTrackingRS in DMRS-UplinkConfig, the UE may assume that the following configurations are not occurring simultaneously for the transmitted PUSCH - any DM-RS ports among 4-7 or 6-11 for DM-RS configurations type 1 and type 2, respectively are scheduled for the UE and PT-RS is transmitted from the UE. For PUSCH scheduled by DCI format 0_1, triggered by activation DCI format 0_1, or configured by configured grant Type 1, the UE shall assume the DM-RS CDM groups indicated in Tables 7.3.1.1.2-6 to 7.3.1.1.2-23 of Subclause 7.3.1.1 of [5, TS38.212] are not used for data transmission, where "1", "2" and "3" for the number of DM-RS CDM group(s) correspond to CDM group 0, {0,1}, {0,1,2}, respectively. For PUSCH scheduled by DCI format 0_0 or triggered by activation DCI format 0_0, the UE shall assume the number of DM-RS CDM groups without data is 1 which corresponds to CDM group 0 for the case of PUSCH with allocation duration of 2 or less OFDM symbols with transform precoding disabled, and the UE shall assume that the number of DM-RS CDM groups without data is 2 which corresponds to CDM group {0,1} for all other cases. For uplink DM-RS with PUSCH, the UE may assume the ratio of PUSCH EPRE to DM-RS EPRE ( [dB]) is given by Table 6.2.2-1 according to the number of DM-RS CDM groups without data. The DM-RS scaling factor specified in subclause 6.4.1.1.3 of [4, TS 38.211] is given by . Table 6.2.2-1: The ratio of PUSCH EPRE to DM-RS EPRE Number of DM-RS CDM groups without data; DM-RS configuration type 1; DM-RS configuration type 2. 1; 0 dB; 0 dB. 2; -3 dB; -3 dB. 3; -; -4.77 dB. < Unchanged parts are omitted >

Suggestion:

- Adopt above the CR.

  - R1-190xxxx (Correction on PUSCH with configured grant, Huawei)

Reason for change:

- The resource mapping for PUSCH transmission with configured grant is not complete.

Summary of change:

- Correct TS 38.211 Section 6.4.1.1.3 as following:

##### 6.4.1.1.3 Precoding and mapping to physical resources < Unchanged parts are omitted > The position(s) of the DM-RS symbols is given by and duration ld where - ld is the duration between the first OFDM symbol of the slot and the last OFDM symbol of the scheduled PUSCH resources in the slot for PUSCH mapping type A according to Tables 6.4.1.1.3-3 and 6.4.1.1.3-4 if intra-slot frequency hopping is not used, or - ld is the duration of scheduled PUSCH resources for PUSCH mapping type B according to Tables 6.4.1.1.3-3 and 6.4.1.1.3-4 if intra-slot frequency hopping is not used, or - ld is the duration per hop according to Table 6.4.1.1.3-6 if intra-slot frequency hopping is used. - if the higher-layer parameter maxLength in DMRS-UplinkConfig is not configured, the tables shall be used according to single-symbol DM-RS - if the higher-layer parameter maxLength in DMRS-UplinkConfig is equal to 'len2', the associated DCI or configured grant determines whether single-symbol or double-symbol DM-RS shall be used - if the higher-layer parameter dmrs-AdditionalPosition is not set to 'pos0' and intra-slot frequency hopping is enabled according to clause 7.3.1.1.2 in [4, TS 38.212] and by higher layer, Tables 6.4.1.1.3-6 shall be used assuming dmrs-AdditionalPosition is equal to 'pos1' for each hop. < Unchanged parts are omitted >

Suggestion:

- Adopt above the CR.