3GPP TSG RAN WG1 #96bis	R1-1904495

Xi’an, China, 8th– 12th April, 2019

Agenda item:	7.2.4.3

Source: 	MediaTek Inc.

Title: 	Discussion on sidelink based synchronization mechanism

Document for: 	Discussion/Decision

# Introduction

In this contribution, we provide our views on several open issues related to S-SSB design and synchronization procedure based on the feature lead summary in RAN1#96 meeting.

# PSS/SSS in S-SSB

Typically, the design target of S-SSB should take into account the coverage, the correlation property of the sequences as well as the feasibility of reusing NR uu interface hardware for S-SSB implementation. From the correlation properties perspective, there has been extensive studies and conclusions for NR PSS/SSS sequences so that they can be reused for S-SSB design. For the feasibility of reusing NR uu interface hardware for S-SSB implementation, it is important for the fast and low-cost implementation of NR V2X.

Proposal 1: S-SSB design should take into account the coverage, the correlation property of the sequences and the feasibility of reusing NR uu interface hardware.

To achieve the same or better performance than LTE V2X with S-SSB over 6 RBs, at most 12 RBs can be used for NR V2X SSB based on the link budget. For PSS/SSS, even though LTE has the narrow bandwidth for more power boosting, the longer sequence length (127 vs. 63) can provide the better correlation performance. In total, the coverage considering the link budget and received SINR will be same. The link budget analysis has been provided in Table 1 and the required SINRs for one-shot detection (NR vs. LTE) are provided in Figure 1.

As shown in Figure 1 with different channels, the PSS/SSS detection performance for NR V2X (based on NR PSS/SSS) can be 3dB better than LTE V2X (based on LTE PSS/SSS) for one-shot detection. With the same number of repetition, the gain is expected to be same. Further considering the 3dB link budget difference due to the different bandwidth (12 RBs in NR V2X vs. 6 RBs in LTE V2X) under the same Tx/Rx configuration, the overall performance for NR V2X and LTE V2X are expected to be same. Further, to secure the power boosting gain for PSS/SSS, there should be no FDM multiplexing between PSS/SSS and other signals/channels, similar to LTE V2X. However, for PBCH, there are still 3dB performance difference based on the link budget analysis.

Table 1. The communication range vs. the corresponding SINR level

| Parameters |  | Description | Freeway NLOSv scenario |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | LTE V2X PSS/SSS/PBCH 6 RBs@15khz | NR PSS/SSS 12RBs@15khz | NR PBCH 12RBs@15khz |
| Common Paras | D | ISD[m] | 1000 | 1000 | 1000 |
|  | f | Frequency[GHz] | 6 | 6 | 6 |
|  | BW | Bandwidth[MHz] | 1.08 | 2.16 | 2.16 |
| Vehicle Tx Paras | Gt_e | Max Tx element gain[dBi] | 3 | 3 | 3 |
|  | Nt | Tx #elements | 1 | 1 | 1 |
|  | Gt | Tx antenna gain[dB] | 3 | 3 | 3 |
| Channel Paras | A | Prop-A | 20 | 20 | 20 |
|  | B | Prop-B | 32.4 | 32.4 | 32.4 |
|  | G | Prop-G | 20 | 20 | 20 |
|  | PL | Pathloss[dB] | -108.0 | -108.0 | -108.0 |
|  | BL | Block[dB] | -12.5 | -12.5 | -12.5 |
|  | S | Shadowing[dB] | -3 | -3 | -3 |
| Vehicle Rx Paras | Gr_e | Max Rx element gain[dB] | 3 | 3 | 3 |
|  | Nr | Rx #elements | 2 | 2 | 2 |
|  | Gr | Rx antenna gain[dB] | 6 | 6 | 6 |
| Loss | Gp | Isotropic gain[dB] | -123.5 | -123.5 | -123.5 |
|  | Gc | Coupling gain[dB] | -114.5 | -114.5 | -114.5 |
| Power related Paras | Pt | Tx power[dBm] | 23 | 23 | 23 |
|  | Pr | Received power[dBm] | -91.5 | -91.5 | -91.5 |
|  | Nf | Noise figure[dB] | 9 | 9 | 9 |
|  | Ni | Interference margin[dB] | 0 | 0 | 0 |
|  | Pn | Noise power[dBm] | -104.7 | -101.7 | -101.7 |
|  |  | PSS/SSS sequence gain （dB） | 0 | 3 | N/A |
|  | SINR | SINR[dB] | 13.2 | 13.2 | 10.2 |

Figure 1. PSS/SSS detection performance for NR V2X (length-127) vs. LTE V2X (length-63).

Proposal 2: Use the same length as NR PSS/SSS sequence (length-127) for NR V2X PSS/SSS.

Proposal 3: at least 2-symbols of PSS and 2-symbols of SSS are supported. FFS on exact positions for SSS symbols and support of AGC symbol.

Proposal 4: There is no FDM multiplexing between PSS/SSS and other signal/channels for power boosting on PSS/SSS.

# PBCH data and PBCH DMRS in S-SSB

If the NR S-SSB design is targeting to have the comparable performance as LTE V2X, then PBCH (including PBCH data and DMRS) could be enhanced by having up to 12 or 11RBs of bandwidth and transmitted twice. Accordingly, the number of symbols for PBCH can be minimized for the better performance to combat the Doppler shift caused by the (relatively) high speed moving. Meanwhile, the payload size, interleaving pattern and polar coding in NR PBCH design can be reused for NR V2X SPBCH Data design for reusing the NR uu receiver as much as possible.

Proposal 5: For NR V2X PBCH data, it is same as NR PBCH data in terms of the payload size, interleaving pattern and polar coding.

To accommodate 10Mhz bandwidth without degrading the performance, there can be different PBCH-DMRS patterns for 15/30khz and 60khz. For example, for 15/30khz subcarrier spacing, the comb-3 DMRS pattern as NR PBCH DMRS can be used. However, the DMRS pattern for 60khz subcarrier spacing can be constructed as such: the DMRS in the first and the last RBs are constructed based on comb-6 pattern to improve the edge PRB channel estimation. For the remaining center 9 RBs in the frequency domain, comb-4 pattern is used. In the time domain, the same DMRS pattern is applied every two symbols of the PBCH channel.

Figure 2. DMRS Pattern for PBCH channel (11 RBs x 4 symbols) with 60khz subcarrier spacing.

The DMRS pattern location in the RB can be fixed in the time/frequency domain without any cyclic shift to ensure the better channel estimation or cancellation in case of collision with other PBCHs from other UEs.

Proposal 6: The sparser NR V2X PBCH DMRS pattern can be supported for 60khz to accommodate the 10Mhz bandwidth without degrading the performance.

Proposal 7: For NR V2X PBCH DMRS, the location in the RB is fixed without any cyclic shift.

# S-SSB structure

Based on discussion in Section 3 and Section 4, the potential candidate for S-SSB structure can be as such as shown in Figure 3. One S-SSB with 11 or 12 RBs by 13 symbols composed of 1 AGC symbol (e.g., based on SSS), 2 PSS symbols, 1 SSS symbol, 8 PBCH symbols by repetition of PBCH channel with 4 symbols per PBCH channel, and 1 SSS symbol in order.

Figure 3. S-SSB structure

Proposal 8: NR V2X S-SSB structure is supported based on Combination 1 as shown in Figure 3.

# Priority rules for NR V2X sidelink synchronization sources

In the last meeting, the following working assumption has been reach as extracted below.

- Whether GNSS-based synchronization or gNB/eNB-based synchronization is used is (pre)-configured. - The following table is a working assumption GNSS-based synchronization; gNB/eNB-based synchronization. - P0: GNSS - P1: the following UE has the same priority: - UE directly synchronized to GNSS - P2: the following UE has the same priority: - UE indirectly synchronized to GNSS - P3: the remaining UEs have the lowest priority.; - P0: gNB/eNB - P1’: UE directly synchronized to gNB/eNB - P2’: UE indirectly synchronized to gNB/eNB - P3’: GNSS - P4’: UE directly synchronized to GNSS - P5’: UE indirectly synchronized to GNSS - P6’: the remaining UEs have the lowest priority..

However, for GNSS-based synchronization, it should be possible to allow the capable UE to synchronize with the BS for timing information.

So to confirm the working assumption, gNB can be added the sync source as below:

GNSS-based synchronization

- P0: GNSS - P1: the following UE has the same priority: - UE directly synchronized to GNSS - P2: gNB - P32: the following UE has the same priority: - UE indirectly synchronized to GNSS - P43: the remaining UEs have the lowest priority.

Proposal 9: Adding gNB as P2 in GNSS-based synchronization rule for confirmation of the working assumption.

# Sidelink synchronization sources

If the UE can support both NR and LTE, as well as NR V2X and LTE V2X communications, GNSS, eNB, gNB, NR UE and LTE UE can be studied for the synchronization sources.

For Case1 with the serving cell available in the V2X or non-V2X communication frequency layer, it is not necessary to use the LTE V2X UE as the sync source. Otherwise, it may mandate the inter-module tunnel between LTE-V2X module and NR-V2X module for fast sync information exchange by imposing more complexity and restriction on UE implementation, even if UE can support NR-V2X and LTE-V2X communication independently.

Observation 1: For the InC case or OoC with serving cell available, there is no need of using LTE V2X UE as the sync source.

Observation 2: LTE V2X UE as the sync source may mandate the inter-module tunnel between NR-V2X module and LTE-V2X module with high complexity and cost.

For Case2 without any serving cell available (i.e., OoC with only pre-configuration information available), the UE may be used as the sync source. However, if the SL communication for LTE V2X and NR V2X does not happen in the same frequency layer, i.e., shared frequency layer, there is no need to use LTE V2X UE as the sync source for NR V2X communication either. Because the NR V2X communication happens in the different frequency layer than the LTE V2X communication, which will not cause any strong interference between two frequency layers. In this case, NR UE synced to other sources or self-promoted NR UE can be taken as the sync source.

Proposal 10: LTE V2X UE is not used as the sync source for NR V2X communication because of the different frequency layer for NR and LTE V2X communication.

In principle, both eNB and gNB can provide the time alignment information for the UE. It is not expected to have any notable performance difference for UE (indirectly) synchronization to eNB and gNB. Thus, there is no strong motivation to differentiate the sync source of eNB and gNB for prioritization in terms of the performance. Accordingly, there is no need to have the different SLSS ID(s) for sync sources based on eNB and gNB, which can also save the SLSS ID number.

Proposal 11: There is no need of SLSS ID partition for differentiation of the sync sources based on eNB and gNB.

# Information carried by PSBCH

In LTE/NR, MIB info carried in PBCH provides the basic information for the UE to access into the network. For V2X, MIB information carried in PSBCH is to provide the timing information and the resource configuration to enable the SL communication for out-of-coverage UEs without serving cell.

In LTE V2X, PSBCH contents is defined considering LTE feature based design, including sl-bandwidth, tdd-ConfigSL, directFrameNumber, directSubFrameNumber and inCoverage indicator. In NR V2X, the information carried in PSBCH should consider NR feature based design, such as SSB structure, whether supporting beam sweeping, bandwidth part (BWP) and different numerologies. At least the following information can be carried in PSBCH:

Timing information

It is reasonable to indicate the frame number in NR V2X PSBCH for frame timing info. For indication of subframe/slot information, there are two options:

- Option A: Slot/subframe information for the corresponding S-SSB is carried in MIB explicitly.

  - Pros: Flexible location for SSB

  - Cons: Large signaling overhead to indicate all possible locations in subframe/slot.

- Option B: Fixed and unique location for S-SSBs in a subframe to derive the slot/subframe information implicitly.

  - Pros: Small signaling overhead with fixed location/pattern to derive the timing information.

  - Cons: Fixed and single location for SSB in a subframe. SSB period can’t be smaller than a subframe.

Considering the different subcarrier spacing in NR V2X, the slot number information should be provided in case of subcarrier spacing is different than 15khz.

Proposal 12: Slot number information should be added for NR V2X.

Proposal 13: The fixed and unique location for S-SSB in a subframe can be used to derive the slot/subframe information implicitly.

Initial SL BWP configuration information

NR V2X has agreed to support BWP. Thus, the information related to initial SL BWP configuration can be carried in PSBCH. The location of SL-BWP can be indicated relative to the SL-SSB. The number of bits for SL-BWP bandwidth and locations should be restricted (or fixed) to save the bits in PBCH and reduce the complexity to avoid unnecessary multiple options.

Proposal 14: The location and bandwidth of the initial SL BWP is carried in PSBCH.

InCoverage Indicator

The inCoverage indicator in LTE V2X PSBCH is used to indicate if the UE is directly or indirectly synced to GNSS/eNB. This is used for the priority group selection since UEs directly synced to GNSS/eNB has higher priority than UEs indirectly synced to them. However, such indicator is carried in MIB in LTE V2X so that UE has to decode PSBCH every time to determine the priority group, which causes the unnecessary complexity for the UEs. In NR V2X, such indicator can be considered to be carried in PSBCH-DMRS to avoid PSBCH decoding for priority group identification.

Observation 3: In LTE V2X, UEs have to decode inCoverage in PSBCH to determine priority groups with unnecessary complexity.

Proposal 15: The inCoverage indicator can be carried in PSBCH-DMRS rather than PSBCH payload to avoid unnecessary decoding for priority group identification.

# Conclusion

The following summarizes the observations:

Observation 1: For the InC case or OoC with serving cell available, there is no need of using LTE V2X UE as the sync source.

Observation 2: LTE V2X UE as the sync source may mandate the inter-module tunnel between NR-V2X module and LTE-V2X module with high complexity and cost.

Observation 3: In LTE V2X, UEs have to decode inCoverage in PSBCH to determine priority groups with unnecessary complexity.

The proposals are summarized as below:

Proposal 1: S-SSB design should take into account the coverage, the correlation property of the sequences and the feasibility of reusing NR uu interface hardware.

Proposal 2: Use the same length as NR PSS/SSS sequence (length-127) for NR V2X PSS/SSS.

Proposal 3: At least 2-symbols of PSS and 2-symbols of SSS are supported. FFS on exact positions for SSS symbols and support of AGC symbol.

Proposal 4: There is no FDM multiplexing between PSS/SSS and other signal/channels for power boosting on PSS/SSS.

Proposal 5: For NR V2X PBCH data, it is same as NR PBCH data in terms of the payload size, interleaving pattern and polar coding.

Proposal 6: The sparser NR V2X PBCH DMRS pattern can be supported for 60khz to accommodate the 10Mhz bandwidth without degrading the performance.

Proposal 7: For NR V2X PBCH DMRS, the location in the RB is fixed without any cyclic shift.

Proposal 8: NR V2X S-SSB structure is supported based on Combination 1 as shown in Figure 3.

Proposal 9: Adding gNB as P2 in GNSS-based synchronization rule for confirmation of the working assumption.

Proposal 10: LTE V2X UE is not used as the sync source for NR V2X communication because of the different frequency layer for NR and LTE V2X communication.

Proposal 11: There is no need of SLSS ID partition for differentiation of the sync sources based on eNB and gNB.

Proposal 12: Slot number information should be added for NR V2X.

Proposal 13: The fixed and unique location for S-SSB in a subframe can be used to derive the slot/subframe information implicitly.

Proposal 14: The location and bandwidth of the initial SL BWP is carried in PSBCH.

Proposal 15: The inCoverage indicator can be carried in PSBCH-DMRS rather than PSBCH payload to avoid unnecessary decoding for priority group identification.

# References

- 3GPP RAN1 #AH1901 Jan 2019 Chairman’s Notes

- 3GPP RAN1 #95 Dec 2018 Chairman’s Notes

- R1-1810454, “On NR V2X synchronization mechanism”, Mediatek Inc., Chengdu, China, October, 2018

- R1-1812364, “Discussion on physical layer structure for NR sidelink”, Mediatek Inc., Spokane, USA, November, 2018

- 3GPP RAN1 #96 Feb 2019 Chairman’s Notes