3GPP TSG RAN WG1 Meeting #97	R1-1905951

Reno, USA, 13th - 17th May, 2019

Title:	Discussion on channel access procedure for NR-U

Source: 	ZTE, Sanechips

Agenda Item:	7.2.2.2.1

Document for:	Discussion and Decision

# Introduction

In RAN #82 meeting, the new WI proposal on “NR-based Access to Unlicensed Spectrum” (NR-U) was approved [1]. This WI targets to specify NR enhancements for a single global solution framework operating in unlicensed bands, e.g., 5 GHz and 6 GHz. In the WID of NR-U, it was described about the specifying support for channel access. During NR-U SI stage, some agreements were reached and can be regarded as a starting point for NR-U WID.

In RAN1 #96 meeting [2], the following agreements about channel access were reached:

Conclusion:

Cat 2 is not used for initiating a UE transmission outside of a gNB COT for the following channels/signals (or any combination of them):

- PUSCH (with or without UCI),

- SRS-only,

- PUCCH-only

Note:

- Cat 4 for these channels was already agreed during the study item

- This does not preclude the use of Cat 2 for transmission on a LBT bandwidth if it is allowed for the case of transmission on multiple LBT bandwidths

Agreement:

For initiation of a gNB transmission:

- LBT other than Cat 4 is not used for DRS multiplexed with unicast data

- LBT other than Cat 4 is not used for PDCCH and/or PDSCH transmission outside of DRS.

Note:

- This does not preclude the use of Cat 2 for transmission on a LBT bandwidth if it is allowed for the case of transmission on multiple LBT bandwidths

Agreement:

- LBT other than Cat4 is not considered for UL transmissions that are part of a RACH procedure that initiate a channel occupancy

- Note: This does not preclude the use of Cat 2 for transmission on a LBT bandwidth if it is allowed for the case of transmission on multiple LBT bandwidths

In RAN1 #96bis meeting [3], the following agreements about channel access were reached:

Conclusion:

If a category 2 LBT with a duration of 25 microseconds is required, no changes to the baseline method as followed in LAA for such LBT are needed

Agreement:

The following agreement from the SI is updated as shown:

Table 7.2.1.3.1-1: Channel access schemes for gNB as LBE device

|  | Cat 2 LBT | Cat 4 LBT |
| --- | --- | --- |
| DRS alone or multiplexed with non-unicast data (e.g. OSI, paging, RAR) | When the DRS duty cycle ≤1/20, and the total duration is up to 1 ms: 25 µs Cat 2 LBT is used (as in LAA) | When DRS duty cycle is > 1/20, or total duration > 1 ms Cat4 with any channel access priority class value can be used |
| DRS multiplexed with unicast data | N/A except for the cases discussed in the Note below | Channel access priority class is selected according to the multiplexed data |
| PDCCH and PDSCH | N/A except for the cases discussed in the Note below | Channel access priority class is selected according to the multiplexed data |

Agreement:

For a UCI-only transmission on PUSCH in a channel occupancy initiated by the UE, Cat4 with lowest channel access priority class value can be used by the UE

Agreement:

For LBT by a UE prior to transmission of a UL burst within a gNB-initiated channel occupancy as LBE device, for gap durations shorter than 25 microseconds, choose one of the following alternatives

- Alt 1: Cat 2 LBT can be indicated (FFS: explicit and/or implicit) to the UE if the gap is 16 microseconds (allowing for implementation tolerances)

- Alt 2: Cat 2 LBT is not indicated to the UE for gaps less than 25 microseconds

- Notes (applicable to both alternatives):

  - This means that the gNB ensures that gaps between 16 and 25 microseconds do not occur

  - This doesn’t change the previous agreement for Cat 1 and Cat 2 LBT for gaps of 16 microseconds or less

- FFS: Conditions on channel occupancy after a Cat. 1 or Cat. 2 LBT after a gap of 16 microseconds or less

In this contribution, we share our views on channel access procedure for NR-U, including CWs adjustment, LBT for DRS, LBT for multiple switching points within a COT, LBT if gap is 16 microseconds, LBT for wideband operation, LBT for multiplexing and frequency (spatial) reuse, LBT for FBE, directional LBT and receiver assisted LBT.

# Channel access procedure for NR-U

## CWS adjustment

In LTE LAA, DL CWS is determined based on the NACK ratio Z of the HARQ-ACKs for the TBs in the DL reference subframe. UL CWS is determined based on the presence of ACK feedback for the TBs in the UL reference subframe or the HARQ process.

CBG based HARQ-ACK feedback

In NR, CBG based HARQ-ACK feedback is introduced. In NR-U, if CBG based HARQ-ACK feedback is configured, it needs to be further studied how to count multiple CBG-ACKs per TB into the ACK/NACK ratio Z in CWS adjustment. The following candidate solutions can be considered.

- Alt-1: CBG level ACK/NACK is converted into TB level ACK/NACK. Wherein, the ACK/NACK ratio Z is of TB level.

For example, if all CBG for the same TB within the reference slot are ACK/NACKs, the CBG level ACK/NACK for the TB is counted as a TB level ACK/NACK. If one of all CBG for the same TB within the reference slot is ACK/NACK, the CBG level ACK/NACK for the TB is counted as a TB level ACK/NACK.

- Alt-2: Redefinition of ACK/NACK ratio Z in order to include TB level ACK/NACK and CBG level ACK/NACK.

ACK/NACK ratio Z can be defined as the ratio of total number of ACK/NACK of TB and CBG and total number of transmitted TB and CBG within the reference slot/subframe.

Proposal 1: If CBG based HARQ-ACK feedback is configured for NR-U, ACK/NACK ratio Z can be counted/determined by one of the following methods:

- Alt-1: CBG level ACK/NACK is converted into TB level ACK/NACK. Wherein, the ACK/NACK ratio Z is of TB level.

- Alt-2: Redefinition ACK/NACK ratio Z in order to include TB level ACK/NACK and CBG level ACK/NACK.

Reference Slot

For DL/UL reference subframe determination, when NR-U supports flexible DL/UL scheduling/HARQ timing, how to determine the reference slot needs to be discussed. Although NR-U can adapt to flexible timing relationship between PDSCH and UL HARQ feedback, between PUSCH transmission and retransmission, and so on, the reference slot for CWS adjustment should still be defined as the first slot or the first and second slots of the latest DL/UL data burst.

Proposal 2: The reference slot for CWs adjustment should be defined as the first slot or the first and second slot of the latest DL/UL date burst for NR-U.

CWS adjustment for Configured Grant

In configured grant operation, the CWS adjustment scheme specified in AUL of R14 eLAA can be used as the baseline, further enhancement can be considered for configured grant specific characteristic once the configured grant UL transmission procedure for NR-U is defined.

Proposal 3: For CWS adjustment for configured grant, the CWS adjustment scheme specified in AUL of R14 eLAA can be used as a starting point for study.

## LBT for DRS

According to the conclusion of the last meeting, Cat-4 LBT is used for DRS duty cycle when DRS transmission is longer than 1/20 or the total of DRS duration is larger than 1ms. Cat-2 LBT can be performed only for DRS duty cycle smaller than 1/20 or the total of DRS duration smaller than 1 ms. For this agreement, our understanding is that Cat-4 LBT is just used before the DRS multiplexed with unicast date are transmitted. Besides, Cat-4 LBT can also be used before DRS burst set starts transmission, wherein, duration of DRS burst set > 1ms or duty cycle of DRS burst set in a DRS period >1/20.

Besides, Cat-2 LBT can be used for the following situations:

Case 1: Within DRS burst set, if there is an empty symbol/gap between DRS bursts (half-slot), Cat-2 LBT can try to be performed as shown in Figure 1.

Figure 1 An example of LBT for DRS burst set and DRS burst

Case 2: Within DRS transmission window, if a new DRS burst set is initiated by gNB and its duration is smaller than or equal to 1 ms and the total duty cycle of DRS transmission in a DRS burst set period is not larger than 1/20, then Cat-2 LBT can try to be adapted before this new DRS burst set as shown in Figure 2. In another case, if gNB uses Cat-4 LBT to detect channel busy all the time and only a few SSB candidate positions remain before the ending of DRS transmission window, then it can be considered to use Cat-2 LBT for DRS burst set transmission.

Figure 2 An example of LBT for DRS burst set and DRS burst transmission (total duty cycle >1/20)

Case 3: Before DRS burst set that only occupies 1ms channel time, which is similar to LTE-LAA DRS. Cat-2 LBT can be considered, as shown in Figure 3. However, if the length of DRS burst set is larger than 1 ms or duty cycle is larger than 1/20 in the next period of DRS burst set, then Cat-4 LBT should be performed.

Figure 3 An example of LBT for DRS burst set >= 1 ms

Case 4: If the adjacent two SSBs (DRS bursts (half-slot)) within DRS burst set is non-QCL’d relationship and there is a gap between non-QCL’d SSB or beam switching occurs, Cat-2 LBT can be used in order to fast access channel and accurately access current channel status. Preferably, directional LBT can be performed for beam-based SSB transmission within DRS burst.

Case 5: Cat-2 LBT mechanism can also be considered for the wideband or CA scenarios, which is similar to LTE-LAA multiple CCs (e.g., Type A/B) manner.

Based on the above analysis, the LBT mechanism for DRS transmission should be considered according to different situations.

Proposal 4: Cat-4 LBT can be used for the following cases:

- Before the DRS multiplexed with unicast date are transmitted (agreed in RAN1#96).

- Before DRS burst set starts transmission and DRS burst set duration is larger than 1 ms or the total duty cycle of DRS transmission in DRS burst set period is larger than 1/20.

Proposal 5: Cat-2 LBT can be considered to be used in the following situations:

- Case 1: Within DRS burst set, if there is an empty symbol/gap between DRS bursts (half-slot),.

- Case 2: Within DRS transmission window, if a new DRS burst set is initiated by gNB and its duration is smaller than or equal to 1ms and the total duty cycle of DRS transmission in a DRS burst set period is not larger than 1/20, or if gNB uses Cat4 LBT to detect channel busy all the time and only a few SSB candidate positions (e.g., 2 SSBs) remain before the ending of DRS transmission window.

- Case 3: Before DRS burst set that only occupies 1 ms channel time, which is similar to LTE-LAA DRS.

- Case 4: If the adjacent two SSBs (DRS bursts (half-slot)) within DRS burst set is non-QCL’d relationship and there is a gap between non-QCL’d SSB.

- Preferably, directional LBT can be performed for beam-based SSB transmission within DRS burst.

- Case 5: for case where multiple sub-band case in the wideband operation or multiple carriers/BWPs case.

## LBT for multiple switching points within a COT

In ETSI regulation, there is no clear restriction about the number of the DL/UL switching potions within a COT. In order to have a flexibility of DL/UL scheduling within a COT, NR-U can support multiple switching points with gap larger than 25 us in a gNB initiated COT using Cat 2 LBT. Besides, if the gap of switching point is larger than 16 us and smaller than 25 us, as last meeting agreed that gNB ensures that this case does not happen. However, if gap is 16 us, some details can be found in the next section. Further, for configured grant operation, the above suggestion can also be applied.

Proposal 6: NR-U support multiple switching points in a gNB initiated COT (notes: the methods can also be applied to CG operation).

- If gap of switching point >= 25 us, Cat 2 LBT is applied.

- If 16 us < gap of switching point < 25 us, gNB ensures that this case does not happen.

- If gap of switching point = 16 us, some details can be found in Proposal 8.

## LBT if gap is 16 microseconds

In LTE-LAA, if the gap between DL and UL is 16 us in a gNB initiated COT, UE can directly transmit UL data without LBT operation. According to regulation of Wi-Fi, if a device does not perform LBT operation, it only can transmit a short duration such as 0.5 ms. Based on this, if the gap between DL and UL is 16 us in NR-U, in order to occupy long channel time, UE can perform a Cat-2 LBT. Whether Cat-2 LBT is performed depends on gNB’s indicator or requirement of channel occupancy time for UE.

If Cat-2 LBT needs to be performed by UE for gap = 16 us, a new definition for Cat-2 LBT with 16 us duration should be considered. In order to minimize specification impact, the definition of Cat-2 LBT with 25 us duration can be considered as a baseline, e.g., the following definition of Cat-2 LBT with 16 us can be considered as shown in Figure 4. If the device senses the channel during 16 us duration, and the power detected by the device for at least 4 us within 16 us duration is less than energy detection threshold, the channel is considered to be idle. Wherein, 16 us duration consists of 7 us duration and 9 us duration

Figure 4 Example definition of Cat-2 LBT with 16 us duration

Proposal 7: Cat-2 LBT should be indicated to UE when the gap between DL and UL is 16 us.

Proposal 8: If Cat-2 LBT with 16 us duration is supported, it is recommended that a new definition for Cat-2 LBT with 16us duration can be introduced.

- Sensing duration consists of an idle duration Tf = 7 us immediately followed by a slot duration Tsl = 9 us. Wherein, 4us in slot duration Tsl is detection duration.

## LBT for Wideband operation

In order to transmit on the wideband carrier, a straightforward way is to perform LBT operation over the entire wideband carrier, which can fully utilize the entire wideband carrier if LBT is successful. When the entire wideband carrier consists of multiple 20 MHz carriers, LBT of wideband carrier would fail if any one of multiple 20 MHz carriers within wideband carrier is busy. In order to increase the chance of channel access, another way is to perform LBT on each 20 MHz carrier. LBT for multiple carriers can be done in a similar manner as it is done for LTE-LAA multiple CCs (e.g., Type A/B) in TS 36.213 Section 15.1.5 [4]. Compared with LBT of wideband carrier, LBT on multiple 20 MHz carriers can improve the spectrum utilization and coexistence fairness with incumbent systems. Based on this, LBT of LTE-LAA multiple CCs (e.g., Type A/B) manner can be used as starting point for research LBT for multiple carriers.

Proposal 9: LBT of LTE-LAA multiple CCs (e.g., Type A/B) manner can be used as starting point for research LBT for multiple carriers.

## LBT for Multiplexing and Frequency Reuse

In LTE-LAA, frequency reuse among the neighbor nodes of the same operator is supported where maximum ED threshold (-52 dBm) has been agreed. In addition, it is recommended that multi-user multiplexing in one UL subframe on the unlicensed SCell by SDM or FDM should be supported in LTE-LAA. Based on this, multi-user multiplexing and frequency reuse could be also considered to be supported in NR-U. Furthermore, spatial reuse in NR-U has been supported to enhance performance beyond the baseline LBT in RAN1 #92bis meeting.

In order to support multiplexing and/or frequency reuse and/or spatial reuse in NR-U, we can consider blank pattern method in addition to CCA detection approach, which was also mentioned in the other companion contribution [5]. Blank pattern can be at the granularity of PRB level, similar to PRB based interlace structure, e.g., one of interlace units can be used as blank pattern for CCA detection as shown in Figure 5. This requires the energy detection at PRB level. The merit of this method is that NR-U nodes can distinguish that the signal observed on the channel is from other NR-U nodes of the same operator or from NR-U nodes of the other operator or from others UE belonging to the same cell or from other RATs and there are less impact on resource allocation. Preferably, different operators/cells configure different blank patterns and NR-U nodes of the same operator/cell configure the same blank pattern. If no energy is detected in those blank patterns, gNB can assume that at least during this period of time, there is no contention from other unlicensed systems, thus it can operate in licensed mode which tends to be more efficient.

Further, blank pattern can be also reserved during its reservation signal and/or data transmission. NR-U nodes can detect energy on the blank pattern to estimate the source of interference. Besides, for the blank pattern scheme, RAN1 can send an LS to RAN4 in order to evaluate the feasibility of this scheme.

Figure 5 Blank pattern for PRB based interlace structure for uplink

An example is given to illustrate the blank pattern method to support multi-user multiplexing in the same cell or frequency reuse of inter-cell of the same operator. As shown in Figure 5, orange color part of the picture denotes the CCA detection pattern. Assuming that node1 and Node 2 belong to operator 1/cell 1, the same CCA detection pattern is configured for Node 1 and Node 2. If the node1 first detects the channel idle before transmission, it transmits on at least one of white part or interlace-based scheduling resources and does not transmit any information on the blank identification area in Figure 5. If the detected energy on the blank pattern for Node 2 is lower than the threshold T1, then Node 2 assumes that the current channel is idle and can be reused or multiplexed. Or if the detected energy on the blank pattern for Node 2 is larger than T1 while detected energy on the resources outside the blank pattern in the whole bandwidth is lower than the threshold T2, Node 2 thinks that frequency reuse/multiplexing can be realized, e.g., T1 < T2. Similarly, if Node 1 and Node 2 is scheduled or allocated on the same time-frequency resource along with different beam direction, the blank pattern method is also applicable to spatial reuse of Node 1 and Node 2.

Proposal 10: Frequency reuse/multiplexing should be supported in NR-U and some methods of frequency reuse/multiplexing can be considered such as PRB based interlace structure of blank pattern method.

Proposal 11: For blank pattern scheme, RAN1 can send an LS to RAN4 in order to evaluate the feasibility of this scheme.

### Simulation

In this section, the coexistence evaluation results for indoor deployment for NR-U+NR-U with FTP traffic are provided. Performance is compared between the proposed multiplexing scheme and the no-multiplexing scheme.

Table 1 NR-U and NR-U coexistence evaluation results for indoor deployment

| Reported parameters |  | -72 dbm |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | NoReuse | ReUse | NoReuse | -ReUse | NoReuse | ReUse |
| DL: UPT CDF [Mbps] | 5% | 55.4 | 55.9 | 34.2 | 34.94 | 16.6 | 20.6 |
|  | 50% | 144.4 | 144.5 | 130.8 | 130.1 | 70.4 | 79.3 |
|  | 95% | 184.6 | 1923 | 155.2 | 169.7 | 112.8 | 132.3 |
|  | Mean | 140.3 | 141.7 | 115.4 | 118.0 | 66.5 | 77.2 |
| DL: Delay CDF [s] | 5% | 0.019 | 0.019 | 0.019 | 0.019 | 0.021 | 0.020 |
|  | 50% | 0.026 | 0.025 | 0.035 | 0.034 | 0.087 | 0.068 |
|  | 95% | 0.113 | 0.114 | 0.182 | 0.177 | 0.837 | 0.566 |
|  | Mean | 0.040 | 0.040 | 0.059 | 0.057 | 0.213 | 0.157 |
| UL: UPT CDF [Mbps] | 5% | 22.1 | 22.2 | 20.2 | 20.8 | 14.0 | 15.1 |
|  | 50% | 162.1 | 162.9 | 143.4 | 144.7 | 96.9 | 107.9 |
|  | 95% | 181.6 | 181.6 | 163.4 | 163.2 | 139.5 | 143.3 |
|  | Mean | 135.2 | 137.0 | 122.8 | 124.8 | 89.1 | 97.1 |
| UL: Delay CDF [s] | 5% | 0.021 | 0.021 | 0.021 | 0.021 | 0.021 | 0.021 |
|  | 50% | 0.025 | 0.025 | 0.030 | 0.029 | 0.051 | 0.046 |
|  | 95% | 0.180 | 0.176 | 0.193 | 0.187 | 0.395 | 0.342 |
|  | Mean | 0.050 | 0.049 | 0.054 | 0.053 | 0.110 | 0.093 |
| BO (DL)(%) |  | 8.952 | 9.024 | 17.128 | 17.674 | 38.534 | 38.626 |
| BO (UL)(%) |  | 8.450 | 8.321 | 14.673 | 14.372 | 30.817 | 27.905 |
| 𝜆 |  | DL: 0.3 files/s UL: 0.3 files/s |  | DL: 0.5 files/s UL: 0.5 files/s |  | DL: 0.8 files/s UL: 0.8 files/s |  |

Simulation results show that multiplexing/Frequency reuse with blank pattern schemes can achieve performance improvement compared to no- multiplexing/Frequency reuse and the gain increases with the increase of traffic load.

Proposal 12: Multiplexing/Frequency reuse with blank pattern schemes can achieve performance improvement compared to no-multiplexing/Frequency reuse the gain increases with the increase of traffic load.

## LBT for FBE

In RAN1 #93 meeting, FBE based frame structure was suggested to be studied. For FBE, low channel access probability due to fixed LBT time and unfair channel access opportunity between different NR-U nodes should be taken into consideration when they are not synchronized. Thus, multiple CCAs opportunities such as enhanced Cat 2 LBT can be configured for FBE based LBT, which was also mentioned in the other companion contribution [6]. Wherein, multiple CCAs opportunities is cell specific.

That is to say, for multiple CCAs method, in each CCA time window, there is more than one CCA time. On the one hand, the first CCA detection position of the UE can be randomly selected in the pre-partitioned CCA detection time window. If the CCA results indicate the channel is unavailable, then the second CCA detection time is still randomly selected in the rest of the CCA detection time window. And so on, until the CCA results indicate the channel is available, the UE would transmit UL data in the scheduled resources, as shown in Figure 6(a). On the other hand, in a given CCA time window, the channel is considered to be available as long as the channel idle time continuously detected is greater than the default CCA detection time, as shown in Figure 6(b). This method can not only solve unfairness problem caused by NR-U node timing difference but also increase the channel access opportunity of NR-U nodes.

(a)

(b)

Figure 6 Two types of multiple CCAs method for FBE

Evaluation results are provided for an indoor deployment of NR-U+NR-U with FTP traffic and compare the fairness performance between enhanced Cat 2 LBT such as multiple CCA opportunity with Cat 2 LBT through simulations.

Table 2 Fairness comparison between enhanced Cat 2 LBT and Cat 2 LBT

| Reported parameters |  | Cat 2 LBT |  | Enhanced Cat 2 LBT |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Opt A | Opt B | Opt A | Opt B |
| DL: UPT CDF [Mbps] | 5% | 33.9 | 18.0 | 22.9 | 29.7 |
|  | 50% | 82.0 | 74.2 | 63.2 | 81.0 |
|  | 95% | 141.8 | 129.4 | 131.3 | 134.3 |
|  | Mean | 96.3 | 74.6 | 81.3 | 84.4 |
| DL: Delay CDF [s] | 5% | 0.019 | 0.020 | 0.020 | 0.020 |
|  | 50% | 0.041 | 0.049 | 0.041 | 0.061 |
|  | 95% | 0.150 | 0.150 | 0.164 | 0.150 |
|  | Mean | 0.062 | 0.076 | 0.080 | 0.065 |
| 𝜌DL(%) |  | 99.8 | 99.8 | 98.1 | 99.6 |
| BO (DL)(%) |  | 25.9 | 22.0 | 30.8 | 22.0 |
| 𝜆 |  | 1.2 files/s |  |  |  |

It is observed that enhanced Cat 2 LBT such as multiple CCAs method has better performance than Cat 2 LBT as shown in Table 2 for the case of asynchronous NR-U nodes.

Proposal 13: Multiple CCAs methods can be considered for FBE to solve unfairness problem caused by NR-U node timing difference and increase the opportunity of channel access for NR-U nodes.

## Directional LBT Mechanism

In LTE-LAA, in order to guarantee fair coexistence with the incumbent Wi-Fi system in the unlicensed carrier, LBT (listen before talk) with energy detection via omni-directional antenna beam was introduced in LAA and eLAA.

In NR, SS/PBCH block transmission with beam has been supported to cover the whole NR cell. In NR-U, SS/PBCH block also needs to be supported in some NR-U deployment scenarios to enable initial access and/or measurement and so on. However, according to the regulation requirement of the unlicensed carrier, LBT needs to be performed before the actual SS/PBCH block transmission. If LBT in LTE-LAA is directly reused for beam-based SS/PBCH block transmission, it will lead to inaccurate CCA detection problems, e.g., high interference detected on the omni-directional beam could block the SS/PBCH block transmission on narrow directional beam even if the SS/PBCH block transmission does not interfere with the transmission of the other nodes in other beam directions. Based on this, it is necessary to study directional LBT mechanism for directional SS/PBCH block transmission in order to improve the probability of channel access and the accuracy of CCA detection.

Proposal 14: Directional LBT should at least be supported for beam-based SS/PBCH block transmission in order to avoid inaccurate CCA detection problems.

In order to better support directional LBT mechanism, some enhancements to current LTE-LAA LBT mechanism are worthy study to avoid interference. For example:

- Enhanced energy calculation method

In order to avoid the inaccuracy of CCA detection results caused by the mismatch between omni-directional LBT mode and directional SS/PBCH block transmission, some enhancements for energy calculation method need to be considered. For example, only the signal energy within the beam range can be used to determine the channel idle/busy condition.

- CCA detection threshold

For LTE-LAA, due to 20MHz channel and omni-directional channel detection mode, the CCA energy detection threshold is unique for the transmission nodes. For NR-U, if channel access uses directional LBT, the CCA energy detection threshold for the transmission node may change due to beamforming gain is introduced in EIRP of  calculation. Wherein, EIRP = Tx power + antenna gain + beamforming gain. Therefore, it is necessary to discuss and study the calculation method of EIRP and appropriate CCA detection threshold for directional LBT in NR-U.

Proposal 15: Directional LBT mechanism should be studied to improve the probability of successful channel access and the accuracy of CCA detection, e.g., enhanced calculation method of observed interference in the beam range, CCA detection threshold for directional transmission.

## Receiver assisted LBT

Receiver assisted LBT has been supported to enhance performance beyond the baseline LBT mechanism in RAN1 #92bis meeting. Based on this, we need to further study the hidden node problems in the directional LBT operation. Due to directional sensing operation, the hidden nodes problem will become more severe as shown in Figure 7. For example, both BS1 and BS2 adopt directional beam mode to transmit/receive. BS1 senses channel idle and transmits to UE1. BS2 in its beam direction may not be able to detect the energy of BS1 and hence transmits to UE2. However, both UE1 and UE2 are within the coverage of BS1 and BS2 transmit beam. Both UE1 and UE2 might fail to receive the respective transmissions due to inter-RAT interference. For this case, the directional LBT can be enhanced by target UE performing a directional LBT and sending a short indication signal. Therefore, receiver assistance should be supported to help mitigation potential hidden node issue.

Figure 7 Directional Tx/Rx

Proposal 16: For directional LBT manner, some receiver assistance methods (e.g., the receiver performs a directional LBT and sends out a short indication signal) should be supported to help mitigation of potential hidden node issue.

# Conclusion

In this contribution, we share our views on channel access procedure for NR-U. We have the following proposals:

Proposal 1: If CBG based HARQ-ACK feedback is configured for NR-U, ACK/NACK ratio Z can be counted/determined by one of the following methods:

- Alt-1: CBG level ACK/NACK is converted into TB level ACK/NACK. Wherein, the ACK/NACK ratio Z is of TB level.

- Alt-2: Redefinition ACK/NACK ratio Z in order to include TB level ACK/NACK and CBG level ACK/NACK.

Proposal 2: The reference slot for CWs adjustment should be defined as the first slot or the first and second slot of the latest DL/UL date burst for NR-U.

Proposal 3: For CWS adjustment for configured grant, the CWS adjustment scheme specified in AUL of R14 eLAA can be used as a starting point for study.

Proposal 4: Cat-4 LBT can be used for the following cases:

- Before the DRS multiplexed with unicast date are transmitted (agreed in RAN1#96).

- Before DRS burst set starts transmission and DRS burst set duration is larger than 1 ms or the total duty cycle of DRS transmission in DRS burst set period is larger than 1/20.

Proposal 5: Cat-2 LBT can be considered to be used in the following situations:

- Case 1: Within DRS burst set, if there is an empty symbol/gap between DRS bursts (half-slot).

- Case 2: Within DRS transmission window, if a new DRS burst set is initiated by gNB and its duration is smaller than or equal to 1 ms and the total duty cycle of DRS transmission in a DRS burst set period is not larger than 1/20, or if gNB uses Cat-4 LBT to detect channel busy all the time and only a few SSB candidate positions (e.g., 2 SSBs) remain before the ending of DRS transmission window..

- Case 3: Before DRS burst set that only occupies 1ms channel time, which is similar to LTE-LAA DRS.

- Case 4: If the adjacent two SSBs (DRS bursts (half-slot)) within DRS burst set is non-QCL’d relationship and there is a gap between non-QCL’d SSB.

- Preferably, directional LBT can be performed for beam-based SSB transmission within DRS burst.

- Case 5: for case where multiple sub-band case in the wideband operation or multiple carriers/BWPs case.

Proposal 6: NR-U support multiple switching points in a gNB initiated COT (notes: the methods can also be applied to CG operation).

- If gap of switching point >= 25 us, Cat 2 LBT is applied.

- If 16us < gap of switching point < 25 us, gNB ensures that this case does not happen.

- If gap of switching point = 16 us, some details can be found in Proposal 8.

Proposal 7: Cat-2 LBT should be indicated to UE when the gap between DL and UL is 16 us.

Proposal 8: If Cat-2 LBT with 16 us duration is supported, it is recommended that a new definition for Cat-2 LBT with 16 us duration can be introduced.

- Sensing duration consists of a idle duration Tf = 7 us immediately followed by a slot duration Tsl = 9 us. Wherein, 4us in slot duration Tsl is detection duration.

Proposal 9: LBT of LTE-LAA multiple CCs (e.g., Type A/B) manner can be used as starting point for research LBT for multiple carriers.

Proposal 10: Frequency reuse/multiplexing should be supported in NR-U and some methods of frequency reuse/multiplexing can be considered such as RE/PRB level of blank pattern method.

Proposal 11: For blank pattern scheme, RAN1 can send an LS to RAN4 in order to evaluate the feasibility of this scheme.

Proposal 12: Multiplexing/Frequency reuse with blank pattern schemes can achieve performance improvement compared to no-multiplexing/Frequency reuse.

Proposal 13: Directional LBT should at least be supported for beam-based SS/PBCH block transmission in order to avoid inaccurate CCA detection problems.

Proposal 14: Multiple CCAs methods can be considered for FBE to solve unfairness problem caused by NR-U node timing difference and increase the opportunity of NR-U nodes access channel.

Proposal 15: Directional LBT mechanism should be studied to improve the probability of successful channel access and the accuracy of CCA detection, e.g., enhanced calculation method of observed interference in the beam range, CCA detection threshold for directional transmission.

Proposal 16: For directional LBT manner, some receiver assistance methods (e.g., the receiver perform a directional LBT and send out a short indication signal) should be supported to help mitigation of potential hidden node issue.

# Reference

- 3GPP, RP-182878, New WID on NR-based access to unlicensed spectrum, Qualcomm, Dec, 2018, RAN #82

- Draft_Minutes_report_RAN1 #96_v1.0, February, 2019.

- Draft_Minutes_report_RAN1 #96bis, April, 2019.

- 3GPP TS 36.213, Physical layer procedures, V14.5.0, 2017-12

- 3GPP, R1-1812194, Coexistence and channel access for NR unlicensed band operations, Huawei, HiSilicon, RAN1#95, November, 2018

- 3GPP, R1-1813413, Channel access procedures for NR unlicensed, Qualcomm, RAN1#95, November, 2018