3GPP TSG RAN WG1 Meeting #97	R1-1906048

Reno, USA, May 13th –17th, 2019

Agenda Item:	7.2.2.2.5

Source:	Huawei, HiSilicon

Title:	NRU wideband BWP operation

Document for:	Discussion

# Introduction

In RAN#82, the WID [1] of NR-based Access to Unlicensed Spectrum was approved. One of the objective is to specify the wideband operation procedure in RAN1 and RAN4.  In this contribution, we clarify our understanding on the wideband operation mode with one serving cell with bandwidth > 20MHz at first. Supporting of dynamic transmission bandwidth in PUSCH (option 3) based on UE capability is further discussed. The impacts are also investigated on the PDCCH/PDSCH/PUCCH/PUSCH when supporting dynamic transmission bandwidth adaptation.  Based on the agreement in RAN1#96bis [2], we compare the different options for UE to detect gNB’s transmission in frequency domain. Finally the behavior of timer adjustment in BWP switch is discussed.

# Support of multiple wideband carriers

According to the WID [1] of NR-based Access to Unlicensed Spectrum, two modes of wideband operation are supported.

- Mode 1: Wideband operation (in integer multiples of 20MHz) for DL and UL for NR-U supported with multiple serving cells.

- Mode 2: Wideband operation (in integer multiples of 20MHz) for DL and UL for NR-U supported with one serving cell with bandwidth > 20MHz.

Mode 1 corresponds to carrier aggregation of multiple component carriers each with channel bandwidth of 20MHz, which is same as LTE LAA. During the discussion, there is confusion on the interpretation of mode 2 whether it allows multiple serving cells among which at least one of the serving cells is configured with carrier bandwidth larger than 20MHz. From our understanding, it is included in the supported mode 2. This mode is efficient for NR-U to make use of wider channel bandwidth with acceptable implementation complexity, such as reduced guard tones and control signaling overhead. In Figure 1, it shows potential aggregation of wide band component carriers according to the regulation in 5GHz [3]. On the other side, additional standard effort would be marginal from RAN1 perspective to support carrier aggregation among carriers with larger than 20MHz bandwidth, when both carrier aggregation among 20MHz carriers and single wideband carrier larger than 20MHz were supported.

Figure 1 channel bonding pattern in 5GHz

Observation 1: It is efficient to allow carrier aggregation among component carriers with bandwidth larger than 20MHz. The additional standard effort is marginal.

# Dynamic transmission bandwidth adaptation in UL

When free of narrow band operation of coexistence systems cannot be guaranteed, subband LBT performed in units of 20MHz is necessary for transmitter to avoid using the LBT bandwidth(s) already occupied. In RAN1 Ad Hoc meeting 1901 [4], both option 2 and option 3 were supported for PDSCH. In Figure 2, we further observed that system suffers even larger UPT loss in PUSCH when wideband transmission is allowed only if all LBT bandwidths succeed channel contention. The simulation assumption is attached in appendix.

Figure 2 DL and UL UPT comparison of NRU with and without dynamic transmission bandwidth adaptation when coexists with 20MHz coexisting systems

In RAN1 RAN1#96bis [2], two alternatives for PUSCH were updated as following and at least Alt 1 is supported.

- Alt. 1: UE transmits the PUSCH only if CCA is successful at UE in all LBT bandwidths of the scheduled PUSCH.

- Alt. 2: UE transmits the PUSCH in all or a subset of LBT bandwidths of the scheduled PUSCH for which CCA is successful at the UE.

Whether supporting Alt2 will depend on the feedback from RAN4 considering the on restrictions to the subset of LBT bandwidths, e.g., only contiguous LBT bandwidths allowed.

According to the LS reply on the DL wideband carrier operation [5], one wideband carrier when LBT is successful in a subset of the LBT sub-bands which are either contiguous (mode 2) or non-contiguous (mode 3) is feasible at least if PRBs within the guard band of two contiguous LBT sub-bands are not scheduled by gNB, considering WiFi-like requirements for in-carrier leakage. Thus, Alt 2 in PUSCH transmission should also be feasible for UE because only baseband processing are required to achieve the requirement. UE could determine to transmit with either Alt 1 or Alt 2 based on the gNB’s configuration or UE capability.  For example, gNB could indicate UE to use Alt1 via high layer signaling, if it do not have capability/intention to handle flexible BW reception of a PUSCH. When UE is notified that Alt 2 is capable at gNB side, UE could decide according to its own capability. For PUSCH spanning multiple LBT bandwidths in configured grant transmission, either Alt 1 or Alt 2 could be configured in the RRC signaling in the resource allocation.

Proposal 1: For UL transmissions in a serving cell with carrier bandwidth greater than LBT bandwidth, for the case where UE performs CCA before UL transmission, Alt 2 should also be supported. UE could determine to transmit with either Alt 1 or Alt 2 based on the gNB’s configuration and its own capability.

# Enhancements on partial BW transmission and receiving

If option 3 in DL or Alt 2 in UL are enabled, a single wideband BWP is activated for a UE within a component carrier, as illustrated in Figure 3. For example, if the LBT bandwidth is 20MHz in 5GHz band, the wideband BWP can consist of multiple LBT bandwidths of 20MHz. The actual transmission bandwidth is subject to subband LBT success, which will result in dynamic transmission bandwidth within this active wideband BWP.

Figure 3 Option 3 (in DL) or Alt 2 (in UL) for BWP operation in NR unlicensed carrier

## Impacts on PDSCH/PDCCH

Considering the hardware restriction, it usually takes some time for the gNB to schedule and prepare the transmission in advance before LBT. The LBT bandwidth(s) available for DL transmission might be different from those scheduled. Similar as the case in time domain, gNB should also avoid changing the pre-determined TBS at least at the beginning of the gNB’s COT, in order to avoid additional gap between end of LBT and start of transmission. For the rest of COT, gNB has enough time to re-schedule based on the acquired bandwidth. Several candidates can be considered for gNB to handle the mismatching between the scheduling and transmission at beginning of COT, similar as the discussion in time domain for flexible start point.

Alternative 1: gNB could schedule multiple PDSCHs, each of which is restricted within an LBT bandwidth at beginning of COT, e.g. 1st slot/mini-slot, as plotted in Figure 4. If LBT on certain LBT bandwidth fails, gNB just drop the PDSCH on that LBT bandwidth. Each PDSCHs can be scheduled by individual PDCCH on the corresponding LBT bandwidth. According to UE capability in NR R15, these PDSCH(s) should be scheduled to different UE respectively. NR-U gNB could also schedule 2 PDSCHs to the same UE on 2 LBT bandwidths, considering the latest agreement in multiple TRP/Panel transmission [4] is also applied to NR-U.

Alternative 2: gNB could schedule one PDSCH across multiple LBT bandwidths assuming all of them are available. As CBG based feedback and retransmission is supported in NR R15, gNB could only retransmit the CBG affected by LBT bandwidth(s) puncturing. Further enhancement can be considered to restrict mapping a CBG within the LBT bandwidth. For example, gNB could map CBG(s) to RE in frequency domain within LBT bandwidth at first, then in time domain then finally across LBT bandwidth(s). It could minimize the number of CBG affected due to LBT bandwidth(s) puncturing, as plotted in Figure 4.

Figure 4 PDSCH and PDCCH in single wideband BWP

Proposal 2: The following two alternatives can be considered for PDSCH scheduling and transmission at beginning of a gNB acquired COT.

- Alternative 1: scheduling multiple PDSCHs, each located within a LBT bandwidth. Do not transmit PDSCH(s) on the LBT bandwidth(s) where CCA fails.

- Alternative 2: scheduling PDSCH assuming all LBT bandwidths are available. Puncturing the REs on the LBT bandwidth(s) where CCA fails.

At beginning of a DL burst (1st slot/mini slot), a PDCCH should be restricted within a LBT bandwidth in order to avoid partial puncturing of a DCI. It is also beneficial for a UE to monitor multiple PDCCH searching spaces in different LBT bandwidths in case CCA fails on part of LBT bandwidths of a wideband BWP. It is straightforward to configure a UE with PDCCH search spaces associating with CORESET(s) in different LBT bandwidths and each CORESET limited within a LBT bandwidth. The number of configured CORESET (at most 3 CORESET in NR R15) might be increased considering the bandwidth of BWP and LBT bandwidth. The number of configured search spaces and blind detection could remain same as NR R15. Self-scheduling (PDCCH and PDSCH in the same LBT bandwidth) is also preferred considering UE cannot decoding PDSCH without PDCCH.  In the remaining of the DL burst, a UE can dynamically reduce PDCCH search spacing monitoring in frequency domain given the knowledge of transmitted LBT bandwidth(s).

Proposal 3: At least at beginning of a DL burst, UE should be configured to monitor PDCCH search spaces associating with CORESETs on different LBT bandwidths. Each CORESET should be within a LBT bandwidth.

- FFS: Maximum number of configured CORESETs and PDCCH search spaces

- FFS: Dynamic switching of PDCCH search spaces configuration in frequency domain in the COT

## Impacts on PUSCH/PUCCH

The mismatch between resource allocation in UL grant and LBT outcome becomes even more severe, as gNB has no idea about the LBT result to be performed by UE later.  Similar as PDSCH, two alternatives are possible.

Alternative 1: gNB schedules multiple PUSCH(s) each located within a LBT bandwidth based on PRB-based interlace design for one LBT bandwidth (e.g. 20MHz). UE does not transmit PUSCH on the LBT bandwidth(s) which fails LBT.

Alternative 2: gNB schedules single PUSCH based on PRB-based interlace design of the wideband BWP (e.g. 80MHz). UE punctures the PRB(s) on the LBT bandwidth(s) which fails LBT. gNB is able to schedule partial interlace corresponding to part of LBT bandwidths when UE do not have enough data to fill the whole wideband interlace. Further enhancement can be considered to restrict mapping a CBG within the LBT bandwidth.

Alternative 2 has the benefits of higher spectrum utilization (no guard tone) and less HARQ processes consumption compared with alternative 1. The detail interlace design can be referred to our companion tdoc [6]. gNB can identify the UL transmission bandwidth as well as of the start point of PUSCH by detecting the UL DMRS at beginning of slot(s).

Proposal 4: When dynamic transmission bandwidth adaption (Alt 2) is adopted to transmit PUSCH, gNB schedules single PUSCH based on PRB-based interlace design of the wideband BWP. UE punctures the PRB(s) on the LBT bandwidth(s) which fails LBT.

A PUCCH resource should be restricted within a LBT bandwidth in order to avoid partial puncturing of UCI due to LBT failure. When PUCCH format(s) of PRB-based interlace design is configured, it occupies PRB of a partial wideband interlace. Multiple PUCCH resource can be configured in different LBT bandwidths. UE can select one of the resource to feedback even if LBT failure occur on part of LBT bandwidths. Detail can refer to our companion tdoc [7].

Proposal 5: A PUCCH resource should be restricted within a LBT bandwidth.

## Contiguous or non-contiguous transmission in frequency domain

According to RAN4 response [5], one wideband carrier when LBT is successful in a subset of the LBT sub-bands which are either contiguous (mode 2) or non-contiguous (mode 3) is feasible at least if PRBs within the guard band of two contiguous LBT sub-bands are not scheduled by gNB, considering WiFi-like requirements for in-carrier leakage. However, some restrictions can still be considered which simplify UE implementation. For example, gNB could still transmit on multiple noncontiguous LBT bandwidths, each contiguous part corresponding to a PDSCH to a different UE as illustrated in Figure 5(a). In UL, UE may transmit on the maximum contiguous portion of the bandwidth where LBT succeeds as illustrated in Figure 5(b).

Figure 5 Contiguous transmission when LBT fails in “internal” LBT sub-band

According to ETSI regulation [3], Load Based Equipment (LBE) may use any combination/grouping of 20 MHz Operating Channels when Type-A multiple carrier channel access mechanism is used. Load Based Equipment that uses a combination/grouping of 20 MHz Operating Channels that is a subset of bonded 40 MHz, 80 MHz or 160 MHz channels, may transmit on any of the 20 MHz Operating Channels when Type-B multi carrier channel access mechanism is used. The channel bonding restriction is only applicable to LBE device use Type-B multi carrier channel access. As long as its configured BWP is a subset of bonded channels, LBE device can transmit on any of the LBT bandwidths where CCA is successful without any restriction.

Observation 2: There is no restriction on the occupied LBT bandwidths for LBE device using Type-A multi carrier channel access mechanism

Observation 3: LBE device using Type-B multi carrier channel access mechanism can transmit on any of the LBT bandwidths where CCA is successful without any restriction as long as its configured BWP is a subset of bonded 40MHz, 80MHz and 160MHz channels as defined in regulation.

# Indication of transmitted LBT bandwidths

In RAN1#96bis, it was agreed that.

- Support a mechanism for a UE to detect gNB is transmitting across

- Multiple carriers

- Multiple LBT bandwidths in a carrier.

- The following mechanisms are to be considered:

- Option 1: Explicit indication via PDCCH

  - FFS: The type of PDCCH (e.g., group common PDCCH or UE-specific PDCCH)

  - FFS: Signaling details of the indication

- Option 2: Explicit indication via selection of a PDCCH DM-RS sequence from a set of PDCCH DM-RS sequences

  - FFS: Details of the indication

- Option 3: Via UE implementation, i.e., implicit method based on NR-based signal such as DM-RS and/or corresponding PDCCH detection

  - FFS: Which signals/channels or combination of signals/channels could be used by the UE

Note: Above options are not mutually exclusive

For option 1, it is preferred to use group common PDCCH as it is expected to be detected by multiple UE in the serving cell(s). A specific field in a GC-DCI could be defined to indicate which LBT bandwidth(s) in a wideband carrier are transmitted together with other COT structure indication. If transmission on non-contiguous LBT bandwidth is supported, a bitmap with each bit corresponding to a LBT bandwidth could be used. If only transmission on contiguous LBT bandwidth is allowed, a compressed indication method could be used, e.g. RIV-like or an index to a table consisting of all allowed LBT bandwidth combination. gNB can transmit GC-PDCCH at beginning of a COT (pre-indication) and/or in pre-defined GC-PDCCH monitoring occasion(s) within a COT (post-indication). If gNB cannot adjust GC-DCI in GC-PDCCH at beginning of a COT immediately after LBT, a predefined value can be configured in advance which informs UE the actual transmitted LBT bandwidth information will be provided later. After UE receives another GC-PDCCH in the COT, the updated transmitted LBT bandwidth information can be applied in the rest of DL burst and for the HARQ process(es) not acknowledged before the latest received GC-PDCCH.  For option 2, it requires UE to blindly detect from a set of PDCCH DMRS sequences to acquire the corresponding transmitted LBT bandwidth(s). The reliability of DMRS detection and number of available DMRS sequences are quite challenging to achieve the functionality. For option 3, the detection reliability would be similar as option 1 if detection of PDCCH and its DMRS were adopted. However, the number of blind decoding at UE side will increase because it requires UE to detect the existence of PDCCH on each LBT bandwidth even when it is transmitted within the COT.

Proposal 6: Explicit indication via GC-PDCCH should be used by UE to detect gNB’s transmission across multiple carriers and/or multiple LBT bandwidths in a carrier.

In UL, if UE transmits the PUSCH in all or a subset of LBT bandwidths of the scheduled PUSCH for which CCA is successful at the UE, gNB can acquire the transmitted bandwidth from UE by detecting front loaded DMRS of PUSCH on each LBT bandwidth.

Proposal 7: gNB could acquire transmitted LBT bandwidth(s) of PUSCH(s) by detection of front loaded DMRS of PUSCH(s).

# Premature active BWP switching due to Inactivity Timer

In NR-U an LBT-based transmission of DCI in a non-default active BWP may be delayed because the wireless channel is occupied, or the DCI transmission may collide with another transmission from a hidden node on the wireless channel and may not be successfully decoded or even recognized by the UE. In such cases, the inactivity timer may expire before it is reset by the DCI reception which would cause the UE to prematurely decide that the DL BWP is no longer active and switch back to the default unpaired DL/UL BWPs. If such premature switching happens, the UE would stop monitoring the CORESETs in the deactivated DL BWP and would not be expected to provide any requested UCI feedback with resources allocated on the deactivated UL BWP.  Therefore, one or more of the following solutions can be adopted in NR-U:

- Larger maximum timer value: One simple solution is that the gNB configures the UE to have a larger maximum inactivity timer value when operating in an NR-U cell. For example, if a maximum inactivity timer value x is typically used with active BWPs in the licensed band, when the UE is active on a BWP on an unlicensed CC, it is configured with a sufficiently larger maximum timer value y>x to provision for potential DCI latency/loss due to the unlicensed channel occupancy. Although this solution is simple, it becomes less efficient as the channel becomes less occupied.

- UE pauses timer during UL LBT failure: The UE can use same inactivity timer maximum value across different BWPs/unlicensed CCs while dynamically accounting for the amount of time the medium access has been blocked on the respective BWP, at least as sensed by the UE. This may be achieved by the UE pausing its inactivity timer on an active BWP for the time duration over which the uplink LBT fails. The inactivity timer therefore does not count during the blocking time associated with an uplink LBT failure. In this way, a BWP on a busy channel with frequent uplink LBT failure and will be effectively associated with a greater inactivity timer value compared to a BWP on a less busy channel and experiencing less frequent uplink LBT failure. Also, the behavior of the inactivity timer on a clear (mostly unoccupied) BWP falls back naturally to the intended behavior of the inactivity timer on a licensed BWP.

Observation 4: Due to the potential DCI latency/loss of detection in the unlicensed band, switching of the single active BWP to the default BWP may occur due to premature expiry of the associated Inactivity Timer.

Proposal 8: To avoid switching of the active BWP due to premature expiry of the respective Inactivity Timer, the UE should pause the timer when the medium access is blocked, at least as sensed by the UE.

# Conclusions

In this contribution, we discussed wideband operation in NR-U mainly from RAN1 perspective. We had following observations and proposals.

Observation 1: It is efficient to allow carrier aggregation among component carriers with bandwidth larger than 20MHz. The additional standard effort is marginal.

Observation 2: There is no restriction on the occupied LBT bandwidths for LBE device using Type-A multi carrier channel access mechanism

Observation 3: LBE device using Type-B multi carrier channel access mechanism can transmit on any of the LBT bandwidths where CCA is successful without any restriction as long as its configured BWP is a subset of bonded 40MHz, 80MHz and 160MHz channels as defined in regulation.

Observation 4: Due to the potential DCI latency/loss of detection in the unlicensed band, switching of the single active BWP to the default BWP may occur due to premature expiry of the associated Inactivity Timer.

Proposal 1: For UL transmissions in a serving cell with carrier bandwidth greater than LBT bandwidth, for the case where UE performs CCA before UL transmission, Alt 2 should also be supported. UE could determine to transmit with either Alt 1 or Alt 2 based on the gNB’s configuration and its own capability.

Proposal 2: The following two alternatives can be considered for PDSCH scheduling and transmission at beginning of a gNB acquired COT.

- Alternative 1: scheduling multiple PDSCHs, each located within a LBT bandwidth. Do not transmit PDSCH(s) on the LBT bandwidth(s) where CCA fails.

- Alternative 2: scheduling PDSCH assuming all LBT bandwidths are available. Puncturing the REs on the LBT bandwidth(s) where CCA fails.

Proposal 3: At least at beginning of a DL burst, UE should be configured to monitor PDCCH search spaces associating with CORESETs on different LBT bandwidths. Each CORESET should be within a LBT bandwidth.

- FFS: Maximum number of configured CORESETs and PDCCH search spaces

- FFS: Dynamic switching of PDCCH search spaces configuration in frequency domain in the COT

Proposal 4: When dynamic transmission bandwidth adaption (Alt 2) is adopted to transmit PUSCH, gNB schedules single PUSCH based on PRB-based interlace design of the wideband BWP. UE punctures the PRB(s) on the LBT bandwidth(s) which fails LBT.

Proposal 5: A PUCCH resource should be restricted within a LBT bandwidth.

Proposal 6: Explicit indication via GC-PDCCH should be used by UE to detect gNB’s transmission across multiple carriers and/or multiple LBT bandwidths in a carrier.

Proposal 7: gNB could acquire transmitted LBT bandwidth(s) of PUSCH(s) by detection of front loaded DMRS of PUSCH(s).

Proposal 8: To avoid switching of the active BWP due to premature expiry of the respective Inactivity Timer, the UE should pause the timer when the medium access is blocked, at least as sensed by the UE.

# References

RP-182878, “New WID on NR-based Access to Unlicensed Spectrum”

Chairman's Notes RAN1#96bis

ETSI EN 301 893 V2.1.1 (2017-05)

Chairman's Notes RAN1 AdHoc 1901

R4-1905209, “LS reply on wideband carrier operation for NR-U”, Nokia

R1-1906043, “UL channels and signals in NR unlicensed band”, Huawei, Hisilicon

R1-1906046, “HARQ enhancement in NR unlicensed”, Huawei, Hisilicon

# Appendix

Table 1 simulation assumption of wideband operation

| Simulation scenario | Indoor Sub-7GHz, 3+3 TRPs with indoor mixed office model (a-b-a = 20-40-20) |
| --- | --- |
| Carrier Frequency | 5GHz |
| Number of users per operator | 5 per gNB per 20MHz |
| Channel bandwidth | NR-U: 80MHz 802.11ac: 20MHz per AP randomly distributed within NR-U’s operating bandwidth |
| Channel Model | NR InH Mixed Office model |
| BS/AP Tx Power | 23dBm (total across all TX antennas) |
| UE/STA Tx Power | 18dBm (total across all TX antennas) |
| BS/AP Antenna gain | 0 dBi |
| UE/STA Antenna gain | 0 dBi |
| BS/AP Noise Figure | 5dB |
| UE/STA Receiver Noise Figure | 9dB |
| Minimum received power from serving cell for UE dropping | -82dBm |
| UE receiver | MMSE-IRC as the baseline receiver |
| BS/AP antenna Array configuration | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = dV = 0.5 λ |
| UE/STA antenna Array configuration | Tx/Rx: (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1), dH = dV = 0.5 λ |
| Traffic model | DL/UL mixed traffic, FTP3 with packet size of 0.5Mbyts for both DL and UL traffic. |