3GPP TSG RAN WG1 #97                                                 R1-1906243

Reno, USA, May 13th - 17th, 2019

Source:         ZTE

Title:            Details and evaluation results on multi-TRP for URLLC

Agenda item:    7.2.8.5

Document for:	Discussion and Decision

Introduction

Based on the approved WID [1], multi-TRP techniques for URLLC requirement should be discussed in MIMO session. To justify the efficient transmission schemes based on simulation results, evaluation assumptions should be consistent among companies. In RAN1#94b meeting, many simulation assumptions for multi-TRP for eMBB were agreed. And a guidance for URLLC requirement is also agreed as follows [2]

Agreement:

- For URLLC multi-TRP/panel performance evaluation, choose a subset of evaluation scenarios/assumptions agreed in the URLLC agenda item

In order to facilitate further down-selection for one or more schemes in RAN1#96bis, schemes for multi-TRP based URLLC, scheduled by single DCI at least, are clarified via email discussion as follows:

- Scheme 1 (SDM):  n (n<=Ns) TCI states within the single slot, with overlapped time and frequency resource allocation

  - Scheme 1a:

    - Each transmission occasion is a layer or a set of layers of the same TB, with each layer or layer set is associated with one TCI and one set of DMRS port(s).

    - Single codeword with one RV is used across all spatial layers or layer sets. From the UE perspective, different coded bits are mapped to different layers or layer sets with the same mapping rule as in Rel-15.

  - Scheme 1b:

    - Each transmission occasion is a layer or a set of layers of the same TB, with each layer or layer set is associated with one TCI and one set of DMRS port(s).

    - Single codeword with one RV is used for each spatial layer or layer set. The RVs corresponding to each spatial layer or layer set can be the same or different.

    - FFS: codeword-to-layer mapping when total number of layers <= 4

  - Scheme 1c:

    - One transmission occasion is one layer of the same TB with one DMRS port associated with multiple TCI state indices, or one layer of the same TB with multiple DMRS ports associated with multiple TCI state indices one by one.

  - Applying different MCS/modulation orders for different layers or layer sets can be discussed.

- Scheme 2 (FDM): n (n<=Nf) TCI states within the single slot, with non-overlapped frequency resource allocation

  - Each non-overlapped frequency resource allocation is associated with one TCI state.

  - Same single/multiple DMRS port(s) are associated with all non-overlapped frequency resource allocations.

  - Scheme 2a:

    - Single codeword with one RV is used across full resource allocation. From UE perspective, the common RB mapping (codeword to layer mapping as in Rel-15) is applied across full resource allocation.

  - Scheme 2b:

    - Single codeword with one RV is used for each non-overlapped frequency resource allocation. The RVs corresponding to each non-overlapped frequency resource allocation can be the same or different.

  - Applying different MCS/modulation orders for different non-overlapped frequency resource allocations can be discussed.

  - Details of frequency resource allocation mechanism for FDM 2a/2b with regarding to allocation granularity, time domain allocation can be discussed.

- Scheme 3 (TDM): n (n<=Nt1) TCI states within the single slot, with non-overlapped time resource allocation

  - Each transmission occasion of the TB has one TCI and one RV with the time granularity of mini-slot.

  - All transmission occasion (s) within the slot use a common MCS with same single or multiple DMRS port(s).

  - RV/TCI state can be same or different among transmission occasions.

  - FFS channel estimation interpolation across mini-slots with the same TCI index

- Scheme 4 (TDM): n (n<=Nt2) TCI states with K (n<=K) different slots.

  - Each transmission occasion of the TB has one TCI and one RV.

  - All transmission occasion (s) across K slots use a common MCS with same single or multiple DMRS port(s)

  - RV/TCI state can be same or different among transmission occasions.

  - FFS channel estimation interpolation across slots with the same TCI index

  - Note that M-TRP/panel based URLLC schemes shall be compared in terms of improved reliability, efficiency, and specification impact.

  - Note: Support of number of layers per TRP may be discussed

In RAN1#96bis meeting, scheme 1a, 3 and 4 were supported for PDSCH reliability enhancement [3]

Agreement:

For multi-TRP based URLLC, scheduled by single DCI,

- Support scheme 1a as agreed in email discussion [96-NR-09]

  - FFS: Whether additional specification impact is necessary for URLLC

- On the support of schemes 2a, 2b

  - Select one of the following: support 2a only, support 2b only, support both 2a and 2b, support none

  - To facilitate further comparisons among 2a, 2b and baseline to understand technical benefits and use cases, consider both SLS and LLS simulation results

  - Specification impact, and UE complexity need to be considered as well.

  - Companies are encouraged to provide simulation results for LLS using at least the following parameters

    - Pathloss delta between two TRPs: 0dB, 3dB, 6dB

    - Details on blockage to be provided by each company if any (for example, the probability that one out of 2 links is blocked is 5% or 10% with 10dB blockage loss for the blocked link)

Based on the above agreements for URLLC, we provide our simulation assumptions and corresponding simulation results on multi-TRP transmission schemes for PDSCH reliability enhancement. The main motivation is to compare the performance of potential transmission schemes for URLLC. In addition, we provide our views for PDCCH, PUCCH and PUSCH reliability enhancements.

Discussion

Simulation results for PDSCH reliability enhancements

Scheme 2a vs. Scheme 2b

For FDM, two TCI states correspond to two non-overlapping frequency resource parts within single slot. Since there is no inter-TRP interference based on FDM repetition, the performance can be significantly increased. For scheme 2a, low code rate is achieved. For scheme 2b, UE can get combining gain at the receiver side. In order to verify the performance of these two schemes, we provide our LLS results in Figure 2.1-1, Figure 2.1-2 and Figure 2.1-3 for QPSK with code rate = 0.12/0.44 for scheme 2a and baseline. In the simulation, the code rate of scheme 2b is double of scheme 2a. Baseline is SFN with small delay CDD. For scheme 2a, single RV=0 is used. For scheme 2b, two RVs = 0 and 2 are used. For each TRP, four PRBs are allocated. In the blockage case, the probability that the transmission of second TRP is blocked is 5% with 10dB blockage loss.

(a) QPSK with code rate = 0.12 for Scheme 2a and baseline, 0.24 for Scheme 2b

(b) QPSK with code rate = 0.44 for Scheme 2a and baseline, 0.88 for Scheme 2b

Figure 2.1-1 BLER comparison of Scheme 2a, Scheme 2b and baseline (no blockage, pathloss delta between 2TRPs is 0dB)

Figure 2.1-2 BLER comparison of Scheme 2a, Scheme 2b and baseline (no blockage, pathloss delta between 2TRPs is 3dB, QPSK with code rate = 0.44 for Scheme 2a and baseline, 0.88 for Scheme 2b)

Figure 2.1-3 BLER comparison of Scheme 2a, Scheme 2b and baseline (blockage, pathloss delta between 2TRPs is 0dB, 5% blocked probability with 10dB blockage loss, QPSK with code rate = 0.44 for Scheme 2a and baseline, 0.88 for Scheme 2b)

Observation 1: Similar performance is observed for scheme 2a and 2b, and both of them outperform the baseline.

Single layer vs. Multiple layer per TRP

In Rel-15, only rank 1 PDSCH transmission is allowed for multi-slot scheduling because only small packet sizes of URLLC traffic were considered. However, many kinds of scenarios are considered in Rel-16, such as Factory automation, AR/VR, Transport Industry. The maximum packet size is over thousands of bytes. To make transmission more efficient, the rank restriction should be relaxed. Furthermore, cross polarization is the most typical antenna structure in the real deployment, more than rank 1 transmission is more suitable.

Here we provide the some simulation results to justify the benefit of more rank transmission per TRP. In the simulation, we compare rank 1 per TRP transmission with rank 2 per TRP transmission for SDM scheme 1a (Figure 2.1-4). More simulation assumptions can be found in section 3. As we can see, rank 2 per TRP can obviously introduce higher reliability because of lower code rate.

(a) QPSK, code rate = 0.2 and 0.1 respectively   (b) 16QAM, code rate = 0.2 and 0.1 respectively

Figure 2.1-4 BLER comparison of one layer per TRP and two layers per TRP

Observation 2: For SDM scheme 1a, rank 2 transmission for each TRP outperforms rank 1 transmission.

For PDCCH, PUCCH and PUSCH reliability enhancements

In order to improve PDCCH reliability, the coordinated TRPs can jointly transmit the same DCI as shown in Figure 2.2(a) and the diversity gain can be achieved. Alternatively, the coordinated TRPs can transmit independent DCIs with the same DCI content as shown in Figure 2.2(b). In this case, correct detection of only one of two DCIs is enough. In order to further improve the robustness of PDCCH detection, it is better to inform UE which DCIs are repeated, then UE can do soft combining between two repeated DCIs. If two DCIs from two TRPs are used to schedule two repeated PDSCH with different DMRS port group, the same DCI bits can be used to convey different DCI content by supporting RRC configuration on different interpretation on DCI content e.g. the order of the codepoints of the DMRS table can be configured.

(a) Same DCI                    (b) independent DCI with the same DCI content

Figure 2.2 Multiple TRPs transmit PDCCH

Proposal 1: Support PDCCH reliability enhancement.

PUCCH and PUSCH repetition is supported in Rel-15. However, the same beam is used for multiple repetitions. In high frequency bands, if blockage exists in the beam direction between UE and gNB, all repetitions cannot be received by gNB. One straightforward way is to use different spatial relations for those multiple repetitions in order to get beam diversity gain. This scheme is similar with TDM repetition (scheme 3 or 4 in section 1) based on multiple TRP transmission for PDSCH.

Proposal 2: Beam diversity scheme is supported for PUSCH/PUCCH repetitions.

Simulation assumptions

In this section, we provide our LLS assumptions in one table for performance comparison of Scheme 2a/2b, Single layer per TRP and Multiple layer per TRP.

Table 3-1 LLS assumptions for URLLC multi-TRP performance evaluation

| Parameters |  |
| --- | --- |
| Carrier frequency for evaluation | 4GHz |
| Channel model | CDL-C (delay spread: 300ns) as described in 38.901 Two hundred meters distance between two TRPs |
| UE speed | 3 km/h for power distribution and Rel-15 enabled use case; |
| BS antenna configuration | 2 Tx antenna ports: (M, N, P, Mg, Ng; Mp, Np) = (8, 1, 2, 1, 1; 1, 2) |
| UE antenna configuration | 4 Rx antenna ports |
| The number of PRBs | 8 |
| Modulation | Scheme 2a: QPSK with code rate = 0.12, 0.44 Single layer per TRP: QPSK, 16 QAM with code rate = 0.1 |
| System bandwidth | 40 MHz |
| Sub-carrier spacing | 30 kHz |
| Channel estimation | Practical |
| Receiver type | MMSE |
| Inter-TRP frequency (time) offsets | 0 Hz |
| Baseline scheme | SFN with CDD, cp/2 delay between two TRPs |

Conclusions

In this contribution, we provide the simulation assumptions for URLLC multi-TRP evaluation for LLS.

Observation 1: Similar performance is observed for scheme 2a and 2b, and both of them outperform the baseline.

Observation 2: For SDM scheme 1a, rank 2 transmission for each TRP outperforms rank 1 transmission.

Proposal 1: Support PDCCH reliability enhancement.

Proposal 2: Beam diversity scheme is supported for PUSCH/PUCCH repetitions.

References

- RP-182318, Revised WID: Enhancements on MIMO for NR , Samsung

- Chairman notes of RAN1#94bis meeting

- Chairman notes of RAN1#96bis meeting