3GPP TSG RAN WG1 Meeting #97                                                                         R1-1906456

Reno, USA, May 13th – 17th, 2019

Source:					ZTE, Sanechips

Title:							Discussion on OTA timing mechanism

Agenda item:			7.2.3.4

Document for:     Discussion and Decision

# Introduction

This contribution further discusses OTA based case-1 timing on IAB backhaul, based on the following agreements made in earlier RAN1 meetings.

Agreements [RAN1 #96]:

- T_delta is indicated by a parent to the child node independently from the existing Rel.15 TA indication from the parent node used to set the UL Tx timing of the child IAB node’s MT

  - T_delta is updated on an aperiodic basis determined by the parent node

  - The child IAB node should trigger its DL TX timing adjustment by TA/2 + T_delta after it receives the timing offset T_delta indication from its parent node, if it is using OTA Timing Case 1 to obtain its DL timing.

    - FFS: behavior if TA/2 + T_delta results in an effective negative timing offset

    - FFS: delay between receiving T_delta and application of T_delta at the child node

  - Separate value ranges/granularities may be considered for T_delta in FR1 and T_delta in FR2

# Further discussions

It is agreed in earlier RAN1 meetings that the DL Tx timing of an IAB node is set to (TA/2 + T_delta) ahead of its DL Rx timing, which is the DL Tx timing of the parent delayed by one-way propagation delay.

Denote the following for further timing analysis in this paper:

- tDL-Tx,pt: the DL-Tx timing at parent node at time t;

- tDL-Rxt: the detected DL-Rx timing at IAB node at time t;

- tUL-Txt: the UL-Tx reference timing at IAB node at time t, satisfying tUL-Txt=tDL-Rxt-NTA+NTA,offsetTc according to [7.1, TS38.133];

- tUL-Txt: the actual UL-Tx timing at IAB node at time t, satisfying tUL-Txt-tUL-Txt≤Te according to [7.1, TS38.133];

- tUL-Rx,pt: the detected UL-Rx timing at parent node at time t;

- tDL-Txt: the DL-Tx timing at IAB node at time t.

Per definitions of TA and T_delta agreed in earlier meetings, TAta≜tDL-Rxta-tUL-Txta, and Tdeltatd≜-12tDL-Tx,ptd-tUL-Rx,ptd, where TA and T_delta are measured by IAB node and parent node at time of ta and td, respectively. In addition, it is a fair assumption that the IAB node measures TA once it applies the received T_delta, which makes the TA information mostly recent, that is to say, td < ta. Given above, the DL-Tx timing at the IAB node that is adjusted at time tn(=ta) is

tDL-Txtn≜tDL-Rxtn-TAta2+Tdeltatd

=tDL-Rxta-12tDL-Rxta-tUL-Txta-tDL-Tx,ptd+tUL-Rx,ptd                            (1)

=tDL-Tx,ptd+12tDL-Rxta+tUL-Txta-tDL-Tx,ptd-tUL-Rx,ptd

Assume both DL-Tx timing at the parent and DL-Rx timing at the IAB node are stable during the procedure of adjusting DL-Tx timing of IAB node, the DL-Tx timing difference between two adjacent hops is given by

εDL-Tx≜tDL-Tx-tDL-Tx,p=12tDL-Rx-tDL-Tx,p-tUL-Rx,ptd-tUL-Txta                              (2)

In (2), tDL-Rx-tDL-Tx,p corresponds to an estimation of one-way propagation delay on backhaul DL (Tp,DL), and tUL-Rx,ptd-tUL-Txta corresponds to an estimation of one-way propagation delay on backhaul UL (Tp,UL). Even though εDL-Tx is inevitably a random value, it is desirable to make its mean as much close to zero as possible. There can be two factors that may cause EεDL-Tx  to be biased from zero.

- The channel power delay profiles for the strongest path are sufficiently different between backhaul DL and backhaul UL, which may only possibly happen in a FDD spectrum. However, this issue breaks the fundamental assumption of OTA-based case-1 timing mechanism which is based on (TA-Tg)/2 adjustment, and therefore is suggested not to be handled by the mechanism itself from specification perspective.

- Even with the same channel power delay profile between backhaul DL and backhaul UL, tUL-Rx,ptd-tUL-Txta can still be an incorrect representation of one-way propagation delay when tUL-Rx,ptd and tUL-Txta are out-of-sync, i.e., tUL-Rx,ptd is driven by a UL-Tx timing differently from tUL-Txta, which may happen when within interval of [td,ta]:

  - There is another TA command applied to IAB node’s UL Tx timing. According to [4], this can be avoided if the parent node knows when, as the latest, the IAB node should apply the T_delta to its DL-Tx timing adjustment.

  - The IAB node autonomously adjusts its UL Tx timing by a very small time amount according to [7.1, TS38.133]. Because this adjustment amount is small (up to 5.5*64*Tc ~ 0.2µs per one adjustment and per aggregated adjustment in 200ms) and can be either positive or negative in somehow the random sense, it can be fairly considered as a timing error within tUL-Txta or tUL-Rx,ptd.

T_delta signalling and time for applying T_delta

It is mentioned above to be beneficial to allow the parent node knowing when the IAB node applies the T_delta to its DL-Tx timing adjustment, or equivalently knowing the delay between receiving the T_delta notification from the parent node and applying T_delta. In addition, because quite some UE/MT PHY layer behaviors count on the timing integrity within radio frame, it is better to restrict the DL-Tx timing adjustment at the radio frame boundary.

Proposal 1: In OTA-based case-1 timing alignment, the IAB node adjusts its DL-Tx timing at the starting of DL radio frame (n+k) after receiving the T_delta notification from parent in radio frame n. FFS on k.

In order to support above timing relation from frame n to frame n+k, it is better to use MAC-CE to deliver T_delta, because using RRC to carry T_delta will make it difficult to fulfill the k-frame delay given the processing time of RRC signaling in higher layer is usually not well specified.

Proposal 2: To use MAC-CE to deliver T_delta.

Case-1 timing error propagation in multi-hop topology

Following up equation (2), the DL-Tx timing error variation of the IAB node, σDLTX, can be decomposed as

σDLTX=σDLTX,p+σparent+σQ+σIABOTA basedGNSS timing errorGNSS based                               (3)

where for OTA-based timing alignment,

- σDLTX,p is the DL-Tx timing error at the parent node;

- σparent counts for the timing errors occurring at the parent and relating to T_delta measurement;

- σQ represents the quantization error for signalling T_delta;

- σIAB is the timing errors occurring at IAB node and relating to TA, including measurement error for tDL-Rx and UL-Tx timing error as mentioned above.

In a multi-hop IAB network, it is beneficial, for timing management purpose, to label each IAB node with certain timing reliability metrics. There are basically two options in the past RAN1 discussion:

- Option-1: Each IAB node has the ability to derive and to indicate to other IAB node its own σDLTX. In this option, the two IAB nodes on the same hop order (but not necessarily with the same parent) may have different σDLTX, even if no IAB node in topology tree relies on GNSS for timing alignment. The option-1 may or may not need the signaling support of indication of σparent. If there is no signaling designed for σparent, it is assumed RAN4 would have certain measurement accuracy requirement upon T_delta, which can be commonly used by IAB node in the timing error derivation. If the signaling of σparent is desirable, it can be carried in RRC to represent the long-term statistics of timing error variance, instead of associating with certain specific T_delta delivered in MAC-CE.

- Option-2: Each IAB node does not explicitly associated with a quantity of σDLTX. The two IAB nodes on the same hop order (but not necessarily with the same parent) counted from certain nodes relying on GNSS are considered to have the same DL-Tx timing accuracy. In this case, the hop order from a node relying on GNSS timing, rather than σDLTX, can be used to implicitly indicate the DL-Tx timing accuracy.

From signaling perspective, the two options can be similar: the IAB nodes need to communicate for σDLTX in Option 1 and a specific hop order in Option 2 (due to a need for an IAB node equipped with GPS to temporarily lose GPS timing functionality). On the other hand, Option-2 may rely on RAN4-defined single-hop timing requirements, which represent the worst-case timing accuracy, to derive multi-hop timing information. In contrast, Option-1 has following advantages:

- To allow node-specific timing error indication which can be more accurate than RAN4 worst-case requirement.

- To give direct quantitative indication of whether the IAB node, when attaching to its parent, can still fulfill the network synchronization requirement (e.g., within 3µs error range).

- To provide better estimation about DL-Tx timing stability.

Proposal 3: It is supported that the IAB node derives and indicates to other IAB node its DL-Tx timing error range.

It should be noted that the following steps relating to equation (3) can be based on implementation and therefore not necessarily specified.

- How the parent determines σparent and how the IAB node determines σIAB;

- How the IAB node derives its σDLTX based on σDLTX of its parent, σparent and σQ, i.e., the equation (3) itself, at least for single parent case.

# Conclusions

Based on the discussion, we have the following observation and proposals:

Proposal 1: In OTA-based case-1 timing alignment, the IAB node adjusts its DL-Tx timing at the starting of DL radio frame (n+k) after receiving the T_delta notification from parent in radio frame n. FFS on k.

Proposal 2: To use MAC-CE to deliver T_delta.

Proposal 3: It is supported that the IAB node derives and indicates to other IAB node its DL-Tx timing error range.

# References

3GPP TR 38.874, v16.0.0

TS 38.133, v15.4.0

TS 38.901, v15.0.0

R1-1904813, Discussion on OTA timing mechanism, ZTE, Sanechips