3GPP TSG RAN WG1 #97                                        R1-1906486

Reno, USA, May 13th - 17th, 2019

Source:	OPPO

Title:	Channel access procedures for NR-U

Agenda Item:	7.2.2.2.1

Document for:	Discussion and Decision

Introduction

In RAN1 meeting #96bis, the following agreements were achieved:

Agreement:

The following agreement from the SI is updated as shown:

Table 7.2.1.3.1-1: Channel access schemes for initiating a COT by gNB as LBE device

|  | Cat 2 LBT | Cat 4 LBT |
| --- | --- | --- |
| DRS alone or multiplexed with non-unicast data (e.g. OSI, paging, RAR) | When the DRS duty cycle ≤1/20, and the total duration is up to 1 ms: 25 µs Cat 2 LBT is used (as in LAA) | When DRS duty cycle is > 1/20, or total duration > 1 ms Cat4 with any channel access priority class value can be used |
| DRS multiplexed with unicast data | N/A except for the cases discussed in the Note below | Channel access priority class is selected according to the multiplexed data |
| PDCCH and PDSCH | N/A except for the cases discussed in the Note below | Channel access priority class is selected according to the multiplexed data |

Agreement:

For a UCI-only transmission on PUSCH in a channel occupancy initiated by the UE, Cat4 with lowest channel access priority class value can be used by the UE

Agreement:

For LBT by a UE prior to transmission of a UL burst within a gNB-initiated channel occupancy as LBE device, for gap durations shorter than 25 microseconds, choose one of the following alternatives

- Alt 1: Cat 2 LBT can be indicated (FFS: explicit and/or implicit) to the UE if the gap is 16 microseconds (allowing for implementation tolerances)

- Alt 2: Cat 2 LBT is not indicated to the UE for gaps less than 25 microseconds

- Notes (applicable to both alternatives):

  - This means that the gNB ensures that gaps between 16 and 25 microseconds do not occur

  - This doesn’t change the previous agreement for Cat 1 and Cat 2 LBT for gaps of 16 microseconds or less

- FFS: Conditions on channel occupancy after a Cat. 1 or Cat. 2 LBT after a gap of 16 microseconds or less

This contribution mainly discusses channel access procedures for NR-U, including LBT mechanism determination for DL and UL channels and signals, CWS adjustments, and LBT for wide-band operation. Discussion of COT sharing for configured grant uplink can be found in our companion contribution [2].

LBT mechanisms for DL and UL

LBT for DL channels and signals

LBT mechanisms identified for gNB as LBE device is shown in Table 1 according to the updated agreements in RAN1 meeting #96bis.

Table 1 Channel access schemes for gNB as LBE device

|  | Cat 2 LBT | Cat 4 LBT |
| --- | --- | --- |
| DRS alone or multiplexed with non-unicast data (e.g. OSI, paging, RAR) | when the DRS duty cycle ≤1/20, and the total duration is up to 1 ms: 25 µs Cat 2 LBT is used (as in LAA) | When DRS duty cycle is > 1/20, or total duration > 1 ms Cat4 with any channel access priority class value can be used |
| DRS multiplexed with unicast data | N/A except for the cases discussed in the Note below | Channel access priority class is selected according to the multiplexed data |
| PDCCH and PDSCH | N/A except for the cases discussed in the Note below | Channel access priority class is selected according to the multiplexed data |

Gaps for multiple switching points within a gNB-initiated COT is allowed. In RAN1 Ad-Hoc Meeting 1901, the following agreement regarding gap creation is achieved.

Agreement:

- A gap (DLUL, ULUL, or UL DL) of a specific duration is created using one or more of:

  - Timing Advance

  - CP extension

    - max value of not more than one OFDM symbol

  - Shortening of DL or UL transmission duration by one or more OFDM-symbol(s) by puncturing or rate matching

  - Note: the mechanisms applied in each case may be different for different SCSs

- FFS: how to signal the way of creating the gap to the UEs

In RAN1 meeting #96bis, it was clarified that the gNB can ensure that gaps between 16 and 25 microseconds do not occur, which means the UE may assume there were two types of gaps within a shared gNB’s COT, one type is gaps with duration larger than or equal to 25 microseconds, the other type is gaps with duration smaller than or equal to 16 microseconds. For the second type of gaps, two alternatives were discussed in the meeting:

- Alt 1: Cat 2 LBT can be indicated (FFS: explicit and/or implicit) to the UE if the gap is 16 microseconds (allowing for implementation tolerances)

- Alt 2: Cat 2 LBT is not indicated to the UE for gaps less than 25 microseconds

According to previous agreement, for gaps smaller than or equal to 16 microseconds within a gNB shared COT, UE can immediately transmit channels/signals without performing LBT. Therefore, Alt 2 that Cat 2 LBT is not indicated to the UE for gaps less than 25 microseconds is preferred.

Proposal 1: Alt 2 that Cat 2 LBT is not indicated to the UE for gaps less than 25 microseconds is supported.

In addition, UE may use no-LBT corresponding to gaps less than or equal to 16 microseconds or one-shot LBT corresponding to gaps larger than or equal to 25 microseconds before transmission occurred within a gNB-initiated COT. Therefore, gNB should indicate which LBT type is used for UE to determine proper channel access procedure. For example, if gNB schedules a PDSCH transmission with its corresponding HARQ-ACK transmitted on a PUCCH resource in the same COT, gNB should indicate no-LBT or one-shot LBT type of the PUCCH resource to UE.

Proposal 2: no LBT or one-shot LBT should be indicated to UE for UE to determine proper channel access procedure within a shared gNB COT.

LBT for UL channels and signals

LBT mechanisms identified during SI phase for UE initiates a COT is shown in Table 2.

Table 2 Channel access schemes for initiating a COT by UE as LBE device

|  | Cat 2 LBT | Cat 4 LBT |
| --- | --- | --- |
| PUSCH (including at least UL-SCH with user plane data) | N/A except for the cases discussed in Note 2 below | Channel access priority class is selected according to the data |
| SRS-only | N/A | Cat4 with lowest channel access priority class value (as in LTE eLAA) |
| RACH-only | (see Note 2) | Cat4 with lowest channel access priority class value |
| PUCCH-only | (see Note 2) | Cat4 with lowest channel access priority class value |

Note 1: If the COT includes multiple signals/channels with different channel access categories / priority classes, the highest channel access priority class value and highest channel access category among the channel access priority classes and channel access categories corresponding to the multiple signals/channels applies.

Note 2: Applicability of a channel access scheme other than Cat 4 for the following signals / channels have been discussed and details are to be determined when the specifications are developed:

-	UL control information including UCI only on PUSCH, e.g. HARQ-ACK, Scheduling Request, and Channel State Information

-	Random Access

Both contention-based RACH procedure and contention-free RACH procedure should be supported for NR-U. For contention-free RACH procedure, the preamble transmission at UE side is triggered by a PDCCH order, so the corresponding LBT mechanism could be indicated through the PDCCH order to give gNB more flexibility, for example, according to the event triggering PRACH transmission, to determine UE’s channel access priority.

Proposal 3: Introduce prioritized LBT mechanisms for contention-free PRACH transmission through PDCCH order.

CWS adjustment procedure

CWS adjustment for PDSCH

In NR-U, the HARQ-ACK corresponding to PDSCH can be transmitted either by licensed carrier or by unlicensed carrier. For HARQ-ACK transmitted via licensed carrier, LAA-LTE rules can be reused. For HARQ-ACK transmitted via unlicensed carrier, the latest available HARQ-ACK values before gNB performing Cat-4 LBT should be used for CWS adjustment, as illustrated in Figure 1. Considering UE would not transmit HARQ-ACKs if the corresponding LBT fails, gNB should detect the presence of the HARQ-ACK values and only use the present HARQ-ACK values for CWS adjustment.

Proposal 4: For HARQ-ACK transmitted via unlicensed carrier, the latest available HARQ-ACK values before gNB performing Cat-4 LBT should be used for CWS adjustment.

Figure 1: Illustration of CWS adjustment based on HARQ-ACKs from unlicensed CC

Similar to LAA-LTE, Cat-4 LBT with different channel access priorities should be reused for PDSCH transmission in NR-U operation. CWS (contention window size) adjustment based on HARQ-ACK feedback for Cat-4 LBT should be considered. Unlike LAA-LTE only support TB based HARQ-ACK feedback, CBG based HARQ-ACK feedback is introduced. Correspondingly, CWS adjustment should be enhanced.

For CBG-based HARQ-ACK operation, each CBG-based HARQ-ACK value should be considered separately when calculating the NACK ratio for CWS adjustment. If some CBGs of the reference PDSCH are not transmitted, for example, for those CBGs which are punctured due to LBT fails, the corresponding CBG-based HARQ-ACK values should not be used for CWS adjustment.

Proposal 5: CBG-based HARQ-ACK values should be considered separately for CWS adjustment for PDSCH transmission.

CWS adjustment for PUSCH

For LAA-LTE, CWS adjustment is based on the NDI indication of the reference HARQ process. For CBG-based HARQ-ACK operation for PUSCH transmission, CBG-based HARQ-ACK values should be interpreted as NDI indication of the reference HARQ process. If the NDI is indicated per CBG, each CBG-based HARQ-ACK value should be considered separately to determine the NDI per CBG. If the NDI is indicated per TB, the NDI indicates new data if all the CBG-based HARQ-ACK values of the TB are ACK, otherwise, the NDI indicates retransmission. The interpreted NDI indication can be used for CWS adjustment for PUSCH transmission.

Proposal 6: CBG-based HARQ-ACK values should be interpreted as NDI indication for CWS adjustment for PUSCH transmission.

CWS adjustment for wideband

For wideband operation, CWS adjustment for one bandwidth should base on the available HARQ-ACK values corresponding to the latest PDSCH transmission within the same bandwidth. Figure 2 gives an example.

Figure 2: Illustration of CWS adjustment for wideband

When adjusting CWS for wideband LBT, the available HARQ-ACK values corresponding to PDSCH transmission on the reference subframes of the latest DL transmission burst within the same wideband should be considered.

When adjusting CWS for subband LBT, if the last DL burst is wideband transmission with CBG based feedback, the available CBG based HARQ-ACK values corresponding to the CBGs transmitted on the same subband should be used.

When adjusting CWS for subband LBT, if the last DL burst is wideband transmission with TB based feedback, and the second last DL burst is subband transmission or wideband transmission with CBG based feedback, HARQ-ACK values corresponding to the second last DL burst on the same subband should be used.

Proposal 7: CWS adjustment for one bandwidth should base on the available HARQ-ACK values corresponding to the latest PDSCH transmission within the same bandwidth.

LBT for wide-band operation

The following options for BWP-based operation within a carrier with bandwidth larger than 20 MHz are supported.

- Option 2: Multiple BWPs can be configured, single BWP activated, gNB transmits PDSCH or UE transmits PUSCH on a single BWP if CCA is successful at UE for the whole BWP

- Option 3: Multiple BWPs can be configured, single BWP activated, gNB transmits PDSCH or UE transmits PUSCH on parts or whole of single BWP where CCA is successful at UE

In LTE-LAA, Type A and Type B procedures are specified for multiple-carrier sensing, where Type A requires eNB perform Cat-4 LBT on each carrier and each carrier should follow a single-carrier Cat-4 LBT procedure, Type B requires eNB perform Cat-4 LBT procedure on one single-carrier of the multiple carriers and perform one-shot LBT on all the carriers before the transmission. Both Type A and Type B can be supported as candidate LBT schemes for wide-band operation.

In addition, wideband LBT that gNB performs a Cat-4 LBT procedure on a BWP with bandwidth larger than 20MHz has been discussed during the SI phase. Such kind of wideband LBT can be supported at both gNB and UE side if the wideband is operated in option 2, in which case the transmission is allowed only after the whole wideband is clear.

On the other hand, if one UE is configured to monitor PDCCH in one active BWP with smaller bandwidth while it is scheduled to receive PDSCH in another active BWP with large bandwidth, how to perform LBT in this case should also be studied.

Proposal 8: Wideband LBT should be supported for wide-band operation option 2 for a single carrier.

Conclusions

In this contribution, channel access procedures for NR-U, including LBT mechanism determination for DL and UL channels and signals, CWS adjustments, and LBT for wide-band operation. The following proposals are made.

Proposal 1: Alt 2 that Cat 2 LBT is not indicated to the UE for gaps less than 25 microseconds is supported.

Proposal 2: no LBT or one-shot LBT should be indicated to UE for UE to determine proper channel access procedure within a shared gNB COT.

Proposal 3: Introduce prioritized LBT mechanisms for contention-free PRACH transmission through PDCCH order.

Proposal 4: For HARQ-ACK transmitted via unlicensed carrier, the latest available HARQ-ACK values before gNB performing Cat-4 LBT should be used for CWS adjustment.

Proposal 5: CBG-based HARQ-ACK values should be considered separately for CWS adjustment for PDSCH transmission.

Proposal 6: CBG-based HARQ-ACK values should be interpreted as NDI indication for CWS adjustment for PUSCH transmission.

Proposal 7: CWS adjustment for one bandwidth should base on the available HARQ-ACK values corresponding to the latest PDSCH transmission within the same bandwidth.

Proposal 8: Wideband LBT should be supported for wide-band operation option 2 for a single carrier.

References

- R1-1904894, “Channel access procedures for NR-U”, OPPO.

- R1-1906489, “On configured grant for NR-U”, OPPO