3GPP TSG RAN WG1 Meeting #97                              R1-1906587

Reno, Nevada, 13th – 17th, May 2019

Source: MediaTek Inc.

Title: Results and Considerations of eMBB Spectral Efficiency in Indoor Hotspot of

IMT2020 Self-Evaluation

Agenda Item: 7.2.7

Document for: Discussion

- Introduction

The IMT2020 self-evaluation results on FR1 eMBB spectral efficiency are updated at RAN#81 in Sept. 2018 in order to deliver the submission to ITU-R WP 5D#31 in Oct. 2018. The SI continues updating the evaluation results and aims at providing the final submission at RAN#84, and then to ITU-R WP 5D#32 in July 2019 [1]. In this contribution, the evaluation results and considerations of eMBB spectral efficiency in the fields of FR2 indoor hotspot are provided.

- Considerations of Evaluation Methodology

  - NR Precoding Matrix

The UE with multiple panels in its antenna configurations is used to provide more spatial diversities to mitigate the effect of severe path loss in mmWave scenarios. However, the NR codebook only supports one-panel precoding matrixes. To keep the properties of uniform linear array in the DFT-based precoding matrix, we have to apply this matrix to one of the multiple panels to avoid the problem of beam aliasing. Beam aliasing is caused by an antenna structure with non-half wavelength distance between two antenna elements, and it will cause beamforming inefficiency and more interference to other radiation beams. Therefore, we have to select an antenna panel with the corresponding precoding matrix to maintain a radiation beam with desired directionality to compete the heavy path loss in mmWave environments.

  - Panel Selection

Panel selection is an important mechanism to optimize the utilization of resources, including power allocation and the transport blocks scheduled to the antenna ports. It is inefficient in system utilization if we allocate power and transport blocks onto an improper panel, with which radiation beam may not reach the receiver side or suffer from heavier path loss. Hence, we propose to select a panel in terms of the RSRP of SRS or channel capacity.

Proposal 1: Panel selection should be implemented in FR2 self evaluations to correctly elaborate the capability of NR precoding matrix.

- Evaluation Results

The evaluation results of eMBB spectral efficiency in FR2 InH environments are provided in Table 3.1. In the DL cases, two carrier frequencies, 30 and 70GHz, are evaluated, and both the results can meet the ITU requirements. In the UL cases, the average and 5th percentile spectral efficiency also meet the ITU requirements. The evaluation assumptions are referred to [2] and the details can be found in Table 5-1 and Table 5-2 in the Appendix below.

Table 3.1 FDD DL spectral efficiency evaluation in different eMBB scenarios

| Test environment | Evaluation configuration | Average spectral efficiency (bps/Hz/TRxP) |  | 5th percentile spectral efficiency (bps/Hz) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | ITU Req. | Value | ITU Req. | Value |
| DL Indoor Hotspot | Configuration B 30GHz | 9 | 12.69 | 0.3 | 0.408 |
| DL Indoor Hotspot | Configuration C 70GHz | 9 | 12.869 | 0.3 | 0.471 |
| UL Indoor Hotspot | Configuration B 30GHz | 6.75 | 10.17 | 0.21 | 0.405 |

Observation 1: NR can fulfill the spectral efficiency requirements in FR2 indoor hotspot scenarios.

- Conclusion

In this document, the evaluation results of eMBB spectral efficiency for NR indoor hotspot cases in FR2 are provided. Based on the evaluation results, the following observation and proposal are summarized.

Observation 1: NR can fulfill the spectral efficiency requirements in FR2 indoor hotspot scenarios

Proposal 1: Panel selection should be implemented in FR2 self evaluations to correctly elaborate the capability of NR precoding matrix.

- Appendix: Evaluation assumption

Table 5-1: Evaluation Assumptions for NR DL

| Parameter |  | Values |  |
| --- | --- | --- | --- |
| Test environment |  | DL Indoor Hotspot – eMBB |  |
| Evaluation configuration |  | Configuration B | Configuration C |
| Channel model |  | InH_B |  |
| ISD |  | 20 m (12TRxP) |  |
| Frame structure |  | Full downlink |  |
| Carrier Frequency |  | 30 GHz | 70 GHz |
| System bandwidth |  | 80MHz |  |
| Subcarrier spacing |  | 60 KHz |  |
| Symbols number per slot |  | 14 |  |
| Number of antenna elements per TRxP |  | 32Tx cross-polarized antenna (M,N,P,Mg,Ng) = (4,4,2,1,1) |  |
| Number of TXRU per TRxP |  | 32TXRU |  |
| Number of antenna elements per UE |  | 32Rx antennas with 0°/90° polarization (M,N,P,Mg,Ng) = (2,4,2,1,2) |  |
| Number of TXRU per UE |  | 8TXRU; Vertical 2-to-1, Horizontal 2-to-1 |  |
| Transmit power per TRxP |  | FDD: 20 dBm |  |
| TRxP number per site |  | 1 |  |
| Mechanic tilt |  | 180o in GCS (pointing to the ground) |  |
| Electronic tilt |  | 90 o in LCS |  |
| Beam set at TRxP |  | Azimuth angle φi = [0], Zenith angle θj = [pi/2] |  |
| Beam set at UE |  | Azimuth angle φi = [-3*pi/4, -pi/4, pi/4, 3*pi/4] Zenith angle θj = [0, pi/4, 2*pi/4, 3*pi/4] |  |
| UT attachment |  | Based on RSRP (Eq. (8.1-1) in TR 36.873) from port 0 |  |
| Scheduling |  | MU-PF |  |
| ACK/NACK delay |  | Next available UL slot |  |
| MIMO mode |  | MU-MIMO with rank 2/4 adaptation per user; Maximum MU layer = 6 |  |
| Guard band ratio |  | FDD: 5.5% (for 80 MHz) |  |
| BS receiver type |  | MMSE-IRC |  |
| CSI feedback |  | 5 slots period based on CSI-RS with delay |  |
| Precoder derivation |  | FDD: NR Type II codebook based |  |
| Overhead | PDCCH | 2 complete symbols |  |
|  | DMRS | Type II, based on MU-layer (dynamic in simulation) |  |
|  | CSI-RS | 32 ports per 10 slots |  |
|  | SSB | 1 SSB per 20 ms |  |
|  | TRS | 2 consecutive slots per 20ms, 1 port, 50PRB |  |
|  | PTRS | 2port PT-RS, (L, K) = (1,4) ,L and K are time and frequency density |  |
| Channel estimation |  | Non-ideal |  |
| Waveform |  | OFDM |  |

Table 5-2: Evaluation Assumptions for NR UL

| Parameter |  | Value |
| --- | --- | --- |
| Test environment |  | UL Indoor Hotspot – eMBB |
| Evaluation configuration |  | Configuration B |
| Channel model |  | InH_B |
| Frame structure |  | Full uplink |
| Carrier Frequency |  | 30 GHz |
| System bandwidth |  | 80MHz |
| Subcarrier spacing |  | 60 KHz |
| Symbols number per slot |  | 14 |
| Number of antenna elements per TRxP |  | 32Rx cross-polarized antenna (M,N,P,Mg,Ng) = (4,4,2,1,1) |
| Number of TXRU per TRxP |  | 32TXRU |
| Number of antenna elements per UE |  | 32Tx antenna with 0°,90° polarization (M,N,P,Mg,Ng) = (2,4,2,1,2) |
| Number of TXRU per UE |  | 8TXRU Vertical 2-to-1, Horizontal 2-to-1 |
| UE power class |  | 23 dBm |
| Mechanic tilt |  | 180 o in GCS (pointing to the ground) |
| Electronic tilt |  | 90 o in LCS |
| Beam set at TRxP |  | Azimuth angle φi = [0], Zenith angle θj = [pi/2] |
| Beam set at UE |  | Azimuth angle φi = [-7*pi/8, -5*pi/8, -3*pi/8, -pi/8, pi/8, 3*pi/8, 5*pi/8 7*pi/8], Zenith angle θj = [0, pi/4, 2*pi/4, 3*pi/4] |
| UT attachment |  | Based on RSRP (Eq. (8.1-1) in TR36.873) from port 0 |
| Scheduling |  | SU-PF |
| MIMO mode |  | SU-MIMO with rank 4 adaptation |
| BS receiver type |  | MMSE-IRC |
| UE precoder scheme |  | Codebook based |
| UL CSI derivation |  | Non-precoded SRS based, with delay |
| Power control |  | α= 0.8, P0 =-70 dBm |
| RB allocation for Power backoff model |  | Continuous: follow TS 38.101; Non-continuous: additional 2 RB reduction |
| Overhead | PUCCH | 2 RBs, 14 OFDM symbols |
|  | DMRS | Type II, 1 additional DMRS symbol, and no FDM with PUSCH |
|  | SRS | 2 symbols per 5 slots, 8 RBs per symbol |
|  | PTRS | 2port PT-RS, (L, K) = (1,4) ,L and K are time and frequency density |
| Channel estimation |  | Non-ideal |
| Waveform |  | OFDM |

- Reference

- [1] RP-181318, “Study on Self-Evaluation towards IMT-2020 submission”, RAN #80, Huawei, Ericsson, and Telecom Italia, Athens

- [2] Report ITU-R M.2412 - Guidelines for evaluation of radio interface technologies for IMT-2020.