3GPP TSG RAN WG1 Meeting #97	R1-1906624

Reno, USA, May 13 – 17, 2019

Agenda Item:	7.2.11.4

Source:	Huawei, HiSilicon

Title:	Consideration on dual/clutter mobility for IIOT

Document for:	Discussion and decision

# Introduction

In 3GPP RAN#81 meeting, a new study item (SI) was approved [1] to study on channel model for indoor industrial scenarios. It was recognized that the current 3GPP channel model in TR 38.901 [2] contains a common channel model with scenario-specific model parameters and settings for scenarios such as Urban Macro, Urban Micro, Rural Macro, and Indoor Hotspot (InH). However, it is noted that InH is based on indoor office / shopping mall environment. To address industrial scenarios that exhibit more diverse and unique environmental features, see the LS from 5G-ACIA in RP-181521 [3], it is needed that the InH in TR 38.901 should be extended to cover additional characteristics of industrial scenarios.

TR38.901 channel model assumes fixed environment and fixed transmitter. The Doppler spread is affected by the UE movement and AoA of each path. However, in IIOT environments, the transmitter and clutter (e.g. robots) may be moving and thus causing additional Doppler frequency shift.

This contribution proposes additional Doppler frequency shift for the moving Tx and clutter.

# Discussion

Radiowave signal from a Tx to Rx in IIOT environment may be shifted in frequency domain due to multiple reasons: Tx may be moving, clutter may be moving, and the Rx may be moving (Figure 1).

Figure 1. Doppler in presence of moving scatterers

Due to the complexity of the environment, the Doppler frequency shift caused by the clutter (robots, robot arms, products, etc.) should be modeled as a stochastic way. The clutter can be modelled as a number of scatterers. Velocity vector of each scatterer could be considered as a random variable. The concept of randomly moving scatterers is illustrated in Figure 2.

Figure 2. Illustration on random motion of clusters.

Proposal 1: The impact of moving scatterers on the Doppler should be modeled as a stochastic method.

For IIOT, dual mobility should be modeled as follows (modified from TR37.885 [5]):

-	Doppler for the LOS path:

,

-	Doppler for the delayed paths:

vn,m=rrx,n,mT∙vrx+rtx,n,mT∙vtx+2αn,mDn,mλ0

where Dn,m is a random variable with uniform distribution from -vscatt to vscatt,  vscatt is the maximum speed of the  clutter. The distributions of αn,m and Dn,m should be FFS.

Proposal 2: The impact of Tx and Rx movement should be modeled as

,

Proposal 3: The impact of scatterer movement should be modeled as

vn,m=rrx,n,mT∙vrx+rtx,n,mT∙vtx+2αn,mDn,mλ0

where Dn,m is a random variable with uniform distribution from -vscatt to vscatt,  vscatt is the maximum speed of the  clutter. The distributions of αn,m and Dn,m should be FFS.

# Conclusion

Proposal 1: The impact of moving scatterers on the Doppler should be modeled as a stochastic method.

Proposal 2: The impact of Tx and Rx movement should be modeled as

,

Proposal 3: The impact of scatterer movement should be modeled as

vn,m=rrx,n,mT∙vrx+rtx,n,mT∙vtx+2αn,mDn,mλ0

where Dn,m is a random variable with uniform distribution from -vscatt to vscatt,  vscatt is the maximum speed of the  clutter. The distributions of αn,m and Dn,m should be FFS.

# References

RP-182138, “New SI proposal: Study on Channel Modeling for Indoor Industrial Scenarios”, Ericsson, September 2018.

3GPP TR 38.901 v15.0.0, “Study on channel model for frequencies from 0.5 to 100 GHz”.

RP-181521, “LS on Channel Model for Indoor Industrial Scenarios”, 5G-ACIA, September 2018.

RP-182862, “SID update: Study on Channel Modeling for Indoor Industrial Scenarios”, Ericsson, December 2018.

3GPP TR37.885 “Study on evaluation methodology of new Vehicle-to-Everything (V2X) use cases for LTE and NR”, V15.2.0 (Release 15), 2018-12.