3GPP TSG-RAN WG1 Meeting #97	 R1-1906749

Reno, USA, 13th – 17th May, 2019

Source:	Nokia, Nokia Shanghai Bell

Title:	2-step RACH evaluation results

Agenda Item:	7.1.2.3

Document for:	Discussion

# Introduction

In this contribution, we present preliminary evaluation results based on a single radio link focusing on the simulation parameters agreed in [1].

# Preliminary link level evaluation results

## Preamble detection

Simulation results for miss-detection probability as a function of receiver SNR are provided in Figure 1 for the preamble format A1 and format 0 in 200m ISD scenario for the case of 2 Rx antennas. For the evaluations, the target false-alarm rate is fixed Pf=1%, illustrated in Figure 2 as example. Based on the simulation assumptions provided in Table A.1, the time offset is within 0 and round-trip time. In that case, the timing estimation accuracy is within +/-0.5 us for all cases and Figure 3 illustrates the evaluation results of the time offset estimation used in the coming evaluations of MsgA PUSCH.

Figure 1. Miss-detection rate of preamble detection for false alarm rate 1%: (a) Format A1, 30 kHz  (b) Format 0, 1.25KHz

Figure 2. False alarm rate.                   Format A1, 30kHz

Figure 3. Standard deviation of TA.      Format A1, 30kHz

Observation 1. Increasing the number of transmitting UEs for a fixed false alarm rate leads to an increase of the miss- detection rate of preamble detection.

The link budget analysis for the preamble formats 0 and A1 is given in Table 1 as example for single UE case, where the SNR value is given for a target BLER of 0.1

Table 1. Link budget calculation of preamble formats A1 and 0

|  | Preamble A1 | Preamble 0 |
| --- | --- | --- |
| (1) Tx Power (dBm) | 23 | 23 |
| (2) Thermal noise density (dBm/Hz) | -174 | -174 |
| (3) eNB receiver noise figure (dB) | 5 | 5 |
| (4) Interference margin (dB) | 0 | 0 |
| (5) Occupied channel bandwidth (Hz) | 4.32e6 | 1.08e6 |
| (6) Effective noise power = (2)+(3)+(4)+10log((5)) (dBm) | -102.79 | -108.66 |
| (7) Required SINR (dB) | -5.5 | -7.1 |
| (8) Receiver sensitivity = (6)+(7) (dBm) | -108.29 | -115.79 |
| (9) Receiver processing gain | 0 | 0 |
| (10) MCL = (1)-(8)+(9) (dB) | 131.29 | 138.79 |

## PUSCH performance

We have compared the BLER performance for single and multiple UE cases for different payload sizes, PRB allocation sizes: 1, 2 and 6 PRB, using 4 or 8 RX antennas and MCS index 4 and 2, in Figures 4 and 5, respectively. Reducing the payload size for more robust MCS values leads to a BLER performance improvement.

The performance degradation for the case of MCS index 4 is much larger when multiplexing multiple UEs within the same PO. When using MCS index 2, BLER performance of 10% can be achieved and higher number of UEs can be multiplexed.

Figure 4. BLER performance for single and multiple UE cases for MCS index 4

Figure 5. BLER performance for single and multiple UE cases for MCS index 2

Observation 2. As the number of users multiplexed in a PUSCH occasion increases, the BLER performance degrades significantly.

Observation 3. Increasing the number of DMRS ports/sequences has diminishing returns as the BLER performance with multiple simultaneously transmitted UEs degrades significantly.

The link budget analysis for the case of 1, 2 or 6 PRBs PUSCH transmission is given in Table 2 for single UE case and 4Rx antennas case as example, where the SNR value is given for a target BLER of 0.1

Table 2. Link budget calculation of MsgA PUSCH

|  | MsgA PUSCH 1 PRB | MsgA PUSCH 2 PRB | MsgA PUSCH 6 PRB |
| --- | --- | --- | --- |
| TBS | 80 | 168 | 528 |
| (1) Tx Power (dBm) | 23 | 23 | 23 |
| (2) Thermal noise density (dBm/Hz) | -174 | -174 | -174 |
| (3) eNB receiver noise figure (dB) | 5 | 5 | 5 |
| (4) Interference margin (dB) | 0 | 0 | 0 |
| (5) Occupied channel bandwidth (Hz) | 0.36e6 | 0.72e6 | 2.16e6 |
| (6) Effective noise power = (2)+(3)+(4)+10log((5)) (dBm) | -113.43 | -110.42 | -105.65 |
| (7) Required SINR (dB) | 2.2 | 0.22 | -0.56 |
| (8) Receiver sensitivity = (6)+(7) (dBm) | -111.23 | -110.20 | -106.21 |
| (9) Receiver processing gain | 0 | 0 | 0 |
| (10) MCL = (1)-(8)+(9) (dB) | 134.23 | 133.20 | 129.21 |

Observation 4. The MCL for low payload sizes in case of 1,2 or 6 PRBs transmissions is closer to the MCL of short preamble format (A1) than that of long preamble format (0).

# Conclusions

In this contribution, we presented first link level performance evaluations of MsgA for 2-step RACH. Based on the evaluation results, we summarize the following observations:

Observation 1. Increasing the number of transmitting UEs for a fixed false alarm rate leads to an increase of the miss- detection rate of preamble detection.

Observation 2. As the number of users multiplexed in a PUSCH occasion increases, the BLER performance degrades significantly.

Observation 3. Increasing the number of DMRS ports/sequences has diminishing returns as the BLER performance with multiple simultaneously transmitted UEs degrades significantly.

Observation 4. The MCL for low payload sizes in case of 1,2 or 6 PRBs transmissions is closer to the MCL of short preamble format (A1) than that of long preamble format (0).

# References

[1] R1-1903834. Email discussion on potential link-level simulation assumption for 2-step RACH. Meeting #96, 3GPP

# Appendix A. Simulation assumptions

Table A.1 General simulation assumptions

| Parameters | Values |
| --- | --- |
| PRB | 1,2 and 6 |
| Subcarrier spacing | 30 kHz |
| Frequency band | 4 GHz |
| PRACH preamble | Format A1 and 0 |
| Propagation channel model | TDL-C DS=300ns |
| UE velocity | 3 kph |
| Waveform | CP-OFDM |
| Antenna configuration | 1Tx2Rx, 1Tx4Rx, 1Tx8Rx |
| DMRS configuration | Type 1 DMRS in symbols 2 and 11 |
| MCS | 1PRB, MCS_index = 2, TBS = 48 2PRB, MCS_index = 2, TBS = 104 6PRB, MCS_index = 2, TBS = 320 1PRB, MCS_index = 4, TBS = 80 2PRB, MCS_index = 4, TBS = 168 6PRB, MCS_index = 4, TBS = 528 |
| Number of UE | 1, 2 |
| Receiver | linear MMSE-IRC |
| Channel estimation | Ideal |
| Timing offset | 200m, UMi: 0.66us |
| Frequency offset | 0.05ppm (fixed) at TRP, and 0.1 ppm (fixed) at UE |