3GPP TSG RAN WG1 Meeting 97                                                                	         R1-1906813

Reno, USA 13th – 17th May, 2019

| Source | : | Intel Corporation |
| --- | --- | --- |
| Title | : | Mobility Evaluation for NR eMBB |
| Agenda Item | : | 7.2.7.2 |
| Document for | : | Discussion and Decision |

# Introduction

This paper is related to the “New Study Item on Self-Evaluation towards IMT-2020 submission” [1]. In the RAN WG1 Meeting #94, 3GPP finalized the self-evaluation results for Rel-15 NR for submission to ITU-R in accordance with the IMT-2020 compliance template. In this paper, we provide updated NR eMBB mobility results for IMT-2020 self-evaluation.

# Mobility Evaluation for NR eMBB

In this section, we provide self-evaluation results for the simulation based metric of Mobility for NR eMBB test environments as outlined in [3]. We present the evaluation methodology and the detailed simulation assumptions considered for this evaluation and show that NR can meet IMT-2020 minimum requirements [2] for all test environments and frequency ranges of operation.

## Evaluation Methodology

Mobility is evaluated for high mobility users for the test environments of Indoor Hotspot (10 km/hr), Dense Urban macro (30 km/hr) in frequency range 1 with carrier frequency 4 GHz as well as frequency range 2 with a carrier frequency of 30 GHz [5][6]; and for Rural macro (120 km/hr, 500 km/hr) for frequency range 1 with carrier frequencies of 700 MHz and 4 GHz as outlined in the IMT-2020 evaluation guidelines in [3]. The evaluation follows a combined system level and link level methodology, wherein the system level simulation with full-buffer traffic assumptions are used to determine the SINR distribution for each test environment and this, in turn, is used to run the link level simulator to determine the normalized link rate (bit/sec/Hz) at a target BLER of 1%. The IMT-2020 minimum requirements for mobility are as follows:

Table 1: IMT-2020 Requirements for Mobility

| Test Environnent | Normalized traffic channel link data rate (bit/s/Hz) | Mobility (km/h) |
| --- | --- | --- |
| Indoor Hotspot – eMBB | 1.5 | 10 |
| Dense Urban – eMBB | 1.12 | 30 |
| Rural – eMBB | 0.8 | 120 |
|  | 0.45 | 500 |

Based on the above requirements, the detailed steps for self-evaluation are as follows [3]:

Step 1: 	Run uplink system-level simulations, identical to those for average spectral efficiency, and 5th percentile user spectral efficiency except for speeds taken from Table 1, using link-level simulations and a link-to-system interface appropriate for these speed values, for the set of selected test environment(s) associated with the candidate RITs/SRITs and collect overall statistics for uplink SINR values, and construct CDF over these values for each test environment.

Step 2:	Use the CDF for the test environment(s) to save the respective 50th-percentile SINR value.

Step 3: 	Run new uplink link-level simulations for the selected test environment(s) for either NLOS or LOS channel conditions using the associated speeds in Table 1, as input parameters, to obtain link data rate and residual packet error ratio as a function of SINR. The link-level simulation shall use air interface configuration(s) supported by the proposal and take into account retransmission, channel estimation and phase noise impact.

Step 4: 	Compare the uplink spectral efficiency values (link data rate normalized by channel bandwidth) obtained from Step 3 using the associated SINR value obtained from Step 2 for selected test environments, with the corresponding threshold values in the Table 1.

Step 5: 	The proposal fulfils the mobility requirement if the spectral efficiency value is larger than or equal to the corresponding threshold value and if also the residual decoded packet error ratio is less than 1%, for all selected test environments. For the selected test environment it is sufficient if one of the spectral efficiency values (using either NLOS or LOS channel conditions) fulfils the threshold.

Based on the outlined methodology, we present detailed results to show that the Mobility requirements for IMT-2020 can be easily met by NR.

## Evaluation Results

In this section, we provide detailed link and system level results to show that NR can meet the minimum requirements for mobility in all test environments. The summary of results is provided in Table 2. For link level simulations, the SINR specified in Table 2, is the minimum SINR required to achieve the target normalized PUSCH data rate (bits/s/Hz) specified in Table 1, at a target BLER of 1%. For system level simulation, the SINR specified in Table 2 is the 50th percentile of the uplink pre-processing SINR. Furthermore, for FR1, we assumed the use of IMT Channel Model A and for FR2, we assumed the use of IMT Channel Model B for the corresponding test environments.

Table 2: System and Link Level Evaluation Results for Mobility

| Test Environ-ment | CF | SCS (kHz) | System Level Results |  |  | Link Level Results |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50th % Pre-Processing SINR (dB) | gNB Antenna Configuration (M,N,P,Mp,Np) | Power Control | Tx Ports (UE) | Rx Ports (gNB) |  | Normalized Link Rate (bits/s/Hz) |
| Indoor Hotspot 10 km/hr | 4 GHz, 12 TRxP (Config. A) | 15 | 9.5 | (8,8,2,2,2) DFT beam switching | (0.9,-90) | 2 | 8 | 3.207 |  |
|  |  | 30 | 9.19 |  |  |  |  | 2.991 |  |
|  | 4 GHz, 36 TRxP (Config. A) | 15 | 3.29 | (16,4,2,2,2) Single Beam 110° downtilt |  |  |  | 1.776 |  |
|  |  | 30 | 3.18 |  |  |  |  | 1.773 |  |
|  | 30 GHz, 12 TRxP (Config. B) | 60 | 16.76 | (4,4,2,2,2) DFT beam switching | (0.8,-70) |  |  | 4.484 |  |
|  |  | 120 | 16.43 |  |  |  |  | 4.270 |  |
|  | 30 GHz, 36 TRxP (Config. B) | 60 | 13.91 | (8,4,2,2,2) DFT beam switching | (0.7,-80) |  |  | 4.127 |  |
|  |  | 120 | 13.29 |  |  |  |  | 3.905 |  |
| Dense Urban 30 km/hr | 4 GHz (Config. A) | 15 | 4.02 | (16,2,2,2,2) Single Beam 102° downtilt | (0.9,-90) |  |  | 1.923 |  |
|  |  | 30 | 3.93 |  |  |  |  | 1.925 |  |
|  | 30 GHz (Config. B) | 60 | 10.56 | (16,8,2,2,2,2) DFT beam switching | (0.7,-80) |  |  | 3.216 |  |
|  |  | 120 | 7.82 |  |  |  |  | 2.665 |  |
| Rural Macro 120 km/hr | 700 MHz (Config. A) | 15 | 7.34 | (8,2,2,2,2) Single Beam 100° downtilt | (0.9,-90) |  |  | 2.464 |  |
|  |  | 30 | 7.17 |  |  |  |  | 2.464 |  |
|  | 4 GHz (Config. B) | 15 | 7.97 | (16,2,2,2,2) Single Beam 100° downtilt |  |  |  | 2.750 |  |
|  |  | 30 | 7.1 |  |  |  |  | 2.462 |  |
| Rural Macro 500 km/hr | 700 MHz (Config. A) | 15 | 7.34 | (8,2,2,2,2) Single Beam 100° downtilt | (0.9,-90) |  |  | 2.730 |  |
|  |  | 30 | 7.17 |  |  |  |  | 2.463 |  |
|  | 4 GHz (Config. B) | 15 | 7.97 | (16,2,2,2,2) Single Beam 100° downtilt |  |  |  | 2.093 |  |
|  |  | 30 | 7.1 |  |  |  |  | 2.463 |  |

Based on the results presented in Table 2, we observe that the normalized link rate or Spectral efficiency obtained from the link level simulations at the 50th percentile pre-processing SINR point from the SLS is greater than the minimum requirements for all cases.

Observation 1:

NR can meet the mobility requirement for all supported test environments and configurations.

### Link Level Simulation Assumptions

For link level simulations, 2 TXU at the UE and 8 RXU at the gNB is assumed for the results presented in Table 2. A more detailed summary of link level evaluation assumption and results are provided in Appendix A and B respectively.

### System Level Simulation Assumptions

For system level simulations, the antenna configuration of the UE at FR1 (carrier frequency below 6 GHz) is  with 2 TXU at the UE. At FR2 (carrier frequency of 30 GHz), two opposite facing panels with the per-panel antenna configuration of   is used with UE having 2 TXU due to panel selection and single panel operation. For each of the test environments and related evaluation configurations, the evaluation assumptions in [3] are followed. The power control parameters for open loop uplink power control are detailed in Table 2. For the uplink SINR evaluation, we assume round robin scheduling and that one UE from each of the non-serving cells interferes with the serving gNB.

# Conclusions

In this paper, we have provided updated link-level results for mobility evaluation and showed that NR can meet IMT-2020 requirements for mobility for all supported configurations.

# References

- RP-170832, New Study Item on Self-Evaluation towards IMT-2020, Huawei, Ericsson, 3GPP TSG RAN Meeting #75, Dubrovnik, Croatia, March 6-9, 2017.

- ITU-R M.2410-0, Minimum requirements related to technical performance for IMT-2020 radio interface(s)

- ITU-R M.2412-0, Guidelines for evaluation of radio interface technologies for IMT-2020

- ITU-R M.2134, Requirements related to technical performance for IMT-Advanced radio interface(s)

- 3GPP TS 38.101-1 v15.1.0, User Equipment radio transmission and reception: Part 1: Range 1 Standalone.

- 3GPP TS 38.101-2 v15.1.0, User Equipment radio transmission and reception: Part 2: Range 2 Standalone.

- 3GPP TS 38.306 v15.1.0, User Equipment (UE) Radio Access Capabilities.

- 3GPP TS 38.214 v15.1.0, NR Physical Layer Procedures for Data (Release 15)

- 3GPP TS 38.211 v15.1.0, NR Physical Channels and Modulation (Release 15)

# Appendix A: Link Level Simulation Assumptions

For the link-level evaluations Table 5.1.3.1-1 from TS 38.214 v15.4.0 is used.

## Indoor Hotspot

| Assumption | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Scenario | Indoor Hotspot |  |  |  |  |  |
| Mobility (km/h) | 10 |  |  |  |  |  |
| UL waveform | CP-OFDM |  |  |  |  |  |
| Carrier frequency (GHz) | 4 |  | 30 |  | 70 |  |
| Numerology | 0 | 1 | 2 | 3 | 2 | 3 |
| Subcarrier spacing (KHz) | 15 | 30 | 60 | 120 | 60 | 120 |
| System bandwidth (MHz) | 10 | 10 | 10 | 50 | 10 | 50 |
| Channel model | TDL-i (TDL-A) | TDL-i (TDL-A) | TDL-i (TDL-A) | TDL-i (TDL-A) | TDL-i (TDL-A) | TDL-i (TDL-A) |
| Delay spread (ns) | 39 | 39 | 26 | 26 | 20 | 20 |
| Channel coding | LDPC |  |  |  |  |  |
| Antenna configuration | 2Tx 8Rx |  |  |  |  |  |
| Data bandwidth | 6 PRBs | 6 PRBs | 6 PRBs | 12 PRBs | 6 PRBs | 12 PRBs |
| Modulation | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM |
| DMRS configuration | NR Type 1 DMRS symbol position [3rd, 6th, 9th, 12th] |  |  |  |  |  |
| Channel estimation | MMSE-Block (No PRB bundling) |  |  |  |  |  |
| Transmission scheme | 1 layer transmission |  |  |  |  |  |
| HARQ | No HARQ |  |  |  |  |  |
| Link adaptation | Fixed MCS |  |  |  |  |  |

## Dense Urban

| Assumption | Value |  |  |  |
| --- | --- | --- | --- | --- |
| Scenario | Dense Urban |  |  |  |
| Mobility (km/h) | 30 |  |  |  |
| UL waveform | CP-OFDM |  |  |  |
| Carrier frequency (GHz) | 4 |  | 30 |  |
| Numerology | 0 | 1 | 2 | 3 |
| Subcarrier spacing (KHz) | 15 | 30 | 60 | 120 |
| System bandwidth (MHz) | 10 | 10 | 10 | 50 |
| Channel model | TDL-iii (TDL-C) | TDL-iii (TDL-C) | TDL-iii (TDL-C) | TDL-iii (TDL-C) |
| Delay spread (ns) | 363 | 363 | 263 | 263 |
| Channel coding | LDPC |  |  |  |
| Antenna configuration | 2Tx 8Rx |  |  |  |
| Data bandwidth | 6 PRBs | 6 PRBs | 6 PRBs | 12 PRBs |
| Modulation | 16QAM | 16QAM | 16QAM | 16QAM |
| DMRS configuration | NR Type 1 DMRS symbol position [3rd, 6th, 9th, 12th] |  |  |  |
| Channel estimation | MMSE-Block (No PRB bundling) |  |  |  |
| Transmission scheme | 1 layer transmission |  |  |  |
| HARQ | No HARQ |  |  |  |
| Link adaptation | Fixed MCS |  |  |  |

## Rural Macro (120 kmph)

| Assumption | Value |  |  |  |
| --- | --- | --- | --- | --- |
| Scenario | Rural |  |  |  |
| Mobility (km/h) | 120 |  |  |  |
| UL waveform | CP-OFDM |  |  |  |
| Carrier frequency (GHz) | 0.7 |  | 4 |  |
| Numerology | 0 | 1 | 0 | 1 |
| Subcarrier spacing (KHz) | 15 | 30 | 15 | 30 |
| System bandwidth (MHz) | 10 | 10 | 10 | 10 |
| Channel model | TDL-iii (TDL-C) | TDL-iii (TDL-C) | TDL-iii (TDL-C) | TDL-iii (TDL-C) |
| Delay spread (ns) | 37 | 37 | 37 | 37 |
| Channel coding | LDPC |  |  |  |
| Antenna configuration | 2Tx 8Rx |  |  |  |
| Data bandwidth | 6 PRBs | 6 PRBs | 6 PRBs | 6 PRBs |
| Modulation | QPSK | QPSK | QPSK | QPSK |
| DMRS configuration | NR Type 1 DMRS symbol position [3rd, 6th, 9th, 12th] |  |  |  |
| Channel estimation | MMSE-Block (No PRB bundling) |  |  |  |
| Transmission scheme | 1 layer transmission |  |  |  |
| HARQ | No HARQ |  |  |  |
| Link adaptation | Fixed MCS |  |  |  |

## Rural Macro (500 kmph)

| Assumption | Value |  |  |  |
| --- | --- | --- | --- | --- |
| Scenario | Rural |  |  |  |
| Mobility (km/h) | 500 |  |  |  |
| UL waveform | CP-OFDM |  |  |  |
| Carrier frequency (GHz) | 0.7 |  | 4 |  |
| Numerology | 0 | 1 | 0 | 1 |
| Subcarrier spacing (KHz) | 15 | 30 | 15 | 30 |
| System bandwidth (MHz) | 10 | 10 | 10 | 10 |
| Channel model | TDL-iii (TDL-C) | TDL-iii (TDL-C) | TDL-iii (TDL-C) | TDL-iii (TDL-C) |
| Delay spread (ns) | 37 | 37 | 37 | 37 |
| Channel coding | LDPC |  |  |  |
| Antenna configuration | 2Tx 8Rx |  |  |  |
| Data bandwidth | 6 PRBs | 6 PRBs | 6 PRBs | 6 PRBs |
| Modulation | QPSK | QPSK | QPSK | QPSK |
| DMRS configuration | NR Type 1 DMRS symbol position [3rd, 6th, 9th, 12th] |  |  |  |
| Channel estimation | MMSE-Block (No PRB bundling) |  |  |  |
| Transmission scheme | 1 layer transmission |  |  |  |
| HARQ | No HARQ |  |  |  |
| Link adaptation | Fixed MCS |  |  |  |

# Appendix B: Detailed Link Level Evaluation Results

In this section, the detailed LLS results for multiple MCS values and corresponding TBS are shown. The selected values presented in Table 2 highlighted in green.

## Indoor Hotspot

| Fc (GHz) | SCS (KHz) | SNR (dB) | MCS Index | Modulation order | TBS | BLER | Spectral efficiency |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 4 | 15 | 3.29 | 11 | 4 | 1288 | 5.00E-05 | 1.193 |
|  |  |  | 12 | 4 | 1480 | 1.00E-04 | 1.370 |
|  |  |  | 13 | 4 | 1672 | 2.50E-04 | 1.548 |
|  |  |  | 14 | 4 | 1928 | 5.00E-03 | 1.776 |
|  |  |  | 15 | 4 | 2088 | 1.73E-02 | 1.900 |
|  |  | 9.5 | 19 | 6 | 2664 | 0.00E+00 | 2.467 |
|  |  |  | 20 | 6 | 2976 | 1.50E-04 | 2.755 |
|  |  |  | 21 | 6 | 3240 | 1.75E-03 | 2.995 |
|  |  |  | 22 | 6 | 3496 | 9.15E-03 | 3.207 |
|  |  |  | 23 | 6 | 3752 | 3.55E-02 | 3.351 |
|  | 30 | 3.18 | 11 | 4 | 1288 | 0.00E+00 | 1.193 |
|  |  |  | 12 | 4 | 1480 | 1.50E-04 | 1.370 |
|  |  |  | 13 | 4 | 1672 | 5.00E-04 | 1.547 |
|  |  |  | 14 | 4 | 1928 | 6.90E-03 | 1.773 |
|  |  |  | 15 | 4 | 2088 | 2.00E-02 | 1.895 |
|  |  | 9.19 | 18 | 6 | 2408 | 0.00E+00 | 2.230 |
|  |  |  | 19 | 6 | 2664 | 0.00E+00 | 2.467 |
|  |  |  | 20 | 6 | 2976 | 3.50E-04 | 2.755 |
|  |  |  | 21 | 6 | 3240 | 3.15E-03 | 2.991 |
|  |  |  | 22 | 6 | 3496 | 1.51E-02 | 3.188 |
| 30 | 60 | 13.91 | 24 | 6 | 3968 | 0.00E+00 | 3.674 |
|  |  |  | 25 | 6 | 4224 | 3.00E-04 | 3.910 |
|  |  |  | 26 | 6 | 4480 | 5.20E-03 | 4.127 |
|  |  |  | 27 | 6 | 4608 | 2.10E-02 | 4.177 |
|  |  |  | 28 | 6 | 4864 | 2.24E-01 | 3.496 |
|  |  | 16.76 | 24 | 6 | 3968 | 0.00E+00 | 3.674 |
|  |  |  | 25 | 6 | 4224 | 0.00E+00 | 3.911 |
|  |  |  | 26 | 6 | 4480 | 0.00E+00 | 4.148 |
|  |  |  | 27 | 6 | 4608 | 0.00E+00 | 4.267 |
|  |  |  | 28 | 6 | 4864 | 4.30E-03 | 4.484 |
|  | 120 | 13.29 | 24 | 6 | 7936 | 0.00E+00 | 3.674 |
|  |  |  | 25 | 6 | 8456 | 2.60E-03 | 3.905 |
|  |  |  | 26 | 6 | 8968 | 2.82E-02 | 4.035 |
|  |  |  | 27 | 6 | 9224 | 1.05E-01 | 3.820 |
|  |  |  | 28 | 6 | 9736 | 6.60E-01 | 1.533 |
|  |  | 16.43 | 24 | 6 | 7936 | 0.00E+00 | 3.674 |
|  |  |  | 25 | 6 | 8456 | 0.00E+00 | 3.915 |
|  |  |  | 26 | 6 | 8968 | 0.00E+00 | 4.152 |
|  |  |  | 27 | 6 | 9224 | 1.00E-04 | 4.270 |
|  |  |  | 28 | 6 | 9736 | 1.43E-02 | 4.443 |

Dense Urban

| Fc (GHz) | SCS (KHz) | SNR (dB) | MCS Index | Modulation order | TBS | BLER | Spectral efficiency |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 4 | 15 | 4.02 | 13 | 4 | 1672 | 5.00E-05 | 1.548 |
|  |  |  | 14 | 4 | 1928 | 8.50E-04 | 1.784 |
|  |  |  | 15 | 4 | 2088 | 5.10E-03 | 1.923 |
|  |  |  | 16 | 4 | 2280 | 2.55E-02 | 2.057 |
|  |  |  | 17 | 6 | 2280 | 4.30E-02 | 2.020 |
|  | 30 | 3.93 | 13 | 4 | 1672 | 0.00E+00 | 1.548 |
|  |  |  | 14 | 4 | 1928 | 9.00E-04 | 1.784 |
|  |  |  | 15 | 4 | 2088 | 4.40E-03 | 1.925 |
|  |  |  | 16 | 4 | 2280 | 2.74E-02 | 2.053 |
|  |  |  | 17 | 6 | 2280 | 4.79E-02 | 2.010 |
| 30 | 60 | 10.56 | 19 | 6 | 2664 | 0.00E+00 | 2.467 |
|  |  |  | 20 | 6 | 2976 | 0.00E+00 | 2.756 |
|  |  |  | 21 | 6 | 3240 | 7.00E-04 | 2.998 |
|  |  |  | 22 | 6 | 3496 | 6.65E-03 | 3.216 |
|  |  |  | 23 | 6 | 3752 | 5.03E-02 | 3.299 |
|  | 120 | 7.82 | 19 | 6 | 5248 | 0.00E+00 | 2.430 |
|  |  |  | 20 | 6 | 5760 | 7.50E-04 | 2.665 |
|  |  |  | 21 | 6 | 6272 | 1.86E-02 | 2.850 |
|  |  |  | 22 | 6 | 6784 | 1.94E-01 | 2.532 |
|  |  |  | 23 | 6 | 7296 | 6.57E-01 | 1.158 |

Rural Macro (120kmh)

| Fc (GHz) | SCS (KHz) | SNR (dB) | MCS Index | Modulation order | TBS | BLER | Spectral efficiency |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0.7 | 15 | 7.34 | 18 | 6 | 1672 | 2.00E-04 | 2.229 |
|  |  |  | 19 | 6 | 1928 | 1.10E-03 | 2.464 |
|  |  |  | 20 | 6 | 2088 | 1.05E-02 | 2.727 |
|  |  |  | 21 | 6 | 2280 | 5.42E-02 | 2.837 |
|  |  |  | 22 | 6 | 2280 | 1.71E-01 | 2.684 |
|  | 30 | 7.17 | 18 | 6 | 1672 | 1.00E-04 | 2.229 |
|  |  |  | 19 | 6 | 1928 | 1.10E-03 | 2.464 |
|  |  |  | 20 | 6 | 2088 | 1.13E-02 | 2.725 |
|  |  |  | 21 | 6 | 2280 | 6.29E-02 | 2.811 |
|  |  |  | 22 | 6 | 2280 | 1.87E-01 | 2.631 |
| 4 | 15 | 7.97 | 18 | 6 | 2664 | 0.00E+00 | 2.230 |
|  |  |  | 19 | 6 | 2976 | 5.00E-05 | 2.467 |
|  |  |  | 20 | 6 | 3240 | 2.15E-03 | 2.750 |
|  |  |  | 21 | 6 | 3496 | 1.86E-02 | 2.944 |
|  |  |  | 22 | 6 | 3752 | 9.71E-02 | 2.923 |
|  | 30 | 7.1 | 18 | 6 | 5248 | 1.50E-04 | 2.229 |
|  |  |  | 19 | 6 | 5760 | 1.85E-03 | 2.462 |
|  |  |  | 20 | 6 | 6272 | 1.55E-02 | 2.713 |
|  |  |  | 21 | 6 | 6784 | 8.25E-02 | 2.752 |
|  |  |  | 22 | 6 | 7296 | 2.44E-01 | 2.446 |

## Rural Macro (500kmh)

| Fc (GHz) | SCS (KHz) | SNR (dB) | MCS Index | Modulation order | TBS | BLER | Spectral efficiency |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0.7 | 15 | 7.34 | 18 | 6 | 2408 | 5.00E-05 | 2.230 |
|  |  |  | 19 | 6 | 2664 | 6.50E-04 | 2.465 |
|  |  |  | 20 | 6 | 2976 | 9.45E-03 | 2.730 |
|  |  |  | 21 | 6 | 3240 | 5.88E-02 | 2.824 |
|  |  |  | 22 | 6 | 3496 | 1.99E-01 | 2.594 |
|  | 30 | 7.17 | 18 | 6 | 2408 | 2.00E-04 | 2.229 |
|  |  |  | 19 | 6 | 2664 | 1.65E-03 | 2.463 |
|  |  |  | 20 | 6 | 2976 | 1.45E-02 | 2.716 |
|  |  |  | 21 | 6 | 3240 | 7.50E-02 | 2.775 |
|  |  |  | 22 | 6 | 3496 | 2.20E-01 | 2.526 |
| 4 | 15 | 7.97 | 17 | 6 | 2280 | 8.70E-03 | 2.093 |
|  |  |  | 18 | 6 | 2408 | 2.70E-02 | 2.170 |
|  |  |  | 19 | 6 | 2664 | 1.59E-01 | 2.076 |
|  |  |  | 20 | 6 | 2976 | 5.59E-01 | 1.215 |
|  |  |  | 21 | 6 | 3240 | 8.79E-01 | 0.362 |
|  | 30 | 7.1 | 18 | 6 | 2408 | 5.00E-05 | 2.230 |
|  |  |  | 19 | 6 | 2664 | 1.40E-03 | 2.463 |
|  |  |  | 20 | 6 | 2976 | 2.51E-02 | 2.686 |
|  |  |  | 21 | 6 | 3240 | 1.73E-01 | 2.480 |
|  |  |  | 22 | 6 | 3496 | 4.89E-01 | 1.653 |