3GPP TSG-RAN WG1 Meeting #97	R1-1907302

Reno, USA, May 13th – 17th, 2019

Agenda item:	7.2.12.1

Source: 	Qualcomm Incorporated

Title: 	On mobility enhancements

Document for:	Discussion/Decision

# Introduction

This contribution discusses our view on NR RACH-less handover during handover.

# RACH-less handover

RACH-less HO might be introduced to eliminate the RACH related latencies during HO. Although RACH-less HO does not provide 0ms HO interruption, it is still useful when source cell connection is deteriorating quickly.

## Timing advance

The timing advance (TA) for Msg3 transmission or for PUSCH transmission (RRC Connection Reconfiguration Complete message) to the target cell is estimated based on PRACH in RACH-based HO and signaled to UE via RAR. With removal of RACH during HO, how should UE get target cell TA? For LTE, RACH-less HO was restricted to the synchronous intra-frequency deployments. Furthermore, it is limited to the cases that the target cell TA is the same as the source cell TA (e.g., collocated cells) or the target cell TA is zero (e.g., small cell). We believe that NR RACH-less HO should support at least scenarios where target cell TA is zero or same as source cell TA for both FR1 and FR2.

Proposal 1: NR RACH-less HO supports at least scenarios where target cell TA is zero or same as source cell TA for both FR1 and FR2.

Although the restriction on TA and synchronous deployment could eliminate the need to obtain TA for the target cell, it limits the usage of RACH-less HO in more practical deployment scenarios e.g., when target cell TA is non-zero or when the target cell TA is not the same as the source cell TA (e.g., non-collocated cells). Hence, for NR, it might be beneficial to investigate mechanisms to enable UE estimate timing advance of the target cell.

Proposal 2: RAN1 should investigate additional TA estimation techniques to extend use cases of NR RACH-less HO.

As noted in [1], the TA calculation error comes from TA error of source cell, UE reception time difference, BS transmission time alignment error, and uplink/downlink imbalance. We could consider these error sources to aid UE with more accurate target cell TA estimation. In particular, the UE might be able to estimate target cell TA based on source cell TA, RSTD, timing difference between DL Tx and UL Rx, and inherent timing offset between source and target cells (∆t) which should be zero in synchronous deployments. Assume DL/UL timing imbalance for a cell can be expressed as

TUL=TDL+∆τ

where ∆τ represents the DL/UL timing imbalance in the cell.

The TA of target cell can be expressed as:

TAtarget=TAsource+ 2RSTD-∆t+ ∆τtarget-∆τsource

where

RSTD=Ttarget,DL- Tsource,DL

is the reference signal time difference between target and source cell. Here, Ttarget,DL is the time when the UE receives the start of one subframe from target cell and Tsource,DL is the time when the UE receives the corresponding start of one subframe from source cell that is closest in time to the subframe received from target cell.

TAtarget=TAsource+ 2RSTD+ ∆τtarget-∆τsource-2∆t
                =TAsource+ 2RSTD+∆

where ∆ =∆τtarget-∆τsource-2∆t representing uplink/downlink imbalance and async between source and target cells.

Proposal 3: For intra-frequency handover, network indicates a timing refinement factor (∆) to UE that can derive the target cell TA by adjusting the source cell TA based on at least the indicated refinement factor.

- The timing refinement factor should represent aspects such as source-target cell timing synchronization and uplink/downlink imbalance at source/target cells.

Another option is to enable target cell to estimate timing advance based on existing uplink RS e.g., SRS that UE transmits to the source cell. Here, the uplink RS configuration is configured to UE and known to the target cell. This option should be at least feasible for FR1 intra-frequency handover.

Proposal 4: At least for FR1 intra-frequency handover, the target cell can estimate the timing advance based on SRS transmitted to the source cell. SRS configuration is configured to UE and known to the target cell.

## PUSCH transmission

Since NR supports multi-beam deployment, it would be necessary to define PUSCH occasion (time-frequency resources) for PUSCH transmission.

Proposal 5: Define PUSCH occasion for PUSCH transmission in multi-beam deployments.

The goal of RACH-less HO is to reduce HO interruption time. It should be noted that the latency of PUSCH transmission might be long. Hence, if NR can support PUSCH repetition, which is expected to be supported for eURLLC, PUSCH transmission latency should be reasonably reduced.

Proposal 6: RACH-less HO supports PUSCH repetition.

Furthermore, NR supports flexible numerology, two uplink waveforms for PUSCH, and different versions of pi/2 BPSK. Hence, it would be necessary to discuss which configuration should be supported for PUSCH in RACH-less HO.

Proposal 7: Discuss PUSCH configuration on numerology, waveform and MCS.

# Conclusion

This contribution provides our discussion on PHY aspects of RACH-less HO. In particular, we make the following observations:

Proposal 1: NR RACH-less HO supports at least scenarios where target cell TA is zero or same as source cell TA for both FR1 and FR2.

Proposal 2: RAN1 should investigate additional TA estimation techniques to extend use cases of NR RACH-less HO.

Proposal 3: For intra-frequency handover, network indicates a timing refinement factor (∆) to UE that can derive the target cell TA by adjusting the source cell TA based on at least the indicated refinement factor.

- The timing refinement factor should represent aspects such as source-target cell timing synchronization and uplink/downlink imbalance at source/target cells.

Proposal 4: At least for FR1 intra-frequency handover, the target cell can estimate the timing advance based on SRS transmitted to the source cell. SRS configuration is configured to UE and known to the target cell.

Proposal 5: Define PUSCH occasion for PUSCH transmission in multi-beam deployments.

Proposal 6: RACH-less HO supports PUSCH repetition.

Proposal 7: Discuss PUSCH configuration on numerology, waveform and MCS.

# Reference

- R4-166817, Reply LS on the feasibility of mobility enhancement solutions, RAN4 #80