3GPP TSG RAN WG1 #97	R1-1907544

Reno, USA, May 13th – 17th, 2019

Agenda Item:	7.2.10.5

Source:	Huawei, HiSilicon

Title:	On hybrid RAT-dependent positioning

Document for:	Discussion and decision

# Introduction

In the conclusions of TR38.885 [1], the following was noted:

Note: It is understood that no single positioning technology has to meet all the requirements for every scenario.

A possible way to improve location performance is to consider using more than one technique at the same time, and to consider hybrid techniques (e.g., UL-TDOA + UL-AOA).

In this contribution, we consider the following hybridization of RAT-dependent techniques:

- UL-TDOA + UL-AoA

- mRTT + UL-AoA.

# Evaluation

## UL-TDOA + UL-AoA

For UL-TDOA + UL-AoA, each gNB measures the RTOA and AoA based on SRS transmitted from the UE, and since both UL-TDOA and UL-AoA can reuse the single procedure, no new physical layer procedure is required.

The CDF error is given in Figure 1, and Table 1, including UL-TDOA only, UL-AoA only, and hybrid UL-TDOA + UL-AoA.

The simulation assumptions are summarized in Table 2.

Table 1 CDF for UL-TDOA and UL-AoA

|  |  | Source | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 | InH 50MHz | UL-TDOA | 2.48 | 3.73 | 5.34 | 8.26 |
|  |  | UL-AoA | 0.21 | 0.38 | 1.16 | 3.47 |
|  |  | UL-TDOA+UL-AoA | 0.18 | 0.31 | 0.78 | 2.89 |
|  | InH 5MHz | UL-TDOA | 13.3 | 16.83 | 20.15 | 26.61 |
|  |  | UL-AoA | 0.31 | 0.68 | 2.1 | 4.21 |
|  |  | UL-TDOA+UL-AoA | 0.22 | 0.44 | 1.09 | 3.68 |

Based on the simulation, we have the following observations:

Observation 1:

- AoA can greatly improve UL-TDOA accuracy, and UL-TDOA can improve AoA.

- UL-TDOA+UL-AoA can reach 1m accuracy @80% for both 50MHz and 5MHz.

Table 2 Simulation assumptions for UL-TDOA and UL-AoA

| Parameter | 50M InH | 5M InH |
| --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | 38.901 | 38.901 |
| Carrier frequency | 4GHz | 2GHz |
| Subcarrier spacing | 15kHz | 15kHz |
| Reference Signal Transmission Bandwidth | 50MHz | 5MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | Comb-4 | Comb-4 |
| Reference signal (type of sequence, number of ports, …) | SRS: ZC, 1-port | SRS: ZC, 1-port |
| Number of sites | 12 | 12 |
| Number of symbols used per occasion | 4 | 4 |
| number of occasions used per positioning estimate | 4 | 4 |
| Power-boosting level | 6dB | 6dB |
| Uplink power control (applied/not applied) | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Regular | Regular |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | UL-TDOA: Chan’s UAoA/UL-TDOA+UAoA: PSO | UL-TDOA: Chan’s UAoA/UL-TDOA+UAoA: PSO |
| Network synchronization assumptions | No sync error | No sync error |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | N/A | N/A |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | SVD | SVD |
| Additional notes, if any |  |  |

Figure 1 CDF for UL-TDOA and UL-AoA

## mRTT + UL-AoA

One of the issues with mRTT is that it does not perform well for low bandwidths: at low bandwidth, the time resolution is not enough for the UE to have good accuracy. On the other hand, AOA accuracy is largely independent of the bandwidth. Therefore, adding UL-AOA on top of mRTT could significantly improve the mRTT drawbacks.

For mRTT + UL-AoA, each gNB measures the Rx – Tx time difference and AoA based on SRS transmitted from the UE, while UE measures the Rx – Tx time difference based PRS transmitted from the gNB. UL-AoA measurement can be obtained in the UL part of the RTT procedure if gNB is equipped with MIMO.

The CDF error is given in Figure 2, and Table 3, including mRTT only, UL-AoA only, and hybrid mRTT + UL-AoA.

The simulation assumptions are summarized in Table 4.

Table 3 CDF for mRTT and UL-AoA

|  |  | Source | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 | InH 50MHz | mRTT | 1.13 | 1.44 | 1.77 | 2.17 |
|  |  | UL-AoA | 0.21 | 0.38 | 1.16 | 3.47 |
|  |  | mRTT+UL-AoA | 0.23 | 0.57 | 1.14 | 1.85 |
|  | InH 5MHz | mRTT | 5.22 | 7.34 | 9.45 | 10.99 |
|  |  | UL-AoA | 0.31 | 0.68 | 2.1 | 4.21 |
|  |  | mRTT+UL-AoA | 0.42 | 1.16 | 2.36 | 4.69 |

Based on the simulation, we have the following observations:

Observation 2:

- UL-AOA improves the performance of mRTT, especially for the low bandwidth case where mRTT alone does not perform well

Table 4 Simulation assumptions for mRTT and UL-AoA

| Parameter | 50M InH | 5M InH |
| --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | 38.901 | 38.901 |
| Carrier frequency | 4GHz | 2GHz |
| Subcarrier spacing | 15kHz | 15kHz |
| Reference Signal Transmission Bandwidth | 50MHz | 5MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | Comb-4 | Comb-4 |
| Reference signal (type of sequence, number of ports, …) | SRS: ZC, 1-port | SRS: ZC, 1-port |
| Number of sites | 12 | 12 |
| Number of symbols used per occasion | 4 | 4 |
| number of occasions used per positioning estimate | 4 | 4 |
| Power-boosting level | 6dB | 6dB |
| Uplink power control (applied/not applied) | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Regular | Regular |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | PSO | PSO |
| Network synchronization assumptions | No sync error | No sync error |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | N/A | N/A |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | SVD | SVD |
| Additional notes, if any |  |  |

Figure 2 CDF for mRTT and UL-AoA

# Potential specification impact

To enable hybrid positioning, especially for soft-combining between two positioning techniques, the measurements should be based on the same path. A common assumption should be that both timing measurement (UL-RTOA or Rx – Tx time difference) and angle measurement should be measured based on the first path. However, in case of multipath scenarios, additional path for timing and angle measurement can be additionally reported. If so, the timing and angle should be grouped by path or with an implicit one-to-one mapping, shown in Figure 3.

Timing/angle based on a “reference” path

Timing/angle based on additional path 1

Timing/angle based on additional path N

⋯

Timing based on a “reference” path

Timing based on additional path 1

Timing based on additional path N

⋯

Angle based on a “reference” path

Angle based on additional path 1

Angle based on additional path N

⋯

Figure 3 Timing and angle grouped by path (left) and with implicit one-to-one mapping (right)

Either way would enable the LMF to associate the timing measurement with angle measurement in a efficient way, and to provide a better location accuracy.

Proposal: NR supports association between timing measurements with angle measurements by path.

# Conclusion

This paper presents the evaluation results for hybridization of RAT-dependent techniques and analysis in specification impact. Based on the discussion we have the following observations:

Observation 1:

- AoA can greatly improve UL-TDOA accuracy, and UL-TDOA can improve AoA.

- UL-TDOA+UL-AoA can reach 1m accuracy @80% for both 50MHz and 5MHz.

Observation 2:

- UL-AOA improves the performance of mRTT, especially for the low bandwidth case where mRTT alone does not perform well

Proposal: NR supports association between timing measurements with angle measurements by path.

# References

- TR 38.855, Study on NR positioning support.