3GPP TSG-RAN WG1 Meeting #98 	R1-1909115

# Prague, CZ, August 26th - 30th, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | - | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft TS 38.213 CR for UL cancellation timeline |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-08-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - From specification, the value of d2,1 is not explicitly stated to be 0 or 1 in some cases involving cancellation of RRC configured transmissions for PUCCH, SRS, and PRACH. This is because the value of d2,1 is dependent upon the PUSCH DMRS location, but in the cases of SRS, PUCCH and PRACH, there is no PUSCH to determine this value. A value definition is needed to clarify operation. - From specification, the SCS used to determine the cancellation timeline N2 in some cases involving cancellation of RRC configured transmissions for PUCCH, SRS, PRACH and PUSCH when UE does not detect a DCI format 2_0 is not clear. When the SCSs of PDCCH providing DCI format 2_0 and PUCCH, SRS, PRACH and PUSCH are different, the SCS used to determine N2 is needed to clarify operation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Define d2,1=1 in cancellation timeline where dynamic downlink scheduling or SFI cancels RRC configured uplink transmissions. This would disambiguate operation for both the UE and network. - Replace N2 with to align the cancellation timeline in the specification in different cases. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Unclear specification on which value to assume between d2,1 = 0 or 1. gNB and UE may be out of sync regarding minimum action time required for cancellation of RRC configured signals, whenever network dynamically scheduled downlink data to the UE. - Unclear specification on which SCS to assume between the SCS of PDCCH for DCI format 2_0 or configured UL transmission. The UL transmission may cause interference to other UEs, whenever network scheduled transissions to other UEs in the overlapped resources. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated Impact Analysis: This CR impacts some cases involving cancellation of RRC configured transmissions for PUCCH, SRS, PRACH and PUSCH. It is expected that the UEs have been implemented in accordance the CR, given the RAN1 agreement. - If the UE is implemented in accordance to this CR and the gNB is not, gNB and UE may be out of sync regarding minimum action time required for cancellation of RRC configured signals, whenever network dynamically scheduled downlink data to the UE. - If the gNB is implemented in accordance to this CR and the UE is not, there is no problem since the value of d2,1=1 or replacing N2 with sets the largest possible value which was previously ambiguous. Since the action time for PUSCH preparation is large relative to d2,1=1 or replacing N2 with , there is no expectation of performance or latency degradation. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

11.1.1	UE procedure for determining slot format

< Unchanged parts are omitted >

If a UE is configured by higher layers to transmit SRS, or PUCCH, or PUSCH, or PRACH in a set of symbols of a slot and the UE detects a DCI format 2_0 with a slot format value other than 255 that indicates a slot format with a subset of symbols from the set of symbols as downlink or flexible, or the UE detects a DCI format 1_0, DCI format 1_1, or DCI format 0_1 indicating to the UE to receive CSI-RS or PDSCH in a subset of symbols from the set of symbols, then

-	the UE does not expect to cancel the transmission in symbols from the subset of symbols that occur, relative to a last symbol of a CORESET where the UE detects the DCI format 2_0 or the DCI format 1_0 or the DCI format 1_1 or the DCI format 0_1, after a number of symbols that is smaller than the PUSCH preparation time  for the corresponding PUSCH processing capability [6, TS 38.214] assuming

-	the UE cancels the PUCCH, or PUSCH, or PRACH transmission in remaining symbols from the set of symbols and cancels the SRS transmission in remaining symbols from the subset of symbols.

A PUSCH preparation time throughout Subclause 11.1.1 is as described in [6, TS 38.214].

A UE assumes that flexible symbols in a CORESET configured to the UE for PDCCH monitoring are downlink symbols if the UE does not detect an SFI-index field value in DCI format 2_0 indicating the set of symbols of the slot as flexible or uplink and the UE does not detect a DCI format 0_0, DCI format 0_1, DCI format 1_0, DCI format 1_1, or DCI format 2_3 indicating to the UE to transmit SRS, PUSCH, PUCCH, or PRACH in the set of symbols.

For a set of symbols of a slot that are indicated as flexible by tdd-UL-DL-ConfigurationCommon, and tdd-UL-DL-ConfigurationDedicated, or when tdd-UL-DL-ConfigurationCommon, and tdd-UL-DL-ConfigurationDedicated are not provided to the UE, and if the UE does not detect a DCI format 2_0 providing a slot format for the slot

-	the UE receives PDSCH or CSI-RS in the set of symbols of the slot if the UE receives a corresponding indication by a DCI format 1_0, DCI format 1_1, or DCI format 0_1

-	the UE transmits PUSCH, PUCCH, PRACH, or SRS in the set of symbols of the slot if the UE receives a corresponding indication by a DCI format 0_0, DCI format 0_1, DCI format 1_0, DCI format 1_1, or DCI format 2_3

-	the UE receives PDCCH as described in Subclause 10.1

-	if the UE is configured by higher layers to receive PDSCH or CSI-RS in the set of symbols of the slot, the UE does not receive the PDSCH or the CSI-RS in the set of symbols of the slot

-	if the UE is configured by higher layers to transmit SRS, or PUCCH, or PUSCH, or PRACH in the set of symbols of the slot, the UE

-	does not transmit the PUCCH, or the PUSCH, or the PRACH in the slot and does not transmit the SRS in symbols from the set of symbols in the slot, if any, starting from a symbol that is a number of symbols equal to the PUSCH preparation time  for the corresponding PUSCH timing capability assuming  after a last symbol of a CORESET where the UE is configured to monitor PDCCH for DCI format 2_0

-	does not expect to cancel the transmission of the SRS, or the PUCCH, or the PUSCH, or the PRACH in symbols from the set of symbols in the slot, if any, starting before a symbol that is a number of symbols equal to the PUSCH preparation time  for the corresponding PUSCH timing capability assuming after a last symbol of a CORESET where the UE is configured to monitor PDCCH for DCI format 2_0

For unpaired spectrum operation for a UE on a cell in a frequency band of FR1, and when the scheduling restrictions due to RRM measurements [10, TS 38.133] are not applicable, if the UE detects a DCI format 0_0, DCI format 0_1, DCI format 1_0, DCI format 1_1, or DCI format 2_3 indicating to the UE to transmit in a set of symbols, the UE is not required to perform RRM measurements [10, TS 38.133] based on a SS/PBCH block or CSI-RS reception on a different cell in the frequency band if the SS/PBCH block or CSI-RS reception includes at least one symbol from the set of symbols.