3GPP TSG RAN WG1 Meeting #98																		    R1-1909909

Prague, Czech Republic, 26th August– 30th August 2019

Agenda Item:		7.2.3

Source:				WI Rapporteur (Qualcomm Incorporated)

Title: 					RAN1 agreements for Rel-16 IAB

Document for:		Information

### Introduction

This contribution provides the RAN1 decisions made for the Rel-16 WI on Integrated Access and Backhaul for NR (WI code NR_IAB; WID in RP-191558), up to and including RAN1#98.

The WI objectives are [1]:

- Specification of SSB/RMSI periodicity for NR initial access assumed by an IAB-node.

- Specification of extensions to Rel. 15 to support the use of SSBs orthogonal to SSBs used for UEs (via TDM and/or FDM), for inter-IAB-node discovery and measurements, including additional SMTC periodicities and time-domain mapping of SSB locations (e.g. enable muting patterns to deal with half-duplex constraint).

- Specification of extension of RACH occasions and periodicities for backhaul RACH resources. w.r.t. access RACH resources, and associated network coordination mechanisms for selection of such parameters (in order to orthogonalize access and BH due to the half-duplex constraints)

- Specification of mechanisms for resource multiplexing among backhaul and access links. This includes:

  - Specification of semi-static configuration for IAB-node/IAB-donor DU resources in case of TDM operation subject to half-duplex constraint. This shall be forward compatible to allow the support of half-duplex scenarios with FDM and SDM resource sharing among backhaul and access links.

  - Specification of time resource types for the DU’s child links: DL hard, DL soft, UL hard, UL soft, Flexible hard, Flexible soft, Not Available

  - Specification of dynamic indication (L1 signalling) of the availability of soft resources for a child IAB-node DU

  - Specification of required transmission/reception rules for IAB-nodes and associated behaviours regarding time resource utilization as discussed in TR 38.874 clause 7.3.3.

- Specification of mechanism to support the “case-1” OTA timing alignment.

### SSB/RMSI periodicity for NR initial access

RAN1 #AH1901 (Taipei, January 2019) [2]

Feature Lead summary R1-1901367

Agreements:

- There is no need to introduce any specification change to enable longer RMSI transmission periodicities.

Feature Lead summary R1-1901400

Agreements:

- For both SA and NSA, for initial cell selection, an IAB-node MT may assume that half frames with SS/PBCH blocks occur with a periodicity of 16 frames.

  - FFS cell re-selection

  - Study further the value of assumed SS/PBCH block periodicity for a backhaul link switching to a new parent node when the link to the current parent node fails.

RAN1 #96 (Athens, February 2019) [3]

Feature Lead summary R1-1903573

Agreements:

- Existing NR mechanisms are used by the network to signal to IAB MTs the SSB periodicity for cell re-selection.

- The IAB-node MT initial access assumption that half frames with SS/PBCH blocks occur with a periodicity of 16 frames does not have an impact on cell re-selection.

Agreements:

- The IAB-node MT initial access assumption that half frames with SS/PBCH blocks occur with a periodicity of 16 frames does not have an impact on backhaul link rerouting to a new parent node in response to a failure in the link to the current parent node.

LS to RAN2/3 capturing agreements from this meeting and RAN1 AdHoc1901 – R1-1903809

### Extensions of SSBs for inter-IAB-node discovery and measurements

RAN1 #AH1901 (Taipei, January 2019) [2]

Feature Lead summary R1-1901369

Agreements:

- SSB transmission configuration information (i.e. the necessary information for the DU to determine when to transmit SSB) and SSB reception configuration information (i.e. the necessary information for the MT to determine when to search for SSB) for IAB inter-node measurements are determined in a centralized manner, i.e. not locally by the IAB node.

- An IAB node is provided with configuration of SSB transmission and reception information for IAB inter-node measurements.

  - FFS details

RAN1 #96 (Athens, February 2019) [3]

Feature Lead summary R1-1903574

Agreements:

- The SSBs for IAB inter-node discovery and measurements are defined with a framework using the characteristics of the Rel 15 SMTC framework with the following enhancements:

  - The maximum number of SMTC windows that can be configured for an IAB node, referred as max( NRX ), is at least 3

    - FFS whether max( NRX ) needs to be greater than 3

    - FFS whether or not to extend existing SMTC window configuration

  - Each SMTC window is provided its own independent configuration (e.g. periodicity, offset, duration)

  - Introduction of SSB transmission configurations (STC) indicating SSB transmission(s)

    - FFS details

  - FFS how to resolve potential collision between SMTC and STC

RAN1 #96bis (Xi’an, April 2019) [4]

Feature Lead summary R1-1905832

Agreements:

- The Rel-15 SSB mapping pattern within a half frame is reused for IAB node discovery and measurements

Agreements:

For IAB node discovery and measurements, at least the following information should be provided in STC

- SSB center frequency

- SSB subcarrier spacing

- SSB transmission periodicity

  - 5, 10, 20, 40, 80, 160, 320, 640 subframes are supported.

- SSB transmission timing offset in half frame(s)

- The index of SSBs to transmit

  - The SSBs to be transmitted in the half frame

- FFS configuration information other than above

- FFS some of the parameters can have default values, e.g. reuse the same value of the cell-defining SSBs.

- Note: The detailed signaling design is up to RAN3.

Agreements:

For IAB node discovery and measurements, an IAB node DU can be configured with zero, one or multiple STCs.

- FFS the maximum number of STCs

- FFS the dependency of multiple STCs

Agreements:

- For IAB node discovery and measurement, the maximum number of SMTC windows that can be configured for an IAB node is 4.

RAN1 #97 (Reno, May 2019) [5]

Feature Lead summary R1-1907834

Agreements:

All parameters in STC for inter-IAB-node discovery and measurement are provided explicitly, i.e. the default values for the parameters in STC will not be discussed further in RAN1.

Agreements:

The configurable values of the parameters in STC for IAB node discovery and measurement are provided in the following

- SSB center frequency:

  - ARFCN-ValueNR

- SSB subcarrier spacing:

  - FR1: 15khz, 30khz

  - FR2: 120khz, 240khz

- SSB transmission periodicity:

  - 5ms, 10ms, 20ms, 40ms, 80ms, 160ms, 320ms, 640ms (agreed in RAN1 #96bis)

- SSB transmission timing offset in half frame(s)

  - [0, … , (number of half frames within SSB transmission periodicity) – 1]

- The index of SSBs to transmit (the SSBs to be transmitted in the half frame)

  - Same as Rel-15

- FFS additional parameter(s) other than above

Agreements:

For IAB node discovery and measurement, the maximum number of STCs that can be configured for an IAB node DU per cell at one frequency location is 4.

- Note: this number does not include the cell-defining SSBs for initial access

Agreements:

Each STC is independently configured when multiple STCs are configured.

- Up to RAN3 to decide that if SSB index is common across two or more STC configurations, whether additional signalling optimization is necessary or not

Agreements:

The configurable values of the parameters in the SSB reception configurations including SMTC for IAB node discovery and measurement are provided in the following

- SMTC window periodicity:

  - 5ms, 10ms, 20ms, 40ms, 80ms, 160ms, 320ms, 640ms, 1280ms

- SMTC window timing offset:

  - [0, ..., (number of subframes within SMTC window periodicity) – 1]

- SMTC window duration:

  - [1, …, 5] (subframes)

    - FFS larger than 5

- List of physical cell IDs to be measured

  - [cell ID 0, …, cell ID M-1 ]

    - FFS value of M

- SSB-ToMeasure

  - Same as Rel-15

RAN1 #98 (Prague, August 2019) [6]

Feature Lead summary R1-1909542

Agreements:

For IAB node discovery and measurements, the Physical cell-ID (0…1007) is provided in STC and the detailed signaling design is up to RAN3.

Agreements:

For IAB node discovery and measurement

  - The maximum SMTC window duration is 5ms.

  - The maximum number of PCIs per SMTC (maxNrofPCIsPerSMTC) is 64.

Extension of RACH occasions and periodicities for backhaul RACH resources

RAN1 #AH1901 (Taipei, January 2019) [2]

Feature Lead summary R1-1901370

No decisions.

RAN1 #96 (Athens, February 2019) [3]

Feature Lead summary R1-1903575

Agreements:

- New RACH configurations specific to IAB nodes are derived with extension of existing Rel 15 RACH configurations obtained by:

  - scaling the parameter ‘x’ from the PRACH configuration table, and

  - by adding an offset y_offset to the parameter ‘y’ (frame-based offset) and/or adding an offset to the slot/subframe number (slot/subframe-based offset) from the PRACH configuration table.

- FFS values and ranges for scaling factor applicable to ‘x’ and for y_offset.

- FFS whether a simple extension rule can be generally applied to all existing configurations leaving it up to the network to not use resulting configurations that may not fit other system constraints.

RAN1 #96bis (Xi’an, April 2019) [4]

Feature Lead summary R1-1905738

Agreements:

The periodicity of a backhaul RACH configuration in frames takes the form x_iab = x * λ where:

- x is the periodicity of an existing RACH configuration,

- λ is a scaling factor taking values in {1, 2, 4, 8, 16, 32, 64} subject to the constraint x_iab ≤ 64.

Agreements:

The frame containing backhaul ROs is identified by (nSFN mod x_iab) = ((y + Δy) mod x_iab) where Δy denotes a time offset in frames taking values in the range from 0 to x_iab – 1.

Agreements:

The subframe (slot) number for a RO of a backhaul RACH configuration is identified by (Sn + Δs,) mod L, where:

- Sn is the subframe (slot) number of an existing RACH configuration,

- Δs denotes a time offset in subframes (slots) taking values in the range from 0 to L – 1, where L is the number of subframes (slots) in a frame.

NOTE: The usage of the terms subframe or slot is meant to align to the terminology used in the existing RACH configuration tables in TS 38.211, e.g. subframe in Tables 6.3.3.2-2 and 6.3.3.2-3, and slot in Table 6.3.3.2-4.

Agreements:

The validity of ROs for backhaul RACH configurations is regulated by the rules defined in Rel-15 for existing RACH configurations.

Agreements:

Partial overlap of ROs between RACH configurations used in two adjacent links (upstream towards the parent and downstream towards the children from an IAB node perspective) is allowed.

RAN1 #97 (Reno, May 2019) [5]

Feature Lead summary R1-1907680

Agreements:

The introduction of scaling factor for PRACH configuration period also scales the SSB-RACH association period and association pattern period accordingly.

Assuming the scaling factor to be λ, for IAB nodes table 8.1-1 of 38.213 gets modified to the following:

Mapping between scaled PRACH Configuration Period (x * λ) and SSB to PRACH Occasion Association Period

| Scaled PRACH configuration period (ms) | Association period |
| --- | --- |
| 10 | {1, 2, 4, 8, 16, 32, 64} |
| 20 | {1, 2, 4, 8, 16, 32} |
| 40 | {1, 2, 4, 8, 16} |
| 80 | {1, 2, 4, 8} |
| 160 | {1, 2, 4} |
| 320 | {1, 2} |
| 640 | {1} |

The definition of the SSB-RACH association pattern period for IAB nodes should be updated as follows: An association pattern period includes one or more association periods and is determined so that a pattern between PRACH occasions and SS/PBCH blocks repeats at most every 640 msec.

Agreements:

The signaling design for the IAB specific backhaul RACH configurations is up to RAN2.

RAN1 has the following recommendations:

- The ability to configure the IAB specific backhaul RACH configuration additionally to the Rel 15 RACH configuration for access UEs is supported.

  - If the IAB specific RACH configuration is not provided, RAN1 assumes that IAB node will use the configured Rel 15 RACH configuration for IAB node initial access.

- Capability to configure IAB specific backhaul RACH configurations need to be provided for both CBRA & CFRA.

RAN1 #98 (Prague, August 2019) [6]

Feature Lead summary R1-1909526

Agreements:

IAB-specific backhaul RACH configurations are also available for intial access of IAB nodes.

### Mechanisms for resource multiplexing among backhaul and access links

RAN1 #AH1901 (Taipei, January 2019) [2]

Feature Lead summary R1-1901455

Agreements:

At least existing resource definitions (D/U/F) and semi-static and dynamic signaling methods defined in Rel-15 for access UEs are reused for configuration and indication of MT resources to be used by the backhaul link between the IAB node and its parent.

- FFS details

Agreements:

IAB-node/IAB-donor DU resources are provided by a semi-static configuration which is provided separately from the MT resource indication

- FFS: whether the configuration is per-link or per-DU

- FFS: details for the configuration

Agreements:

Support indication of the dynamic availability of soft resources at an IAB node from a parent

- FFS details, including explicit vs. implicit indication

RAN1 #96 (Athens, February 2019) [3]

Feature Lead summary R1-1903582

Agreements:

- If a resource is configured as not available, the DU cannot assume it can use the resource

- In case of hard DU resources, the DU can assume it can use the resource regardless of the MT’s configuration

  - FFS: Exception cases for specific signals/channels to be transmitted or received by the MT in the same resource (e.g. SS/PBCH blocks, SI reception, RACH)

- In case of soft DU resources:

  - If the soft resource is indicated as available, the DU can assume it can use the resource

  - If the soft resource is not indicated as available, the DU cannot assume it can use the resource

  - The use of soft resources at least corresponds to transmission/reception of specific signals and channels (e.g. PDSCH/PUSCH) at the DU

    - FFS the use of soft resources in case of cell-specific (e.g. SS/PBCH blocks, SI reception, RACH) signals and channels to be potentially transmitted/received at the DU

  - Both implicit and explicit indication of the availability of soft resources at an IAB node is supported

    - In case of implicit indication of DU soft-resource availability, the IAB node knows that the DU resource can be used without impacting the MTs ability to transmit/receive according to its configuration and scheduling based on indirect means. Examples of such means may include:

      - the lack of uplink scheduling grant at the MT

      - no data available for MT transmission

      - the configured MT search space,

      - configured RS measurement occasions (e.g. SSB/CSI-RS)

      - FFS: consider whether the parent should be able to always be aware of/control the outcome of implicit indications at child nodes

    - Explicit indication that a resource is available is based on DCI indication. The following options can be considered:

      - SFI-like indication via DCI Format 2_0

      - Use of 2 SFI indications (e.g. based on multi-slot scheduling mechanism)

      - Define a new DCI format

      - Other options are not precluded

      - FFS: whether an explicit indication that a resource is available always has priority over any implicit determination of the availability of a resource

  - Further consider factors impacting the usage of soft resources at a child DU, including:

    - MT’s decoding delay

    - Information exchange delay between MT and DU

    - DU’s PDSCH preparation time

    - UE PUSCH preparation time

    - Accumulated delay across hops

Agreements:

- Inter-IAB node conflict resolution can be supported by the following options (to be down-selected)

  - Alt1: the parent node is aware of all of the DU resource configurations (D/U/F/H/S/NA) of its child IAB node DUs,

  - Alt2: the parent node may be made aware of a subset of the DU resource configurations (D/U/F/H/S/NA) of its child IAB node DUs

  - FFS: whether the indication of the child DU resources at the parent is via explicit (e.g. F1-AP signaling) or implicit (e.g. based on child MT configuration) means

Agreements:

- Both slot and symbol-level multiplexing of access and backhaul links are supported.

RAN1 #96bis (Xi’an, April 2019) [4]

Feature Lead summary R1-1905739

Agreements:

- In case of Hard or Soft Indicated Available DU resources, no additional exception cases need to be defined for cell specific signals/channels to be transmitted or received by the MT in the same resource (e.g. SS/PBCH blocks, SI reception, RACH).

  - The decision on whether to give priority to the DU or to the MT for the use of the resource (e.g. in case of MT RACH transmission) is left to the IAB node implementation.

    - The IAB shall fulfill its performance requirements in terms of measurement and transmission of cell specific signals / channels.

- In case of NA or Soft not Indicated Available DU resources, for potential conflicting configuration between NA and Soft not Indicated Available DU resources and cell-specific signals/channels (e.g. SSB, SI, PRACH) configured at the DU, the following alternatives are to be considered (to be down-selected):

  - Alt 1): A resource with cell-specific signals/channels configured at DU shall not be configured as type NA or Soft without being Indicated Available.

  - Alt 2): If a DU NA or Soft resource is configured with cell-specific signals/channels, the resource is treated as if it were a Hard DU resource.

- FFS the cell specific signals/channels. The list may include (not necessarily an exhaustive list)

  - SSB transmissions

  - Broadcast system information

  - Configured periodic CSI-RS

  - PRACH resources

  - Resources for scheduling requests

Agreements:

- An IAB node has the ability to be made aware of the semi-static DU resource configuration (D/U/F/H/S/NA) of all its child IAB nodes.

  - in the case the full DU resource configuration information of its child IAB nodes is not necessary, ensure that only the necessary configuration information is signaled to the IAB node.

    - FFS the necessary configuration information.

Agreements:

- An explicit indication of availability of a Soft resource by the parent node makes the resource available to the child node, irrespective of the outcome of any implicit determination of availability by the child node.

- The parent node does not need to be aware of the outcome of an implicit determination of availability of a DU Soft resources at a child node.

RAN1 #97 (Reno, May 2019) [5]

Feature Lead summary R1-1907679

Agreements:

For the semi-static DU resource configuration, the following is supported:

- The resources are configured on a per DU (cell) basis

  - FFS: indication of additional supplemental per-link resource configurations of child DUs

- Indication of D/U/F resources in the semi-static DU resource configuration includes the following:

  - The flexibility to configure all of the slot patterns and formats supported by the existing Rel-15 TDD-UL-DL-Config RRC configurations and slot format table defined in Table 11.1.1-1 in TS38.213

    - FFS: additional pattern durations than supported in Rel-15

    - FFS: default resources or pattern

  - New slot formats defined only for IAB nodes (DU and MTs) which begin with uplink slots, uplink symbols, or flexible symbols.

    - Note: usage of these slot formats should be compatible with Rel-15 access UEs sharing the same link

    - FFS: whether these slot formats also need to be included in the MT RRC configuration and/or SFI carried on DCI Format 2_0

- Indication of H/S/NA for the DU resource configuration is based on one of the following alternatives:

  - Alt. 1: H/S/NA is additionally explicitly indicated per-resource type (D/U/F) in each slot

    - To handle potential misalignment in time of the configured DU and MT resources when determining the validity of H/S/NA at the DU one of the following sub-alternatives need to be supported:

      - 1a: H/S/NA is applied relative to the DU resource configuration (D/U/F) slot timing without considering the MT resource configuration or timing.

        - FFS: definition of additional restrictions on the usage of the semi-static configuration (e.g. guard symbols) based on deployment scenario or DL/UL switching times within an IAB node, etc.

        - FFS: How the CU can get information about the required guard symbols for a given DU configuration if needed

      - 1b: H/S/NA is applied relative to the MT resource configuration (D/U/F) slot timing.

        - FFS: Whether and/or how the CU will know the actual H/S/NA resources at the child DU

        - FFS whether S is explicitly indicated or not

      - 1c: H is applied relative to the DU resource configuration (D/U/F) slot timing. S is not explicitly indicated, but implicitly determined by the DU based on whether the corresponding MT configuration indicates the MT resources is F (DU-S). The remaining resources are assumed to be NA at the child DU.

        - FFS: Whether and/or how the CU will know the actual S/NA resources at the child DU

  - Alt. 2: NA is explicitly indicated as a resource type in each slot for both the DU and MT configuration. H/S is not explicitly indicated, but implicitly determined by the DU based on the corresponding MT configuration

    - FFS: how to handle the case where there is not a 1-1 mapping of DUs and MTs in the child IAB node

- Details of the F1-AP signaling design is up to RAN3

Agreements:

If a DU NA or Soft resource is configured with cell-specific signals/channels, the resource is treated as if it were a Hard DU resource (Alt. 2 from RAN1#96bis).

  - The list of cell-specific signals/channels includes:

- resources for SSB transmission at DU, including both CD-SSB and non-CD-SSB;

- configured RACH occasions for receiving at the DU

- periodic CSI-RS transmission at the DU

- scheduled resource for receving SR at DU

- The parent does not need to be aware of the cell-specific signals/channel configurations of the child DU

RAN1 #98 (Prague, August 2019) [6]

Feature Lead summary R1-1909531

Agreements:

- In Rel-16 for IAB backhaul, only existing slot pattern durations are supported.

Agreements:

Slot formats with the sequence order UL-Flexible-DL are supported for IAB-node DU resource configurations (F1-AP) and IAB-node MT resource configurations provided in dedicated RRC signaling (e.g. enhancements to the existing TDD-UL-DL-SlotConfig in TDD-UL-DL-ConfigDedicated – details up to RAN2).

- FFS: Support of dynamic indication of UL-Flexible-DL

Agreements:

The H/S/NA attributes for the per-cell DU resource configuration should take into account the associated MT carrier frequency(ies).

- Note: RAN1 assumes that this is mainly needed for intra-band cases

Agreements:

To handle potential misalignment in time of the configured DU and MT resources when determining the validity of H/S/NA at the DU, the following is supported:

- H/S/NA is applied/determined relative to the DU resource configuration (D/U/F) slot timing without considering the MT resource configuration or timing

- The parent DU should know the information about the misalignment between the MT and DU DL Tx/Rx / UL Tx/Rx timing(s) in order to avoid resource type conflicts

  - FFS whether any explicit signalling is necessary or not

- It is up to the parent IAB node implementation and network configuration to handle any potential resource type conflicts (e.g. MT scheduling restrictions or configuration of guard symbols). The child IAB node should handle any remaining conflicts using the existing conflict resolution rules

Agreements:

For the explicit indication of DU-IA:

- At least DU-IA can be indicated independently of the MT resource configuration:

  - FFS the DCI indicates DU-IA per-resource type (D/U/F), slot, etc.

- FFS: Whether this indication is via a new DCI format or DCI Format 2_0 using existing and/or reserved entries of Table 11.1.1-1 in 38.213

- FFS: Whether for the purpose of signaling optimization a DU/MT resource type can additionally be jointly indicated with DU-IA which overrides the flexible resources in the semi-static DU/MT resource configuration

### Mechanism to support the “case-1” OTA timing alignment

RAN1 #AH1901 (Taipei, January 2019) [2]

Feature Lead summary R1-1901417

Agreements:

An IAB node should set its DL TX timing ahead of its DL Rx timing by TA/2 + T_delta

- T_delta is signalled from the parent node, where the value is intended to account for factors such the offset between parent DL Tx and UL Rx, if any due to factors such as Tx to Rx switching time, HW impairments, etc.

- TA is the timing gap between UL Tx timing and DL Rx timing, which is derived based on existing Rel-15 mechanism

- FFS (not necessarily an exhaustive list):

  - value range and granularity of Tdelta

  - need for aperiodic/periodic updates of Tdelta

  - other timing impairment factors for adjusting IAB node timing to be included in Tdelta

  - timing alignment when the IAB node has multiple parents

  - Note: once the design of the above FFS points is in a good shape, an LS to RAN4 may be necessary to solicit their input

RAN1 #96 (Athens, February 2019) [3]

Feature Lead summary R1-1903583

Agreements:

- T_delta is indicated by a parent to the child node independently from the existing Rel.15 TA indication from the parent node used to set the UL Tx timing of the child IAB node’s MT

  - T_delta is updated on an aperiodic basis determined by the parent node

  - The child IAB node should trigger its DL TX timing adjustment by TA/2 + T_delta after it receives the timing offset T_delta indication from its parent node, if it is using OTA Timing Case 1 to obtain its DL timing.

    - FFS: behavior if TA/2 + T_delta results in an effective negative timing offset

    - FFS: delay between receiving T_delta and application of T_delta at the child node

  - Separate value ranges/granularities may be considered for T_delta in FR1 and T_delta in FR2

- Send LS to RAN4 asking them to determine the exact values and granularity of T_delta and provide confirmation on RAN1’s assumption on the DL timing accuracy requirements for IAB nodes in case of OTA Case 1 timing is applied across multiple hops – Final LS in R1-1903810.

RAN1 #96bis (Xi’an, April 2019) [4]

Feature Lead summary R1-1905778

Agreements:

In order to align the DL TX timing of the IAB node with the DL TX timing of the parent node by setting DL TX timing of the IAB node (TA/2 + T_delta) ahead of its DL Rx timing, T_delta should be set to the (-1/2) of time interval at the parent node between the start of UL RX frame i for the IAB node and the start of DL TX frame i.

- The setting of T_delta is not necessarily specified.

- Note: The above setting of T_delta assumes that, for the same purpose, TA should be the time interval at the IAB node between the start of UL TX frame i and the start of DL RX frame i.

- Send LS to RAN4 for timing clarification. Final LS in R1-1905842.

Agreements:

- In case the calculated TA/2 + T_delta at IAB node is negative, the IAB node should not adjust its DL-Tx timing.

RAN1 #97 (Reno, May 2019) [5]

Feature Lead summary R1-1907667

Agreements:

In Rel-16, an IAB node is not expected to receive T_delta when the IAB node MT is not in RRC_Connected mode.

RAN1 #98 (Prague, August 2019) [6]

Feature Lead summary R1-1909804

Agreements:

- According to RAN1 #96bis agreement, whether T_delta is a “target value” or an “actual value” is up to parent node implementation.

- For the TA and T_delta in (TA/2+T_delta), to down-select:

  - Opt-A: T_delta is given by the latest T_delta signaling, and TA is the current time interval at the IAB node between the start of UL TX frame i and the start of DL RX frame i, which is updated with the received TA command per Rel-15.

  - Opt-B: T_delta is given by the target T_delta signaling, and TA is an average of timing advance intervals (e.g., TA1, TA2, TA3…) updated by a series TA commands.

- Once down-selected, further discuss how to reflect it in RAN1 specs

### References

[1]	RP-191558 - Revised WID: Integrated Access and Backhaul for NR – Qualcomm Incorporated – June 2019

[2]	RAN1 #1901 - Chairman's Notes RAN1 AdHoc 1901 final – January 2019

[3]	RAN1 #96 - Chairman's Notes RAN1#96 final – February 2019

[4]	RAN1 #96bis - Chairman's Notes RAN1#96bis final – April 2019

[5]	RAN1 #97 - Chairman's Notes RAN1#97 final – May 2019

[6]	RAN1 #98 - Chairman's Notes RAN1#98 final – August 2019